USB Modem Test Case

Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | 03/22/2012 | Zhonghua Xu | Initial draft |
| 0.2 | 03/23/2012 | Zhonghua Xu | Add HM cases, Chapter 6.9 |
| 0.3 | 03/27/2012 | Zhonghua Xu | Add browser related HM cases; Add usbmodem mode delta switch cases |
| 0.4 | 03/28/2012 | Zhonghua Xu | Add negative cases to check CLI config after usbmodem process crash; Add HM cases about Config/ Auto Provision pages |
| 0.5 | 04/06/2012 | Zhonghua Xu | Complete HiveOS test |
| 0.6 | 04/11/2012 | Zhonghua Xu | Delete HM related cases; Refer to the standard HM cases on test link and case execution record from the progress excel |
| 0.7 | 04/16/2012 | Zhonghua Xu | Add cases for eth0 with pppoe |
| 0.8 | 04/17/2012 | Zhonghua Xu | Add cases for LED state, considering capwap connection |
| 0.9 | 04/26/2012 | Zhonghua Xu | Update test result for 6.9.3 |
| 1.0 | 04/26/2012 | Zhonghua Xu | Correct the case id of UsbModem_PrimaryWan_2 |
| 1.1 | 06/14/2012 | Zhonghua Xu | Add check for DNS switch before/ after failover/ failback, for cases, UsbModem_OnDemand_2 UsbModem_ AlwaysConnected_2 UsbModem_PrimaryWan_2 |
| 1.2 | 06/20/2012 | Zhonghua Xu | Correct the typo, “modem is PPPoE required” |

Table of Contents

Glossary and Abbreviations

Introduction

This project adds support for 3G mobile broadband modems to Aerohive’s access points equipped with a USB port. These modems will be used to provide primary and backup backhaul access over 3G networks.

Test Objectives

The goal is to validate feature functionality on both HiveOS and HiveManager (configuration), testing shall also provide customer and negative scenarios

- USB modem should support 3G network service

- USB modem should support 4G network service

- USB modem can work as backup WAN, when eth0 WAN is unavailable

- USB modem can work as primary WAN, even eth0 WAN is also available

- USB modem can be configured as one of 3 WAN mode successfully

- WAN should failover/ failback between eth0 and USB modem without issue

- Traffic should not be affected between WAN failover/ failback

- Error msg about USB modem should be reported

Test Acceptance Criterion from Development

- Approved – MRD

http://saturn.aerohive.com/view.php?fDocumentId=3310

- Approved – Functional Specifications

N/A

- Approved – Unit Test Plans

N/A

Product Pass Criterion

Feature testing is considered pass when test result meets the requirement defined in the expected result field. The expected result field is defined by the requirements stated in the functional specification and/ or MRD; whichever is stricter; plus additional quality and usability expectations set by the test engineer.

Test Bed/Topo Design

Topo1- In headquater, connect internet through both eth0 and usbmodem WAN are available

Figure01- Topo1

Topo2- Outside headquater, connect internet through both eth0 and usbmodem WAN are available

Figure02- Topo2

Test Case

UsbModem_CLI

UsbModem_CLI_1

| Case ID | UsbModem_CLI_1 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | CLI check, 1) show usbmodem |  |  |
| Pre-condition |  |  |  |
| Test procedure | 1) Show the default status of USB modem show usbmodem |  |  |
| Expect result | 1) There should be “Enabled” state 2) There should be “Mode” state 3) There should be 6 supported modem listed AH-8c66c0#show usbmodem <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: ISP.CINGULAR Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# ----------------- Modem Id: sierra_313 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68aa Vendor Id: 0x1199, Product Id: 0x68aa APN: broadband Dialup Username: Dialup Password: Dialup Number: ATD*99***1# ----------------- Modem Id: pantech_uml USB Id(s): Vendor Id: 0x106c, Product Id: 0x3718 APN: Dialup Username: Dialup Password: Dialup Number: ATD*99***3# ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: Dialup Username: Dialup Password: Dialup Number: ATD*99***1# ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c APN: Dialup Username: Dialup Password: Dialup Number: ATD*99***1# ----------------- Modem Id: huawei_e366 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x14ac APN: Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> |  |  |
| Test Result | PASS |  |  |

UsbModem_CLI_2

| Case ID | UsbModem_CLI_2 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | CLI check, 1) [no] usbmodem modem-id <string> apn [[<string>]] 2) show usbmodem [modem-id <string>] |  |  |
| Pre-condition |  |  |  |
| Test procedure | 1) Config apn as "3gnet” for modem-id huawei_e220 usbmodem modem-id huawei_e220 apn 3gnet 2) Show the detail of modem huawei_e220 sh usbmodem modem-id huawei_e220 3) Delete the config in step 1) no usbmodem modem-id huawei_e220 apn 4) Repeat step 2) |  |  |
| Expect result | 1) apn configured for modem huawei_e220 successfully AH-8c66c0#sh usbmodem modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> 2) There is no apn once delete the apn config of modem huawei_e220 AH-8c66c0#sh usbmodem modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> |  |  |
| Test Result | PASS |  |  |

UsbModem_CLI_3

| Case ID | UsbModem_CLI_3 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | CLI check, 1) [no] usbmodem modem-id <string> dialup-number [[<string>]] 2) [no] usbmodem modem-id <string> dialup-username [[<string>]] 3) [no] usbmodem modem-id <string> dialup-password [[<string>]] 4) show usbmodem [modem-id <string>] |  |  |
| Pre-condition |  |  |  |
| Test procedure | 1) Config dialup-number for modem huawei_e220 usbmodem modem-id huawei_e220 dialup-number 12345678 2) Config dialup-username for modem huawei_e220 usbmodem modem-id huawei_e220 dialup-username admin 3) Config dialup-password for modem huawei_e220 usbmodem modem-id huawei_e220 dialup-password ******** 4) Show the detail of modem huawei_e220 sh usbmodem modem-id huawei_e220 5) Delete the config in step 1) ~ 3) 6) Repeat step 4) |  |  |
| Expect result | 1) dialup-number/ username/ passwd configured for modem huawei_e220 successfully AH-8c66c0#sh usbmodem modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: Dialup Username: admin Dialup Password: aerohive Dialup Number: 12345678 <End of USB modem configuration> 2) There is no dialup-username/ passwd once delete these config of modem huawei_e220, and dialup-number is configured as “ATD*99***1#” by default. AH-8c66c0#sh usbmodem modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> |  |  |
| Test Result | PASS |  |  |

UsbModem_CLI_4

| Case ID | UsbModem_CLI_4 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | CLI check, 1) [no] usbmodem mode {on-demand¦always-connected¦primary-wan} 2) show usbmodem [modem-id <string>] |  |  |
| Pre-condition |  |  |  |
| Test procedure | 1) Config usbmodem mode as on-demand, usbmodem mode on-demand 2) Show the usbmodem mode of modem huawei_e220, sh usbmodem modem-id huawei_e220 3) Config usbmodem mode as always-connected, usbmodem mode always-connected 4) Repeat step 2) 5) Config usbmodem mode as primary-wan, usbmodem mode primary-wan 6) Repeat step 2) 7) Delete mode config for USB modem no usbmodem mode 8) Repeat step 2) |  |  |
| Expect result | 1) usbmodem mode is configured as on-demand successfully AH-8c66c0#sh usbmodem modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> 2) usbmodem mode is configured as always-connected successfully AH-8c66c0#sh usbmodem modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> 3) usbmodem mode is configured as primary-wan successfully AH-8c66c0#sh usbmodem modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> 4) usbmodem mode will be on-demand by default once delete mode config AH-8c66c0#sh usbmodem modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> |  |  |
| Test Result | PASS |  |  |

UsbModem_CLI_5

| Case ID | UsbModem_CLI_5 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | CLI check, 1) [no] usbmodem enable 2) show usbmodem [modem-id <string>] |  |  |
| Pre-condition |  |  |  |
| Test procedure | 1) Show the default usbmodem availability show usbmodem 2) Disable the availability of usbmodem no usbmodem enable 3) Repeat step 1) |  |  |
| Expect result | 1) The default availability of usbmodem is "Enabled", zhxu-8c66c0#sh usbmodem <Begin of USB modem configuration> Enabled: yes 2) The usbmodem availability is "Disabled" after run "no" operation zhxu-8c66c0#sh usbmodem <Begin of USB modem configuration> Enabled: no |  |  |
| Test Result | PASS |  |  |

UsbModem_CLI_6

| Case ID | UsbModem_CLI_6 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | CLI check, 1) show usbmodem status |  |  |
| Pre-condition |  |  |  |
| Test procedure | 1) Show the usbmodem/ WAN status of BR show usbmodem status |  |  |
| Expect result | 1) The usbmodem/ WAN status of BR is displayed accordingly, AH-8c66c0#show usbmodem status USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 22105 State: S (sleeping) PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 22 07:30:45 2012) DOWN: 0 times (since Thu Jan 1 00:00:08 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:08 1970) DOWN: 0 times (since Thu Jan 1 00:00:08 1970) Active: no Cached Gateway IP: 0.0.0.0 [Backup] Interface: ppp0 WAN Monitor State: down UP: 1 times (since Thu Mar 22 09:28:46 2012) DOWN: 21 times (since Thu Mar 22 10:20:22 2012) Local Link State: UP: 1 times (since Thu Mar 22 09:28:42 2012) DOWN: 1 times (since Thu Mar 22 09:33:18 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTING Pending Event(s): NONE |  |  |
| Test Result | PASS |  |  |

UsbModem_CLI_7

| Case ID | UsbModem_CLI_7 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | CLI check, 1) show usbmodem rssi |  |  |
| Pre-condition | 1) USB modem is plugged with BR and enabled |  |  |
| Test procedure | 1) Show the rssi of 3g signal received at usbmodem show usbmodem rssi |  |  |
| Expect result | 1) The rssi is displayed accordingly |  |  |
| Test Result | FAIL, BUG 16848 |  |  |

UsbModem_CLI_8

| Case ID | UsbModem_CLI_8 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | CLI check, 1) show usbmodem info |  |  |
| Pre-condition | 1) USB modem is plugged with BR and enabled |  |  |
| Test procedure | 1) Show the hardware/ provisioned data of usbmodem show usbmodem info |  |  |
| Expect result | 1) The provisioned USB modem info is displayed accordingly, AH-8c66c0#sh usbmodem info times (since Thu Mar 22 09:28:42 2012) DOWN: 1 times (since Thu Mar 22 09:33:18 2012) Active: no Cached Gateway IP: 0.0.0.0 |  |  |
| Test Result | FAIL, BUG 16849 |  |  |

UsbModem_CLI_9

| Case ID | UsbModem_CLI_9 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | CLI check, 1) Input invalid modem-id should fail |  |  |
| Pre-condition |  |  |  |
| Test procedure | 1) Try to input an invalid modem-id, which is not in the modem list usbmodem modem-id xxx apn 3gnet |  |  |
| Expect result | 1) Invalid modem-id should not be allowed, AH-8c66c0#usbmodem modem-id xxx apn 3gnet No entry for modem id [xxx] ERROR: Invalid parameter(s) |  |  |
| Test Result | PASS |  |  |

UsbModem_ServiceProvider

UsbModem_ServiceProvider_1

| Case ID | UsbModem_ServiceProvider_1 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | UMTS supported |  |  |
| Pre-condition | 1) The UMTS service is activated and available 2) Configured the dialup username/ passwd/ number if the modem if necessary |  |  |
| Test procedure | 1) Config USB modem as on-demand mode 2) Unplug the eth0 cable from BR 3) Show USB modem status in the console 4) Ping an internet address |  |  |
| Expect result | 1) The mode of USB modem is displayed as configured accordingly AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> 2) Failover to usbmodem once WAN is unavailable through eth0 AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 11521 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 2800 39 0 0 0 0 39 ¦ 5567 71 0 0 71 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 4 times (since Mon Mar 26 10:18:01 2012) DOWN: 4 times (since Mon Mar 26 10:21:08 2012) Local Link State: UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 1 times (since Mon Mar 26 10:21:00 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Mon Mar 26 10:21:47 2012) DOWN: 8 times (since Mon Mar 26 10:04:08 2012) Local Link State: UP: 1 times (since Mon Mar 26 10:21:40 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE 3) USB modem get access to the APN successfully AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.209.203 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 4) Success to ping internet address AH-8c66c0#ping 8.8.8.8 PING 8.8.8.8 (8.8.8.8) 56(84) bytes of data. 64 bytes from 8.8.8.8: icmp_seq=1 ttl=46 time=120 ms 64 bytes from 8.8.8.8: icmp_seq=2 ttl=46 time=110 ms 64 bytes from 8.8.8.8: icmp_seq=3 ttl=46 time=118 ms 64 bytes from 8.8.8.8: icmp_seq=4 ttl=46 time=127 ms 64 bytes from 8.8.8.8: icmp_seq=5 ttl=46 time=116 ms --- 8.8.8.8 ping statistics --- 5 packets transmitted, 5 received, 0% packet loss, time 4003ms rtt min/avg/max/mdev = 110.006/118.396/127.005/5.619 ms |  |  |
| Test Result |  |  |  |

UsbModem_ServiceProvider_2

| Case ID | UsbModem_ServiceProvider_2 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | CDMA supported |  |  |
| Pre-condition | 1) The CDMA service is activated and available 2) Configured the dialup username/ passwd/ number if the modem if necessary |  |  |
| Test procedure | 1) Config USB modem as on-demand mode 2) Unplug the eth0 cable from BR 3) Show USB modem status in the console 4) Ping an internet address |  |  |
| Expect result | 1) The mode of USB modem is displayed as configured accordingly 2) Failover to usbmodem once WAN is unavailable through eth0 3) USB modem get access to the APN successfully 4) Success to ping internet address |  |  |
| Test Result |  |  |  |

UsbModem_ServiceProvider_3

| Case ID | UsbModem_ServiceProvider_3 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | LTE supported |  |  |
| Pre-condition | 1) The LTE service is activated and available 2) Configured the dialup username/ passwd/ number if the modem if necessary |  |  |
| Test procedure | 1) Config USB modem as on-demand mode 2) Unplug the eth0 cable from BR 3) Show USB modem status in the console 4) Ping an internet address |  |  |
| Expect result | 1) The mode of USB modem is displayed as configured accordingly 2) Failover to usbmodem once WAN is unavailable through eth0 3) USB modem get access to the APN successfully 4) Success to ping internet address |  |  |
| Test Result |  |  |  |

UsbModem_ServiceProvider_4

| Case ID | UsbModem_ServiceProvider_4 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | EDGE supported |  |  |
| Pre-condition | 1) The EDGE service is activated and available 2) Configured the dialup username/ passwd/ number if the modem if necessary |  |  |
| Test procedure | 1) Config USB modem as on-demand mode 2) Unplug the eth0 cable from BR 3) Show USB modem status in the console 4) Ping an internet address |  |  |
| Expect result | 1) The mode of USB modem is displayed as configured accordingly 2) Failover to usbmodem once WAN is unavailable through eth0 3) USB modem get access to the APN successfully 4) Success to ping internet address |  |  |
| Test Result |  |  |  |

UsbModem_ServiceProvider_5

| Case ID | UsbModem_ServiceProvider_5 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | APN config |  |  |
| Pre-condition | 1) The 3g service is activated and available |  |  |
| Test procedure | 1) Config a modem-id with an APN selected from the list on wiki.ubuntu.com 2) Config the dialup username/ passwd/ number if the modem if necessary 3) Show modem status of the modem-id 4) Reboot BR, check the APN config after get up 5) Select another APN from the list, repeat step 1)-4) 6) Config an APN which is not in the list, repeat step 1)-4) |  |  |
| Expect result | 1) APN config successfully for each scenario 2) APN config won't lost after reboot |  |  |
| Test Result |  |  |  |

UsbModem_ServiceProvider_6

| Case ID | UsbModem_ServiceProvider_6 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | SIM card from other providers |  |  |
| Pre-condition | 1) The 3g service is activated and available 2) WAN through 3g modem with original SIM card works well |  |  |
| Test procedure | 1) Unplug the origianl SIM card and plug a new SIM card from other providers 2) WAN through usbmodem 3) Ping an internet address |  |  |
| Expect result | 1) USB modem get access to the new APN successfully 2) Ping successfully through usbmodem WAN |  |  |
| Test Result |  |  |  |

UsbModem_ServiceProvider_7

| Case ID | UsbModem_ServiceProvider_7 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Update APN list |  |  |
| Pre-condition | 1) The 3g service is activated and available |  |  |
| Test procedure | 1) Show the APN list in BR console and HM 2) Refresh the APN list in BR console and HM |  |  |
| Expect result | 1) There is APN list printed in console as well as in HM 2) The APN list will be updated in console as well as in HM |  |  |
| Test Result |  |  |  |

UsbModem_OnDemand

UsbModem_OnDemand_1

| Case ID | UsbModem_OnDemand_1 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | 3g connection status |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server name 10.155.33.220 capwap client vhm-name JingLi-VHM routing route-request enable usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Check the usbmodem status while WAN is eth0 by default 2) Trigger failover to usbmodem WAN via shut eth0 3) Check the WAN and usbmodem status 4) Trigger failback to eth0 WAN via enable eth0 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is eth0 by default, and 3g connection is “Backup” and “Attached” AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE 2) WAN is usbmodem once failover, and 3g connection is "Backup" and "Failover” AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 8889 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 3792 49 0 0 0 0 49 ¦ 6385 67 0 0 67 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 3 times (since Wed Mar 28 10:30:43 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 1 times (since Wed Mar 28 10:30:38 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 3 times (since Wed Mar 28 10:30:58 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 3 times (since Wed Mar 28 10:30:53 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE 3) WAN is eth0 after failback, and 3g connection is "Backup" and "Attached” AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 3 times (since Wed Mar 28 10:35:12 2012) DOWN: 3 times (since Wed Mar 28 10:30:43 2012) Local Link State: UP: 2 times (since Wed Mar 28 10:34:57 2012) DOWN: 1 times (since Wed Mar 28 10:30:38 2012) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 3 times (since Wed Mar 28 10:30:58 2012) DOWN: 6 times (since Wed Mar 28 10:35:20 2012) Local Link State: UP: 3 times (since Wed Mar 28 10:30:53 2012) DOWN: 3 times (since Wed Mar 28 10:35:12 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE |  |  |
| Test Result |  |  |  |

UsbModem_OnDemand_2

| Case ID | UsbModem_OnDemand_2 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Failover usbmodem via unplug eth0 |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server name 10.155.33.220 capwap client vhm-name JingLi-VHM routing route-request enable usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, and check the DNS of BR, while WAN is eth0 by default 2) Trigger failover to usbmodem WAN via shut eth0 in eth0 sh 3) Continue ping process and check the DNS of BR, and check the WAN and usbmodem status 4) Trigger failback to eth0 WAN via enable eth0 no in eth0 sh 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is eth0 by default, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=127ms TTL=44 Reply from 8.8.8.8: bytes=32 time=139ms TTL=44 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from default: Domain name suffix: Primary : 168.143.87.77 2) WAN is usbmodem once failover, and ping without error, and DNS changed accordingly AH-8c66c0#in eth0 sh - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 11912 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 1848 33 0 0 0 0 33 ¦ 4106 44 0 0 44 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 5 times (since Wed Mar 28 10:43:15 2012) DOWN: 6 times (since Wed Mar 28 10:52:03 2012) Local Link State: UP: 2 times (since Wed Mar 28 10:34:57 2012) DOWN: 2 times (since Wed Mar 28 10:51:53 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 6 times (since Wed Mar 28 10:52:18 2012) DOWN: 10 times (since Wed Mar 28 10:43:23 2012) Local Link State: UP: 6 times (since Wed Mar 28 10:52:13 2012) DOWN: 5 times (since Wed Mar 28 10:43:15 2012) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping during failover - - - - - - - - - - - - - - - - - - - - - - Reply from 192.168.85.1: Destination net unreachable. Reply from 192.168.85.1: Destination net unreachable. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping after failover - - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=136ms TTL=45 Reply from 8.8.8.8: bytes=32 time=138ms TTL=45 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: Primary : 221.12.1.227 Secondary : 221.12.33.227 Tertiary : 0.0.0.0 3) WAN is eth0 after failback, and ping without error, and DNS changed accordingly AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 6 times (since Wed Mar 28 10:55:49 2012) DOWN: 6 times (since Wed Mar 28 10:52:03 2012) Local Link State: UP: 3 times (since Wed Mar 28 10:55:42 2012) DOWN: 2 times (since Wed Mar 28 10:51:53 2012) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 6 times (since Wed Mar 28 10:52:18 2012) DOWN: 12 times (since Wed Mar 28 10:56:41 2012) Local Link State: UP: 6 times (since Wed Mar 28 10:52:13 2012) DOWN: 6 times (since Wed Mar 28 10:55:49 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=32ms TTL=44 Reply from 8.8.8.8: bytes=32 time=32ms TTL=44 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from default: Domain name suffix: Primary : 168.143.87.77 |  |  |
| Test Result |  |  |  |

UsbModem_OnDemand_3

| Case ID | UsbModem_OnDemand_3 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Failover usbmodem when no internet access through eth0 |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server name 10.155.33.220 capwap client vhm-name JingLi-VHM routing route-request enable usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, while WAN is eth0 by default 2) Trigger failover to usbmodem WAN when internet access is unavailable Unplug the internet cable from the HUB which the BR is connected with 3) Continue ping process and check the WAN and usbmodem status 4) Trigger failback to eth0 WAN when internet access through eth0 is restored plug the internet cable back to the HUB which the BR is connected with 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is eth0 by default, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=127ms TTL=44 Reply from 8.8.8.8: bytes=32 time=139ms TTL=44 2) WAN is usbmodem once failover, and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 14427 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 6471 103 0 0 0 0 103 ¦ 6361 105 0 0 105 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 7 times (since Wed Mar 28 11:06:26 2012) DOWN: 8 times (since Wed Mar 28 11:11:13 2012) Local Link State: UP: 4 times (since Wed Mar 28 11:06:16 2012) DOWN: 3 times (since Wed Mar 28 11:04:57 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 8 times (since Wed Mar 28 11:11:29 2012) DOWN: 14 times (since Wed Mar 28 11:06:34 2012) Local Link State: UP: 8 times (since Wed Mar 28 11:11:24 2012) DOWN: 7 times (since Wed Mar 28 11:06:26 2012) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.26.197.44 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 32766 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - debug console - - - - - - - - - - - - - - - - - - - - - - - - 2012-03-28 11:14:43 debug ah_brd: [brd_wanmon]: WFOIFM: Ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 8.8.8.8] executing 2012-03-28 11:14:45 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via eth0: 100% packet loss 2012-03-28 11:14:45 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via eth0 failed 2012-03-28 11:14:45 debug ah_brd: [brd_wanmon]: WFOIFM: Interface eth0: 1 targets tested, result: FAIL 2012-03-28 11:14:45 debug ah_brd: [brd_wanmon]: WFOIFM: Use cached gw IP 0x0 for ppp0 2012-03-28 11:14:45 debug ah_brd: [brd_wanmon]: WFOIFM: Interface ppp0 disallows ARP (ifr_flags = 0x10d1) 2012-03-28 11:14:45 debug ah_brd: [brd_wanmon]: WFOIFM: Going to test 1 targets for ppp0 ... 2012-03-28 11:14:45 debug ah_brd: [brd_wanmon]: WFOIFM: Ping cmd [/bin/ping -q -c 1 -I ppp0 -w 2 8.8.8.8] executing 2012-03-28 11:14:45 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via ppp0: 0% packet loss 2012-03-28 11:14:45 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via ppp0: RTT min/avg/max = 131.177002/131.177002/131.177002 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping during failover - - - - - - - - - - - - - - - - - - - - - - Reply from 192.168.85.1: Destination net unreachable. Reply from 192.168.85.1: Destination net unreachable. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping after failover - - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=136ms TTL=45 Reply from 8.8.8.8: bytes=32 time=138ms TTL=45 3) WAN is eth0 after failback, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=127ms TTL=44 Reply from 8.8.8.8: bytes=32 time=139ms TTL=44 |  |  |
| Test Result |  |  |  |

UsbModem_OnDemand_4

| Case ID | UsbModem_OnDemand_4 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Failover usbmodem via fail to track default gateway |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test default-gateway track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server name 10.155.33.220 capwap client vhm-name JingLi-VHM routing route-request enable usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, while WAN is eth0 by default 2) Trigger failover to usbmodem WAN when fail to track default gateway of eth0 no in eth0 dhcp client 3) Continue ping process and check the WAN and usbmodem status 4) Trigger failback to eth0 WAN when success to track default gateway of eth0 in eth0 dhcp client 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is eth0 by default, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - debug console - - - - - - - - - - - - - - - - - - - - - - - - 2012-03-28 11:45:34 debug ah_brd: [brd_wanmon]: WFOIFM: Interface eth0: 1 targets tested, result: PASS 2012-03-28 11:45:34 debug ah_brd: [brd_wanmon]: WFOIFM: Interface ppp0 does not exist 2012-03-28 11:45:34 debug ah_brd: [brd_wanmon]: WFOIFM: Going to pause for 2 secs before next test 2012-03-28 11:45:36 debug ah_brd: [brd_wanmon]: WFOIFM: Determined current gw IP as 0xa9b1efe for eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=127ms TTL=44 Reply from 8.8.8.8: bytes=32 time=139ms TTL=44 2) WAN is usbmodem once failover, and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 14427 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 6471 103 0 0 0 0 103 ¦ 6361 105 0 0 105 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 7 times (since Wed Mar 28 11:06:26 2012) DOWN: 8 times (since Wed Mar 28 11:11:13 2012) Local Link State: UP: 4 times (since Wed Mar 28 11:06:16 2012) DOWN: 3 times (since Wed Mar 28 11:04:57 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 8 times (since Wed Mar 28 11:11:29 2012) DOWN: 14 times (since Wed Mar 28 11:06:34 2012) Local Link State: UP: 8 times (since Wed Mar 28 11:11:24 2012) DOWN: 7 times (since Wed Mar 28 11:06:26 2012) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 0.0.0.0 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.40.12 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - debug console - - - - - - - - - - - - - - - - - - - - - - - - 2012-03-28 11:42:40 debug ah_brd: [brd_wanmon]: WFOIFM: Arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.254] executing 2012-03-28 11:42:41 debug ah_brd: [brd_wanmon]: WFOIFM: Arping 10.155.30.254 via eth0 failed 2012-03-28 11:42:41 debug ah_brd: [brd_wanmon]: WFOIFM: Interface eth0: 1 targets tested, result: FAIL 2012-03-28 11:42:41 debug ah_brd: [brd_wanmon]: WFOIFM: Use cached gw IP 0x0 for ppp0 2012-03-28 11:42:41 debug ah_brd: [brd_wanmon]: WFOIFM: Interface ppp0 disallows ARP (ifr_flags = 0x10d1) 2012-03-28 11:42:41 debug ah_brd: [brd_wanmon]: WFOIFM: Mark test "passed" for ppp0 (non-ARPable and default gw test only) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping during failover - - - - - - - - - - - - - - - - - - - - - - Reply from 192.168.85.1: Destination net unreachable. Reply from 192.168.85.1: Destination net unreachable. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping after failover - - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=136ms TTL=45 Reply from 8.8.8.8: bytes=32 time=138ms TTL=45 3) WAN is eth0 after failback, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - debug console - - - - - - - - - - - - - - - - - - - - - - - - 2012-03-28 11:45:34 debug ah_brd: [brd_wanmon]: WFOIFM: Interface eth0: 1 targets tested, result: PASS 2012-03-28 11:45:34 debug ah_brd: [brd_wanmon]: WFOIFM: Interface ppp0 does not exist 2012-03-28 11:45:34 debug ah_brd: [brd_wanmon]: WFOIFM: Going to pause for 2 secs before next test 2012-03-28 11:45:36 debug ah_brd: [brd_wanmon]: WFOIFM: Determined current gw IP as 0xa9b1efe for eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=127ms TTL=44 Reply from 8.8.8.8: bytes=32 time=139ms TTL=44 |  |  |
| Test Result |  |  |  |

UsbModem_OnDemand_5

| Case ID | UsbModem_OnDemand_5 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Ping with different pkt size through usbmodem WAN |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) MTU of each peer of network (Client/ BR/ Switch) is 1518 5) Client get association with BR either by eth or wifi AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan ip 23.15.247.43 track wan use-for-wan-testing routing route-request enable usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, with pkt size 32 bytes, while WAN is eth0 by default 2) Trigger failover to usbmodem WAN via shut eth0 in e0 sh 3) Continue ping process and check the WAN and usbmodem status 4) Trigger failback to eth0 WAN via enable eth0 no in e0 sh 5) Repeat step 3) 6) Repeat step 1)-5) with ping pkt size 1472 7) Repeat step 1)-5) with ping pkt size 1473 |  |  |
| Expect result | 1) WAN is eth0 by default, and ping without error AH-8c66c0#sh usbm m huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Sun Apr 1 02:50:10 2012) DOWN: 1 times (since Sun Apr 1 02:49:52 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 23.15.247.43 with 32 bytes of data: Reply from 23.15.247.43: bytes=32 time=253ms TTL=50 Reply from 23.15.247.43: bytes=32 time=256ms TTL=50 2) WAN is usbmodem once failover, and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 4800 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 996 20 0 0 0 0 20 ¦ 1236 22 0 0 22 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 2 times (since Sun Apr 1 02:50:10 2012) DOWN: 2 times (since Sun Apr 1 03:08:43 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 1 times (since Sun Apr 1 03:08:38 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Sun Apr 1 03:08:59 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Sun Apr 1 03:08:54 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.38.208.207 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 192.168.85.1: Destination net unreachable. Reply from 192.168.85.1: Destination net unreachable. Reply from 23.15.247.43: bytes=32 time=204ms TTL=51 Reply from 23.15.247.43: bytes=32 time=213ms TTL=51 3) WAN is eth0 after failback, and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 3 times (since Sun Apr 1 03:10:50 2012) DOWN: 2 times (since Sun Apr 1 03:08:43 2012) Local Link State: UP: 2 times (since Sun Apr 1 03:10:40 2012) DOWN: 1 times (since Sun Apr 1 03:08:38 2012) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 1 times (since Sun Apr 1 03:08:59 2012) DOWN: 1 times (since Sun Apr 1 03:10:51 2012) Local Link State: UP: 1 times (since Sun Apr 1 03:08:54 2012) DOWN: 1 times (since Sun Apr 1 03:10:50 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 23.15.247.43: bytes=32 time=234ms TTL=50 Reply from 23.15.247.43: bytes=32 time=229ms TTL=50 4) When ping pkt size < 1473, there is no ip fragment 5) When ping pkt size ≥ 1473, there is ip fragment |  |  |
| Test Result |  |  |  |

UsbModem_OnDemand_6

| Case ID | UsbModem_OnDemand_6 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Http video through usbmodem WAN |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server port 80 vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Check the WAN status as eth0 is the default WAN 2) Try to play http video from client, http://www.letv.com/ptv/vplay/1257936.html 3) Trigger failover to usbmodem WAN via shut eth0 4) Continue playing the http video 5) Trigger failback to eth0 WAN via enable eth0 6) Repeat step 4) |  |  |
| Expect result | 1) Play http video successfully before/ after faiover/ failback 2) failover/ failback without error 3) Have to replay the video after failover/ failback AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Before failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 11912 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 1848 33 0 0 0 0 33 ¦ 4106 44 0 0 44 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 5 times (since Wed Mar 28 10:43:15 2012) DOWN: 6 times (since Wed Mar 28 10:52:03 2012) Local Link State: UP: 2 times (since Wed Mar 28 10:34:57 2012) DOWN: 2 times (since Wed Mar 28 10:51:53 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 6 times (since Wed Mar 28 10:52:18 2012) DOWN: 10 times (since Wed Mar 28 10:43:23 2012) Local Link State: UP: 6 times (since Wed Mar 28 10:52:13 2012) DOWN: 5 times (since Wed Mar 28 10:43:15 2012) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failback - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 |  |  |
| Test Result |  |  |  |

UsbModem_OnDemand_7

| Case ID | UsbModem_OnDemand_7 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Http download through usbmodem WAN |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server port 80 vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Check the WAN status as eth0 is the default WAN 2) Try to download file “QQ2012Beta1_QQProtect2.5.exe” from client http://im.qq.com/qq/2012/safe/ 3) Trigger failover to usbmodem WAN via shut eth0 4) Continue downloading the file 5) Trigger failback to eth0 WAN via enable eth0 6) Repeat step 4) |  |  |
| Expect result | 1) Http download successfully before/ after faiover/ failback 2) failover/ failback without error 3) Have to re-download the file after failover/ failback AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Before failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 15290 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 2556 16 0 0 0 0 16 ¦ 2157 22 0 0 22 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 4 times (since Wed Apr 4 06:31:05 2012) DOWN: 4 times (since Wed Apr 4 07:04:04 2012) Local Link State: UP: 2 times (since Wed Apr 4 06:30:50 2012) DOWN: 2 times (since Wed Apr 4 07:03:57 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 3 times (since Wed Apr 4 07:04:19 2012) DOWN: 6 times (since Wed Apr 4 06:31:13 2012) Local Link State: UP: 4 times (since Wed Apr 4 07:04:14 2012) DOWN: 3 times (since Wed Apr 4 06:31:05 2012) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 172.28.106.7 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.0.1 0.0.0.0 255.255.255.255 UH 0 0 0 tunnel0 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.85.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.30.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 10.155.33.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 192.168.10.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 172.28.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.18.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failback - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 |  |  |
| Test Result |  |  |  |

UsbModem_OnDemand_8

| Case ID | UsbModem_OnDemand_8 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | VPN tunnel accessibility |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi 5) VPN tunnel for usbmodem WAN has been configured AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Ping another VPN endpoint through tunnel ping 10.155.33.67 2) Trigger failover to usbmodem WAN via shut eth0, continue ping process 3) Continue ping process and check the WAN and usbmodem status from both HM and console 4) Trigger failback to eth0 WAN via enable eth0, continue ping process 5) Repeat step 3) |  |  |
| Expect result | 1) Ping through VPN tunnel successfully with failover and failback 2) failover/ failback without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Before failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 10.155.33.67 with 32 bytes of data: Reply from 10.155.33.67: bytes=32 time=1ms TTL=126 Reply from 10.155.33.67: bytes=32 time=1ms TTL=126 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 4200 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 9034 59 0 0 0 0 59 ¦ 9435 74 0 0 74 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 2 times (since Wed Apr 4 08:25:50 2012) DOWN: 2 times (since Wed Apr 4 08:26:23 2012) Local Link State: UP: 1 times (since Wed Apr 4 08:25:38 2012) DOWN: 2 times (since Wed Apr 4 08:26:20 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 2 times (since Wed Apr 4 08:26:38 2012) DOWN: 2 times (since Wed Apr 4 08:25:58 2012) Local Link State: UP: 2 times (since Wed Apr 4 08:26:33 2012) DOWN: 1 times (since Wed Apr 4 08:25:50 2012) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.31.218.171 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.0.1 0.0.0.0 255.255.255.255 UH 0 0 0 tunnel0 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.85.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.30.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 10.155.33.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 192.168.10.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 172.28.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.18.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 10.155.33.67: bytes=32 time=142ms TTL=126 Reply from 10.155.33.67: bytes=32 time=139ms TTL=126 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failback - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 10.155.33.67: bytes=32 time=1ms TTL=126 Reply from 10.155.33.67: bytes=32 time=1ms TTL=126 |  |  |
| Test Result |  |  |  |

UsbModem_OnDemand_9

| Case ID | UsbModem_OnDemand_9 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Play video through vpn tunnel when WAN is usbmodem |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi 5) VPN tunnel for usbmodem WAN has been configured AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Check the WAN status as eth0 is the default WAN 2) Access the video on HQ server, \\10.155.33.67\share\cctv.rm 3) Try to play the video from client 4) Trigger failover to usbmodem WAN via shut eth0 5) Continue playing the video through vpn tunnel 6) Trigger failback to eth0 WAN via enable eth0 7) Continue playing the video |  |  |
| Expect result | 1) Play the video through vpn tunnel from client successfully 2) Have to replay the video after failover/ failback 3) failover/ failback without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Before failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 4200 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 9034 59 0 0 0 0 59 ¦ 9435 74 0 0 74 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 2 times (since Wed Apr 4 08:25:50 2012) DOWN: 2 times (since Wed Apr 4 08:26:23 2012) Local Link State: UP: 1 times (since Wed Apr 4 08:25:38 2012) DOWN: 2 times (since Wed Apr 4 08:26:20 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 2 times (since Wed Apr 4 08:26:38 2012) DOWN: 2 times (since Wed Apr 4 08:25:58 2012) Local Link State: UP: 2 times (since Wed Apr 4 08:26:33 2012) DOWN: 1 times (since Wed Apr 4 08:25:50 2012) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.31.218.171 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.0.1 0.0.0.0 255.255.255.255 UH 0 0 0 tunnel0 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.85.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.30.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 10.155.33.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 192.168.10.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 172.28.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.18.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Have to replay the video since the WAN connection has changed - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failback - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Have to replay the video since the WAN connection has changed |  |  |
| Test Result |  |  |  |

UsbModem_OnDemand_10

| Case ID | UsbModem_OnDemand_10 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | FTP download file through vpn tunnel when WAN is usbmodem |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi 5) VPN tunnel for usbmodem WAN has been configured AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Check the WAN status as eth0 is the default WAN 2) Access HQ FTP server, ftp 10.155.33.67 3) Download file “hivemanager-5.0r3-beta-1-1U-patch-20120217.tar.gz” to client 4) Trigger failover to usbmodem WAN via shut eth0 5) Continue downloading through vpn tunnel 6) Trigger failback to eth0 WAN via enable eth0 7) Continue downloading the file |  |  |
| Expect result | 1) Download file through vpn tunnel from client successfully 2) FTP resume file transfer after failover/ failback 3) failover/ failback without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Before failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 4200 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 9034 59 0 0 0 0 59 ¦ 9435 74 0 0 74 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 2 times (since Wed Apr 4 08:25:50 2012) DOWN: 2 times (since Wed Apr 4 08:26:23 2012) Local Link State: UP: 1 times (since Wed Apr 4 08:25:38 2012) DOWN: 2 times (since Wed Apr 4 08:26:20 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 2 times (since Wed Apr 4 08:26:38 2012) DOWN: 2 times (since Wed Apr 4 08:25:58 2012) Local Link State: UP: 2 times (since Wed Apr 4 08:26:33 2012) DOWN: 1 times (since Wed Apr 4 08:25:50 2012) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.31.218.171 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.0.1 0.0.0.0 255.255.255.255 UH 0 0 0 tunnel0 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.85.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.30.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 10.155.33.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 192.168.10.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 172.28.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.18.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue FTP process since FTP support resuming transfer - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failback - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue FTP process since FTP support resuming transfer |  |  |
| Test Result |  |  |  |

UsbModem_OnDemand_11

| Case ID | UsbModem_OnDemand_11 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Delta switch from on-demand to always-connect mode |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server name 10.155.33.220 capwap client vhm-name JingLi-VHM routing route-request enable usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, while WAN is eth0 by default 2) Delta config usbmodem mode to always-connect without reboot BR 3) Continue ping process and check the WAN and usbmodem status 4) Switch usbmodem mode back to on-demand 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is eth0 by default, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=127ms TTL=44 Reply from 8.8.8.8: bytes=32 time=139ms TTL=44 2) WAN is eth0 once config usbmodem mode to always-connect, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 20769 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 1200 19 0 0 0 0 19 ¦ 1206 20 0 0 20 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 10 times (since Wed Mar 28 11:44:45 2012) DOWN: 10 times (since Wed Mar 28 11:41:20 2012) Local Link State: UP: 4 times (since Wed Mar 28 11:06:16 2012) DOWN: 3 times (since Wed Mar 28 11:04:57 2012) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 11 times (since Wed Mar 28 11:54:49 2012) DOWN: 18 times (since Wed Mar 28 11:45:01 2012) Local Link State: UP: 11 times (since Wed Mar 28 11:54:44 2012) DOWN: 10 times (since Wed Mar 28 11:44:45 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.39.4.37 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=127ms TTL=44 Reply from 8.8.8.8: bytes=32 time=139ms TTL=44 3) WAN is eth0 after switch usbmodem mode back to on-demand, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=127ms TTL=44 Reply from 8.8.8.8: bytes=32 time=139ms TTL=44 |  |  |
| Test Result |  |  |  |

UsbModem_OnDemand_12

| Case ID | UsbModem_OnDemand_12 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Delta switch from on-demand to primary-wan mode |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server name 10.155.33.220 capwap client vhm-name JingLi-VHM routing route-request enable usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, while WAN is eth0 by default 2) Delta config usbmodem mode to primary-wan without reboot BR 3) Continue ping process and check the WAN and usbmodem status 4) Switch usbmodem mode back to on-demand 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is eth0 by default, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=127ms TTL=44 Reply from 8.8.8.8: bytes=32 time=139ms TTL=44 2) WAN is usbmodem once config its mode to primary-wan, and ping without error AH-8c66c0#sh usbm mode AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 21944 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 2664 45 0 0 0 0 45 ¦ 2920 48 0 0 48 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 13 times (since Wed Mar 28 12:04:06 2012) DOWN: 20 times (since Wed Mar 28 11:57:57 2012) Local Link State: UP: 12 times (since Wed Mar 28 12:03:59 2012) DOWN: 11 times (since Wed Mar 28 11:57:49 2012) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: down UP: 10 times (since Wed Mar 28 11:44:45 2012) DOWN: 11 times (since Wed Mar 28 12:04:14 2012) Local Link State: UP: 4 times (since Wed Mar 28 11:06:16 2012) DOWN: 3 times (since Wed Mar 28 11:04:57 2012) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.58.46 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - debug console - - - - - - - - - - - - - - - - - - - - - - - - 2012-03-28 12:04:56 debug ah_brd: [brd_wanmon]: WFOIFM: Ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 8.8.8.8] executing 2012-03-28 12:04:58 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via eth0: 100% packet loss 2012-03-28 12:04:58 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via eth0 failed 2012-03-28 12:04:58 debug ah_brd: [brd_wanmon]: WFOIFM: Interface eth0: 1 targets tested, result: FAIL 2012-03-28 12:04:58 debug ah_brd: [brd_wanmon]: WFOIFM: Use cached gw IP 0x0 for ppp0 2012-03-28 12:04:58 debug ah_brd: [brd_wanmon]: WFOIFM: Interface ppp0 disallows ARP (ifr_flags = 0x10d1) 2012-03-28 12:04:58 debug ah_brd: [brd_wanmon]: WFOIFM: Going to test 1 targets for ppp0 ... 2012-03-28 12:04:58 debug ah_brd: [brd_wanmon]: WFOIFM: Ping cmd [/bin/ping -q -c 1 -I ppp0 -w 2 8.8.8.8] executing 2012-03-28 12:04:58 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via ppp0: 0% packet loss 2012-03-28 12:04:58 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via ppp0: RTT min/avg/max = 141.242004/141.242004/141.242004 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=127ms TTL=44 Reply from 8.8.8.8: bytes=32 time=139ms TTL=44 3) WAN is eth0 after switch usbmodem mode back to on-demand, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=127ms TTL=44 Reply from 8.8.8.8: bytes=32 time=139ms TTL=44 |  |  |
| Test Result |  |  |  |

UsbModem_AlwaysConnected

UsbModem_AlwaysConnected_1

| Case ID | UsbModem_AlwaysConnected_1 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | 3g connection status |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connected 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server name 10.155.33.220 capwap client vhm-name JingLi-VHM vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Check the usbmodem status while WAN is eth0 by default 2) Trigger failover to usbmodem WAN via shut eth0 3) Check the WAN and usbmodem status 4) Trigger failback to eth0 WAN via enable eth0 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is eth0 by default, and 3g connection is “Backup” and “CONNECTED” AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE 2) WAN is usbmodem once failover, and 3g connection is “Backup” and “FAILOVER” AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 19680 239 0 0 0 0 239 ¦ 19686 240 0 0 240 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 1 times (since Thu Mar 29 07:15:52 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 1 times (since Thu Mar 29 07:15:42 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE 3) WAN is eth0 after failback, and 3g connection is “Backup” and “CONNECTED” AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE |  |  |
| Test Result |  |  |  |

UsbModem_AlwaysConnected_2

| Case ID | UsbModem_AlwaysConnected_2 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Failover usbmodem via unplug eth0 |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connected 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server name 10.155.33.220 capwap client vhm-name JingLi-VHM vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, and check the DNS of BR, while WAN is eth0 by default 2) Trigger failover to usbmodem WAN via shut eth0 in eth0 sh 3) Continue ping process and check the DNS of BR, and check the WAN and usbmodem status 4) Trigger failback to eth0 WAN via enable eth0 no in eth0 sh 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is eth0 by default, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=34ms TTL=44 Reply from 8.8.8.8: bytes=32 time=33ms TTL=44 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from default: Domain name suffix: Primary : 168.143.87.77 2) WAN is usbmodem once failover, and ping without error, and DNS changed accordingly AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 61906 513 0 0 0 0 513 ¦ 52199 501 0 0 501 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 2 times (since Thu Mar 29 07:17:34 2012) DOWN: 2 times (since Thu Mar 29 07:27:28 2012) Local Link State: UP: 2 times (since Thu Mar 29 07:17:17 2012) DOWN: 2 times (since Thu Mar 29 07:27:15 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping during failover - - - - - - - - - - - - - - - - - - - - - - Reply from 192.168.85.1: Destination net unreachable. Reply from 192.168.85.1: Destination net unreachable. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping after failover - - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=136ms TTL=45 Reply from 8.8.8.8: bytes=32 time=138ms TTL=45 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: Primary : 221.12.1.227 Secondary : 221.12.33.227 Tertiary : 0.0.0.0 3) WAN is eth0 after failback, and ping without error, and DNS changed accordingly AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=32ms TTL=44 Reply from 8.8.8.8: bytes=32 time=32ms TTL=44 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from default: Domain name suffix: Primary : 168.143.87.77 |  |  |
| Test Result |  |  |  |

UsbModem_AlwaysConnected_3

| Case ID | UsbModem_AlwaysConnected_3 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Failover usbmodem when no internet access through eth0 |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connected 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server name 10.155.33.220 capwap client vhm-name JingLi-VHM vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, while WAN is eth0 by default 2) Trigger failover to usbmodem WAN when internet access is unavailable Unplug the internet cable from the HUB which the BR is connected with 3) Continue ping process and check the WAN and usbmodem status 4) Trigger failback to eth0 WAN when internet access through eth0 is restored Plug the internet cable back to the HUB which the BR is connected with 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is eth0 by default, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=34ms TTL=44 Reply from 8.8.8.8: bytes=32 time=33ms TTL=44 2) WAN is usbmodem once failover, and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 451674 3187 0 0 0 0 3187 ¦ 253391 2734 0 0 2734 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 3 times (since Thu Mar 29 07:30:51 2012) DOWN: 3 times (since Thu Mar 29 07:36:50 2012) Local Link State: UP: 3 times (since Thu Mar 29 07:30:46 2012) DOWN: 2 times (since Thu Mar 29 07:27:15 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 2 times (since Thu Mar 29 07:49:12 2012) DOWN: 1 times (since Thu Mar 29 07:48:58 2012) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 32766 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - debug console - - - - - - - - - - - - - - - - - - - - - - - - 2012-03-29 07:53:54 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via eth0: 100% packet loss 2012-03-29 07:53:54 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via eth0 failed 2012-03-29 07:53:54 debug ah_brd: [brd_wanmon]: WFOIFM: Interface eth0: 1 targets tested, result: FAIL 2012-03-29 07:53:54 debug ah_brd: [brd_wanmon]: WFOIFM: Use cached gw IP 0x0 for ppp0 2012-03-29 07:53:54 debug ah_brd: [brd_wanmon]: WFOIFM: Interface ppp0 disallows ARP (ifr_flags = 0x10d1) 2012-03-29 07:53:54 debug ah_brd: [brd_wanmon]: WFOIFM: Going to test 1 targets for ppp0 ... 2012-03-29 07:53:54 debug ah_brd: [brd_wanmon]: WFOIFM: Ping cmd [/bin/ping -q -c 1 -I ppp0 -w 2 8.8.8.8] executing 2012-03-29 07:53:54 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via ppp0: 0% packet loss 2012-03-29 07:53:54 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via ppp0: RTT min/avg/max = 132.227997/132.227997/132.227997 2012-03-29 07:53:56 debug ah_brd: [brd_wanmon]: WFOIFM: Ping cmd [/bin/ping -q -c 1 -I ppp0 -w 2 8.8.8.8] executing 2012-03-29 07:53:57 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via ppp0: 0% packet loss 2012-03-29 07:53:57 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via ppp0: RTT min/avg/max = 119.209000/119.209000/119.209000 2012-03-29 07:53:59 debug ah_brd: [brd_wanmon]: WFOIFM: Ping cmd [/bin/ping -q -c 1 -I ppp0 -w 2 8.8.8.8] executing 2012-03-29 07:53:59 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via ppp0: 0% packet loss 2012-03-29 07:53:59 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via ppp0: RTT min/avg/max = 110.170998/110.170998/110.170998 2012-03-29 07:54:01 debug ah_brd: [brd_wanmon]: WFOIFM: Interface ppp0: 1 targets tested, result: PASS - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping during failover - - - - - - - - - - - - - - - - - - - - - - Reply from 10.155.30.55: Destination net unreachable. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping after failover - - - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=136ms TTL=45 Reply from 8.8.8.8: bytes=32 time=138ms TTL=45 3) WAN is eth0 after failback, and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=34ms TTL=44 Reply from 8.8.8.8: bytes=32 time=33ms TTL=44 |  |  |
| Test Result |  |  |  |

UsbModem_AlwaysConnected_4

| Case ID | UsbModem_AlwaysConnected_4 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Failover usbmodem via fail to track default gateway |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connected 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test default-gateway track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server name 10.155.33.220 capwap client vhm-name JingLi-VHM vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, while WAN is eth0 by default 2) Trigger failover to usbmodem WAN when fail to track default gateway of eth0 no in eth0 dhcp client 3) Continue ping process and check the WAN and usbmodem status 4) Trigger failback to eth0 WAN when success to track default gateway of eth0 in eth0 dhcp client 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is eth0 by default, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=34ms TTL=44 Reply from 8.8.8.8: bytes=32 time=33ms TTL=44 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - debug console - - - - - - - - - - - - - - - - - - - - - - - - 2012-03-29 08:04:13 debug ah_brd: [brd_wanmon]: WFOIFM: Going to test 1 targets for eth0 ... 2012-03-29 08:04:13 debug ah_brd: [brd_wanmon]: WFOIFM: Arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.254] executing 2012-03-29 08:04:14 debug ah_brd: [brd_wanmon]: WFOIFM: Arping OK: RTT avg = 4.207000 (out of 1 counts) 2012-03-29 08:04:16 debug ah_brd: [brd_wanmon]: WFOIFM: Arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.254] executing 2012-03-29 08:04:17 debug ah_brd: [brd_wanmon]: WFOIFM: Arping OK: RTT avg = 10.685000 (out of 1 counts) 2012-03-29 08:04:19 debug ah_brd: [brd_wanmon]: WFOIFM: Arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.254] executing 2012-03-29 08:04:20 debug ah_brd: [brd_wanmon]: WFOIFM: Arping OK: RTT avg = 2.162000 (out of 1 counts) 2012-03-29 08:04:22 debug ah_brd: [brd_wanmon]: WFOIFM: Interface eth0: 1 targets tested, result: PASS 2012-03-29 08:04:22 debug ah_brd: [brd_wanmon]: WFOIFM: Use cached gw IP 0x0 for ppp0 2012-03-29 08:04:22 debug ah_brd: [brd_wanmon]: WFOIFM: Interface ppp0 disallows ARP (ifr_flags = 0x10d1) 2012-03-29 08:04:22 debug ah_brd: [brd_wanmon]: WFOIFM: Mark test "passed" for ppp0 (non-ARPable and default gw test only) 2) WAN is usbmodem once failover, and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 562438 3941 0 0 0 0 3941 ¦ 309729 3352 0 0 3352 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 5 times (since Thu Mar 29 08:03:38 2012) DOWN: 5 times (since Thu Mar 29 08:06:51 2012) Local Link State: UP: 3 times (since Thu Mar 29 07:30:46 2012) DOWN: 2 times (since Thu Mar 29 07:27:15 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 3 times (since Thu Mar 29 08:03:38 2012) DOWN: 2 times (since Thu Mar 29 08:02:59 2012) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 0.0.0.0 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - debug console - - - - - - - - - - - - - - - - - - - - - - - - 2012-03-29 08:07:28 debug ah_brd: [brd_wanmon]: WFOIFM: Going to test 1 targets for eth0 ... 2012-03-29 08:07:28 debug ah_brd: [brd_wanmon]: WFOIFM: Arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.254] executing 2012-03-29 08:07:29 debug ah_brd: [brd_wanmon]: WFOIFM: Arping 10.155.30.254 via eth0 failed 2012-03-29 08:07:29 debug ah_brd: [brd_wanmon]: WFOIFM: Arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.254] executing 2012-03-29 08:07:30 debug ah_brd: [brd_wanmon]: WFOIFM: Arping 10.155.30.254 via eth0 failed 2012-03-29 08:07:30 debug ah_brd: [brd_wanmon]: WFOIFM: Arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.254] executing 2012-03-29 08:07:31 debug ah_brd: [brd_wanmon]: WFOIFM: Arping 10.155.30.254 via eth0 failed 2012-03-29 08:07:31 debug ah_brd: [brd_wanmon]: WFOIFM: Interface eth0: 1 targets tested, result: FAIL 2012-03-29 08:07:31 debug ah_brd: [brd_wanmon]: WFOIFM: Use cached gw IP 0x0 for ppp0 2012-03-29 08:07:31 debug ah_brd: [brd_wanmon]: WFOIFM: Interface ppp0 disallows ARP (ifr_flags = 0x10d1) 2012-03-29 08:07:31 debug ah_brd: [brd_wanmon]: WFOIFM: Mark test "passed" for ppp0 (non-ARPable and default gw test only) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping during failover - - - - - - - - - - - - - - - - - - - - - - Reply from 192.168.85.1: Destination net unreachable. Reply from 192.168.85.1: Destination net unreachable. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping after failover - - - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=136ms TTL=45 Reply from 8.8.8.8: bytes=32 time=138ms TTL=45 3) WAN is eth0 after failback, and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=34ms TTL=44 Reply from 8.8.8.8: bytes=32 time=33ms TTL=44 |  |  |
| Test Result |  |  |  |

UsbModem_AlwaysConnected_5

| Case ID | UsbModem_AlwaysConnected_5 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Ping with different pkt size through usbmodem WAN |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connected 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) MTU of each peer of network (Client/ BR/ Switch) is 1518 5) Client get association with BR either by eth or wifi AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan ip 23.15.247.43 track wan use-for-wan-testing routing route-request enable usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, with pkt size 64, while WAN is eth0 by default 2) Trigger failover to usbmodem WAN via shut eth0 3) Continue ping process and check the WAN and usbmodem status 4) Trigger failback to eth0 WAN via enable eth0 5) Repeat step 3) 6) Repeat step 1)-5) with ping pkt size 1472 7) Repeat step 1)-5) with ping pkt size 1473 |  |  |
| Expect result | 1) WAN is eth0 by default, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=34ms TTL=44 Reply from 8.8.8.8: bytes=32 time=33ms TTL=44 2) WAN is usbmodem once failover, and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 61906 513 0 0 0 0 513 ¦ 52199 501 0 0 501 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 2 times (since Thu Mar 29 07:17:34 2012) DOWN: 2 times (since Thu Mar 29 07:27:28 2012) Local Link State: UP: 2 times (since Thu Mar 29 07:17:17 2012) DOWN: 2 times (since Thu Mar 29 07:27:15 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping during failover - - - - - - - - - - - - - - - - - - - - - - Reply from 192.168.85.1: Destination net unreachable. Reply from 192.168.85.1: Destination net unreachable. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping after failover - - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=136ms TTL=45 Reply from 8.8.8.8: bytes=32 time=138ms TTL=45 3) WAN is eth0 after failback, and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=32ms TTL=44 Reply from 8.8.8.8: bytes=32 time=32ms TTL=44 4) When ping pkt size < 1473, there is no ip fragment 5) When ping pkt size ≥ 1473, there is ip fragment |  |  |
| Test Result |  |  |  |

UsbModem_AlwaysConnected_6

| Case ID | UsbModem_AlwaysConnected_6 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Http video through usbmodem WAN |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connected 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi |  |  |
| Test procedure | 1) Check the WAN status as eth0 is the default WAN 2) Try to play http video from client, http://www.letv.com/ptv/vplay/1257936.html 3) Trigger failover to usbmodem WAN via shut eth0 4) Continue playing the http video 5) Trigger failback to eth0 WAN via enable eth0 6) Repeat step 4) |  |  |
| Expect result | 1) Play http video successfully before/ after faiover/ failback 2) failover/ failback without error 3) Have to replay the video after failover/ failback |  |  |
| Test Result |  |  |  |

UsbModem_AlwaysConnected_7

| Case ID | UsbModem_AlwaysConnected_7 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Http download through usbmodem WAN |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connected 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi |  |  |
| Test procedure | 1) Check the WAN status as eth0 is the default WAN 2) Try to download file “QQ2012Beta1_QQProtect2.5.exe” from client http://im.qq.com/qq/2012/safe/ 3) Trigger failover to usbmodem WAN via shut eth0 4) Continue downloading the file 5) Trigger failback to eth0 WAN via enable eth0 6) Repeat step 4) |  |  |
| Expect result | 1) Http download successfully before/ after faiover/ failback 2) failover/ failback without error 3) Have to re-download the file after failover/ failback |  |  |
| Test Result |  |  |  |

UsbModem_AlwaysConnected_8

| Case ID | UsbModem_AlwaysConnected_8 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | VPN tunnel accessibility |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connected 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi 5) VPN tunnel for usbmodem WAN has been configured AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Ping another VPN endpoint through tunnel 2) Trigger failover to usbmodem WAN via shut eth0, continue ping process 3) Continue ping process and check the WAN and usbmodem status from both HM and console 4) Trigger failback to eth0 WAN via enable eth0, continue ping process 5) Repeat step 3) |  |  |
| Expect result | 1) Ping through VPN tunnel successfully with failover and failback 2) failover/ failback without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Before failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 10.155.33.67 with 32 bytes of data: Reply from 10.155.33.67: bytes=32 time=1ms TTL=126 Reply from 10.155.33.67: bytes=32 time=1ms TTL=126 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2291 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 13701 113 0 0 0 0 113 ¦ 14101 124 0 0 124 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 1 times (since Wed Apr 4 08:58:19 2012) DOWN: 1 times (since Wed Apr 4 09:03:29 2012) Local Link State: UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 1 times (since Wed Apr 4 09:03:24 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Wed Apr 4 08:58:26 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Wed Apr 4 08:58:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.40.57.15 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.0.1 0.0.0.0 255.255.255.255 UH 0 0 0 tunnel0 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.85.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.30.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 10.155.33.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 192.168.10.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 172.28.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.18.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 10.155.33.67: bytes=32 time=142ms TTL=126 Reply from 10.155.33.67: bytes=32 time=139ms TTL=126 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failback - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 10.155.33.67: bytes=32 time=1ms TTL=126 Reply from 10.155.33.67: bytes=32 time=1ms TTL=126 |  |  |
| Test Result |  |  |  |

UsbModem_AlwaysConnected_9

| Case ID | UsbModem_AlwaysConnected_9 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Play video through vpn tunnel when WAN is usbmodem |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connected 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi 5) VPN tunnel for usbmodem WAN has been configured AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Check the WAN status as eth0 is the default WAN 2) Access the video on HQ server, \\10.155.33.67\share\cctv.rm 3) Try to play the video from client 4) Trigger failover to usbmodem WAN via shut eth0 5) Continue playing the video through vpn tunnel 6) Trigger failback to eth0 WAN via enable eth0 7) Continue playing the video |  |  |
| Expect result | 1) Play the video through vpn tunnel from client successfully 2) Have to replay the video after failover/ failback 3) failover/ failback without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Before failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2291 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 13701 113 0 0 0 0 113 ¦ 14101 124 0 0 124 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 1 times (since Wed Apr 4 08:58:19 2012) DOWN: 1 times (since Wed Apr 4 09:03:29 2012) Local Link State: UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 1 times (since Wed Apr 4 09:03:24 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Wed Apr 4 08:58:26 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Wed Apr 4 08:58:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.40.57.15 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.0.1 0.0.0.0 255.255.255.255 UH 0 0 0 tunnel0 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.85.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.30.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 10.155.33.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 192.168.10.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 172.28.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.18.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Have to replay the video since the WAN connection has changed - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failback - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Have to replay the video since the WAN connection has changed |  |  |
| Test Result |  |  |  |

UsbModem_AlwaysConnected_10

| Case ID | UsbModem_AlwaysConnected_10 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | FTP download file through vpn tunnel when WAN is usbmodem |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connected 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi 5) VPN tunnel for usbmodem WAN has been configured AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Check the WAN status as eth0 is the default WAN 2) Access HQ FTP server, ftp 10.155.33.67 3) Download file “hivemanager-5.0r3-beta-1-1U-patch-20120217.tar.gz” to client 4) Trigger failover to usbmodem WAN via shut eth0 5) Continue downloading through vpn tunnel 6) Trigger failback to eth0 WAN via enable eth0 7) Continue downloading the file |  |  |
| Expect result | 1) Download file through vpn tunnel from client successfully 2) FTP resume file transfer after failover/ failback 3) failover/ failback without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Before failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2291 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 13701 113 0 0 0 0 113 ¦ 14101 124 0 0 124 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 1 times (since Wed Apr 4 08:58:19 2012) DOWN: 1 times (since Wed Apr 4 09:03:29 2012) Local Link State: UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 1 times (since Wed Apr 4 09:03:24 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Wed Apr 4 08:58:26 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Wed Apr 4 08:58:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.40.57.15 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.0.1 0.0.0.0 255.255.255.255 UH 0 0 0 tunnel0 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.85.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.30.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 10.155.33.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 192.168.10.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 172.28.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.18.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue FTP process since FTP support resuming transfer - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failback - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue FTP process since FTP support resuming transfer |  |  |
| Test Result |  |  |  |

UsbModem_AlwaysConnected_11

| Case ID | UsbModem_AlwaysConnected_11 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Delta switch from always-connect to on-demand mode |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connect 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test default-gateway track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server name 10.155.33.220 capwap client vhm-name JingLi-VHM vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, while WAN is eth0 by default 2) Delta config usbmodem mode to on-demand without reboot BR 3) Continue ping process and check the WAN and usbmodem status 4) Switch usbmodem mode back to always-connect 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is eth0 by default, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=34ms TTL=44 Reply from 8.8.8.8: bytes=32 time=33ms TTL=44 2) WAN is eth0 once config usbmodem mode to on-demand, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=127ms TTL=44 Reply from 8.8.8.8: bytes=32 time=139ms TTL=44 3) WAN is eth0 after switch its mode back to always-connect, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=32ms TTL=44 Reply from 8.8.8.8: bytes=32 time=32ms TTL=44 |  |  |
| Test Result |  |  |  |

UsbModem_AlwaysConnected_12

| Case ID | UsbModem_AlwaysConnected_12 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Delta switch from always-connect to primary-wan mode |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connect 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, while WAN is eth0 by default 2) Delta config usbmodem mode to primary-wan without reboot BR 3) Continue ping process and check the WAN and usbmodem status 4) Switch usbmodem mode back to always-connect 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is eth0 by default, and ping without error 2) WAN is usbmodem once config its mode to primary-wan, and ping without error 3) WAN is eth0 after switch its mode back to always-connect, and ping without error |  |  |
| Test Result |  |  |  |

UsbModem_PrimaryWan

UsbModem_PrimaryWan_1

| Case ID | UsbModem_PrimaryWan_1 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | 3g connection status |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) eth0 WAN is available and healthy AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test default-gateway track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server name 10.155.33.220 capwap client vhm-name JingLi-VHM vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem mode primary-wan usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Check the usbmodem status while WAN is usbmodem by default 2) Trigger failover to eth0 WAN via shut usbmodem 3) Check the WAN and usbmodem status 4) Trigger failback to eth0 WAN via enable usbmodem 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is usbmodem by default, and 3g connection is "Primary" and "CONNECTED" AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE 2) WAN is eth0 once failover, and 3g connection is " Primary" and “FAILOVER” AH-8c66c0#sh usbm s USB Modem Attached: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: down UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 1 times (since Thu Mar 29 08:48:19 2012) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 1 times (since Thu Mar 29 08:48:13 2012) Active: no Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE 3) WAN is usbmodem after failback, and 3g connection is "Primary" and "CONNECTED" AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE |  |  |
| Test Result |  |  |  |

UsbModem_PrimaryWan_2

| Case ID | UsbModem_PrimaryWan_2 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Failover eth0 via unplug usbmodem |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi 5) eth0 WAN is available and healthy AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test default-gateway track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server name 10.155.33.220 capwap client vhm-name JingLi-VHM vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem mode primary-wan usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, and check the DNS of BR, while WAN is usbmodem by default 2) Trigger failover to eth0 WAN via shut usbmodem unplug usbmodem from BR (Or with CLI “no usbm en”) 3) Continue ping processand check the DNS of BR, and check the WAN and usbmodem status 4) Trigger failback to usbmodem WAN via enable usbmodem plug usbmodem back to BR (Or with CLI “usbm en”) 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is usbmodem by default, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.138.74 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=122ms TTL=45 Reply from 8.8.8.8: bytes=32 time=132ms TTL=45 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: Primary : 221.12.1.227 Secondary : 221.12.33.227 Tertiary : 0.0.0.0 2) WAN is eth0 once failover, and ping without error, and DNS changed accordingly AH-8c66c0#sh usbm s USB Modem Attached: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: down UP: 3 times (since Thu Mar 29 08:58:04 2012) DOWN: 3 times (since Thu Mar 29 08:58:48 2012) Local Link State: UP: 3 times (since Thu Mar 29 08:57:59 2012) DOWN: 3 times (since Thu Mar 29 08:58:38 2012) Active: no Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping during failover - - - - - - - - - - - - - - - - - - - - - - Reply from 192.168.85.1: Destination net unreachable. Reply from 192.168.85.1: Destination net unreachable. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping after failover - - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=33ms TTL=45 Reply from 8.8.8.8: bytes=32 time=33ms TTL=45 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: Primary : 221.12.1.227 Secondary : 221.12.33.227 Tertiary : 0.0.0.0 3) WAN is usbmodem after failback, and ping without error, and DNS changed accordingly AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.138.74 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=122ms TTL=45 Reply from 8.8.8.8: bytes=32 time=132ms TTL=45 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: Primary : 221.12.1.227 Secondary : 221.12.33.227 Tertiary : 0.0.0.0 |  |  |
| Test Result |  |  |  |

UsbModem_PrimaryWan_3

| Case ID | UsbModem_PrimaryWan_3 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Failover eth0 when 3g service is unavailable |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi 5) eth0 WAN is available and healthy AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test default-gateway track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server name 10.155.33.220 capwap client vhm-name JingLi-VHM vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem mode primary-wan usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, while WAN is usbmodem by default 2) Trigger failover to eth0 WAN when internet access through 3g is unavailable go to shield room where no 3g signal 3) Continue ping process and check the WAN and usbmodem status 4) Trigger failback to usbmodem WAN when internet access through 3g is restored 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is usbmodem by default, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.138.74 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=122ms TTL=45 Reply from 8.8.8.8: bytes=32 time=132ms TTL=45 2) WAN is eth0 once failover, and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2290 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 22948 174 0 0 0 0 174 ¦ 39731 454 0 0 454 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: down UP: 1 times (since Thu Apr 5 11:40:10 2012) DOWN: 1 times (since Thu Apr 5 11:43:20 2012) Local Link State: UP: 1 times (since Thu Apr 5 11:39:58 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Apr 5 11:40:04 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.35.254 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - debug console - - - - - - - - - - - - - - - - - - - - - - - - 2012-03-29 11:07:37 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via eth0: 0% packet loss 2012-03-29 11:07:37 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via eth0: RTT min/avg/max = 32.004002/32.004002/32.004002 2012-03-29 11:07:39 debug ah_brd: [brd_wanmon]: WFOIFM: Ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 8.8.8.8] executing 2012-03-29 11:07:40 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via eth0: 0% packet loss 2012-03-29 11:07:40 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via eth0: RTT min/avg/max = 32.841000/32.841000/32.841000 2012-03-29 11:07:42 debug ah_brd: [brd_wanmon]: WFOIFM: Interface eth0: 1 targets tested, result: PASS 2012-03-29 11:07:42 debug ah_brd: [brd_wanmon]: WFOIFM: Interface ppp0 does not exist 3) WAN is usbmodem after failback, and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 4126 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 4896 79 0 0 0 0 79 ¦ 5314 88 0 0 88 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 2 times (since Thu Mar 29 11:09:35 2012) DOWN: 3 times (since Thu Mar 29 11:06:12 2012) Local Link State: UP: 2 times (since Thu Mar 29 11:09:30 2012) DOWN: 1 times (since Thu Mar 29 11:04:30 2012) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: down UP: 4 times (since Thu Mar 29 11:00:14 2012) DOWN: 4 times (since Thu Mar 29 11:09:43 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.138.74 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=122ms TTL=45 Reply from 8.8.8.8: bytes=32 time=132ms TTL=45 |  |  |
| Test Result |  |  |  |

UsbModem_PrimaryWan_4

| Case ID | UsbModem_PrimaryWan_4 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Failover eth0 via fail to track a local endpoint |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi 5) eth0 WAN is available and healthy AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 10.155.30.72 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server port 80 routing route-request enable usbmodem mode primary-wan usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, while WAN is usbmodem by default 2) Trigger failover to eth0 WAN via track a local ip track wan_failover_test ip 10.155.30.72 3) Continue ping process and check the WAN and usbmodem status 4) Trigger failback to usbmodem WAN via track an internet ip no track wan_failover_test ip 10.155.30.72 track wan_failover_test ip 8.8.8.8 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is usbmodem by default, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.138.74 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=156ms TTL=44 Reply from 8.8.8.8: bytes=32 time=172ms TTL=44 2) WAN is eth0 once failover, and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 3807 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 1076 12 0 0 0 0 12 ¦ 8337 102 0 0 102 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Thu Mar 29 11:37:37 2012) DOWN: 2 times (since Thu Mar 29 11:38:35 2012) Local Link State: UP: 2 times (since Thu Mar 29 11:37:40 2012) DOWN: 1 times (since Thu Mar 29 11:30:16 2012) Active: no Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 11:27:08 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.33.44.195 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - debug console - - - - - - - - - - - - - - - - - - - - - - - - 2012-03-29 11:42:40 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 10.155.30.72 via eth0: 0% packet loss 2012-03-29 11:42:40 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 10.155.30.72 via eth0: RTT min/avg/max = 8.882000/8.882000/8.882000 2012-03-29 11:42:42 debug ah_brd: [brd_wanmon]: WFOIFM: Interface eth0: 1 targets tested, result: PASS 2012-03-29 11:42:48 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 10.155.30.72 via ppp0: 100% packet loss 2012-03-29 11:42:48 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 10.155.30.72 via ppp0 failed 2012-03-29 11:42:48 debug ah_brd: [brd_wanmon]: WFOIFM: Interface ppp0: 1 targets tested, result: FAIL - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=36ms TTL=44 Reply from 8.8.8.8: bytes=32 time=33ms TTL=44 3) WAN is usbmodem after failback, and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 3807 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 2140 25 0 0 0 0 25 ¦ 22095 266 0 0 266 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 3 times (since Thu Mar 29 11:48:37 2012) DOWN: 2 times (since Thu Mar 29 11:38:35 2012) Local Link State: UP: 2 times (since Thu Mar 29 11:37:40 2012) DOWN: 1 times (since Thu Mar 29 11:30:16 2012) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: down UP: 1 times (since Thu Mar 29 11:27:08 2012) DOWN: 1 times (since Thu Mar 29 11:48:45 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.33.44.195 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - debug console - - - - - - - - - - - - - - - - - - - - - - - - 2012-03-29 11:50:13 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via eth0: 100% packet loss 2012-03-29 11:50:13 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via eth0 failed 2012-03-29 11:50:13 debug ah_brd: [brd_wanmon]: WFOIFM: Interface eth0: 1 targets tested, result: FAIL 2012-03-29 11:50:17 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via ppp0: 0% packet loss 2012-03-29 11:50:17 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via ppp0: RTT min/avg/max = 180.139999/180.139999/180.139999 2012-03-29 11:50:19 debug ah_brd: [brd_wanmon]: WFOIFM: Interface ppp0: 1 targets tested, result: PASS - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=156ms TTL=44 Reply from 8.8.8.8: bytes=32 time=172ms TTL=44 |  |  |
| Test Result |  |  |  |

UsbModem_PrimaryWan_5

| Case ID | UsbModem_PrimaryWan_5 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Ping with different pkt size through usbmodem WAN |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) MTU of each peer of network (Client/ BR/ Switch) is 1518 5) Client get association with BR either by eth or wifi 6) eth0 WAN is available and healthy |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, with pkt size 64, while WAN is usbmodem by default 2) Trigger failover to eth0 WAN via shut usbmodem 3) Continue ping process and check the WAN and usbmodem status 4) Trigger failback to usbmodem WAN via enable usbmodem 5) Repeat step 3) 6) Repeat step 1)-5) with ping pkt size 1472 7) Repeat step 1)-5) with ping pkt size 1473 |  |  |
| Expect result | 1) WAN is usbmodem by default, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.138.74 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.0.1 0.0.0.0 255.255.255.255 UH 0 0 0 tunnel0 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.85.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.30.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 10.155.33.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 192.168.10.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 172.28.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.18.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 10.155.33.67 with 1473 bytes of data: Reply from 10.155.33.67: bytes=1473 time=385ms TTL=126 Reply from 10.155.33.67: bytes=1473 time=391ms TTL=126 2) WAN is eth0 once failover, and ping without error AH-8c66c0#sh usbm s USB Modem Attached: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: down UP: 3 times (since Thu Mar 29 08:58:04 2012) DOWN: 3 times (since Thu Mar 29 08:58:48 2012) Local Link State: UP: 3 times (since Thu Mar 29 08:57:59 2012) DOWN: 3 times (since Thu Mar 29 08:58:38 2012) Active: no Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - Reply from 10.155.33.67: bytes=32 time<1ms TTL=126 Reply from 10.155.33.67: bytes=32 time<1ms TTL=126 3) WAN is usbmodem after failback, and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.138.74 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.0.1 0.0.0.0 255.255.255.255 UH 0 0 0 tunnel0 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.85.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.30.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 10.155.33.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 192.168.10.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 172.28.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.18.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 10.155.33.67: bytes=1473 time=385ms TTL=126 Reply from 10.155.33.67: bytes=1473 time=391ms TTL=126 4) When ping pkt size < 1473, there is no ip fragment 5) When ping pkt size ≥ 1473, there is ip fragment |  |  |
| Test Result |  |  |  |

UsbModem_PrimaryWan_6

| Case ID | UsbModem_PrimaryWan_6 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Upgrade HiveOS for BR with scp through usbmodem WAN |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) VPN tunnel for usbmodem WAN has been configured AH-8c66c0#sh ru ssid usb interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid usb report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 1 track wan_failover_test use-for-wan-testing capwap client server port 80 vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem mode primary-wan usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Check the WAN and usbmodem status from both HM and console 2) Upgrade BR with scp CLI in console save image scp://Administrator@10.155.33.67:/tmp/ br200-HiveOS-040212044119-0848.img 3) From HM, upgrade BR through usbmodem WAN |  |  |
| Expect result | 1) WAN is usbmodem initially by default AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.138.74 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.0.1 0.0.0.0 255.255.255.255 UH 0 0 0 tunnel0 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.85.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.30.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 10.155.33.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 192.168.10.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 172.28.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.18.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 2) Upgrade through usbmodem WAN successfully AH-8c66c0#save image scp://Administrator@10.155.33.67:/tmp/br200-HiveOS-040212044119-0848.img Do you really want to update image?(Y/N)y Warning: Permanently added '10.155.33.67' (ECDSA) to the list of known hosts. Administrator@10.155.33.67's password: br200-HiveOS-040212044119-0848.img 8% 1664KB 10.1KB/s 29:08 ETA br200-HiveOS-040212044119-0848.img 97% 18MB 7.9KB/s 00:57 ETA br200-HiveOS-040212044119-0848.img 100% 19MB 9.5KB/s 34:00 The bootloader image currently in flash is the same as the image you are attempting to load. Programming DTB ... Erasing blocks: 1/1 (100%) Writing data: 8k/8k (100%) Verifying data: 8k/8k (100%) Program DTB finished. Programming kernel image ... Erasing blocks: 16/16 (100%) Writing data: 2026k/2026k (100%) Verifying data: 2026k/2026k (100%) Program kernel finished. Programming application ... Erasing blocks: 83/132 (62%) Erasing blocks: 132/132 (100%) Writing data: 16784k/16784k (100%) Verifying data: 16784k/16784k (100%) Program application finished. 3) From HM, Upgrade through usbmodem WAN successfully |  |  |
| Test Result |  |  |  |

UsbModem_PrimaryWan_7

| Case ID | UsbModem_PrimaryWan_7 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Http video through usbmodem WAN |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi 5) eth0 WAN is available and healthy |  |  |
| Test procedure | 1) Check the WAN status as usbmodem is the default WAN 2) Try to play http video from client, http://www.letv.com/ptv/vplay/1257936.html 3) Trigger failover to eth0 WAN via shut usbmodem 4) Continue playing the http video 5) Trigger failback to usbmodem WAN via enable usbmodem 6) Repeat step 4) |  |  |
| Expect result | 1) Play http video successfully before/ after faiover/ failback 2) failover/ failback without error 3) Have to replay the video after failover/ failback |  |  |
| Test Result |  |  |  |

UsbModem_PrimaryWan_8

| Case ID | UsbModem_PrimaryWan_8 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Http download through usbmodem WAN |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi 5) eth0 WAN is available and healthy |  |  |
| Test procedure | 1) Check the WAN status as usbmodem is the default WAN 2) Try to download file “QQ2012Beta1_QQProtect2.5.exe” from client http://im.qq.com/qq/2012/safe/ 3) Trigger failover to eth0 WAN via shut usbmodem 4) Continue downloading the file 5) Trigger failback to usbmodem WAN via enable usbmodem 6) Repeat step 4) |  |  |
| Expect result | 1) Http download successfully before/ after faiover/ failback 2) failover/ failback without error 3) Have to re-download the file after failover/ failback |  |  |
| Test Result |  |  |  |

UsbModem_PrimaryWan_9

| Case ID | UsbModem_PrimaryWan_9 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | VPN tunnel accessibility |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi 5) VPN tunnel for usbmodem WAN has been configured 6) eth0 WAN is available and healthy AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server name 10.155.33.220 capwap client vhm-name JingLi-VHM vpn client-ipsec-tunnel 0019778C66C0_client_ipsec_1 vpn-mode layer-3 vpn ipsec-tunnel 0019778C66C0_client_ipsec_1 gateway 115.236.187.186 client-name 7FJRFnPmu9LA894kZB7GSKzJGWZtNLwJ password *** vpn tunnel-policy vpn_tunnel_policy client ipsec-tunnel 0019778C66C0_client_ipsec_1 primary routing route-request enable routing internal-sub-network 192.168.85.0/24 usbmodem mode primary-wan usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Ping another VPN endpoint through tunnel 2) Trigger failover to eth0 WAN via shut usbmodem, continue ping process 3) Continue ping process and check the WAN and usbmodem status from both HM and console 4) Trigger failback to usbmodem WAN via enable usbmodem, continue ping process 5) Repeat step 3) |  |  |
| Expect result | 1) Ping through VPN tunnel successfully with failover and failback 2) failover/ failback without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Before failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 23269 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 7108072 47033 0 0 0 0 47033 ¦ 7822641 53851 0 0 53851 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 41 times (since Thu Mar 29 02:05:21 2012) DOWN: 48 times (since Thu Mar 29 02:05:07 2012) Local Link State: UP: 14 times (since Wed Mar 28 12:13:32 2012) DOWN: 13 times (since Wed Mar 28 12:12:29 2012) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: down UP: 37 times (since Thu Mar 29 02:05:15 2012) DOWN: 38 times (since Thu Mar 29 02:05:29 2012) Local Link State: UP: 4 times (since Wed Mar 28 11:06:16 2012) DOWN: 3 times (since Wed Mar 28 11:04:57 2012) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 172.27.33.130 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.0.1 0.0.0.0 255.255.255.255 UH 0 0 0 tunnel0 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.85.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.30.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 10.155.33.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 192.168.10.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 172.28.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.18.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh vpn ipsec-tunnel IPsec Tunnel Duration: Source Destination Created Duration ------------------------ ------------------------ -------------------- ---------------------------------------- 172.27.33.130[4500] 115.236.187.186[4500] 2012-03-29 02:05:54 0 days 0 hours 10 minutes 30 seconds Total IPsec Tunnel Sessions: 1 Tunnel Statistic Information:: Src IP Dst IP Pkts Bytes Auth-Err Other-Err SPI Remaining-Lifetime ------------------------ ------------------------ ---------- ---------- ---------- ---------- ---------- ------------------ 172.27.33.130[4500] 115.236.187.186[4500] 211 28399 0 0 0x09faf4d3 2251(s) rekey 115.236.187.186[4500] 172.27.33.130[4500] 181 25244 0 0 0x0b3011e8 2251(s) rekey - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 10.155.33.67 with 32 bytes of data: Reply from 10.155.33.67: bytes=32 time=334ms TTL=126 Reply from 10.155.33.67: bytes=32 time=335ms TTL=126 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: down UP: 41 times (since Thu Mar 29 02:05:21 2012) DOWN: 50 times (since Thu Mar 29 02:18:10 2012) Local Link State: UP: 14 times (since Wed Mar 28 12:13:32 2012) DOWN: 14 times (since Thu Mar 29 02:18:01 2012) Active: no Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 38 times (since Thu Mar 29 02:18:10 2012) DOWN: 38 times (since Thu Mar 29 02:05:29 2012) Local Link State: UP: 4 times (since Wed Mar 28 11:06:16 2012) DOWN: 3 times (since Wed Mar 28 11:04:57 2012) Active: yes Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 10.155.33.67: bytes=32 time=3ms TTL=126 Reply from 10.155.33.67: bytes=32 time=1ms TTL=126 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failback - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 23269 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 7108072 47033 0 0 0 0 47033 ¦ 7822641 53851 0 0 53851 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 41 times (since Thu Mar 29 02:05:21 2012) DOWN: 48 times (since Thu Mar 29 02:05:07 2012) Local Link State: UP: 14 times (since Wed Mar 28 12:13:32 2012) DOWN: 13 times (since Wed Mar 28 12:12:29 2012) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: down UP: 37 times (since Thu Mar 29 02:05:15 2012) DOWN: 38 times (since Thu Mar 29 02:05:29 2012) Local Link State: UP: 4 times (since Wed Mar 28 11:06:16 2012) DOWN: 3 times (since Wed Mar 28 11:04:57 2012) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 172.27.33.130 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.0.1 0.0.0.0 255.255.255.255 UH 0 0 0 tunnel0 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.85.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.30.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 10.155.33.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 192.168.10.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 172.28.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.18.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh vpn ipsec-tunnel IPsec Tunnel Duration: Source Destination Created Duration ------------------------ ------------------------ -------------------- ---------------------------------------- 172.27.33.130[4500] 115.236.187.186[4500] 2012-03-29 02:05:54 0 days 0 hours 10 minutes 30 seconds Total IPsec Tunnel Sessions: 1 Tunnel Statistic Information:: Src IP Dst IP Pkts Bytes Auth-Err Other-Err SPI Remaining-Lifetime ------------------------ ------------------------ ---------- ---------- ---------- ---------- ---------- ------------------ 172.27.33.130[4500] 115.236.187.186[4500] 211 28399 0 0 0x09faf4d3 2251(s) rekey 115.236.187.186[4500] 172.27.33.130[4500] 181 25244 0 0 0x0b3011e8 2251(s) rekey - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 10.155.33.67: bytes=32 time=334ms TTL=126 Reply from 10.155.33.67: bytes=32 time=335ms TTL=126 |  |  |
| Test Result |  |  |  |

UsbModem_PrimaryWan_10

| Case ID | UsbModem_PrimaryWan_10 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Play video through vpn tunnel when WAN is usbmodem |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi 5) VPN tunnel for usbmodem WAN has been configured 6) eth0 WAN is available and healthy |  |  |
| Test procedure | 1) Check the WAN status as usbmodem is the default WAN 2) Access the video on remote HQ server through vpn tunnel \\10.155.33.67\share\cctv.rm 3) Try to play the video from client 4) Trigger failover to eth0 WAN via shut usbmodem 5) Continue playing the video 6) Trigger failback to usbmodem WAN via enable usbmodem 7) Repeat step 5) |  |  |
| Expect result | 1) Play the video through vpn tunnel from client successfully 2) Have to replay the video after failover/ failback 3) failover/ failback without error |  |  |
| Test Result |  |  |  |

UsbModem_PrimaryWan_11

| Case ID | UsbModem_PrimaryWan_11 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | FTP download file through vpn tunnel when WAN is usbmodem |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi 5) VPN tunnel for usbmodem WAN has been configured 6) eth0 WAN is available and healthy |  |  |
| Test procedure | 1) Check the WAN status as usbmodem is the default WAN 2) Access remote HQ FTP server through vpn tunnel ftp 10.155.33.67 3) Download file “hivemanager-5.0r3-beta-1-1U-patch-20120217.tar.gz” to client 4) Trigger failover to eth0 WAN via shut usbmodem 5) Continue downloading 6) Trigger failback to usbmodem WAN via enable usbmodem 7) Repeat step 5) |  |  |
| Expect result | 1) Download file through vpn tunnel from client successfully 2) FTP resume file transfer after failover/ failback 3) failover/ failback without error |  |  |
| Test Result |  |  |  |

UsbModem_PrimaryWan_12

| Case ID | UsbModem_PrimaryWan_12 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Delta switch from primary-wan to on-demand mode |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, while WAN is usbmodem by default 2) Delta config usbmodem mode to on-demand without reboot BR 3) Continue ping process and check the WAN and usbmodem status 4) Switch usbmodem mode back to primary-wan 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is usbmodem by default, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.138.74 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 2) WAN is eth0 once config usbmodem mode to on-demand, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 3) WAN is usbmodem after switch its mode back to primary-wan, and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.138.74 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 |  |  |
| Test Result |  |  |  |

UsbModem_PrimaryWan_13

| Case ID | UsbModem_PrimaryWan_13 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Delta switch from primary-wan to always-connect mode |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Client get association with BR either by eth or wifi |  |  |
| Test procedure | 1) From client, ping an external endpoint that is outside of the BR's vlan, while WAN is usbmodem by default 2) Delta config usbmodem mode to always-connect without reboot BR 3) Continue ping process and check the WAN and usbmodem status 4) Switch usbmodem mode back to primary-wan 5) Repeat step 3) |  |  |
| Expect result | 1) WAN is usbmodem by default, and ping without error 2) WAN is eth0 once config usbmodem mode to always-connect, and ping without error 3) WAN is usbmodem after switch its mode back to primary-wan, and ping without error |  |  |
| Test Result |  |  |  |

UsbModem_Eth0IsPPPoE

UsbModem_Eth0IsPPPoE_1

| Case ID | UsbModem_Eth0IsPPPoE_1 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Check the ppp state for eth0 and usbmodem, on-demand mode |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) eth0 WAN is working as PPPoE 3) The 3g service is activated and available 4) Configured the dialup username/ passwd/ number if the modem if necessary 5) Ping through eth0 WAN works well AH-218cc0#sh ru pppoe interface eth0 pppoe username wcai password *** interface eth0 pppoe enable |  |  |
| Test procedure | 1) Trigger failover to usbmodem WAN via unplug eth0 2) Continue ping process and check the WAN and usbmodem status 3) Trigger failback to eth0 WAN via plug eth0 back 4) Repeat step 3) |  |  |
| Expect result | 1) WAN is eth0 by default, and ping without error AH-218cc0#sh usbm modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh in e0 pp PPPoE status: enabled Username: wcai Password: *** Authentication: ANY Connection status: connected Local IP: 10.5.16.23 Remote IP: 10.5.16.1 DNS server: 10.155.3.250 DNS server: 10.155.32.190 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x141b USB Network Interface Status: NONE PPP Network Interface Status: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: ppp1 WAN Monitor State: up UP: 2 times (since Fri Jun 8 20:05:45 2012) DOWN: 1 times (since Fri Jun 8 20:02:17 2012) Local Link State: UP: 2 times (since Fri Jun 8 20:04:49 2012) DOWN: 1 times (since Fri Jun 8 20:02:03 2012) Active: yes Cached Gateway IP: 10.5.16.1 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Fri Jun 8 20:03:20 2012) DOWN: 3 times (since Fri Jun 8 20:05:47 2012) Local Link State: UP: 2 times (since Fri Jun 8 20:02:29 2012) DOWN: 2 times (since Fri Jun 8 20:05:45 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0# sAH-218cc0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 0.0.0.0 wan - 08ea:4421:8cc0 U Mgt0 172.18.216.1 - 4094 08ea:4421:8cc0 U Mgt0.3 172.29.255.1 - 10 08ea:4421:8cc0 U Mgt0.8 172.28.253.129 - 1 08ea:4421:8cc0 U Mgt0.12 172.30.148.1 - 20 08ea:4421:8cc0 U Ppp1 10.5.16.21 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.5.16.1 0.0.0.0 255.255.255.255 UH 0 0 0 ppp1 172.18.216.0 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 172.28.253.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0.8 172.30.148.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.12 172.29.255.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.3 1.2.5.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 1.1.1.0 0.0.0.0 255.255.255.0 U 0 0 0 wifi0.1 0.0.0.0 10.5.16.1 0.0.0.0 UG 0 0 0 ppp1 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=94ms TTL=45 Reply from 8.8.8.8: bytes=32 time=37ms TTL=45 Reply from 8.8.8.8: bytes=32 time=48ms TTL=45 Reply from 8.8.8.8: bytes=32 time=51ms TTL=45 Ping statistics for 8.8.8.8: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 37ms, Maximum = 94ms, Average = 57ms 2) WAN is usbmodem once failover, and ping without error AH-218cc0#sh in e0 pp PPPoE status: enabled Username: wcai Password: *** Authentication: ANY Connection status: unconnect Local IP: 0.0.0.0 Remote IP: 0.0.0.0 DNS server: 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x141b USB Network Interface Status: NONE PPP Network Interface Status: ppp0 Link encap:Point-to-Point Protocol inet addr:10.31.94.103 P-t-P:10.64.64.64 Mask:255.255.255.255 UP POINTOPOINT RUNNING NOARP MULTICAST MTU:1500 Metric:1 RX packets:70 errors:0 dropped:0 overruns:0 frame:0 TX packets:91 errors:0 dropped:0 overruns:0 carrier:0 collisions:0 txqueuelen:3 RX bytes:12964 (12.6 KiB) TX bytes:11875 (11.5 KiB) PPP Process Status: PID: 4489 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 12964 70 0 0 0 0 70 ¦ 11875 91 0 0 91 0 0 WAN Interface(s) Status: [Primary] Interface: ppp1 WAN Monitor State: down UP: 1 times (since Fri Jun 8 20:01:41 2012) DOWN: 1 times (since Fri Jun 8 20:02:17 2012) Local Link State: UP: 1 times (since Fri Jun 8 20:00:35 2012) DOWN: 1 times (since Fri Jun 8 20:02:03 2012) Active: no Cached Gateway IP: 10.5.16.1 [Backup] Interface: ppp0 WAN Monitor State: up UP: 2 times (since Fri Jun 8 20:03:20 2012) DOWN: 2 times (since Fri Jun 8 20:01:44 2012) Local Link State: UP: 2 times (since Fri Jun 8 20:02:29 2012) DOWN: 1 times (since Fri Jun 8 20:01:42 2012) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 0.0.0.0 wan - 08ea:4421:8cc0 D Mgt0 172.18.216.1 - 4094 08ea:4421:8cc0 U Mgt0.3 172.29.255.1 - 10 08ea:4421:8cc0 U Mgt0.8 172.28.253.129 - 1 08ea:4421:8cc0 U Mgt0.12 172.30.148.1 - 20 08ea:4421:8cc0 U Ppp0 10.31.94.103 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 172.18.217.129 0.0.0.0 255.255.255.255 UH 0 0 0 tunnel0 172.18.216.0 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 172.28.253.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0.8 172.30.148.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.12 172.29.255.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.3 1.2.5.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 172.16.60.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 10.155.33.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 1.1.1.0 0.0.0.0 255.255.255.0 U 0 0 0 wifi0.1 172.30.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.28.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.29.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.18.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=94ms TTL=45 Reply from 8.8.8.8: bytes=32 time=37ms TTL=45 Reply from 8.8.8.8: bytes=32 time=48ms TTL=45 Reply from 8.8.8.8: bytes=32 time=51ms TTL=45 Ping statistics for 8.8.8.8: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 37ms, Maximum = 94ms, Average = 57ms 3) WAN is eth0 after failback, and ping without error AH-218cc0#sh in e0 pp PPPoE status: enabled Username: wcai Password: *** Authentication: ANY Connection status: connected Local IP: 10.5.16.23 Remote IP: 10.5.16.1 DNS server: 10.155.3.250 DNS server: 10.155.32.190 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x141b USB Network Interface Status: NONE PPP Network Interface Status: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: ppp1 WAN Monitor State: up UP: 2 times (since Fri Jun 8 20:05:45 2012) DOWN: 1 times (since Fri Jun 8 20:02:17 2012) Local Link State: UP: 2 times (since Fri Jun 8 20:04:49 2012) DOWN: 1 times (since Fri Jun 8 20:02:03 2012) Active: yes Cached Gateway IP: 10.5.16.1 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Fri Jun 8 20:03:20 2012) DOWN: 3 times (since Fri Jun 8 20:05:47 2012) Local Link State: UP: 2 times (since Fri Jun 8 20:02:29 2012) DOWN: 2 times (since Fri Jun 8 20:05:45 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0# sAH-218cc0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 0.0.0.0 wan - 08ea:4421:8cc0 U Mgt0 172.18.216.1 - 4094 08ea:4421:8cc0 U Mgt0.3 172.29.255.1 - 10 08ea:4421:8cc0 U Mgt0.8 172.28.253.129 - 1 08ea:4421:8cc0 U Mgt0.12 172.30.148.1 - 20 08ea:4421:8cc0 U Ppp1 10.5.16.21 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.5.16.1 0.0.0.0 255.255.255.255 UH 0 0 0 ppp1 172.18.216.0 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 172.28.253.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0.8 172.30.148.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.12 172.29.255.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.3 1.2.5.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 1.1.1.0 0.0.0.0 255.255.255.0 U 0 0 0 wifi0.1 0.0.0.0 10.5.16.1 0.0.0.0 UG 0 0 0 ppp1 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=94ms TTL=45 Reply from 8.8.8.8: bytes=32 time=37ms TTL=45 Reply from 8.8.8.8: bytes=32 time=48ms TTL=45 Reply from 8.8.8.8: bytes=32 time=51ms TTL=45 Ping statistics for 8.8.8.8: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 37ms, Maximum = 94ms, Average = 57ms |  |  |
| Test Result |  |  |  |

UsbModem_Eth0IsPPPoE_2

| Case ID | UsbModem_Eth0IsPPPoE_2 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Check the ppp state for eth0 and usbmodem, always-connected mode |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connected 2) eth0 WAN is working as PPPoE 3) The 3g service is activated and available 4) Configured the dialup username/ passwd/ number if the modem if necessary 5) Ping through eth0 WAN works well AH-218cc0#sh ru pppoe interface eth0 pppoe username wcai password *** interface eth0 pppoe enable |  |  |
| Test procedure | 1) Trigger failover to usbmodem WAN via unplug eth0 2) Continue ping process and check the WAN and usbmodem status 3) Trigger failback to eth0 WAN via plug eth0 back 4) Repeat step 3) |  |  |
| Expect result | 1) WAN is eth0 by default, and ping without error AH-218cc0#sh usbm modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh in e0 pp PPPoE status: enabled Username: wcai Password: *** Authentication: ANY Connection status: connected Local IP: 10.5.16.29 Remote IP: 10.5.16.1 DNS server: 10.155.3.250 DNS server: 10.155.32.190 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x141b USB Network Interface Status: NONE PPP Network Interface Status: ppp0 Link encap:Point-to-Point Protocol inet addr:10.40.102.3 P-t-P:10.64.64.64 Mask:255.255.255.255 UP POINTOPOINT RUNNING NOARP MULTICAST MTU:1500 Metric:1 RX packets:9 errors:0 dropped:0 overruns:0 frame:0 TX packets:10 errors:0 dropped:0 overruns:0 carrier:0 collisions:0 txqueuelen:3 RX bytes:336 (336.0 B) TX bytes:366 (366.0 B) PPP Process Status: PID: 3051 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 336 9 0 0 0 0 9 ¦ 366 10 0 0 10 0 0 WAN Interface(s) Status: [Primary] Interface: ppp1 WAN Monitor State: up UP: 1 times (since Fri Jun 8 20:35:30 2012) DOWN: 0 times (since Thu Jan 1 00:00:55 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:01:01 1970) DOWN: 0 times (since Thu Jan 1 00:00:55 1970) Active: yes Cached Gateway IP: 10.5.16.1 [Backup] Interface: ppp0 WAN Monitor State: up UP: 0 times (since Thu Jan 1 00:00:31 1970) DOWN: 1 times (since Fri Jun 8 20:35:30 2012) Local Link State: UP: 1 times (since Fri Jun 8 20:36:15 2012) DOWN: 0 times (since Thu Jan 1 00:00:31 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 0.0.0.0 wan - 08ea:4421:8cc0 U Mgt0 172.18.216.1 - 4094 08ea:4421:8cc0 U Mgt0.3 172.29.255.1 - 10 08ea:4421:8cc0 U Mgt0.8 172.28.253.129 - 1 08ea:4421:8cc0 U Mgt0.12 172.30.148.1 - 20 08ea:4421:8cc0 U Ppp1 10.5.16.29 wan - 0000:0000:0000 U Ppp0 10.40.102.3 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.5.16.1 0.0.0.0 255.255.255.255 UH 0 0 0 ppp1 172.18.216.0 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 172.28.253.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0.8 172.30.148.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.12 172.29.255.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.3 1.2.5.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 1.1.1.0 0.0.0.0 255.255.255.0 U 0 0 0 wifi0.1 0.0.0.0 10.5.16.1 0.0.0.0 UG 0 0 0 ppp1 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=94ms TTL=45 Reply from 8.8.8.8: bytes=32 time=37ms TTL=45 Reply from 8.8.8.8: bytes=32 time=48ms TTL=45 Reply from 8.8.8.8: bytes=32 time=51ms TTL=45 Ping statistics for 8.8.8.8: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 37ms, Maximum = 94ms, Average = 57ms 2) WAN is usbmodem once failover, and ping without error AH-218cc0#sh in e0 pp PPPoE status: enabled Username: wcai Password: *** Authentication: ANY Connection status: unconnect Local IP: 0.0.0.0 Remote IP: 0.0.0.0 DNS server: 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x141b USB Network Interface Status: NONE PPP Network Interface Status: ppp0 Link encap:Point-to-Point Protocol inet addr:10.40.102.3 P-t-P:10.64.64.64 Mask:255.255.255.255 UP POINTOPOINT RUNNING NOARP MULTICAST MTU:1500 Metric:1 RX packets:112 errors:0 dropped:0 overruns:0 frame:0 TX packets:131 errors:0 dropped:0 overruns:0 carrier:0 collisions:0 txqueuelen:3 RX bytes:13852 (13.5 KiB) TX bytes:16169 (15.7 KiB) PPP Process Status: PID: 3051 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 13852 112 0 0 0 0 112 ¦ 16169 131 0 0 131 0 0 WAN Interface(s) Status: [Primary] Interface: ppp1 WAN Monitor State: down UP: 1 times (since Fri Jun 8 20:35:30 2012) DOWN: 1 times (since Fri Jun 8 20:43:06 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:01:01 1970) DOWN: 1 times (since Fri Jun 8 20:42:30 2012) Active: no Cached Gateway IP: 10.5.16.1 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Fri Jun 8 20:37:19 2012) DOWN: 1 times (since Fri Jun 8 20:35:30 2012) Local Link State: UP: 1 times (since Fri Jun 8 20:36:15 2012) DOWN: 0 times (since Thu Jan 1 00:00:31 1970) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 0.0.0.0 wan - 08ea:4421:8cc0 D Mgt0 172.18.216.1 - 4094 08ea:4421:8cc0 U Mgt0.3 172.29.255.1 - 10 08ea:4421:8cc0 U Mgt0.8 172.28.253.129 - 1 08ea:4421:8cc0 U Mgt0.12 172.30.148.1 - 20 08ea:4421:8cc0 U Ppp0 10.40.102.3 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 172.18.217.129 0.0.0.0 255.255.255.255 UH 0 0 0 tunnel0 172.18.216.0 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 172.28.253.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0.8 172.30.148.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.12 172.29.255.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.3 1.2.5.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 172.16.60.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 10.155.33.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 1.1.1.0 0.0.0.0 255.255.255.0 U 0 0 0 wifi0.1 172.30.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.28.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.29.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.18.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=94ms TTL=45 Reply from 8.8.8.8: bytes=32 time=37ms TTL=45 Reply from 8.8.8.8: bytes=32 time=48ms TTL=45 Reply from 8.8.8.8: bytes=32 time=51ms TTL=45 Ping statistics for 8.8.8.8: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 37ms, Maximum = 94ms, Average = 57ms 3) WAN is eth0 after failback, and ping without error AH-218cc0#sh in e0 pp PPPoE status: enabled Username: wcai Password: *** Authentication: ANY Connection status: connected Local IP: 10.5.16.29 Remote IP: 10.5.16.1 DNS server: 10.155.3.250 DNS server: 10.155.32.190 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x141b USB Network Interface Status: NONE PPP Network Interface Status: ppp0 Link encap:Point-to-Point Protocol inet addr:10.40.102.3 P-t-P:10.64.64.64 Mask:255.255.255.255 UP POINTOPOINT RUNNING NOARP MULTICAST MTU:1500 Metric:1 RX packets:9 errors:0 dropped:0 overruns:0 frame:0 TX packets:10 errors:0 dropped:0 overruns:0 carrier:0 collisions:0 txqueuelen:3 RX bytes:336 (336.0 B) TX bytes:366 (366.0 B) PPP Process Status: PID: 3051 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 336 9 0 0 0 0 9 ¦ 366 10 0 0 10 0 0 WAN Interface(s) Status: [Primary] Interface: ppp1 WAN Monitor State: up UP: 1 times (since Fri Jun 8 20:35:30 2012) DOWN: 0 times (since Thu Jan 1 00:00:55 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:01:01 1970) DOWN: 0 times (since Thu Jan 1 00:00:55 1970) Active: yes Cached Gateway IP: 10.5.16.1 [Backup] Interface: ppp0 WAN Monitor State: up UP: 0 times (since Thu Jan 1 00:00:31 1970) DOWN: 1 times (since Fri Jun 8 20:35:30 2012) Local Link State: UP: 1 times (since Fri Jun 8 20:36:15 2012) DOWN: 0 times (since Thu Jan 1 00:00:31 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 0.0.0.0 wan - 08ea:4421:8cc0 U Mgt0 172.18.216.1 - 4094 08ea:4421:8cc0 U Mgt0.3 172.29.255.1 - 10 08ea:4421:8cc0 U Mgt0.8 172.28.253.129 - 1 08ea:4421:8cc0 U Mgt0.12 172.30.148.1 - 20 08ea:4421:8cc0 U Ppp1 10.5.16.29 wan - 0000:0000:0000 U Ppp0 10.40.102.3 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.5.16.1 0.0.0.0 255.255.255.255 UH 0 0 0 ppp1 172.18.216.0 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 172.28.253.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0.8 172.30.148.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.12 172.29.255.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.3 1.2.5.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 1.1.1.0 0.0.0.0 255.255.255.0 U 0 0 0 wifi0.1 0.0.0.0 10.5.16.1 0.0.0.0 UG 0 0 0 ppp1 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=94ms TTL=45 Reply from 8.8.8.8: bytes=32 time=37ms TTL=45 Reply from 8.8.8.8: bytes=32 time=48ms TTL=45 Reply from 8.8.8.8: bytes=32 time=51ms TTL=45 Ping statistics for 8.8.8.8: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 37ms, Maximum = 94ms, Average = 57ms |  |  |
| Test Result |  |  |  |

UsbModem_Eth0IsPPPoE_3

| Case ID | UsbModem_Eth0IsPPPoE_3 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Check the ppp state for eth0 and usbmodem, primary-wan mode |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) eth0 WAN is working as PPPoE 3) The 3g service is activated and available 4) Configured the dialup username/ passwd/ number if the modem if necessary 5) Ping through usbmodem WAN works well AH-218cc0#sh ru pppoe interface eth0 pppoe username wcai password *** interface eth0 pppoe enable |  |  |
| Test procedure | 1) Trigger failover to eth0 WAN via unplug usbmodem 2) Continue ping process and check the WAN and usbmodem status 3) Trigger failback to usbmodem WAN via plug usbmodem back 4) Repeat step 3) |  |  |
| Expect result | 1) WAN is usbmodem by default, and ping without error AH-218cc0#sh usbm modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh in e0 pp PPPoE status: enabled Username: wcai Password: *** Authentication: ANY Connection status: connected Local IP: 10.5.16.30 Remote IP: 10.5.16.1 DNS server: 10.155.3.250 DNS server: 10.155.32.190 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x141b USB Network Interface Status: NONE PPP Network Interface Status: ppp0 Link encap:Point-to-Point Protocol inet addr:10.24.42.76 P-t-P:10.64.64.64 Mask:255.255.255.255 UP POINTOPOINT RUNNING NOARP MULTICAST MTU:1500 Metric:1 RX packets:68 errors:0 dropped:0 overruns:0 frame:0 TX packets:75 errors:0 dropped:0 overruns:0 carrier:0 collisions:0 txqueuelen:3 RX bytes:11027 (10.7 KiB) TX bytes:11369 (11.1 KiB) PPP Process Status: PID: 5213 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 11027 68 0 0 0 0 68 ¦ 11369 75 0 0 75 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Fri Jun 8 20:37:19 2012) DOWN: 1 times (since Fri Jun 8 20:35:30 2012) Local Link State: UP: 2 times (since Fri Jun 8 20:49:39 2012) DOWN: 1 times (since Fri Jun 8 20:49:24 2012) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: ppp1 WAN Monitor State: up UP: 2 times (since Fri Jun 8 20:47:53 2012) DOWN: 1 times (since Fri Jun 8 20:43:06 2012) Local Link State: UP: 2 times (since Fri Jun 8 20:47:15 2012) DOWN: 1 times (since Fri Jun 8 20:42:30 2012) Active: no Cached Gateway IP: 10.5.16.1 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 0.0.0.0 wan - 08ea:4421:8cc0 U Mgt0 172.18.216.1 - 4094 08ea:4421:8cc0 U Mgt0.3 172.29.255.1 - 10 08ea:4421:8cc0 U Mgt0.8 172.28.253.129 - 1 08ea:4421:8cc0 U Mgt0.12 172.30.148.1 - 20 08ea:4421:8cc0 U Ppp1 10.5.16.30 wan - 0000:0000:0000 U Ppp0 10.24.42.76 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.5.16.1 0.0.0.0 255.255.255.255 UH 0 0 0 ppp1 172.18.217.129 0.0.0.0 255.255.255.255 UH 0 0 0 tunnel0 172.18.216.0 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 172.28.253.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0.8 172.30.148.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.12 172.29.255.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.3 1.2.5.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 172.16.60.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 10.155.33.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 1.1.1.0 0.0.0.0 255.255.255.0 U 0 0 0 wifi0.1 172.30.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.28.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.29.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.18.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 0.0.0.0 10.5.16.1 0.0.0.0 UG 32766 0 0 ppp1 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=94ms TTL=45 Reply from 8.8.8.8: bytes=32 time=37ms TTL=45 Reply from 8.8.8.8: bytes=32 time=48ms TTL=45 Reply from 8.8.8.8: bytes=32 time=51ms TTL=45 Ping statistics for 8.8.8.8: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 37ms, Maximum = 94ms, Average = 57ms 2) WAN is eth0 once failover, and ping without error AH-218cc0#sh in e0 pp PPPoE status: enabled Username: wcai Password: *** Authentication: ANY Connection status: connected Local IP: 10.5.16.30 Remote IP: 10.5.16.1 DNS server: 10.155.3.250 DNS server: 10.155.32.190 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x141b USB Network Interface Status: NONE PPP Network Interface Status: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: down UP: 1 times (since Fri Jun 8 20:37:19 2012) DOWN: 2 times (since Fri Jun 8 20:54:17 2012) Local Link State: UP: 2 times (since Fri Jun 8 20:49:39 2012) DOWN: 2 times (since Fri Jun 8 20:53:38 2012) Active: no Cached Gateway IP: 0.0.0.0 [Backup] Interface: ppp1 WAN Monitor State: up UP: 2 times (since Fri Jun 8 20:47:53 2012) DOWN: 1 times (since Fri Jun 8 20:43:06 2012) Local Link State: UP: 2 times (since Fri Jun 8 20:47:15 2012) DOWN: 1 times (since Fri Jun 8 20:42:30 2012) Active: yes Cached Gateway IP: 10.5.16.1 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 0.0.0.0 wan - 08ea:4421:8cc0 U Mgt0 172.18.216.1 - 4094 08ea:4421:8cc0 U Mgt0.3 172.29.255.1 - 10 08ea:4421:8cc0 U Mgt0.8 172.28.253.129 - 1 08ea:4421:8cc0 U Mgt0.12 172.30.148.1 - 20 08ea:4421:8cc0 U Ppp1 10.5.16.30 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.5.16.1 0.0.0.0 255.255.255.255 UH 0 0 0 ppp1 172.18.216.0 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 172.28.253.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0.8 172.30.148.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.12 172.29.255.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.3 1.2.5.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 1.1.1.0 0.0.0.0 255.255.255.0 U 0 0 0 wifi0.1 0.0.0.0 10.5.16.1 0.0.0.0 UG 0 0 0 ppp1 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=94ms TTL=45 Reply from 8.8.8.8: bytes=32 time=37ms TTL=45 Reply from 8.8.8.8: bytes=32 time=48ms TTL=45 Reply from 8.8.8.8: bytes=32 time=51ms TTL=45 Ping statistics for 8.8.8.8: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 37ms, Maximum = 94ms, Average = 57ms 3) WAN is usbmodem after failback, and ping without error AH-218cc0#sh in e0 pp PPPoE status: enabled Username: wcai Password: *** Authentication: ANY Connection status: connected Local IP: 10.5.16.30 Remote IP: 10.5.16.1 DNS server: 10.155.3.250 DNS server: 10.155.32.190 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x141b USB Network Interface Status: NONE PPP Network Interface Status: ppp0 Link encap:Point-to-Point Protocol inet addr:10.24.42.76 P-t-P:10.64.64.64 Mask:255.255.255.255 UP POINTOPOINT RUNNING NOARP MULTICAST MTU:1500 Metric:1 RX packets:68 errors:0 dropped:0 overruns:0 frame:0 TX packets:75 errors:0 dropped:0 overruns:0 carrier:0 collisions:0 txqueuelen:3 RX bytes:11027 (10.7 KiB) TX bytes:11369 (11.1 KiB) PPP Process Status: PID: 5213 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 11027 68 0 0 0 0 68 ¦ 11369 75 0 0 75 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Fri Jun 8 20:37:19 2012) DOWN: 1 times (since Fri Jun 8 20:35:30 2012) Local Link State: UP: 2 times (since Fri Jun 8 20:49:39 2012) DOWN: 1 times (since Fri Jun 8 20:49:24 2012) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: ppp1 WAN Monitor State: up UP: 2 times (since Fri Jun 8 20:47:53 2012) DOWN: 1 times (since Fri Jun 8 20:43:06 2012) Local Link State: UP: 2 times (since Fri Jun 8 20:47:15 2012) DOWN: 1 times (since Fri Jun 8 20:42:30 2012) Active: no Cached Gateway IP: 10.5.16.1 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 0.0.0.0 wan - 08ea:4421:8cc0 U Mgt0 172.18.216.1 - 4094 08ea:4421:8cc0 U Mgt0.3 172.29.255.1 - 10 08ea:4421:8cc0 U Mgt0.8 172.28.253.129 - 1 08ea:4421:8cc0 U Mgt0.12 172.30.148.1 - 20 08ea:4421:8cc0 U Ppp1 10.5.16.30 wan - 0000:0000:0000 U Ppp0 10.24.42.76 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-218cc0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.5.16.1 0.0.0.0 255.255.255.255 UH 0 0 0 ppp1 172.18.217.129 0.0.0.0 255.255.255.255 UH 0 0 0 tunnel0 172.18.216.0 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 172.28.253.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0.8 172.30.148.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.12 172.29.255.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0.3 1.2.5.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 172.16.60.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 10.155.33.0 0.0.0.0 255.255.255.0 U 1 0 0 tunnel0 1.1.1.0 0.0.0.0 255.255.255.0 U 0 0 0 wifi0.1 172.30.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.28.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.29.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 172.18.0.0 0.0.0.0 255.255.0.0 U 1 0 0 tunnel0 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 0.0.0.0 10.5.16.1 0.0.0.0 UG 32766 0 0 ppp1 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=94ms TTL=45 Reply from 8.8.8.8: bytes=32 time=37ms TTL=45 Reply from 8.8.8.8: bytes=32 time=48ms TTL=45 Reply from 8.8.8.8: bytes=32 time=51ms TTL=45 Ping statistics for 8.8.8.8: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 37ms, Maximum = 94ms, Average = 57ms |  |  |
| Test Result |  |  |  |

UsbModem_ErrorReport

UsbModem_ErrorReport_1

| Case ID | UsbModem_ErrorReport_1 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Store modem error messages |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary |  |  |
| Test procedure | 1) Unplug usbmodem from BR 2) Check the error msg from the console log |  |  |
| Expect result | 1) The related error msg is printd in the console of BR - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Before unplug usbmodem - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh log bu ¦ in fosm 2012-04-01 02:32:09 debug ah_brd: [brd_fosm]: WFO: PPP ifchk timer: PPP is UP (IP:0xa26d9a8, netmask:0xffffffff) 2012-04-01 02:32:09 debug ah_brd: [brd_fosm]: WFO: Interface ppp0 status is UP (ifr_flags = 0x10d1) 2012-04-01 02:32:09 debug ah_brd: [brd_fosm]: WFO: PPP ifchk timer popped ... 2012-04-01 02:31:42 info ah_cli: admin:<_debug brd fosm> 2012-04-01 02:31:22 debug ah_brd: WFOSM: state IDLE -> IDLE 2012-04-01 02:31:22 debug ah_brd: WFOSM: Added event MAIN_WAN_IF_UP 2012-04-01 02:31:15 debug ah_brd: WFOSM: state IDLE -> IDLE 2012-04-01 02:31:15 debug ah_brd: WFOSM: Added event PPP_IF_UP 2012-04-01 02:31:10 debug ah_brd: WFOSM: state IDLE -> IDLE 2012-04-01 02:31:10 debug ah_brd: WFOSM: state FAILOVER -> IDLE 2012-04-01 02:31:10 debug ah_brd: WFOSM: state ATTACHED -> FAILOVER 2012-04-01 02:31:10 debug ah_brd: WFOSM: state CONNECTING -> ATTACHED 2012-04-01 02:31:10 debug ah_brd: WFOSM: Added event START_PPP 2012-04-01 02:31:10 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 2012-04-01 02:31:10 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-01 02:31:10 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-01 02:31:10 debug ah_brd: WFOSM: Added event PPP_STARTED 2012-04-01 02:31:07 debug ah_brd: WFOSM: Added event BKUP_WAN_IF_UP 2012-04-01 02:31:04 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-01 02:31:04 debug ah_brd: WFOSM: state CONNECTING -> ATTACHED 2012-04-01 02:31:04 debug ah_brd: WFOSM: Added event PPP_STARTED 2012-04-01 02:30:58 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-01 02:30:58 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 2012-04-01 02:30:58 debug ah_brd: WFOSM: state CONNECTING -> ATTACHED 2012-04-01 02:30:58 debug ah_brd: WFOSM: Added event START_PPP 2012-04-01 02:30:58 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 2012-04-01 02:30:58 debug ah_brd: WFOSM: state CONNECTING -> ATTACHED 2012-04-01 02:30:58 debug ah_brd: WFOSM: Added event START_PPP 2012-04-01 02:30:58 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 2012-04-01 02:30:58 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-01 02:30:58 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-01 02:30:58 debug ah_brd: WFOSM: Added event PPP_STARTED 2012-04-01 02:30:52 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-01 02:30:52 debug ah_brd: WFOSM: state CONNECTING -> ATTACHED 2012-04-01 02:30:52 debug ah_brd: WFOSM: Added event PPP_STARTED 1970-01-01 00:00:24 debug ah_brd: WFOSM: Added event PPP_STOPPED 1970-01-01 00:00:24 debug ah_brd: WFOSM: state CONNECTING -> CONNECTING 1970-01-01 00:00:24 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 1970-01-01 00:00:24 debug ah_brd: WFOSM: Added event PPP_STARTED 1970-01-01 00:00:24 debug ah_brd: WFOSM: Added event USB_DEV_ADD 1970-01-01 00:00:20 debug ah_brd: WFOSM: Added event START_PPP 1970-01-01 00:00:20 debug ah_brd: WFOSM: Added event START_PPP 1970-01-01 00:00:19 debug ah_brd: WFOSM: Added event USB_DEV_ADD 1970-01-01 00:00:19 debug ah_brd: WFOSM: Added event USB_DEV_ADD 1970-01-01 00:00:19 debug ah_brd: WFOSM: Added event USB_DEV_ADD 1970-01-01 00:00:19 debug ah_brd: WFOSM: Added event USB_DEV_ADD 1970-01-01 00:00:19 debug ah_brd: WFOSM: Added event USB_DEV_ADD 1970-01-01 00:00:19 debug ah_brd: WFOSM: Added event USB_DEV_ADD 1970-01-01 00:00:19 debug ah_brd: WFOSM: Added event USB_DEV_ADD 1970-01-01 00:00:19 debug ah_brd: WFOSM: state IDLE -> ATTACHED 1970-01-01 00:00:19 debug ah_brd: WFOSM: Added event MAIN_WAN_IF_DOWN 1970-01-01 00:00:19 debug ah_brd: WFOSM: Added event USB_DEV_ADD - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh log bu ¦ in hot 2012-04-01 02:31:42 info ah_cli: admin:<_debug brd hotplug> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After unplug usbmodem - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh log bu ¦ in fosm 2012-04-01 02:33:18 debug ah_brd: WFOSM: state IDLE -> FAILOVER 2012-04-01 02:33:18 debug ah_brd: [brd_fosm]: WFO: Failover OK 2012-04-01 02:33:18 debug ah_brd: [brd_fosm]: WFO: Switch route: dft route (gw: cur=0xa9b1efe cached=0xa9b1efe) via eth0 existed 2012-04-01 02:33:18 debug ah_brd: [brd_fosm]: WFO: Switch route: ppp0 dft gw IP 0x0 now cached 2012-04-01 02:33:18 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: IDLE 2012-04-01 02:33:18 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:IDLE + event:MAIN_WAN_IF_DOWN [0x200] 2012-04-01 02:33:18 debug ah_brd: WFOSM: Added event MAIN_WAN_IF_DOWN 2012-04-01 02:33:13 debug ah_brd: WFOSM: state IDLE -> IDLE 2012-04-01 02:33:13 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: IDLE 2012-04-01 02:33:13 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:IDLE + event:USB_DEV_REMOVE [0x2] 2012-04-01 02:33:13 debug ah_brd: WFOSM: Added event USB_DEV_REMOVE 2012-04-01 02:33:13 debug ah_brd: [brd_fosm]: WFO: USB device id is set (vid=0x0 pid=0x0) 2012-04-01 02:33:13 debug ah_brd: [brd_fosm]: WFO: Retrieved USB device id (vid=0x12d1 pid=0x1003) 2012-04-01 02:33:13 debug ah_brd: [brd_fosm]: WFO: Retrieved USB device id (vid=0x12d1 pid=0x1003) 2012-04-01 02:33:13 debug ah_brd: [brd_fosm]: WFO: Retrieved USB device id (vid=0x12d1 pid=0x1003) 2012-04-01 02:33:13 debug ah_brd: [brd_fosm]: WFO: Retrieved USB device id (vid=0x12d1 pid=0x1003) 2012-04-01 02:33:13 debug ah_brd: [brd_fosm]: WFO: Retrieved USB device id (vid=0x12d1 pid=0x1003) 2012-04-01 02:33:13 debug ah_brd: [brd_fosm]: WFO: Retrieved USB device id (vid=0x12d1 pid=0x1003) 2012-04-01 02:33:13 debug ah_brd: [brd_fosm]: WFO: Retrieved USB device id (vid=0x12d1 pid=0x1003) 2012-04-01 02:33:13 debug ah_brd: [brd_fosm]: WFO: Retrieved USB device id (vid=0x12d1 pid=0x1003) 2012-04-01 02:33:13 debug ah_brd: WFOSM: state IDLE -> IDLE 2012-04-01 02:33:13 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: IDLE 2012-04-01 02:33:13 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:IDLE + event:PPP_IF_DOWN [0x80] 2012-04-01 02:33:13 debug ah_brd: [brd_fosm]: WFO: ppp0: Interface DOWN :-( 2012-04-01 02:33:13 debug ah_brd: WFOSM: Added event PPP_IF_DOWN - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh log bu ¦ in hot 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list end] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [0]: USB id=0x1d6b0002 vid=0x1d6b pid=0x2 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list begin (count:1)] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/../uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/./uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Hotplug event REMOVE trapped 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Found USB hotplug event remove@ for dev usb1 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list end] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Found USB dev vid=0x12d1 pid=0x1003 attached 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [1]: USB id=0x12d11003 vid=0x12d1 pid=0x1003 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [0]: USB id=0x1d6b0002 vid=0x1d6b pid=0x2 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list begin (count:2)] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/../uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/./uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Hotplug event REMOVE trapped 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Found USB hotplug event remove@ for dev usb1 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list end] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Found USB dev vid=0x12d1 pid=0x1003 attached 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [1]: USB id=0x12d11003 vid=0x12d1 pid=0x1003 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [0]: USB id=0x1d6b0002 vid=0x1d6b pid=0x2 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list begin (count:2)] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/../uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/./uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Hotplug event REMOVE trapped 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Found USB hotplug event remove@ for dev usb1 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list end] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Found USB dev vid=0x12d1 pid=0x1003 attached 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [1]: USB id=0x12d11003 vid=0x12d1 pid=0x1003 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [0]: USB id=0x1d6b0002 vid=0x1d6b pid=0x2 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list begin (count:2)] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/../uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/./uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Hotplug event REMOVE trapped 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Found USB hotplug event remove@ for dev usb1 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list end] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Found USB dev vid=0x12d1 pid=0x1003 attached 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [1]: USB id=0x12d11003 vid=0x12d1 pid=0x1003 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [0]: USB id=0x1d6b0002 vid=0x1d6b pid=0x2 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list begin (count:2)] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/../uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/./uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Hotplug event REMOVE trapped 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Found USB hotplug event remove@ for dev usb1 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list end] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Found USB dev vid=0x12d1 pid=0x1003 attached 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [1]: USB id=0x12d11003 vid=0x12d1 pid=0x1003 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [0]: USB id=0x1d6b0002 vid=0x1d6b pid=0x2 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list begin (count:2)] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/../uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/./uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Hotplug event REMOVE trapped 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Found USB hotplug event remove@ for dev USB1 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list end] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Found USB dev vid=0x12d1 pid=0x1003 attached 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [1]: USB id=0x12d11003 vid=0x12d1 pid=0x1003 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [0]: USB id=0x1d6b0002 vid=0x1d6b pid=0x2 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list begin (count:2)] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/../uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/./uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Hotplug event REMOVE trapped 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Found USB hotplug event remove@ for dev usb1 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list end] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Found USB dev vid=0x12d1 pid=0x1003 attached 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [1]: USB id=0x12d11003 vid=0x12d1 pid=0x1003 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [0]: USB id=0x1d6b0002 vid=0x1d6b pid=0x2 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list begin (count:2)] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/../uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/./uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Hotplug event REMOVE trapped 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Found USB hotplug event remove@ for dev usb1 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list end] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Found USB dev vid=0x12d1 pid=0x1003 attached 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [1]: USB id=0x12d11003 vid=0x12d1 pid=0x1003 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [0]: USB id=0x1d6b0002 vid=0x1d6b pid=0x2 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: [USB dev list begin (count:2)] 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/../uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Skip sysfs entry [/sys/bus/usb/devices/./uevent] ... 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Hotplug event REMOVE trapped 2012-04-01 02:33:13 debug ah_brd: [brd_hotplug]: WFOHP: Found USB hotplug event remove@ for dev USB0 |  |  |
| Test Result |  |  |  |

UsbModem_ErrorReport_2

| Case ID | UsbModem_ErrorReport_2 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Report modem errors to HiveManager |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary |  |  |
| Test procedure | 1) Run some illegal action to trigger modem error 2) Check the error msg of modem on HM |  |  |
| Expect result | 1) There are errors displayed in HM for this BR/ modem 2) The error msg is the same as the error printed in BR console 3) The error msg will be updated according to the retrieve period of HM 4) The history error msg can still be displayed in HM, after BR reboot and restore the connection with HM |  |  |
| Test Result |  |  |  |

UsbModem_Electric

UsbModem_Electric_1

| Case ID | UsbModem_Electric_1 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Status of LED when 3g modem mode is on-demand |  |  |
| Pre-condition | 1) WAN is available by eth0 2) The 3g service is activated and available 3) capwap connection works well by either eth0 or usbmodem WAN 4) Configured the dialup username/ passwd/ number if the modem if necessary |  |  |
| Test procedure | 1) Set the mode of USB modem as on-demand in BR 2) Unplug the cable of eth0 from BR 3) Unplug the USB modem from BR |  |  |
| Expect result | 1) When WAN through eth0 is available, LED is white solid 2) When WAN through 3g instead of eth0 is available, LED is white blinking 3) When WAN is unavailalbe either through 3g or eth0, LED is amber solid |  |  |
| Test Result |  |  |  |

UsbModem_Electric_2

| Case ID | UsbModem_Electric_2 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Status of LED when 3g modem mode is always-connected |  |  |
| Pre-condition | 1) WAN is available by eth0 2) The 3g service is activated and available 3) capwap connection works well by either eth0 or usbmodem WAN 4) Configured the dialup username/ passwd/ number if the modem if necessary |  |  |
| Test procedure | 1) Set the mode of USB modem as always-connected in BR 2) Unplug the cable of eth0 from BR 3) Unplug the USB modem from BR |  |  |
| Expect result | 1) When WAN through eth0 is available, LED is white solid 2) When WAN through 3g instead of eth0 is available, LED is white blinking 3) When WAN is unavailalbe either through 3g or eth0, LED is amber solid |  |  |
| Test Result |  |  |  |

UsbModem_Electric_3

| Case ID | UsbModem_Electric_3 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Status of LED when 3g modem mode is primary-wan |  |  |
| Pre-condition | 1) WAN is available by eth0 2) The 3g service is activated and available 3) capwap connection works well by either eth0 or usbmodem WAN 4) Configured the dialup username/ passwd/ number if the modem if necessary |  |  |
| Test procedure | 1) Set the mode of USB modem as primary-wan in BR 2) Unplug the USB modem from BR 3) Unplug the cable of eth0 from BR |  |  |
| Expect result | 1) When WAN through 3g is available, LED is white solid 2) When WAN through eth0 instead of 3g is available, LED is white blinking 3) When WAN is unavailalbe either through 3g or eth0, LED is amber solid |  |  |
| Test Result |  |  |  |

UsbModem_Electric_4

| Case ID | UsbModem_Electric_4 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Status of LED when track invalid ip and eth0 is primary wan |  |  |
| Pre-condition | 1) WAN is available by eth0 2) capwap connection works well by eth0 WAN 3) Unplug usbmodem zhxu-8c66c0#sh ru ¦ in track track QS-IP-Track-Router default-gateway track QS-IP-Track-Router ip 206.80.44.205 track QS-IP-Track-Router ip 206.80.44.206 track QS-IP-Track-Router interval 30 track QS-IP-Track-Router timeout 5 track QS-IP-Track-Router multi-dst-logic and track QS-IP-Track-Router use-for-wan-testing |  |  |
| Test procedure | 1) Set the mode of USB modem as on-demand or always-connected in BR zhxu-8c66c0#sh usbm ¦ in mode: Mode: on-demand 2) Track invalid ip track QS-IP-Track-Router ip 1.1.1.1 3) Check the state of WAN and LED sh sys l |  |  |
| Expect result | 1) Before track invalid ip, both WAN and capwap connection are up, LED is white solid zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 172.18.255.129 - 1 0019:778c:66c0 U Mgt0.16 192.168.1.249 - 10 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.1.248 0.0.0.0 255.255.255.248 U 0 0 0 mgt0.16 172.18.255.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh usbm s USB Modem Attached: NONE USB Network Interface Status: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Tue Apr 17 11:22:12 2012) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: IDLE Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh capw c ¦ in RUN RUN state: Connected securely to the CAPWAP server - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh sys l Brightness configuration: bright System LED status: No Led Colour Blinking --------------------------------------------------------- 1 Power Off No blinking 2 Status White No blinking 2) After track invalid ip, WAN monitor state is down, LED is amber solid, though capwap connection still is ok. zhxu-8c66c0#sh ru ¦ in track track QS-IP-Track-Router default-gateway track QS-IP-Track-Router ip 206.80.44.205 track QS-IP-Track-Router ip 206.80.44.206 track QS-IP-Track-Router ip 1.1.1.1 track QS-IP-Track-Router interval 30 track QS-IP-Track-Router timeout 5 track QS-IP-Track-Router use-for-wan-testing - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0# zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 172.18.255.129 - 1 0019:778c:66c0 U Mgt0.16 192.168.1.249 - 10 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.1.248 0.0.0.0 255.255.255.248 U 0 0 0 mgt0.16 172.18.255.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh usbm s USB Modem Attached: NONE USB Network Interface Status: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 1 times (since Tue Apr 17 11:22:12 2012) DOWN: 1 times (since Tue Apr 17 11:47:32 2012) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: IDLE Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh capw c ¦ in RUN RUN state: Connected securely to the CAPWAP server - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh sys l Brightness configuration: bright System LED status: No Led Colour Blinking --------------------------------------------------------- 1 Power Red No blinking 2 Status Off No blinking |  |  |
| Test Result |  |  |  |

UsbModem_Electric_5

| Case ID | UsbModem_Electric_5 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Status of LED when track invalid ip and usbmodem is primary wan |  |  |
| Pre-condition | 1) WAN is available by usbmodem 2) capwap connection works well by usbmodem WAN 3) Unplug eth0 zhxu-8c66c0#sh ru ¦ in track track QS-IP-Track-Router default-gateway track QS-IP-Track-Router ip 206.80.44.205 track QS-IP-Track-Router ip 206.80.44.206 track QS-IP-Track-Router interval 30 track QS-IP-Track-Router timeout 5 track QS-IP-Track-Router multi-dst-logic and track QS-IP-Track-Router use-for-wan-testing |  |  |
| Test procedure | 1) Set the mode of USB modem as primary-wan in BR zhxu-8c66c0#sh usbm ¦ in mode: Mode: primary-wan 2) Track invalid ip track QS-IP-Track-Router ip 1.1.1.1 3) Check the state of WAN and LED |  |  |
| Expect result | 1) Before track invalid ip, both WAN and capwap connection are up, LED is white solid zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 0.0.0.0 wan - 0019:778c:66c0 D Mgt0 172.18.255.129 - 1 0019:778c:66c0 U Mgt0.16 192.168.1.249 - 10 0019:778c:66c0 U Ppp0 10.32.136.87 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.1.248 0.0.0.0 255.255.255.248 U 0 0 0 mgt0.16 172.18.255.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x141b USB Network Interface Status: NONE PPP Process Status: PID: 2597 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 920228 4301 0 0 0 0 4301 ¦ 1640737 18426 0 0 18426 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Jan 1 08:39:11 1970) DOWN: 1 times (since Thu Jan 1 08:02:27 1970) Local Link State: UP: 1 times (since Thu Jan 1 08:01:40 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh capw c ¦ in RUN RUN state: Connected securely to the CAPWAP server - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh sys l Brightness configuration: bright System LED status: No Led Colour Blinking --------------------------------------------------------- 1 Power Off No blinking 2 Status White No blinking 2) After track invalid ip, WAN monitor state is down, LED is amber solid, though capwap connection still is ok. zhxu-8c66c0#sh ru ¦ in track track QS-IP-Track-Router default-gateway track QS-IP-Track-Router ip 206.80.44.205 track QS-IP-Track-Router ip 206.80.44.206 track QS-IP-Track-Router ip 1.1.1.1 track QS-IP-Track-Router interval 30 track QS-IP-Track-Router timeout 5 track QS-IP-Track-Router use-for-wan-testing - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - |  |  |
| Test Result |  |  |  |

UsbModem_Electric_6

| Case ID | UsbModem_Electric_6 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Status of LED when disable/ enable capwap connection, eth0 is prim WAN |  |  |
| Pre-condition | 1) WAN is available by eth0 2) Set the mode of USB modem as on-demand or always-connecte 3) capwap is disabled initially 4) Unplug usbmodem zhxu-8c66c0#sh ru ¦ in track track QS-IP-Track-Router default-gateway track QS-IP-Track-Router ip 206.80.44.205 track QS-IP-Track-Router ip 206.80.44.206 track QS-IP-Track-Router interval 30 track QS-IP-Track-Router timeout 5 track QS-IP-Track-Router multi-dst-logic and track QS-IP-Track-Router use-for-wan-testing - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ru ¦ i capw no capwap client enable capwap client server name 10.155.33.220 capwap client dtls hm-defined-passphrase *** key-id 1 capwap client vhm-name JingLi-VHM no capwap client dtls negotiation enable |  |  |
| Test procedure | 1) Startup BR with track valid ip 2) Check the state of WAN and LED 3) Enable capwap capw c en 4) Repeat step 2) 5) Track invalid ip track QS-IP-Track-Router ip 1.1.1.1 6) Repeat step 2) 7) Disable capwap no capw c en 8) Repeat step 2) 9) Enable capwap again capw c en 10) Repeat step 2) |  |  |
| Expect result | 1) Before track invalid ip, WAN is up, but no capwap connection, and LED is amber solid zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 172.18.255.129 - 1 0019:778c:66c0 U Mgt0.16 192.168.1.249 - 10 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.1.248 0.0.0.0 255.255.255.248 U 0 0 0 mgt0.16 172.18.255.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh usbm s USB Modem Attached: NONE USB Network Interface Status: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Tue Apr 17 14:41:32 2012) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: IDLE Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh capw c CAPWAP client: Disabled Discovery interval: 5 seconds Heartbeat interval: 30 seconds Max discovery interval: 10 seconds Neighbor dead interval:105 seconds Silent interval: 15 seconds Wait join interval: 60 seconds Discovery count: 0 Max discovery count: 3 Retransmit count: 0 Max retransmit count: 2 Keepalives lost/sent: 0/0 Event packet drop due to buffer shortage: 0 Event packet drop due to loss connection: 6 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh sy l Brightness configuration: bright System LED status: No Led Colour Blinking --------------------------------------------------------- 1 Power Red No blinking 2 Status Off No blinking 2) After enable capwap, both WAN and capwap connection are up, and LED is white solid zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 172.18.255.129 - 1 0019:778c:66c0 U Mgt0.16 192.168.1.249 - 10 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.1.248 0.0.0.0 255.255.255.248 U 0 0 0 mgt0.16 172.18.255.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh usbm s USB Modem Attached: NONE USB Network Interface Status: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Tue Apr 17 14:41:32 2012) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: IDLE Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh capw c CAPWAP client: Enabled CAPWAP transport mode: UDP RUN state: Connected securely to the CAPWAP server CAPWAP client IP: 172.18.255.129 CAPWAP server IP: 10.155.33.220 HiveManager Primary Name:10.155.33.220 HiveManager Backup Name: CAPWAP Default Server Name: staging.aerohive.com Virtual HiveManager Name: JingLi-VHM Server destination Port: 12222 CAPWAP send event: Enabled CAPWAP DTLS state: Enabled CAPWAP DTLS negotiation: Disabled DTLS next connect status: Enable DTLS always accept bootstrap passphrase: Enabled DTLS session status: Connected DTLS key type: passphrase DTLS session cut interval: 5 seconds DTLS handshake wait interval: 60 seconds DTLS Max retry count: 3 DTLS authorize failed: 0 DTLS reconnect count: 0 Discovery interval: 5 seconds Heartbeat interval: 30 seconds Max discovery interval: 10 seconds Neighbor dead interval:105 seconds Silent interval: 15 seconds Wait join interval: 60 seconds Discovery count: 0 Max discovery count: 3 Retransmit count: 0 Max retransmit count: 2 Keepalives lost/sent: 0/133 Event packet drop due to buffer shortage: 0 Event packet drop due to loss connection: 7 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh sy l Brightness configuration: bright System LED status: No Led Colour Blinking --------------------------------------------------------- 1 Power Off No blinking 2 Status White No blinking 3) After track invalid ip, WAN is down, and LED is amber solid zhxu-8c66c0#sh ru ¦ i track track QS-IP-Track-Router default-gateway track QS-IP-Track-Router ip 206.80.44.205 track QS-IP-Track-Router ip 206.80.44.206 track QS-IP-Track-Router ip 1.1.1.1 track QS-IP-Track-Router interval 30 track QS-IP-Track-Router timeout 5 track QS-IP-Track-Router use-for-wan-testing - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 172.18.255.129 - 1 0019:778c:66c0 U Mgt0.16 192.168.1.249 - 10 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.1.248 0.0.0.0 255.255.255.248 U 0 0 0 mgt0.16 172.18.255.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh usbm s USB Modem Attached: NONE USB Network Interface Status: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 1 times (since Tue Apr 17 14:41:32 2012) DOWN: 1 times (since Tue Apr 17 16:09:42 2012) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: IDLE Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh capw c CAPWAP client: Enabled CAPWAP transport mode: UDP RUN state: Connected securely to the CAPWAP server CAPWAP client IP: 172.18.255.129 CAPWAP server IP: 10.155.33.220 HiveManager Primary Name:10.155.33.220 HiveManager Backup Name: CAPWAP Default Server Name: staging.aerohive.com Virtual HiveManager Name: JingLi-VHM Server destination Port: 12222 CAPWAP send event: Enabled CAPWAP DTLS state: Enabled CAPWAP DTLS negotiation: Disabled DTLS next connect status: Enable DTLS always accept bootstrap passphrase: Enabled DTLS session status: Connected DTLS key type: passphrase DTLS session cut interval: 5 seconds DTLS handshake wait interval: 60 seconds DTLS Max retry count: 3 DTLS authorize failed: 0 DTLS reconnect count: 0 Discovery interval: 5 seconds Heartbeat interval: 30 seconds Max discovery interval: 10 seconds Neighbor dead interval:105 seconds Silent interval: 15 seconds Wait join interval: 60 seconds Discovery count: 0 Max discovery count: 3 Retransmit count: 0 Max retransmit count: 2 Keepalives lost/sent: 0/233 Event packet drop due to buffer shortage: 0 Event packet drop due to loss connection: 7 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh sy l Brightness configuration: bright System LED status: No Led Colour Blinking --------------------------------------------------------- 1 Power Red No blinking 2 Status Off No blinking 4) After disable capwap again, both WAN and capwap connection are down, and LED is amber solid zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 172.18.255.129 - 1 0019:778c:66c0 U Mgt0.16 192.168.1.249 - 10 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.1.248 0.0.0.0 255.255.255.248 U 0 0 0 mgt0.16 172.18.255.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh usbm s USB Modem Attached: NONE USB Network Interface Status: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 1 times (since Tue Apr 17 14:41:32 2012) DOWN: 1 times (since Tue Apr 17 16:09:42 2012) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: IDLE Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh capw c CAPWAP client: Disabled Discovery interval: 5 seconds Heartbeat interval: 30 seconds Max discovery interval: 10 seconds Neighbor dead interval:105 seconds Silent interval: 15 seconds Wait join interval: 60 seconds Discovery count: 0 Max discovery count: 3 Retransmit count: 0 Max retransmit count: 2 Keepalives lost/sent: 0/240 Event packet drop due to buffer shortage: 0 Event packet drop due to loss connection: 7 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh sy l Brightness configuration: bright System LED status: No Led Colour Blinking --------------------------------------------------------- 1 Power Red No blinking 2 Status Off No blinking 5) After enable capwap again, WAN still is down, and LED is amber solid though capwap connection is up zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 172.18.255.129 - 1 0019:778c:66c0 U Mgt0.16 192.168.1.249 - 10 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.1.248 0.0.0.0 255.255.255.248 U 0 0 0 mgt0.16 172.18.255.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh usbm s USB Modem Attached: NONE USB Network Interface Status: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 1 times (since Tue Apr 17 14:41:32 2012) DOWN: 1 times (since Tue Apr 17 16:09:42 2012) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: IDLE Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh capw c CAPWAP client: Enabled CAPWAP transport mode: UDP RUN state: Connected securely to the CAPWAP server CAPWAP client IP: 172.18.255.129 CAPWAP server IP: 10.155.33.220 HiveManager Primary Name:10.155.33.220 HiveManager Backup Name: CAPWAP Default Server Name: staging.aerohive.com Virtual HiveManager Name: JingLi-VHM Server destination Port: 12222 CAPWAP send event: Enabled CAPWAP DTLS state: Enabled CAPWAP DTLS negotiation: Disabled DTLS next connect status: Enable DTLS always accept bootstrap passphrase: Enabled DTLS session status: Connected DTLS key type: passphrase DTLS session cut interval: 5 seconds DTLS handshake wait interval: 60 seconds DTLS Max retry count: 3 DTLS authorize failed: 0 DTLS reconnect count: 0 Discovery interval: 5 seconds Heartbeat interval: 30 seconds Max discovery interval: 10 seconds Neighbor dead interval:105 seconds Silent interval: 15 seconds Wait join interval: 60 seconds Discovery count: 0 Max discovery count: 3 Retransmit count: 0 Max retransmit count: 2 Keepalives lost/sent: 0/264 Event packet drop due to buffer shortage: 0 Event packet drop due to loss connection: 7 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh sy l Brightness configuration: bright System LED status: No Led Colour Blinking --------------------------------------------------------- 1 Power Off No blinking 2 Status White No blinking |  |  |
| Test Result |  |  |  |

UsbModem_Electric_7

| Case ID | UsbModem_Electric_7 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Status of LED when disable/ enable capwap connection, usbmodem is prim WAN |  |  |
| Pre-condition | 1) WAN is available by usbmodem 2) Set the mode of USB modem as primary-wan 3) capwap is disabled initially 4) Unplug eth0 |  |  |
| Test procedure | 1) Startup BR with track valid ip 2) Check the state of WAN and LED 3) Enable capwap capw c en 4) Repeat step 2) 5) Track invalid ip track QS-IP-Track-Router ip 1.1.1.1 6) Repeat step 2) 7) Disable capwap no capw c en 8) Repeat step 2) 9) Enable capwap again capw c en 10) Repeat step 2) |  |  |
| Expect result | 1) Before track invalid ip, WAN is up, but no capwap connection, and LED is amber solid 2) After enable capwap, both WAN and capwap connection are up, and LED is white solid 3) After track invalid ip, WAN is down, and LED is amber solid 4) After disable capwap again, both WAN and capwap connection are down, and LED is amber solid 5) After enable capwap again, WAN still is down, and LED is amber solid though capwap connection is up |  |  |
| Test Result |  |  |  |

UsbModem_Electric_8

| Case ID | UsbModem_Electric_8 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Status of LED with/ without track invalid ip after capwap is disabled, eth0 is prim WAN |  |  |
| Pre-condition | 1) WAN is available by eth0 2) Set the mode of USB modem as on-demand or always-connecte 3) capwap is disabled initially 4) Unplug usbmodem zhxu-8c66c0#sh ru ¦ i track track QS-IP-Track-Router default-gateway track QS-IP-Track-Router ip 206.80.44.205 track QS-IP-Track-Router ip 206.80.44.206 track QS-IP-Track-Router interval 30 track QS-IP-Track-Router timeout 5 track QS-IP-Track-Router multi-dst-logic and track QS-IP-Track-Router use-for-wan-testing - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ru ¦ i capw no capw client enable capwap client server name 10.155.33.220 capwap client dtls hm-defined-passphrase *** key-id 1 capwap client vhm-name JingLi-VHM no capwap client dtls negotiation enable |  |  |
| Test procedure | 1) Startup BR with track valid ip 2) Check the state of WAN and LED 3) Track invalid ip track QS-IP-Track-Router ip 1.1.1.1 4) Repeat step 2) 5) Stop track invalid ip no track QS-IP-Track-Router ip 1.1.1.1 6) Repeat step 2) |  |  |
| Expect result | 1) Before track invalid ip , capwap connection is down, and LED is amber solid though WAN is up zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 172.18.255.129 - 1 0019:778c:66c0 U Mgt0.16 192.168.1.249 - 10 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.1.248 0.0.0.0 255.255.255.248 U 0 0 0 mgt0.16 172.18.255.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh usbm s USB Modem Attached: NONE USB Network Interface Status: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Tue Apr 17 16:22:50 2012) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: IDLE Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh capw c CAPWAP client: Disabled Discovery interval: 5 seconds Heartbeat interval: 30 seconds Max discovery interval: 10 seconds Neighbor dead interval:105 seconds Silent interval: 15 seconds Wait join interval: 60 seconds Discovery count: 0 Max discovery count: 3 Retransmit count: 0 Max retransmit count: 2 Keepalives lost/sent: 0/128 Event packet drop due to buffer shortage: 0 Event packet drop due to loss connection: 8 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh sy l Brightness configuration: bright System LED status: No Led Colour Blinking --------------------------------------------------------- 1 Power Red No blinking 2 Status Off No blinking 2) After track invalid ip, both WAN and capwap connection are down, and LED is amber solid zhxu-8c66c0#sh ru ¦ i track track QS-IP-Track-Router default-gateway track QS-IP-Track-Router ip 206.80.44.205 track QS-IP-Track-Router ip 206.80.44.206 track QS-IP-Track-Router ip 1.1.1.1 track QS-IP-Track-Router interval 30 track QS-IP-Track-Router timeout 5 track QS-IP-Track-Router use-for-wan-testing zhxu-8c66c0# - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 172.18.255.129 - 1 0019:778c:66c0 U Mgt0.16 192.168.1.249 - 10 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.1.248 0.0.0.0 255.255.255.248 U 0 0 0 mgt0.16 172.18.255.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh usbm s USB Modem Attached: NONE USB Network Interface Status: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 1 times (since Tue Apr 17 16:22:50 2012) DOWN: 1 times (since Tue Apr 17 16:35:11 2012) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: IDLE Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh capw c CAPWAP client: Disabled Discovery interval: 5 seconds Heartbeat interval: 30 seconds Max discovery interval: 10 seconds Neighbor dead interval:105 seconds Silent interval: 15 seconds Wait join interval: 60 seconds Discovery count: 0 Max discovery count: 3 Retransmit count: 0 Max retransmit count: 2 Keepalives lost/sent: 0/128 Event packet drop due to buffer shortage: 0 Event packet drop due to loss connection: 8 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh sy l Brightness configuration: bright System LED status: No Led Colour Blinking --------------------------------------------------------- 1 Power Red No blinking 2 Status Off No blinking 3) After stop track invalid ip, WAN is up, and LED is amber solid since capwap connection is still down zhxu-8c66c0#sh ru ¦ i track track QS-IP-Track-Router default-gateway track QS-IP-Track-Router ip 206.80.44.205 track QS-IP-Track-Router ip 206.80.44.206 track QS-IP-Track-Router interval 30 track QS-IP-Track-Router timeout 5 track QS-IP-Track-Router multi-dst-logic and track QS-IP-Track-Router use-for-wan-testing - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 172.18.255.129 - 1 0019:778c:66c0 U Mgt0.16 192.168.1.249 - 10 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.1.248 0.0.0.0 255.255.255.248 U 0 0 0 mgt0.16 172.18.255.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh usbm s USB Modem Attached: NONE USB Network Interface Status: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Tue Apr 17 16:38:16 2012) DOWN: 1 times (since Tue Apr 17 16:35:11 2012) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: IDLE Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh capw c CAPWAP client: Disabled Discovery interval: 5 seconds Heartbeat interval: 30 seconds Max discovery interval: 10 seconds Neighbor dead interval:105 seconds Silent interval: 15 seconds Wait join interval: 60 seconds Discovery count: 0 Max discovery count: 3 Retransmit count: 0 Max retransmit count: 2 Keepalives lost/sent: 0/128 Event packet drop due to buffer shortage: 0 Event packet drop due to loss connection: 8 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh sy l Brightness configuration: bright System LED status: No Led Colour Blinking --------------------------------------------------------- 1 Power Red No blinking 2 Status Off No blinking |  |  |
| Test Result |  |  |  |

UsbModem_Electric_9

| Case ID | UsbModem_Electric_9 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Status of LED with/ without track invalid ip after capwap is disabled, usbmodem is prim WAN |  |  |
| Pre-condition | 1) WAN is available by usbmodem 2) Set the mode of USB modem as primary-wan 3) capwap is disabled initially 4) Unplug eth0 |  |  |
| Test procedure | 1) Startup BR with track valid ip 2) Check the state of WAN and LED 3) Track invalid ip track QS-IP-Track-Router ip 1.1.1.1 4) Repeat step 2) 5) Stop track invalid ip no track QS-IP-Track-Router ip 1.1.1.1 6) Repeat step 2) |  |  |
| Expect result | 1) Before track invalid ip, capwap connection is down, and LED is amber solid though WAN is up 2) After track invalid ip, both WAN and capwap connection are down, and LED is amber solid 3) After stop track invalid ip, WAN is up, and LED is amber solid since capwap connection is still down |  |  |
| Test Result |  |  |  |

UsbModem_Electric_10

| Case ID | UsbModem_Electric_10 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Status of LED with/ without track invalid ip after capwap is enabled, eth0 is prim WAN |  |  |
| Pre-condition | 1) WAN is available by eth0 2) Set the mode of USB modem as on-demand or always-connecte 3) capwap is enabled initially 4) Unplug usbmodem zhxu-8c66c0#sh ru ¦ i track track QS-IP-Track-Router default-gateway track QS-IP-Track-Router ip 206.80.44.205 track QS-IP-Track-Router ip 206.80.44.206 track QS-IP-Track-Router interval 30 track QS-IP-Track-Router timeout 5 track QS-IP-Track-Router multi-dst-logic and track QS-IP-Track-Router use-for-wan-testing - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ru ¦ i capw capwap client server name 10.155.33.220 capwap client dtls hm-defined-passphrase *** key-id 1 capwap client vhm-name JingLi-VHM no capwap client dtls negotiation enable |  |  |
| Test procedure | 1) Startup BR with track valid ip 2) Check the state of WAN and LED 3) Track invalid ip track QS-IP-Track-Router ip 1.1.1.1 4) Repeat step 2) 5) Stop track invalid ip no track QS-IP-Track-Router ip 1.1.1.1 6) Repeat step 2) |  |  |
| Expect result | 1) Before track invalid ip, both WAN and capwap connection are up, and LED is white solid zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 172.18.255.129 - 1 0019:778c:66c0 U Mgt0.16 192.168.1.249 - 10 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.1.248 0.0.0.0 255.255.255.248 U 0 0 0 mgt0.16 172.18.255.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh usbm s USB Modem Attached: NONE USB Network Interface Status: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Tue Apr 17 11:22:12 2012) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: IDLE Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh capw c ¦ in RUN RUN state: Connected securely to the CAPWAP server - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh sys l Brightness configuration: bright System LED status: No Led Colour Blinking --------------------------------------------------------- 1 Power Off No blinking 2 Status White No blinking 2) After track invalid ip, WAN is down, and LED is amber solid though capwap connection is up zhxu-8c66c0#sh ru ¦ in track track QS-IP-Track-Router default-gateway track QS-IP-Track-Router ip 206.80.44.205 track QS-IP-Track-Router ip 206.80.44.206 track QS-IP-Track-Router ip 1.1.1.1 track QS-IP-Track-Router interval 30 track QS-IP-Track-Router timeout 5 track QS-IP-Track-Router use-for-wan-testing - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0# zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 172.18.255.129 - 1 0019:778c:66c0 U Mgt0.16 192.168.1.249 - 10 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.1.248 0.0.0.0 255.255.255.248 U 0 0 0 mgt0.16 172.18.255.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh usbm s USB Modem Attached: NONE USB Network Interface Status: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 1 times (since Tue Apr 17 11:22:12 2012) DOWN: 1 times (since Tue Apr 17 11:47:32 2012) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: IDLE Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh capw c ¦ in RUN RUN state: Connected securely to the CAPWAP server - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh sys l Brightness configuration: bright System LED status: No Led Colour Blinking --------------------------------------------------------- 1 Power Red No blinking 2 Status Off No blinking 3) After stop track invalid ip, both WAN and capwap connection are up, and LED is white solid zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 172.18.255.129 - 1 0019:778c:66c0 U Mgt0.16 192.168.1.249 - 10 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.1.248 0.0.0.0 255.255.255.248 U 0 0 0 mgt0.16 172.18.255.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh usbm s USB Modem Attached: NONE USB Network Interface Status: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Tue Apr 17 11:22:12 2012) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Local Link State: UP: 0 times (since Thu Jan 1 08:00:10 1970) DOWN: 0 times (since Thu Jan 1 08:00:10 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: IDLE Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh capw c ¦ in RUN RUN state: Connected securely to the CAPWAP server - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh sys l Brightness configuration: bright System LED status: No Led Colour Blinking --------------------------------------------------------- 1 Power Off No blinking 2 Status White No blinking |  |  |
| Test Result |  |  |  |

UsbModem_Electric_11

| Case ID | UsbModem_Electric_11 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Status of LED with/ without track invalid ip after capwap is enabled, usbmodem is prim WAN |  |  |
| Pre-condition | 1) WAN is available by usbmodem 2) Set the mode of USB modem as primary-wan 3) capwap is enabled initially 4) Unplug eth0 |  |  |
| Test procedure | 1) Startup BR with track valid ip 2) Check the state of WAN and LED 3) Track invalid ip track QS-IP-Track-Router ip 1.1.1.1 4) Repeat step 2) 5) Stop track invalid ip no track QS-IP-Track-Router ip 1.1.1.1 6) Repeat step 2) |  |  |
| Expect result | 1) Before track invalid ip, both WAN and capwap connection are up, and LED is white solid 2) After track invalid ip, WAN is down, and LED is amber solid though capwap connection is up 3) After stop track invalid ip, both WAN and capwap connection are up, and LED is white solid |  |  |
| Test Result |  |  |  |

UsbModem_Electric_12

| Case ID | UsbModem_Electric_12 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | DC power is necessary for BR connected with usbmodem |  |  |
| Pre-condition | 1) Disconnect usbmodem from BR 2) Disconnect DC power from BR |  |  |
| Test procedure | 1) Connect BR with PoE ethernet port 2) Plug usbmodem in the USB port of BR 3) Plug DC power with BR |  |  |
| Expect result | 1) BR is powerd up with PoE 2) usbmodem cannot power up when BR is PoE 3) usbmodem will work when BR is DC power supplied |  |  |
| Test Result |  |  |  |

UsbModem_FailureScenario

UsbModem_FailureScenario_1

| Case ID | UsbModem_FailureScenario_1 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Abnormal 3g WAN when there is no eth0 WAN, on-demand mode |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Ping through eth0 WAN works well AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Trigger failover to usbmodem WAN via shut eth0 2) Continue ping process 3) Disable 3g service (fault the account/ out of service) 4) Check the WAN status |  |  |
| Expect result | 1) WAN is eth0 initially, and usbmodem state is “Backup” and “Attached” AH-8c66c0#sh usbm m huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Fri Mar 30 10:14:53 2012) DOWN: 1 times (since Fri Mar 30 09:44:01 2012) Local Link State: UP: 1 times (since Fri Mar 30 10:14:33 2012) DOWN: 1 times (since Fri Mar 30 09:41:22 2012) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 1 times (since Fri Mar 30 09:44:30 2012) DOWN: 3 times (since Fri Mar 30 10:15:03 2012) Local Link State: UP: 1 times (since Fri Mar 30 09:44:12 2012) DOWN: 1 times (since Fri Mar 30 10:14:53 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 23.15.247.43 with 32 bytes of data: Reply from 23.15.247.43: bytes=32 time=559ms TTL=50 Reply from 23.15.247.43: bytes=32 time=572ms TTL=50 2) Failover to usbmodem WAN successfully AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 11912 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 1848 33 0 0 0 0 33 ¦ 4106 44 0 0 44 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 5 times (since Wed Mar 28 10:43:15 2012) DOWN: 6 times (since Wed Mar 28 10:52:03 2012) Local Link State: UP: 2 times (since Wed Mar 28 10:34:57 2012) DOWN: 2 times (since Wed Mar 28 10:51:53 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 6 times (since Wed Mar 28 10:52:18 2012) DOWN: 10 times (since Wed Mar 28 10:43:23 2012) Local Link State: UP: 6 times (since Wed Mar 28 10:52:13 2012) DOWN: 5 times (since Wed Mar 28 10:43:15 2012) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.35.39 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.57.108 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.35.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 3) Ping works well via usbmodem WAN Reply from 8.8.8.8: bytes=32 time=136ms TTL=45 Reply from 8.8.8.8: bytes=32 time=138ms TTL=45 4) Ping break once 3g service is unavailable Reply from 192.168.85.1: Destination net unreachable. Reply from 192.168.85.1: Destination net unreachable. 5) Keep ppp connecting and won’t failback to eth0 AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 5203 State: S (sleeping) PPP Interface Status: State: down Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 1288 46 0 0 0 0 46 ¦ 542 48 0 0 48 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 2 times (since Thu Apr 5 12:04:56 2012) DOWN: 3 times (since Thu Apr 5 12:09:20 2012) Local Link State: UP: 1 times (since Thu Apr 5 12:04:44 2012) DOWN: 2 times (since Thu Apr 5 12:09:15 2012) Active: no Cached Gateway IP: 10.155.35.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Thu Apr 5 12:09:36 2012) DOWN: 4 times (since Thu Apr 5 12:12:41 2012) Local Link State: UP: 2 times (since Thu Apr 5 12:09:29 2012) DOWN: 2 times (since Thu Apr 5 12:12:38 2012) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTING Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.35.39 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.57.108 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.35.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh log bu ¦ in fosm 2012-04-05 12:15:08 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 2012-04-05 12:15:08 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-05 12:15:08 debug ah_brd: WFOSM: state CONNECTING -> ATTACHED 2012-04-05 12:15:08 debug ah_brd: WFOSM: Added event PPP_STARTED 2012-04-05 12:15:02 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-05 12:15:02 debug ah_brd: WFOSM: Added event START_L3 2012-04-05 12:14:02 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 2012-04-05 12:14:02 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-05 12:14:02 debug ah_brd: WFOSM: state CONNECTING -> ATTACHED 2012-04-05 12:14:02 debug ah_brd: WFOSM: Added event PPP_STARTED 2012-04-05 12:13:56 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-05 12:13:56 debug ah_brd: WFOSM: Added event START_L3 2012-04-05 12:12:57 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 2012-04-05 12:12:57 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-05 12:12:57 debug ah_brd: WFOSM: state CONNECTING -> ATTACHED 2012-04-05 12:12:57 debug ah_brd: WFOSM: Added event PPP_STARTED 2012-04-05 12:12:51 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-05 12:12:51 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 2012-04-05 12:12:51 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-05 12:12:51 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-05 12:12:51 debug ah_brd: WFOSM: state CONNECTING -> ATTACHED 2012-04-05 12:12:51 debug ah_brd: WFOSM: Added event START_L3 2012-04-05 12:12:51 debug ah_brd: WFOSM: state FAILOVER -> CONNECTING 2012-04-05 12:12:51 debug ah_brd: WFOSM: Added event PPP_STARTED 2012-04-05 12:12:41 debug ah_brd: WFOSM: Added event BKUP_WAN_IF_DOWN 2012-04-05 12:12:38 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-05 12:12:38 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-05 12:12:38 debug ah_brd: WFOSM: Added event PPP_IF_DOWN |  |  |
| Test Result |  |  |  |

UsbModem_FailureScenario_2

| Case ID | UsbModem_FailureScenario_2 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Abnormal 3g WAN when there is no eth0 WAN, always-connected mode |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connected 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Ping through eth0 WAN works well AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Trigger failover to usbmodem WAN via shut eth0 2) Continue ping process 3) Disable 3g service (fault the account/ out of service) 4) Check the WAN status |  |  |
| Expect result | 1) WAN is eth0 initially, and usbmodem state is “Backup” and “Connected” AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=34ms TTL=44 Reply from 8.8.8.8: bytes=32 time=33ms TTL=44 2) Failover to usbmodem WAN successfully AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 11912 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 1848 33 0 0 0 0 33 ¦ 4106 44 0 0 44 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 5 times (since Wed Mar 28 10:43:15 2012) DOWN: 6 times (since Wed Mar 28 10:52:03 2012) Local Link State: UP: 2 times (since Wed Mar 28 10:34:57 2012) DOWN: 2 times (since Wed Mar 28 10:51:53 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 6 times (since Wed Mar 28 10:52:18 2012) DOWN: 10 times (since Wed Mar 28 10:43:23 2012) Local Link State: UP: 6 times (since Wed Mar 28 10:52:13 2012) DOWN: 5 times (since Wed Mar 28 10:43:15 2012) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.35.39 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.91 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.35.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 3) Ping works well via usbmodem WAN Reply from 8.8.8.8: bytes=32 time=136ms TTL=45 Reply from 8.8.8.8: bytes=32 time=138ms TTL=45 4) Ping break once 3g service is unavailable Reply from 192.168.85.1: Destination net unreachable. Reply from 192.168.85.1: Destination net unreachable. 5) Keep ppp connecting and won’t failback to eth0 AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 5203 State: S (sleeping) PPP Interface Status: State: down Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 1288 46 0 0 0 0 46 ¦ 542 48 0 0 48 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 2 times (since Thu Apr 5 12:04:56 2012) DOWN: 3 times (since Thu Apr 5 12:09:20 2012) Local Link State: UP: 1 times (since Thu Apr 5 12:04:44 2012) DOWN: 2 times (since Thu Apr 5 12:09:15 2012) Active: no Cached Gateway IP: 10.155.35.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Thu Apr 5 12:09:36 2012) DOWN: 4 times (since Thu Apr 5 12:12:41 2012) Local Link State: UP: 2 times (since Thu Apr 5 12:09:29 2012) DOWN: 2 times (since Thu Apr 5 12:12:38 2012) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTING Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.35.39 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.35.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh log bu ¦ in fosm 2012-04-05 12:28:04 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 2012-04-05 12:28:04 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-05 12:28:04 debug ah_brd: WFOSM: state CONNECTING -> ATTACHED 2012-04-05 12:28:04 debug ah_brd: WFOSM: Added event PPP_STARTED 2012-04-05 12:27:58 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-05 12:27:58 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 2012-04-05 12:27:58 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-05 12:27:58 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-05 12:27:58 debug ah_brd: WFOSM: state CONNECTING -> ATTACHED 2012-04-05 12:27:58 debug ah_brd: WFOSM: Added event START_L3 2012-04-05 12:27:58 debug ah_brd: WFOSM: state FAILOVER -> CONNECTING 2012-04-05 12:27:58 debug ah_brd: WFOSM: Added event PPP_STARTED 2012-04-05 12:27:51 debug ah_brd: WFOSM: Added event BKUP_WAN_IF_DOWN 2012-04-05 12:27:45 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-05 12:27:45 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-05 12:27:45 debug ah_brd: WFOSM: Added event PPP_IF_DOWN |  |  |
| Test Result |  |  |  |

UsbModem_FailureScenario_3

| Case ID | UsbModem_FailureScenario_3 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Abnormal eth0 WAN when there is no 3g WAN, primary-wan mode |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Ping through usbmodem WAN works well AH-8c66c0#sh ver Aerohive Networks Inc. Copyright (C) 2006-2011 Version: HiveOS 5.1r1 release build0720 Build time: Wed Apr 25 17:40:12 UTC 2012 Build cookie: 042512054110 Platform: BR200-WP Bootloader ver: v1.0.2.7 TPM ver: v1.2.35.8 Uptime: 0 weeks, 0 days, 0 hours, 2 minutes, 13 seconds - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing usbmodem mode primary-wan usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Trigger failover to eth0 WAN via disable 3g service 2) Continue ping process 3) Disable eth0 WAN via shut eth0 4) Check the WAN status |  |  |
| Expect result | 1) WAN is usbmodem initially, and usbmodem state is “Primary” and “Connected” AH-8c66c0#sh usbm modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x141b USB Network Interface Status: NONE PPP Process Status: PID: 2328 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17148 143 0 0 0 0 143 ¦ 29671 236 0 0 236 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Jan 1 00:00:52 1970) DOWN: 1 times (since Thu Jan 1 00:00:40 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:43 1970) DOWN: 0 times (since Thu Jan 1 00:00:10 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Apr 26 02:25:02 2012) DOWN: 1 times (since Thu Jan 1 00:00:40 1970) Local Link State: UP: 1 times (since Thu Apr 26 02:24:52 2012) DOWN: 0 times (since Thu Jan 1 00:00:10 1970) Active: no Cached Gateway IP: 10.155.35.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.35.39 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.27.168.61 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.0.1 0.0.0.0 255.255.255.255 UH 0 0 0 tunnel0 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.35.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 0.0.0.0 10.155.35.254 0.0.0.0 UG 32766 0 0 eth0 2) Failover to eth0 WAN successfully when no 3g signal AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x141b USB Network Interface Status: NONE PPP Process Status: PID: 3652 State: S (sleeping) PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: down UP: 1 times (since Thu Jan 1 00:00:52 1970) DOWN: 2 times (since Thu Apr 26 02:31:00 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:00:43 1970) DOWN: 1 times (since Thu Apr 26 02:31:00 2012) Active: no Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Apr 26 02:25:02 2012) DOWN: 1 times (since Thu Jan 1 00:00:40 1970) Local Link State: UP: 1 times (since Thu Apr 26 02:24:52 2012) DOWN: 0 times (since Thu Jan 1 00:00:10 1970) Active: yes Cached Gateway IP: 10.155.35.254 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.35.39 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.35.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.35.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh lo b ¦ i fosm 2012-04-26 02:31:13 debug ah_brd: WFOSM: state FAILOVER -> FAILOVER 2012-04-26 02:31:13 debug ah_brd: WFOSM: state FAILOVER -> FAILOVER 2012-04-26 02:31:13 debug ah_brd: WFOSM: state ATTACHED -> FAILOVER 2012-04-26 02:31:13 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-26 02:31:13 debug ah_brd: WFOSM: state CONNECTED -> ATTACHED 2012-04-26 02:31:13 debug ah_brd: WFOSM: Added event MAIN_WAN_IF_DOWN 2012-04-26 02:31:13 debug ah_brd: WFOSM: Added event PPP_STARTED 2012-04-26 02:31:00 debug ah_brd: WFOSM: Added event MAIN_WAN_IF_DOWN 2012-04-26 02:31:00 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-26 02:31:00 debug ah_brd: WFOSM: Added event PPP_IF_DOWN 2012-04-26 02:25:02 debug ah_brd: WFOSM: state CONNECTED -> CONNECTED 3) Unplug eth0, still won’t failback to usbmodem AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x141b USB Network Interface Status: NONE PPP Process Status: PID: 4747 State: S (sleeping) PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: down UP: 1 times (since Thu Jan 1 00:00:52 1970) DOWN: 2 times (since Thu Apr 26 02:31:00 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:00:43 1970) DOWN: 1 times (since Thu Apr 26 02:31:00 2012) Active: no Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: down UP: 1 times (since Thu Apr 26 02:25:02 2012) DOWN: 2 times (since Thu Apr 26 02:35:57 2012) Local Link State: UP: 1 times (since Thu Apr 26 02:24:52 2012) DOWN: 1 times (since Thu Apr 26 02:35:56 2012) Active: yes Cached Gateway IP: 10.155.35.254 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.35.39 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.35.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.35.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh lo b ¦ i fosm 2012-04-26 02:35:57 debug ah_brd: WFOSM: state FAILOVER -> FAILOVER 2012-04-26 02:35:57 debug ah_brd: WFOSM: Added event BKUP_WAN_IF_DOWN 2012-04-26 02:35:32 debug ah_brd: WFOSM: state CONNECTING -> FAILOVER 2012-04-26 02:35:32 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 2012-04-26 02:35:32 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-26 02:35:32 debug ah_brd: WFOSM: state FAILOVER -> ATTACHED 2012-04-26 02:35:32 debug ah_brd: WFOSM: Added event MAIN_WAN_IF_DOWN |  |  |
| Test Result |  |  |  |

UsbModem_FailureScenario_4

| Case ID | UsbModem_FailureScenario_4 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | No either eth0 or 3g WAN, on-demand mode |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is unavailable 3) Ping through eth0 WAN works well AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Trigger failover to usbmodem WAN via shut eth0 2) Continue ping process 3) Check the WAN status |  |  |
| Expect result | 1) WAN is eth0 initially, and usbmodem state is “Backup” and “Attached” AH-8c66c0#sh usbm m huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Fri Mar 30 10:14:53 2012) DOWN: 1 times (since Fri Mar 30 09:44:01 2012) Local Link State: UP: 1 times (since Fri Mar 30 10:14:33 2012) DOWN: 1 times (since Fri Mar 30 09:41:22 2012) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 1 times (since Fri Mar 30 09:44:30 2012) DOWN: 3 times (since Fri Mar 30 10:15:03 2012) Local Link State: UP: 1 times (since Fri Mar 30 09:44:12 2012) DOWN: 1 times (since Fri Mar 30 10:14:53 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 23.15.247.43 with 32 bytes of data: Reply from 23.15.247.43: bytes=32 time=559ms TTL=50 Reply from 23.15.247.43: bytes=32 time=572ms TTL=50 2) Won’t failover to usbmodem AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 3084 State: S (sleeping) PPP Interface Status: State: down Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 924 33 0 0 0 0 33 ¦ 412 35 0 0 35 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 1 times (since Thu Apr 5 11:49:47 2012) DOWN: 1 times (since Thu Apr 5 11:55:33 2012) Local Link State: UP: 0 times (since Thu Jan 1 00:00:08 1970) DOWN: 1 times (since Thu Apr 5 11:55:28 2012) Active: yes Cached Gateway IP: 10.155.35.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 00:00:08 1970) DOWN: 0 times (since Thu Jan 1 00:00:08 1970) Local Link State: UP: 0 times (since Thu Jan 1 00:00:08 1970) DOWN: 0 times (since Thu Jan 1 00:00:08 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTING Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.35.39 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 0.0.0.0 wan - 0000:0000:0000 D - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.35.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.35.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh log bu ¦ in fosm 2012-04-05 11:55:38 debug ah_brd: WFOSM: state CONNECTING -> CONNECTING 2012-04-05 11:55:38 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 2012-04-05 11:55:38 debug ah_brd: WFOSM: Added event PPP_STARTED 2012-04-05 11:55:33 debug ah_brd: WFOSM: Added event MAIN_WAN_IF_DOWN 3) Ping break since 3g service is either unavailable Reply from 10.155.35.39: Destination host unreachable. Reply from 10.155.35.39: Destination host unreachable. 4) Keep ppp connecting AH-8c66c0#sh log bu ¦ in fosm 2012-04-05 12:01:07 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 2012-04-05 12:01:07 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-05 12:01:07 debug ah_brd: WFOSM: state CONNECTING -> ATTACHED 2012-04-05 12:01:07 debug ah_brd: WFOSM: Added event PPP_STARTED 2012-04-05 12:01:01 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-05 12:01:01 debug ah_brd: WFOSM: Added event START_L3 2012-04-05 12:00:01 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 2012-04-05 12:00:01 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-05 12:00:01 debug ah_brd: WFOSM: state CONNECTING -> ATTACHED 2012-04-05 12:00:01 debug ah_brd: WFOSM: Added event PPP_STARTED 2012-04-05 11:59:55 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-05 11:59:55 debug ah_brd: WFOSM: Added event START_L3 2012-04-05 11:58:55 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING |  |  |
| Test Result |  |  |  |

UsbModem_FailureScenario_5

| Case ID | UsbModem_FailureScenario_5 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | No either eth0 or 3g WAN, always-connected mode |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connected 2) The 3g service is unavailable 3) Ping through eth0 WAN works well AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Trigger failover to usbmodem WAN via shut eth0 2) Continue ping process 3) Check the WAN status |  |  |
| Expect result | 1) WAN is eth0 initially, and usbmodem state is “Backup” and “Connected” AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2287 State: S (sleeping) PPP Interface Status: State: down Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 812 29 0 0 0 0 29 ¦ 372 31 0 0 31 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Apr 5 12:34:03 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTING Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=34ms TTL=44 Reply from 8.8.8.8: bytes=32 time=33ms TTL=44 2) Won’t failover to usbmodem WAN AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2536 State: S (sleeping) PPP Interface Status: State: down Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 1400 50 0 0 0 0 50 ¦ 582 52 0 0 52 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 1 times (since Fri Apr 6 02:29:18 2012) DOWN: 2 times (since Fri Apr 6 02:30:57 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 1 times (since Fri Apr 6 02:30:52 2012) Active: no Cached Gateway IP: 0.0.0.0 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTING Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.35.39 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 0.0.0.0 wan - 0000:0000:0000 D - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.35.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.35.254 0.0.0.0 UG 0 0 0 eth0 3) Ping break since 3g service is either unavailable Reply from 10.155.35.39: Destination host unreachable. Reply from 10.155.35.39: Destination host unreachable. 4) Keep ppp connecting AH-8c66c0#sh log bu ¦ in fosm 2012-04-06 02:31:11 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 2012-04-06 02:31:11 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: ATTACHED 2012-04-06 02:31:11 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:ATTACHED + event:PPP_STARTED [0x10] 2012-04-06 02:31:11 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-06 02:31:11 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: ATTACHED 2012-04-06 02:31:11 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:ATTACHED + event:PPP_STOPPED [0x20] 2012-04-06 02:31:11 debug ah_brd: WFOSM: state CONNECTING -> ATTACHED 2012-04-06 02:31:11 debug ah_brd: [brd_fosm]: WFO: Requested to start PPP 2012-04-06 02:31:11 debug ah_brd: [brd_fosm]: WFO: Start: L3 ifchk timer started: rc=0 2012-04-06 02:31:11 debug ah_brd: [brd_fosm]: WFO: Created L3 ifchk timer to expire in 60 secs 2012-04-06 02:31:11 debug ah_brd: WFOSM: Added event PPP_STARTED 2012-04-06 02:31:07 debug ah_brd: [brd_fosm]: WFO: Ethernet iface eth0 link status is DOWN 2012-04-06 02:31:06 debug ah_brd: [brd_fosm]: WFO: Retrieved USB device id (vid=0x12d1 pid=0x1003) 2012-04-06 02:31:05 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-06 02:31:05 debug ah_brd: [brd_fosm]: WFO: L3 ifchk timer cancelled: rc=0 2012-04-06 02:31:05 debug ah_brd: [brd_fosm]: WFO: L3 ifchk timer destroyed 2012-04-06 02:31:05 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: CONNECTING 2012-04-06 02:31:05 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:CONNECTING + event:START_L3 [0x4] 2012-04-06 02:31:05 debug ah_brd: WFOSM: Added event START_L3 2012-04-06 02:31:05 debug ah_brd: [brd_fosm]: WFO: L3 ifchk timer popped ... 2012-04-06 02:31:02 debug ah_brd: [brd_fosm]: WFO: Ethernet iface eth0 link status is DOWN 2012-04-06 02:30:59 debug ah_brd: [brd_fosm]: WFO: Retrieved USB device id (vid=0x12d1 pid=0x1003) 2012-04-06 02:30:57 debug ah_brd: WFOSM: state CONNECTING -> CONNECTING 2012-04-06 02:30:57 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: CONNECTING 2012-04-06 02:30:57 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:CONNECTING + event:MAIN_WAN_IF_DOWN [0x200] 2012-04-06 02:30:57 debug ah_brd: WFOSM: Added event MAIN_WAN_IF_DOWN 2012-04-06 02:30:57 debug ah_brd: [brd_fosm]: WFO: Ethernet iface eth0 link status is DOWN 2012-04-06 02:30:55 debug ah_brd: WFOSM: state CONNECTING -> CONNECTING 2012-04-06 02:30:55 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: CONNECTING 2012-04-06 02:30:55 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:CONNECTING + event:MAIN_WAN_IF_DOWN [0x200] 2012-04-06 02:30:55 debug ah_brd: WFOSM: Added event MAIN_WAN_IF_DOWN 2012-04-06 02:30:54 debug ah_brd: [brd_fosm]: WFO: Retrieved USB device id (vid=0x12d1 pid=0x1003) 2012-04-06 02:30:52 debug ah_brd: [brd_fosm]: WFO: eth0: Interface DOWN :-( |  |  |
| Test Result |  |  |  |

UsbModem_FailureScenario_6

| Case ID | UsbModem_FailureScenario_6 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | No either eth0 or 3g WAN, primary-wan mode |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) eth0 WAN is unavailable 3) Ping through usbmodem WAN works well AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing usbmodem mode primary-wan usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Trigger failover to eth0 WAN via disable 3g service go to shield room where no 3g signal 2) Continue ping process 3) Check the WAN status |  |  |
| Expect result | 1) WAN is usbmodem initially, and usbmodem state is “Primary” and “Connected” AH-8c66c0#sh usbm m huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2271 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 7147 100 0 0 0 0 100 ¦ 7090 104 0 0 104 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Jan 1 00:00:55 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:44 1970) DOWN: 1 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 0.0.0.0 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.26.176.109 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 23.15.247.43 with 32 bytes of data: Reply from 23.15.247.43: bytes=32 time=218ms TTL=52 Reply from 23.15.247.43: bytes=32 time=218ms TTL=52 2) Won’t failover to eth0 AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2870 State: S (sleeping) PPP Interface Status: State: down Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 420 15 0 0 0 0 15 ¦ 232 17 0 0 17 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: down UP: 1 times (since Thu Jan 1 00:00:55 1970) DOWN: 2 times (since Fri Apr 6 02:18:33 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:00:44 1970) DOWN: 2 times (since Fri Apr 6 02:18:31 2012) Active: no Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTING Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 0.0.0.0 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 0.0.0.0 wan - 0000:0000:0000 D - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 3) Ping break since eth0 WAN is either unavailable Reply from 192.168.85.1: Destination net unreachable. Reply from 192.168.85.1: Destination net unreachable. 4) Keep ppp connecting AH-8c66c0#sh log bu ¦ in fosm 2012-04-06 02:18:50 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 2012-04-06 02:18:50 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: ATTACHED 2012-04-06 02:18:50 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:ATTACHED + event:PPP_STARTED [0x10] 2012-04-06 02:18:50 debug ah_brd: WFOSM: state CONNECTING -> ATTACHED 2012-04-06 02:18:50 debug ah_brd: WFOSM: Added event START_L3 2012-04-06 02:18:50 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: CONNECTING 2012-04-06 02:18:50 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:CONNECTING + event:PPP_STOPPED [0x20] 2012-04-06 02:18:50 debug ah_brd: WFOSM: state CONNECTING -> CONNECTING 2012-04-06 02:18:50 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: CONNECTING 2012-04-06 02:18:50 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:CONNECTING + event:PPP_STARTED [0x10] 2012-04-06 02:18:50 debug ah_brd: WFOSM: state ATTACHED -> CONNECTING 2012-04-06 02:18:50 debug ah_brd: [brd_fosm]: WFO: PPP started 2012-04-06 02:18:50 debug ah_brd: [brd_fosm]: WFO: Start: L3 ifchk timer started: rc=0 2012-04-06 02:18:50 debug ah_brd: [brd_fosm]: WFO: Created L3 ifchk timer to expire in 60 secs 2012-04-06 02:18:50 debug ah_brd: WFOSM: Added event PPP_STARTED 2012-04-06 02:18:48 debug ah_brd: [brd_fosm]: WFO: Ethernet iface eth0 link status is DOWN 2012-04-06 02:18:45 debug ah_brd: [brd_fosm]: WFO: Retrieved USB device id (vid=0x12d1 pid=0x1003) 2012-04-06 02:18:44 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-06 02:18:44 debug ah_brd: [brd_fosm]: WFO: L3 ifchk timer cancelled: rc=0 2012-04-06 02:18:44 debug ah_brd: [brd_fosm]: WFO: L3 ifchk timer destroyed 2012-04-06 02:18:44 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: ATTACHED 2012-04-06 02:18:44 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:ATTACHED + event:MAIN_WAN_IF_DOWN [0x200] 2012-04-06 02:18:44 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-06 02:18:44 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: ATTACHED 2012-04-06 02:18:44 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:ATTACHED + event:PPP_STOPPED [0x20] 2012-04-06 02:18:44 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-06 02:18:44 debug ah_brd: [brd_fosm]: WFO: Mode: primary-wan => PPP started 2012-04-06 02:18:44 debug ah_brd: [brd_fosm]: WFO: Start: L3 ifchk timer started: rc=0 2012-04-06 02:18:44 debug ah_brd: [brd_fosm]: WFO: Created L3 ifchk timer to expire in 60 secs 2012-04-06 02:18:44 debug ah_brd: WFOSM: Added event PPP_STARTED 2012-04-06 02:18:43 debug ah_brd: [brd_fosm]: WFO: Ethernet iface eth0 link status is DOWN 2012-04-06 02:18:39 debug ah_brd: [brd_fosm]: WFO: Retrieved USB device id (vid=0x12d1 pid=0x1003) 2012-04-06 02:18:38 debug ah_brd: [brd_fosm]: WFO: Ethernet iface eth0 link status is DOWN 2012-04-06 02:18:33 debug ah_brd: WFOSM: Added event MAIN_WAN_IF_DOWN 2012-04-06 02:18:33 debug ah_brd: [brd_fosm]: WFO: Ethernet iface eth0 link status is DOWN 2012-04-06 02:18:31 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-06 02:18:31 debug ah_brd: [brd_fosm]: WFO: L3 ifchk timer cancelled: rc=0 2012-04-06 02:18:31 debug ah_brd: [brd_fosm]: WFO: L3 ifchk timer not active 2012-04-06 02:18:31 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: ATTACHED 2012-04-06 02:18:31 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:ATTACHED + event:START_L3 [0x4] 2012-04-06 02:18:31 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-04-06 02:18:31 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: ATTACHED 2012-04-06 02:18:31 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:ATTACHED + event:PPP_STOPPED [0x20] 2012-04-06 02:18:31 debug ah_brd: WFOSM: state CONNECTED -> ATTACHED 2012-04-06 02:18:31 debug ah_brd: WFOSM: Added event START_L3 2012-04-06 02:18:31 debug ah_brd: [brd_fosm]: WFO: Stopping PPP ... 2012-04-06 02:18:31 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-04-06 02:18:31 debug ah_brd: [brd_fosm]: WFO: L3 ifchk timer cancelled: rc=0 2012-04-06 02:18:31 debug ah_brd: [brd_fosm]: WFO: L3 ifchk timer destroyed 2012-04-06 02:18:31 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: CONNECTED 2012-04-06 02:18:31 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:CONNECTED + event:PPP_IF_DOWN [0x80] 2012-04-06 02:18:31 debug ah_brd: [brd_fosm]: WFO: ppp0: Interface DOWN :-( 2012-04-06 02:18:31 debug ah_brd: WFOSM: Added event PPP_IF_DOWN 2012-04-06 02:18:31 debug ah_brd: [brd_fosm]: WFO: ppp0: kevent type=7 is ignored 2012-04-06 02:18:25 debug ah_brd: [brd_fosm]: WFO: Ethernet iface eth0 link status is DOWN 2012-04-06 02:18:23 debug ah_brd: WFOSM: state CONNECTED -> CONNECTED 2012-04-06 02:18:23 debug ah_brd: [brd_fosm]: WFO: Switch route: dft route of gw:0x0 via eth0 not added 2012-04-06 02:18:23 debug ah_brd: [brd_fosm]: WFO: Switch route: ppp0 dft gw IP 0x0 now cached 2012-04-06 02:18:23 debug ah_brd: [brd_fosm]: WFO: Switch route: del dft route via ppp0 (gw:0x0) OK 2012-04-06 02:18:23 debug ah_brd: [brd_fosm]: WFO: Del default route (ifname=ppp0 gwip=0x0) OK 2012-04-06 02:18:23 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: CONNECTED 2012-04-06 02:18:23 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:CONNECTED + event:MAIN_WAN_IF_DOWN [0x200] 2012-04-06 02:18:23 debug ah_brd: WFOSM: Added event MAIN_WAN_IF_DOWN 2012-04-06 02:18:17 debug ah_brd: [brd_fosm]: WFO: Ethernet iface eth0 link status is DOWN |  |  |
| Test Result |  |  |  |

UsbModem_FailureScenario_7

| Case ID | UsbModem_FailureScenario_7 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Track invalid ip, on-demand mode |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is activated and available 3) Ping through eth0 WAN works well AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan ip 23.15.247.43 track wan ip 1.1.1.1 track wan use-for-wan-testing routing route-request enable usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Track an invalid ip 2) Continue ping process 3) Check the WAN status |  |  |
| Expect result | 1) Before track invalid ip, WAN is eth0 AH-8c66c0#sh usbm m huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Fri Mar 30 10:33:57 2012) DOWN: 2 times (since Fri Mar 30 10:29:19 2012) Local Link State: UP: 2 times (since Fri Mar 30 10:33:47 2012) DOWN: 2 times (since Fri Mar 30 10:29:11 2012) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 1 times (since Fri Mar 30 09:44:30 2012) DOWN: 11 times (since Fri Mar 30 10:36:03 2012) Local Link State: UP: 1 times (since Fri Mar 30 09:44:12 2012) DOWN: 1 times (since Fri Mar 30 10:14:53 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=122ms TTL=45 Reply from 8.8.8.8: bytes=32 time=226ms TTL=45 2) Failover to usbmodem WAN once track invalid ip AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 3250 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 11756 160 0 0 0 0 160 ¦ 14177 216 0 0 216 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 1 times (since Sat Mar 31 02:06:50 2012) DOWN: 2 times (since Sat Mar 31 02:11:41 2012) Local Link State: UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 1 times (since Sat Mar 31 02:12:09 2012) Local Link State: UP: 1 times (since Sat Mar 31 02:11:52 2012) DOWN: 1 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.27.134.108 wan - 0000:0000:0000 U AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 32766 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=128ms TTL=45 Reply from 8.8.8.8: bytes=32 time=126ms TTL=45 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - debug console - - - - - - - - - - - - - - - - - - - - - - - - 2012-03-31 02:18:06 debug kernel: L*: (i) eth1 192.168.85.10->8.8.8.8(15124) ttl(128) icmp-echo-req(2/20927) 60 bytes 2012-03-31 02:18:06 debug kernel: [fe]: ip forward: ip pkt with dst-ip 8.8.8.8 received on eth1 2012-03-31 02:18:06 debug kernel: L*: (u) mgt0 192.168.85.10->8.8.8.8(15124) ttl(128) icmp-echo-req(2/20927) 60 bytes 2012-03-31 02:18:06 debug kernel: [fe]: iptables PRE_ROUTING pkt 192.168.85.10(mgt0)->8.8.8.8(<NULL>) proto(1) 60 bytes: ACCEPT 2012-03-31 02:18:06 debug kernel: [fe]: routing done, 192.168.85.10 -> 8.8.8.8 ttl(128) proto(1) mark(0) 60 bytes, found route in table:Main(Split) 2012-03-31 02:18:06 debug kernel: [fe]: create routing cache:192.168.85.10 -> 8.8.8.8 fwmark 0 dev ppp0 2012-03-31 02:18:06 debug kernel: [fe]: iptables FORWARD pkt 192.168.85.10(mgt0)->8.8.8.8(ppp0) proto(1) 60 bytes: ACCEPT 2012-03-31 02:18:06 debug kernel: [fe]: iptables POST_ROUTING pkt 10.27.134.108(<NULL>)->8.8.8.8(ppp0) proto(1) 60 bytes: ACCEPT 2012-03-31 02:18:06 debug kernel: L*: (o) ppp0 0000:7f01:6016->4500:003c:3b14(0x0a1b) 60 bytes 2012-03-31 02:18:06 debug kernel: [fe]: bypass fe egress procesing and deliver packet to stack on ppp0 3) Will not failback to eth0 WAN though track invalid ip AH-8c66c0#sh log bu ¦ in fosm 2012-03-31 02:13:58 debug ah_brd: [brd_fosm]: WFO: Interface ppp0 status is UP (ifr_flags = 0x10d1) 2012-03-31 02:13:58 debug ah_brd: [brd_fosm]: WFO: Retrieved USB device id (vid=0x12d1 pid=0x1003) 2012-03-31 02:12:46 debug ah_brd: [brd_fosm]: WFO: PPP ifchk timer: PPP is UP (IP:0xa1b866c, netmask:0xffffffff) 2012-03-31 02:12:46 debug ah_brd: [brd_fosm]: WFO: Interface ppp0 status is UP (ifr_flags = 0x10d1) 2012-03-31 02:12:46 debug ah_brd: [brd_fosm]: WFO: PPP ifchk timer popped ... 2012-03-31 02:12:09 debug ah_brd: WFOSM: state FAILOVER -> FAILOVER 2012-03-31 02:12:09 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: FAILOVER 2012-03-31 02:12:09 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:FAILOVER + event:BKUP_WAN_IF_DOWN [0x800] 2012-03-31 02:12:09 debug ah_brd: WFOSM: Added event BKUP_WAN_IF_DOWN 2012-03-31 02:11:52 debug ah_brd: WFOSM: state CONNECTED -> FAILOVER 2012-03-31 02:11:52 debug ah_brd: [brd_fosm]: WFO: Failover OK 2012-03-31 02:11:52 debug ah_brd: [brd_fosm]: WFO: Switch route: added dft route (gw:0x0) via ppp0 OK 2012-03-31 02:11:52 debug ah_brd: [brd_fosm]: WFO: Update: state=CONNECTED, WAN index: primary=0 bkup=1 2012-03-31 02:11:52 debug ah_brd: [brd_fosm]: WFO: Update: state=CONNECTED, WAN index: primary=0 bkup=1 2012-03-31 02:11:52 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: CONNECTED 2012-03-31 02:11:52 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:CONNECTED + event:MAIN_WAN_IF_DOWN [0x200] 2012-03-31 02:11:52 debug ah_brd: WFOSM: state CONNECTING -> CONNECTED 2012-03-31 02:11:52 debug ah_brd: [brd_fosm]: WFO: FO: Primary WAN interface (eth0, idx=0) is DOWN 2012-03-31 02:11:52 debug ah_brd: WFOSM: Added event MAIN_WAN_IF_DOWN - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=128ms TTL=45 Reply from 8.8.8.8: bytes=32 time=126ms TTL=45 4) Failback to eth0 once end track invalid ip AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Fri Mar 30 10:33:57 2012) DOWN: 2 times (since Fri Mar 30 10:29:19 2012) Local Link State: UP: 2 times (since Fri Mar 30 10:33:47 2012) DOWN: 2 times (since Fri Mar 30 10:29:11 2012) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 1 times (since Fri Mar 30 09:44:30 2012) DOWN: 11 times (since Fri Mar 30 10:36:03 2012) Local Link State: UP: 1 times (since Fri Mar 30 09:44:12 2012) DOWN: 1 times (since Fri Mar 30 10:14:53 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh log bu ¦ in fosm 2012-03-31 05:38:06 debug ah_brd: [brd_fosm]: WFO: Retrieved USB device id (vid=0x12d1 pid=0x1003) 2012-03-31 05:38:01 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-03-31 05:38:01 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: ATTACHED 2012-03-31 05:38:01 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:ATTACHED + event:BKUP_WAN_IF_DOWN [0x800] 2012-03-31 05:38:01 debug ah_brd: WFOSM: Added event BKUP_WAN_IF_DOWN 2012-03-31 05:37:51 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: ATTACHED 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:ATTACHED + event:START_PPP [0x4] 2012-03-31 05:37:51 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: ATTACHED 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:ATTACHED + event:PPP_STOPPED [0x20] 2012-03-31 05:37:51 debug ah_brd: WFOSM: state ATTACHED -> ATTACHED 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: ATTACHED 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:ATTACHED + event:PPP_STOPPED [0x20] 2012-03-31 05:37:51 debug ah_brd: WFOSM: state CONNECTED -> ATTACHED 2012-03-31 05:37:51 debug ah_brd: WFOSM: Added event START_PPP 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Stopping PPP ... 2012-03-31 05:37:51 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: PPP ifchk timer cancelled: rc=0 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: PPP ifchk timer not active 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: CONNECTED 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:CONNECTED + event:PPP_IF_DOWN [0x80] 2012-03-31 05:37:51 debug ah_brd: WFOSM: state FAILOVER -> CONNECTED 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Stopping PPP ... 2012-03-31 05:37:51 debug ah_brd: WFOSM: Added event PPP_STOPPED 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: ppp0: Interface DOWN :-( 2012-03-31 05:37:51 debug ah_brd: WFOSM: Added event PPP_IF_DOWN 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: ppp0: kevent type=7 is ignored 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: PPP ifchk timer cancelled: rc=0 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: PPP ifchk timer destroyed 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Failback OK 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Switch DNS: previous entries restored OK 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Cleared [2] DNS server IP from current cfg: rc=0 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Found 2 DNS server entries in DNS resolver cfg '/etc/resolv.conf.failback' 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Set [1] DNS sever IP as 0xdd0c21e3 (221.12.33.227) from resolver cfg '/etc/reso lv.conf.failback' 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Set [0] DNS sever IP as 0xdd0c01e3 (221.12.1.227) from resolver cfg '/etc/resol v.conf.failback' 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Switch route: set eth0 dft route zero metric (gw:0xa9b1efe) OK 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Set Metric default route (ifname=eth0 gwip=0xa9b1efe) OK 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Added default route with metric (0) (ifname=eth0 gwip=0xa9b1efe) OK 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Update: state=FAILOVER, WAN index: primary=0 bkup=1 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Recv'd event default gateway route change 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Deleted existing default route (ifname=eth0 gwip=0xa9b1efe) OK 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Update: state=FAILOVER, WAN index: primary=0 bkup=1 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Recv'd event default gateway route change 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Switch route: ppp0 dft gw IP 0x0 now cached 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Switch route: del dft route via ppp0 (gw:0x0) OK 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Del default route (ifname=ppp0 gwip=0x0) OK 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: FAILOVER 2012-03-31 05:37:51 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:FAILOVER + event:MAIN_WAN_IF_UP [0x100] 2012-03-31 05:37:51 debug ah_brd: WFOSM: Added event MAIN_WAN_IF_UP - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=128ms TTL=45 Reply from 8.8.8.8: bytes=32 time=126ms TTL=45 |  |  |
| Test Result |  |  |  |

UsbModem_FailureScenario_8

| Case ID | UsbModem_FailureScenario_8 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Track invalid ip, always-connected mode |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connected 2) The 3g service is activated and available 3) Ping through eth0 WAN works well AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan ip 23.15.247.43 track wan use-for-wan-testing capwap client server port 80 routing route-request enable usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Track an invalid ip 2) Continue ping process 3) Check the WAN status |  |  |
| Expect result | 1) Before track invalid ip, WAN is eth0 AH-8c66c0#sh usbm m huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2290 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 2124 30 0 0 0 0 30 ¦ 2192 32 0 0 32 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Sat Mar 31 05:50:28 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Sat Mar 31 05:50:34 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Sat Mar 31 05:50:26 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.24.6.224 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 23.15.247.43 with 32 bytes of data: Reply from 23.15.247.43: bytes=32 time=366ms TTL=50 Reply from 23.15.247.43: bytes=32 time=367ms TTL=50 2) Failover to usbmodem WAN once track invalid ip AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2290 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 90096 1081 0 0 0 0 1081 ¦ 90986 1091 0 0 1091 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 1 times (since Sat Mar 31 05:50:28 2012) DOWN: 1 times (since Sat Mar 31 07:22:59 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 1 times (since Sat Mar 31 05:50:34 2012) DOWN: 1 times (since Sat Mar 31 07:23:12 2012) Local Link State: UP: 1 times (since Sat Mar 31 05:50:26 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.24.6.224 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 32766 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 23.15.247.43: bytes=32 time=367ms TTL=50 Reply from 23.15.247.43: bytes=32 time=366ms TTL=50 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - debug console - - - - - - - - - - - - - - - - - - - - - - - - 2012-03-31 07:26:36 debug kernel: L*: (i) eth1 192.168.85.10->23.15.247.43(21808) ttl(128) icmp-echo-req(2/26534) 60 bytes 2012-03-31 07:26:36 debug kernel: [fe]: MAC session (id [3]) found 2012-03-31 07:26:36 debug kernel: [fe]: fflow d067:e533:8dd0->0019:778c:66c0 flag 0x41002, rflow 0019:778c:66c0->d067:e533:8dd0 flag 0xc1210 2012-03-31 07:26:36 debug kernel: [fe]: fflow acl 0x0/0x0, rflow acl 0x0/0x0 2012-03-31 07:26:36 debug kernel: [fe]: QoS: ingress pkt fwd(eth1) profile=4096 qos=2 2012-03-31 07:26:36 debug kernel: [fe]: set pkt to self 2012-03-31 07:26:36 debug kernel: [fe]: swap incoming dev eth1 -> mgt0 2012-03-31 07:26:36 debug kernel: [fe]: allow to self-pak, dst-ip 23.15.247.43,allow-arp 0 allow-dhcp 0 allow-forwarding 1 2012-03-31 07:26:36 debug kernel: [fe]: ip forward: ip pkt with dst-ip 23.15.247.43 received on eth1 2012-03-31 07:26:36 debug kernel: [fe]: deliver pak to self on mgt0 with fw mark 0 2012-03-31 07:26:36 debug kernel: L*: (u) mgt0 192.168.85.10->23.15.247.43(21808) ttl(128) icmp-echo-req(2/26534) 60 bytes 2012-03-31 07:26:36 debug kernel: [fe]: iptables PRE_ROUTING pkt 192.168.85.10(mgt0)->23.15.247.43(<NULL>) proto(1) 60 bytes: ACCEPT 2012-03-31 07:26:36 debug kernel: [fe]: iptables FORWARD pkt 192.168.85.10(mgt0)->23.15.247.43(ppp0) proto(1) 60 bytes: ACCEPT 2012-03-31 07:26:36 debug kernel: [fe]: iptables POST_ROUTING pkt 10.24.6.224(<NULL>)->23.15.247.43(ppp0) proto(1) 60 bytes: ACCEPT 2012-03-31 07:26:36 debug kernel: L*: (o) ppp0 0000:7f01:c75e->4500:003c:5530(0x0a18) 60 bytes 2012-03-31 07:26:36 debug kernel: [fe]: bypass fe egress procesing and deliver packet to stack on ppp0 3) Will not failback to eth0 WAN though track invalid ip AH-8c66c0#sh log bu ¦ in fosm 2012-03-31 07:37:36 debug ah_brd: WFOSM: state FAILOVER -> FAILOVER 2012-03-31 07:37:36 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: FAILOVER 2012-03-31 07:37:36 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:FAILOVER + event:BKUP_WAN_IF_DOWN [0x800] 2012-03-31 07:37:36 debug ah_brd: WFOSM: Added event BKUP_WAN_IF_DOWN 2012-03-31 07:37:23 debug ah_brd: WFOSM: state CONNECTED -> FAILOVER 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Failover OK 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Switch DNS: current entries updated OK 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Cleared [2] DNS server IP from current cfg: rc=0 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Found 2 DNS server entries in DNS resolver cfg '/etc/ppp/resolv.conf.ppp0' 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Set [1] DNS sever IP as 0xdd0c21e3 (221.12.33.227) from resolver cfg '/etc/ppp/ resolv.conf.ppp0' 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Set [0] DNS sever IP as 0xdd0c01e3 (221.12.1.227) from resolver cfg '/etc/ppp/r esolv.conf.ppp0' 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Switch DNS: current entries saved OK 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Switch route: added dft route (gw:0x0) via ppp0 OK 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Add default route (ifname=ppp0 gwip=0x0) OK 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Switch route: set dft route max metric via eth0 (gw:0xa9b1efe) OK 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Set Metric default route (ifname=eth0 gwip=0xa9b1efe) OK 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Added default route with metric (32767) (ifname=eth0 gwip=0xa9b1efe) OK 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Update: state=CONNECTED, WAN index: primary=0 bkup=1 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Recv'd event default gateway route change 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Deleted existing default route (ifname=eth0 gwip=0xa9b1efe) OK 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Update: state=CONNECTED, WAN index: primary=0 bkup=1 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Recv'd event default gateway route change 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Interface eth0 status is UP (ifr_flags = 0x1043) 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Ethernet iface eth0 link status is UP 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: CONNECTED 2012-03-31 07:37:23 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:CONNECTED + event:MAIN_WAN_IF_DOWN [0x200] 2012-03-31 07:37:23 debug ah_brd: WFOSM: Added event MAIN_WAN_IF_DOWN - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Pinging 23.15.247.43 with 32 bytes of data: Reply from 23.15.247.43: bytes=32 time=366ms TTL=50 Reply from 23.15.247.43: bytes=32 time=367ms TTL=50 4) Failback to eth0 once end track invalid ip AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2290 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 2124 30 0 0 0 0 30 ¦ 2192 32 0 0 32 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Sat Mar 31 05:50:28 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Sat Mar 31 05:50:34 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Sat Mar 31 05:50:26 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.24.6.224 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh log bu ¦ in log 2012-03-31 07:33:14 info ah_cli: admin:<sh log bu > AH-8c66c0#sh log bu ¦ in fosm 2012-03-31 07:33:12 debug ah_brd: WFOSM: state CONNECTED -> CONNECTED 2012-03-31 07:33:12 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: CONNECTED 2012-03-31 07:33:12 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:CONNECTED + event:BKUP_WAN_IF_UP [0x400] 2012-03-31 07:33:12 debug ah_brd: WFOSM: Added event BKUP_WAN_IF_UP 2012-03-31 07:33:05 debug ah_brd: WFOSM: state FAILOVER -> CONNECTED 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Failback OK 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Switch DNS: previous entries restored OK 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Cleared [2] DNS server IP from current cfg: rc=0 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Cleared [1] DNS server IP from current cfg: rc=0 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Found 1 DNS server entries in DNS resolver cfg '/etc/resolv.conf.failback' 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Set [0] DNS sever IP as 0xa9b03fa (10.155.3.250) from resolver cfg '/etc/resolv .conf.failback' 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Switch route: set eth0 dft route zero metric (gw:0xa9b1efe) OK 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Set Metric default route (ifname=eth0 gwip=0xa9b1efe) OK 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Added default route with metric (0) (ifname=eth0 gwip=0xa9b1efe) OK 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Update: state=FAILOVER, WAN index: primary=0 bkup=1 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Recv'd event default gateway route change 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Deleted existing default route (ifname=eth0 gwip=0xa9b1efe) OK 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Update: state=FAILOVER, WAN index: primary=0 bkup=1 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Recv'd event default gateway route change 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Switch route: ppp0 dft gw IP 0x0 now cached 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Switch route: del dft route via ppp0 (gw:0x0) OK 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Del default route (ifname=ppp0 gwip=0x0) OK 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: FAILOVER 2012-03-31 07:33:05 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:FAILOVER + event:MAIN_WAN_IF_UP [0x100] 2012-03-31 07:33:05 debug ah_brd: WFOSM: Added event MAIN_WAN_IF_UP - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Pinging 23.15.247.43 with 32 bytes of data: Reply from 23.15.247.43: bytes=32 time=366ms TTL=50 Reply from 23.15.247.43: bytes=32 time=367ms TTL=50 |  |  |
| Test Result |  |  |  |

UsbModem_FailureScenario_9

| Case ID | UsbModem_FailureScenario_9 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Track invalid ip, primary-wan mode |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Ping through usbmodem WAN works well AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan ip 23.15.247.43 track wan use-for-wan-testing capwap client server port 80 routing route-request enable usbmodem mode primary-wan usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Track an invalid ip 2) Continue ping process 3) Check the WAN status |  |  |
| Expect result | 1) Before track invalid ip, WAN is usbmodem AH-8c66c0#sh usbm m huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2271 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 21266 225 0 0 0 0 225 ¦ 23379 253 0 0 253 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Sat Mar 31 07:43:48 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Sat Mar 31 07:43:38 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: down UP: 1 times (since Sat Mar 31 07:43:41 2012) DOWN: 1 times (since Sat Mar 31 07:43:56 2012) Local Link State: UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.184.167 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 23.15.247.43 with 32 bytes of data: Reply from 23.15.247.43: bytes=32 time=218ms TTL=52 Reply from 23.15.247.43: bytes=32 time=218ms TTL=52 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - debug console - - - - - - - - - - - - - - - - - - - - - - - - 2012-03-31 08:02:04 debug kernel: L*: (i) eth1 192.168.85.10->23.15.247.43(22691) ttl(128) icmp-echo-req(2/27011) 60 bytes 2012-03-31 08:02:04 debug kernel: [fe]: MAC session (id [2]) found 2012-03-31 08:02:04 debug kernel: [fe]: fflow d067:e533:8dd0->0019:778c:66c0 flag 0x41002, rflow 0019:778c:66c0->d067:e533:8dd0 flag 0xc1210 2012-03-31 08:02:04 debug kernel: [fe]: fflow acl 0x0/0x0, rflow acl 0x0/0x0 2012-03-31 08:02:04 debug kernel: [fe]: QoS: ingress pkt fwd(eth1) profile=4096 qos=2 2012-03-31 08:02:04 debug kernel: [fe]: set pkt to self 2012-03-31 08:02:04 debug kernel: [fe]: swap incoming dev eth1 -> mgt0 2012-03-31 08:02:04 debug kernel: [fe]: allow to self-pak, dst-ip 23.15.247.43,allow-arp 0 allow-dhcp 0 allow-forwarding 1 2012-03-31 08:02:04 debug kernel: [fe]: ip forward: ip pkt with dst-ip 23.15.247.43 received on eth1 2012-03-31 08:02:04 debug kernel: [fe]: deliver pak to self on mgt0 with fw mark 0 2012-03-31 08:02:04 debug kernel: L*: (u) mgt0 192.168.85.10->23.15.247.43(22691) ttl(128) icmp-echo-req(2/27011) 60 bytes 2012-03-31 08:02:04 debug kernel: [fe]: iptables PRE_ROUTING pkt 192.168.85.10(mgt0)->23.15.247.43(<NULL>) proto(1) 60 bytes: ACCEPT 2012-03-31 08:02:04 debug kernel: [fe]: iptables FORWARD pkt 192.168.85.10(mgt0)->23.15.247.43(ppp0) proto(1) 60 bytes: ACCEPT 2012-03-31 08:02:04 debug kernel: [fe]: iptables POST_ROUTING pkt 10.28.184.167(<NULL>)->23.15.247.43(ppp0) proto(1) 60 bytes: ACCEPT 2012-03-31 08:02:04 debug kernel: L*: (o) ppp0 0000:7f01:1220->4500:003c:58a3(0x0a1c) 60 bytes 2012-03-31 08:02:04 debug kernel: [fe]: bypass fe egress procesing and deliver packet to stack on ppp0 2012-03-31 08:02:04 debug ah_brd: [brd_wanmon]: WFOIFM: Ping cmd [/bin/ping -q -c 1 -I ppp0 -w 2 23.15.247.43] executing 2012-03-31 08:02:04 debug kernel: L*: (o) ppp0 0000:4001:edd5->4500:0054:bbd5(0x0a1c) 84 bytes 2012-03-31 08:02:04 debug kernel: [fe]: bypass fe egress procesing and deliver packet to stack on ppp0 2) Failover to eth0 WAN once track invalid ip AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2271 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 63886 810 0 0 0 0 810 ¦ 67078 868 0 0 868 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: down UP: 1 times (since Sat Mar 31 07:43:48 2012) DOWN: 1 times (since Sat Mar 31 08:05:31 2012) Local Link State: UP: 1 times (since Sat Mar 31 07:43:38 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: down UP: 1 times (since Sat Mar 31 07:43:41 2012) DOWN: 1 times (since Sat Mar 31 07:43:56 2012) Local Link State: UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.184.167 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 3) Will not failback to usbmodem WAN though track invalid ip AH-8c66c0#sh log bu ¦ in fosm 2012-03-31 08:09:50 debug ah_brd: WFOSM: state FAILOVER -> FAILOVER 2012-03-31 08:09:50 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: FAILOVER 2012-03-31 08:09:50 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:FAILOVER + event:BKUP_WAN_IF_UP [0x400] 2012-03-31 08:09:50 debug ah_brd: WFOSM: Added event BKUP_WAN_IF_UP - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Pinging 23.15.247.43 with 32 bytes of data: Reply from 23.15.247.43: bytes=32 time=366ms TTL=50 Reply from 23.15.247.43: bytes=32 time=367ms TTL=50 4) Failback to usbmodem once end track invalid ip AH-8c66c0#sh log bu ¦ in fosm 2012-03-31 08:10:05 debug ah_brd: WFOSM: state CONNECTED -> CONNECTED 2012-03-31 08:10:05 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: CONNECTED 2012-03-31 08:10:05 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:CONNECTED + event:BKUP_WAN_IF_DOWN [0x800] 2012-03-31 08:10:05 debug ah_brd: WFOSM: Added event BKUP_WAN_IF_DOWN 2012-03-31 08:09:57 debug ah_brd: WFOSM: state FAILOVER -> CONNECTED 2012-03-31 08:09:57 debug ah_brd: [brd_fosm]: WFO: Failback OK 2012-03-31 08:09:57 debug ah_brd: [brd_fosm]: WFO: Switch DNS: current entries updated OK 2012-03-31 08:09:57 debug ah_brd: [brd_fosm]: WFO: Cleared [2] DNS server IP from current cfg: rc=0 2012-03-31 08:09:57 debug ah_brd: [brd_fosm]: WFO: Found 2 DNS server entries in DNS resolver cfg '/etc/ppp/resolv.conf.ppp0' 2012-03-31 08:09:57 debug ah_brd: [brd_fosm]: WFO: Set [1] DNS sever IP as 0xdd0c21e3 (221.12.33.227) from resolver cfg '/etc/ppp/ resolv.conf.ppp0' 2012-03-31 08:09:57 debug ah_brd: [brd_fosm]: WFO: Set [0] DNS sever IP as 0xdd0c01e3 (221.12.1.227) from resolver cfg '/etc/ppp/r esolv.conf.ppp0' 2012-03-31 08:09:57 debug ah_brd: [brd_fosm]: WFO: Switch DNS: current entries saved OK 2012-03-31 08:09:57 debug ah_brd: [brd_fosm]: WFO: Switch route: added dft route (gw:0x0) via ppp0 OK 2012-03-31 08:09:57 debug ah_brd: [brd_fosm]: WFO: Add default route (ifname=ppp0 gwip=0x0) OK 2012-03-31 08:09:57 debug ah_brd: [brd_fosm]: WFO: Switch route: del dft route via eth0 (gw:0xa9b1efe) OK 2012-03-31 08:09:57 debug ah_brd: [brd_fosm]: WFO: Del default route (ifname=eth0 gwip=0xa9b1efe) OK 2012-03-31 08:09:57 debug ah_brd: [brd_fosm]: WFO: Update: state=FAILOVER, WAN index: primary=1 bkup=0 2012-03-31 08:09:57 debug ah_brd: [brd_fosm]: WFO: Recv'd event default gateway route change 2012-03-31 08:09:57 debug ah_brd: [brd_fosm]: WFO: >>> Enter state: FAILOVER 2012-03-31 08:09:57 debug ah_brd: [brd_fosm]: WFO: Engine: @ state:FAILOVER + event:MAIN_WAN_IF_UP [0x100] 2012-03-31 08:09:57 debug ah_brd: WFOSM: Added event MAIN_WAN_IF_UP - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Pinging 23.15.247.43 with 32 bytes of data: Reply from 23.15.247.43: bytes=32 time=366ms TTL=50 Reply from 23.15.247.43: bytes=32 time=367ms TTL=50 |  |  |
| Test Result |  |  |  |

UsbModem_FailureScenario_10

| Case ID | UsbModem_FailureScenario_10 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Check the WAN state once crash usbmodem process, on-demand mode |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is activated and available AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan ip 23.15.247.43 track wan use-for-wan-testing routing route-request enable usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Check the usbmodem config initially 2) Crash the usbmodem process 3) Check usbmodem config after recover usbmodem process |  |  |
| Expect result | 1) usbmodem is configured as on-demand mode initially AH-8c66c0#sh usbm m huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Sat Mar 31 09:44:29 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 2 times (since Sat Mar 31 09:44:38 2012) Local Link State: UP: 1 times (since Sat Mar 31 09:44:15 2012) DOWN: 1 times (since Sat Mar 31 09:44:29 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#_shell password: BusyBox v1.18.3 (2011-07-11 10:52:56 PDT) built-in shell (ash) Enter 'help' for a list of built-in commands. AH-8c66c0:/tmp/home/admin$ ps ¦ grep brd 1441 root 104m S /opt/ah/bin/ah_brd 2) There is no brd process once kill process ah_brd AH-8c66c0:/tmp/home/admin$ killall ah_brd AH-8c66c0:/tmp/home/admin$ ps ¦ grep brd AH-8c66c0:/tmp/home/admin$ 3) usbmodem config does not lost AH-8c66c0#sh usbm m huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> 4) WAN is still eth0 after restore brd process AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Sat Mar 31 09:44:29 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 2 times (since Sat Mar 31 09:44:38 2012) Local Link State: UP: 1 times (since Sat Mar 31 09:44:15 2012) DOWN: 1 times (since Sat Mar 31 09:44:29 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#_shell password: BusyBox v1.18.3 (2011-07-11 10:52:56 PDT) built-in shell (ash) Enter 'help' for a list of built-in commands. AH-8c66c0:/tmp/home/admin$ ps ¦ grep brd 2842 root 104m S /opt/ah/bin/ah_brd - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - debug console - - - - - - - - - - - - - - - - - - - - - - - - 2012-03-31 08:53:21 debug ah_brd: [brd_wanmon]: WFOIFM: Use cached gw IP 0x0 for eth0 2012-03-31 08:53:21 debug ah_brd: [brd_wanmon]: WFOIFM: Skip test for eth0: no targets defined 2012-03-31 08:53:21 debug ah_brd: [brd_wanmon]: WFOIFM: Use cached gw IP 0x0 for ppp0 2012-03-31 08:53:21 debug ah_brd: [brd_wanmon]: WFOIFM: Skip test for ppp0: no targets defined 2012-03-31 08:53:21 debug ah_brd: [brd_wanmon]: WFOIFM: Going to pause for 30 secs before next test |  |  |
| Test Result |  |  |  |

UsbModem_FailureScenario_11

| Case ID | UsbModem_FailureScenario_11 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Check the WAN state once crash usbmodem process, always-connected mode |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connected 2) The 3g service is activated and available AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan ip 23.15.247.43 track wan use-for-wan-testing routing route-request enable usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Check the usbmodem config initially 2) Crash the usbmodem process 3) Check usbmodem config after recover usbmodem process |  |  |
| Expect result | 1) usbmodem is configured as always-connected mode initially AH-8c66c0#sh usbm m huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2366 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 1872 27 0 0 0 0 27 ¦ 1878 28 0 0 28 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Sat Mar 31 09:31:00 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Sat Mar 31 09:31:07 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Sat Mar 31 09:30:58 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.26.86.120 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#_shell password: BusyBox v1.18.3 (2011-07-11 10:52:56 PDT) built-in shell (ash) Enter 'help' for a list of built-in commands. AH-8c66c0:/tmp/home/admin$ ps ¦ grep brd 1441 root 104m S /opt/ah/bin/ah_brd AH-8c66c0:/tmp/home/admin$ ps ¦ grep pppd 2275 root 4108 S /sbin/pppd noaccomp nopcomp novj nobsdcomp noipdefau 2) There is no brd/ ppd process once kill process ah_brd/ pppd AH-8c66c0:/tmp/home/admin$ killall ah_brd AH-8c66c0:/tmp/home/admin$ killall pppd AH-8c66c0:/tmp/home/admin$ ps ¦ grep brd AH-8c66c0:/tmp/home/admin$ ps ¦ grep pppd AH-8c66c0:/tmp/home/admin$ 3) usbmodem config does not lost AH-8c66c0#sh usbm m huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> 4) WAN is still eth0 after restore brd/ pppd process AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2366 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 1872 27 0 0 0 0 27 ¦ 1878 28 0 0 28 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Sat Mar 31 09:31:00 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Sat Mar 31 09:31:07 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Sat Mar 31 09:30:58 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.26.86.120 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#_shell password: BusyBox v1.18.3 (2011-07-11 10:52:56 PDT) built-in shell (ash) Enter 'help' for a list of built-in commands. AH-8c66c0:/tmp/home/admin$ ps ¦ grep brd 2989 root 104m S /opt/ah/bin/ah_brd AH-8c66c0:/tmp/home/admin$ ps ¦ grep pppd 3196 root 4108 S /sbin/pppd noaccomp nopcomp novj nobsdcomp noipdefau - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - debug console - - - - - - - - - - - - - - - - - - - - - - - - 2012-03-31 08:53:21 debug ah_brd: [brd_wanmon]: WFOIFM: Use cached gw IP 0x0 for eth0 2012-03-31 08:53:21 debug ah_brd: [brd_wanmon]: WFOIFM: Skip test for eth0: no targets defined 2012-03-31 08:53:21 debug ah_brd: [brd_wanmon]: WFOIFM: Use cached gw IP 0x0 for ppp0 2012-03-31 08:53:21 debug ah_brd: [brd_wanmon]: WFOIFM: Skip test for ppp0: no targets defined 2012-03-31 08:53:21 debug ah_brd: [brd_wanmon]: WFOIFM: Going to pause for 30 secs before next test |  |  |
| Test Result |  |  |  |

UsbModem_FailureScenario_12

| Case ID | UsbModem_FailureScenario_12 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Check the WAN state once crash usbmodem process, primary-wan mode |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan ip 23.15.247.43 track wan use-for-wan-testing routing route-request enable usbmodem mode primary-wan usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Check the usbmodem config initially 2) Crash the usbmodem process 3) Check usbmodem config after recover usbmodem process |  |  |
| Expect result | 1) usbmodem is configured as primary-wan mode initially AH-8c66c0#sh usbm m huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2275 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 612 12 0 0 0 0 12 ¦ 852 14 0 0 14 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Sat Mar 31 08:45:39 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Sat Mar 31 08:45:30 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: down UP: 1 times (since Sat Mar 31 08:45:33 2012) DOWN: 1 times (since Sat Mar 31 08:45:47 2012) Local Link State: UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.41.125.72 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#_shell password: BusyBox v1.18.3 (2011-07-11 10:52:56 PDT) built-in shell (ash) Enter 'help' for a list of built-in commands. AH-8c66c0:/tmp/home/admin$ ps ¦ grep brd 1441 root 104m S /opt/ah/bin/ah_brd AH-8c66c0:/tmp/home/admin$ ps ¦ grep pppd 2275 root 4108 S /sbin/pppd noaccomp nopcomp novj nobsdcomp noipdefau 2) There is no brd/ ppd process once kill process ah_brd/ pppd AH-8c66c0:/tmp/home/admin$ killall ah_brd AH-8c66c0:/tmp/home/admin$ killall pppd AH-8c66c0:/tmp/home/admin$ ps ¦ grep brd AH-8c66c0:/tmp/home/admin$ ps ¦ grep pppd AH-8c66c0:/tmp/home/admin$ 3) usbmodem config does not lost AH-8c66c0#sh usbm m huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> 4) WAN is still down even restore brd/ pppd process AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 3091 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 108 6 0 0 0 0 6 ¦ 114 7 0 0 7 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Sat Mar 31 08:50:21 2012) DOWN: 0 times (since Sat Mar 31 08:50:21 2012) Local Link State: UP: 1 times (since Sat Mar 31 08:50:32 2012) DOWN: 0 times (since Sat Mar 31 08:50:21 2012) Active: no Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 0 times (since Sat Mar 31 08:50:21 2012) DOWN: 0 times (since Sat Mar 31 08:50:21 2012) Local Link State: UP: 0 times (since Sat Mar 31 08:50:21 2012) DOWN: 0 times (since Sat Mar 31 08:50:21 2012) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.30.41.171 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#deb co 2012-03-31 08:53:21 debug ah_brd: [brd_wanmon]: WFOIFM: Use cached gw IP 0x0 for eth0 2012-03-31 08:53:21 debug ah_brd: [brd_wanmon]: WFOIFM: Skip test for eth0: no targets defined 2012-03-31 08:53:21 debug ah_brd: [brd_wanmon]: WFOIFM: Use cached gw IP 0x0 for ppp0 2012-03-31 08:53:21 debug ah_brd: [brd_wanmon]: WFOIFM: Skip test for ppp0: no targets defined 2012-03-31 08:53:21 debug ah_brd: [brd_wanmon]: WFOIFM: Going to pause for 30 secs before next test - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#_shell password: BusyBox v1.18.3 (2011-07-11 10:52:56 PDT) built-in shell (ash) Enter 'help' for a list of built-in commands. AH-8c66c0:/tmp/home/admin$ ps ¦ grep brd 2888 root 104m S /opt/ah/bin/ah_brd AH-8c66c0:/tmp/home/admin$ ps ¦ grep pppd 3091 root 4108 S /sbin/pppd noaccomp nopcomp novj nobsdcomp noipdefau - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0:/tmp/home/admin$ ping 8.8.8.8 PING 8.8.8.8 (8.8.8.8): 56 data bytes ping: sendto: Network is unreachable |  |  |
| Test Result |  |  |  |

UsbModem_FailureScenario_13

| Case ID | UsbModem_FailureScenario_13 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Invalid track ip parameter |  |  |
| Pre-condition |  |  |  |
| Test procedure | 1) Config track ip with interval (6s)/ timeout (2s)/ retry (2times) 2) Conifg track ip interval as 181 3) Conifg track ip timeout as 181 4) Conifg track ip retry as 1025 |  |  |
| Expect result | 1) Config track ip successfully with valid para value interval, 1-180 AH-8c66c0#track wan in 1 AH-8c66c0#sh ru ¦ in tr track wan ip 23.15.247.43 track wan interval 1 track wan use-for-wan-testing AH-8c66c0#track wan in 180 AH-8c66c0#sh ru ¦ in tr track wan ip 23.15.247.43 track wan interval 180 track wan use-for-wan-testing timeout, 1-180 AH-8c66c0#track wan in 1 AH-8c66c0#sh ru ¦ in tr track wan ip 23.15.247.43 track wan timeout 1 track wan use-for-wan-testing AH-8c66c0#track wan ti 180 AH-8c66c0#sh ru ¦ in tr track wan ip 23.15.247.43 track wan timeout 180 track wan use-for-wan-testing retry, 0-1024 AH-8c66c0#track wan re 0 AH-8c66c0#sh ru ¦ in tr track wan ip 23.15.247.43 track wan retry 0 track wan use-for-wan-testing AH-8c66c0#track wan re 1024 AH-8c66c0#sh ru ¦ in tr track wan ip 23.15.247.43 track wan retry 1024 track wan use-for-wan-testing 2) Fail to config paras with invalid value interval, AH-8c66c0#track wan in 0 ^-- unknown keyword or invalid input AH-8c66c0#track wan in 181 ^-- unknown keyword or invalid input AH-8c66c0#track wan in 1800 ^-- unknown keyword or invalid input timeout, AH-8c66c0#track wan ti 0 ^-- unknown keyword or invalid input AH-8c66c0#track wan ti 181 ^-- unknown keyword or invalid input AH-8c66c0#track wan ti 1800 ^-- unknown keyword or invalid input retry, AH-8c66c0#track wan re 1025 ^-- unknown keyword or invalid input AH-8c66c0#track wan re 10240 ^-- unknown keyword or invalid input |  |  |
| Test Result |  |  |  |

UsbModem_Stress

UsbModem_Stress_1

| Case ID | UsbModem_Stress_1 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Ping duration via 3g WAN |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary |  |  |
| Test procedure | 1) Client get association via eth1 2) Ping an internal address which is in the vlan of BR for 24 hours 3) Ping an internet address for 24 hours 4) Repeat step 2)-3) for a client associated via wifi0 |  |  |
| Expect result | 1) Client get association via eth1 or wifi0 successfully 2) No error with duration ping |  |  |
| Test Result |  |  |  |

UsbModem_Stress_2

| Case ID | UsbModem_Stress_2 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | All eth ports connected via 3g WAN |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server port 80 usbmodem mode primary-wan usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Check the WAN status as usbmodem is the default WAN 2) Get clients connected with eth port 3) On each client, try to download file “QQ2012Beta1_QQProtect2.5.exe” http://im.qq.com/qq/2012/safe/ |  |  |
| Expect result | 1) All clients get connection successfully 2) Http download successfully on each client AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - WAN is usbmodem - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.138.74 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - 2 clients connected - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in eth1 mac-learning al NO mac address Vlan ageout 1 d067:e533:8dd0 1 148 AH-8c66c0#sh in eth2 mac-learning al NO mac address Vlan ageout 1 d067:e531:4cf3 1 171 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh arp-cache ¦ in 192 0019:778c:66c0 862 MGT0 192.168.85.1 d067:e531:4cf3 880 ETH 192.168.85.11 d067:e533:8dd0 804 ETH 192.168.85.12 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Download file on clients successfully- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - |  |  |
| Test Result |  |  |  |

UsbModem_Stress_3

| Case ID | UsbModem_Stress_3 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Wifi connected via 3g WAN with filled ssid |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Config 16 ssid for wifi0 AH-8c66c0#sh ru ssid usb interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid usb report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 23.15.247.43 track wan_failover_test interval 1 track wan_failover_test use-for-wan-testing usbmodem mode primary-wan usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Check the WAN status as usbmodem is the default WAN 2) Get clients associated with ssid 3) On each client, try to download file “QQ2012Beta1_QQProtect2.5.exe” http://im.qq.com/qq/2012/safe/ |  |  |
| Expect result | 1) All clients get association successfully 2) Http download successfully on each client AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - WAN is usbmodem - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.138.74 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - 2 clients associated - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=usb: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.13 1 195M 175.5M -25(59) open none 00:04:49 1 Yes 0 11ng disabled 20MHz No Good 0024:d7f4:3114 192.168.85.14 1 195M 6M -26(58) open none 00:04:42 1 Yes 0 11ng disabled 20MHz No Good - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Download file on clients successfully- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - |  |  |
| Test Result |  |  |  |

UsbModem_Stress_4

| Case ID | UsbModem_Stress_4 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | All eth ports and wifi connected via 3g WAN |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Config 16 ssid for wifi0 AH-8c66c0#sh ru ssid usb interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid usb report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 23.15.247.43 track wan_failover_test interval 1 track wan_failover_test use-for-wan-testing usbmodem mode primary-wan usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Check the WAN status as usbmodem is the default WAN 2) Get clients connected with each eth port (eth1-eth4) 3) Get clients associated with ssid (wifi0.1-wifi0.4) 4) On each client, try to download file “QQ2012Beta1_QQProtect2.5.exe” http://im.qq.com/qq/2012/safe/ |  |  |
| Expect result | 1) All clients get connection/ association successfully 2) Http download successfully on each client AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - WAN is usbmodem - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.138.74 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - 2 clients associated - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in eth2 mac-learning al NO mac address Vlan ageout 1 d067:e533:8d7e 1 163 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh arp-cache ¦ in 8d7e d067:e533:8d7e 844 ETH 192.168.85.10 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=usb: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.13 1 195M 195M -25(59) open none 00:11:04 1 Yes 0 11ng disabled 20MHz No Good - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Download file on clients successfully- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - |  |  |
| Test Result |  |  |  |

UsbModem_Stress_5

| Case ID | UsbModem_Stress_5 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Repetition failover and back, on-demand mode |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Trigger failover to usbmodem WAN via shut eth0 2) Trigger failback to eth WAN via enable eth0 3) Repeat step 1)-2) for 100 times |  |  |
| Expect result | 1) No error with failover 2) No error with failback AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Before failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 11912 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 1848 33 0 0 0 0 33 ¦ 4106 44 0 0 44 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 5 times (since Wed Mar 28 10:43:15 2012) DOWN: 6 times (since Wed Mar 28 10:52:03 2012) Local Link State: UP: 2 times (since Wed Mar 28 10:34:57 2012) DOWN: 2 times (since Wed Mar 28 10:51:53 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 6 times (since Wed Mar 28 10:52:18 2012) DOWN: 10 times (since Wed Mar 28 10:43:23 2012) Local Link State: UP: 6 times (since Wed Mar 28 10:52:13 2012) DOWN: 5 times (since Wed Mar 28 10:43:15 2012) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failback - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 |  |  |
| Test Result |  |  |  |

UsbModem_Stress_6

| Case ID | UsbModem_Stress_6 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Repetition failover and back, always-connected mode |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connected 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Trigger failover to usbmodem WAN via shut eth0 2) Trigger failback to eth WAN via enable eth0 3) Repeat step 1)-2) for 100 times |  |  |
| Expect result | 1) No error with failover 2) No error with failback AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Before failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 61906 513 0 0 0 0 513 ¦ 52199 501 0 0 501 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 2 times (since Thu Mar 29 07:17:34 2012) DOWN: 2 times (since Thu Mar 29 07:27:28 2012) Local Link State: UP: 2 times (since Thu Mar 29 07:17:17 2012) DOWN: 2 times (since Thu Mar 29 07:27:15 2012) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 D Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failback - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 |  |  |
| Test Result |  |  |  |

UsbModem_Stress_7

| Case ID | UsbModem_Stress_7 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Repetition failover and back, primary-wan mode |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing usbmodem mode primary-wan usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Trigger failover to eth WAN via shut usbmodem 2) Trigger failback to usbmodem WAN via enable usbmodem 3) Repeat step 1)-2) for 100 times |  |  |
| Expect result | 1) No error with failover 2) No error with failback AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Before failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.138.74 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failover - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: NONE PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: down UP: 3 times (since Thu Mar 29 08:58:04 2012) DOWN: 3 times (since Thu Mar 29 08:58:48 2012) Local Link State: UP: 3 times (since Thu Mar 29 08:57:59 2012) DOWN: 3 times (since Thu Mar 29 08:58:38 2012) Active: no Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - After failback - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.138.74 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 |  |  |
| Test Result |  |  |  |

UsbModem_Stress_8

| Case ID | UsbModem_Stress_8 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Track an invalid ip for long time then track a valid ip, on-demand mode |  |  |
| Pre-condition | 1) Config usbmodem mode as on-demand 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Ping through eth0 WAN works well AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test ip 1.1.1.1 track wan_failover_test interval 1 track wan_failover_test multi-dst-logic and track wan_failover_test use-for-wan-testing usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Track an invalid ip as well as ping internet from client track wan_failover_test ip 1.1.1.1 track wan_failover_test use-for-wan-testing 2) Check the usbmodem/ WAN status and continue ping process 3) After 24 hours, change track ip to 8.8.8.8 track wan_failover_test ip 1.1.1.1 track wan_failover_test use-for-wan-testing 4) Repeat step 2) |  |  |
| Expect result | 1) WAN is eth0 initially before track invalid ip,and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 2 times (since Wed Mar 28 09:57:31 2012) DOWN: 2 times (since Wed Mar 28 09:57:22 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:03:38 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 2 times (since Wed Mar 28 09:57:38 2012) DOWN: 4 times (since Wed Mar 28 10:17:18 2012) Local Link State: UP: 2 times (since Wed Mar 28 09:57:33 2012) DOWN: 2 times (since Wed Mar 28 10:17:08 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=127ms TTL=44 Reply from 8.8.8.8: bytes=32 time=139ms TTL=44 2) WAN failover to usbmodem once track invalid ip 3) Keep WAN as usbmodem during track invalid ip,and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2899 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 840 17 0 0 0 0 17 ¦ 846 18 0 0 18 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 1 times (since Thu Apr 5 06:28:18 2012) DOWN: 1 times (since Thu Apr 5 06:32:19 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 0 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Apr 5 06:32:30 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.210.238 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping after failover - - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=136ms TTL=45 Reply from 8.8.8.8: bytes=32 time=138ms TTL=45 4) WAN failback to eth0 once track valid ip,and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: NONE PPP Interface Status: NONE WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 6 times (since Wed Mar 28 10:55:49 2012) DOWN: 6 times (since Wed Mar 28 10:52:03 2012) Local Link State: UP: 3 times (since Wed Mar 28 10:55:42 2012) DOWN: 2 times (since Wed Mar 28 10:51:53 2012) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 6 times (since Wed Mar 28 10:52:18 2012) DOWN: 12 times (since Wed Mar 28 10:56:41 2012) Local Link State: UP: 6 times (since Wed Mar 28 10:52:13 2012) DOWN: 6 times (since Wed Mar 28 10:55:49 2012) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: ATTACHED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=32ms TTL=44 Reply from 8.8.8.8: bytes=32 time=32ms TTL=44 |  |  |
| Test Result |  |  |  |

UsbModem_Stress_9

| Case ID | UsbModem_Stress_9 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Track an invalid ip for long time then track a valid ip, always-connected mode |  |  |
| Pre-condition | 1) Config usbmodem mode as always-connected 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Ping through eth0 WAN works well AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test ip 1.1.1.1 track wan_failover_test interval 1 track wan_failover_test multi-dst-logic and track wan_failover_test use-for-wan-testing usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Track an invalid ip as well as ping internet from client track wan_failover_test ip 1.1.1.1 track wan_failover_test use-for-wan-testing 2) Check the usbmodem/ WAN status and continue ping process 3) After 24 hours, change track ip to 8.8.8.8 track wan_failover_test ip 1.1.1.1 track wan_failover_test use-for-wan-testing 4) Repeat step 2) |  |  |
| Expect result | 1) WAN is eth0 initially before track invalid ip,and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=34ms TTL=44 Reply from 8.8.8.8: bytes=32 time=33ms TTL=44 2) WAN failover to usbmodem once track invalid ip 3) Keep WAN as usbmodem during track invalid ip,and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2348 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 2287 34 0 0 0 0 34 ¦ 4209 60 0 0 60 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: down UP: 1 times (since Thu Apr 5 06:38:19 2012) DOWN: 1 times (since Thu Apr 5 06:39:40 2012) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: down UP: 1 times (since Thu Apr 5 06:38:31 2012) DOWN: 1 times (since Thu Apr 5 06:39:53 2012) Local Link State: UP: 1 times (since Thu Apr 5 06:38:11 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.17.87 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping after failover - - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=136ms TTL=45 Reply from 8.8.8.8: bytes=32 time=138ms TTL=45 4) WAN failback to eth0 once track valid ip,and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2307 State: R (running) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 17748 216 0 0 0 0 216 ¦ 17754 217 0 0 217 0 0 WAN Interface(s) Status: [Primary] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 10.155.30.254 [Backup] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 06:57:18 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 06:57:12 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 0.0.0.0 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.28.188.69 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=32ms TTL=44 Reply from 8.8.8.8: bytes=32 time=32ms TTL=44 |  |  |
| Test Result |  |  |  |

UsbModem_Stress_10

| Case ID | UsbModem_Stress_10 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | No |
| Topology to use | Topo1/ Topo2 |  |  |
| Description | Track an invalid ip for long time then track a valid ip, primary-wan mode |  |  |
| Pre-condition | 1) Config usbmodem mode as primary-wan 2) The 3g service is activated and available 3) Configured the dialup username/ passwd/ number if the modem if necessary 4) Ping through usbmodem WAN works well AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable report statistic period 1 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track wan_failover_test ip 8.8.8.8 track wan_failover_test interval 5 track wan_failover_test use-for-wan-testing capwap client server port 80 usbmodem mode primary-wan usbmodem modem-id huawei_e220 apn 3gnet |  |  |
| Test procedure | 1) Track an invalid ip as well as ping internet from client track wan_failover_test ip 1.1.1.1 track wan_failover_test use-for-wan-testing 2) Check the usbmodem/ WAN status and continue ping process 3) After 24 hours, change track ip to 8.8.8.8 track wan_failover_test ip 1.1.1.1 track wan_failover_test use-for-wan-testing 4) Repeat step 2) |  |  |
| Expect result | 1) WAN is usbmodem initially before track invalid ip,and ping without error AH-8c66c0#sh usbm modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.138.74 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Ping Internet from Client - - - - - - - - - - - - - - - - - - Pinging 8.8.8.8 with 32 bytes of data: Reply from 8.8.8.8: bytes=32 time=156ms TTL=44 Reply from 8.8.8.8: bytes=32 time=172ms TTL=44 2) WAN failover to eth0 once track invalid ip 3) Keep WAN as eth0 during track invalid ip,and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 4542 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 2480 30 0 0 0 0 30 ¦ 3637 56 0 0 56 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: down UP: 1 times (since Thu Apr 5 05:55:04 2012) DOWN: 1 times (since Thu Apr 5 05:56:43 2012) Local Link State: UP: 1 times (since Thu Apr 5 05:54:53 2012) DOWN: 0 times (since Thu Jan 1 00:00:08 1970) Active: no Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: down UP: 2 times (since Thu Apr 5 05:48:51 2012) DOWN: 2 times (since Thu Apr 5 05:56:30 2012) Local Link State: UP: 0 times (since Thu Jan 1 00:00:08 1970) DOWN: 0 times (since Thu Jan 1 00:00:08 1970) Active: yes Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: FAILOVER Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.33.44.195 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 0 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=36ms TTL=44 Reply from 8.8.8.8: bytes=32 time=33ms TTL=44 4) WAN failback to usbmodem once track valid ip,and ping without error AH-8c66c0#sh usbm s USB Modem Attached: Vendor ID: 0x12d1 Product ID: 0x1003 PPP Process Status: PID: 2304 State: S (sleeping) PPP Interface Status: State: up Statistics: IN PACK VJCOMP VJUNC VJERR VJTOSS NON-VJ ¦ OUT PACK VJCOMP VJUNC NON-VJ VJSRCH VJMISS 39242 184 0 0 0 0 184 ¦ 20268 115 0 0 115 0 0 WAN Interface(s) Status: [Primary] Interface: ppp0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Mar 29 08:38:33 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: yes Cached Gateway IP: 0.0.0.0 [Backup] Interface: eth0 WAN Monitor State: up UP: 1 times (since Thu Mar 29 08:38:35 2012) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Local Link State: UP: 1 times (since Thu Jan 1 00:00:09 1970) DOWN: 0 times (since Thu Jan 1 00:00:09 1970) Active: no Cached Gateway IP: 10.155.30.254 Failover SM Snapshot: Current State: CONNECTED Pending Event(s): NONE - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 in Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- Eth0 10.155.30.55 wan - 0019:778c:66c0 U Mgt0 192.168.85.1 - 1 0019:778c:66c0 U Ppp0 10.25.138.74 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 0.0.0.0 0.0.0.0 U 0 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Continue Ping Process - - - - - - - - - - - - - - - - - - - - - Reply from 8.8.8.8: bytes=32 time=156ms TTL=44 Reply from 8.8.8.8: bytes=32 time=172ms TTL=44 |  |  |
| Test Result |  |  |  |