AerohiveNetworks Inc.

QA TestCase Template

Revision History

| Version | Date | Author | Comments |
| --- | --- | --- | --- |
| 0.1 | 9/17/2012 | Zhonghua Xu | Initial draft |
| 0.2 | 9/24/2012 | Zhonghua Xu | Update after review |
| 0.3 | 10/9/2012 | Zhonghua Xu | Finalize before test |
| 0.4 | 10/29/2012 | Zhonghua Xu | Remove platform BRAP121/ BRAP141, and related cases |
| 0.5 | 11/14/2012 | Zhonghua Xu | Remove DHCP related cases from Function suite |
| 0.6 | 02/22/2013 | Zhonghua Xu | Add log for some cases |
| 0.7 | 03/06/2013 | Zhonghua Xu | Add log for some cases |
| 0.8 | 04/10/2013 | Zhonghua Xu | Add track-wan related case in 6.2.3 for bug 25662, 6.2.3.6~ 6.2.3.12 |

# Table of Contents

Glossary and Abbreviations

# Introduction

Currently BR products only support two types of WAN interface: eth0 and ppp0 (or usb0 for USB modem with directIP support). Policy based routing support will allow multiple WAN interfaces by enable any ethernet interface into WAN mode which includes ethX, usbnetX and wifiX.

# TestStrategy

# Test Acceptance Criterion from Development

- Approved – MRD

https://intranet-aerohive.jivesbs.com/docs/DOC-3993

- Approved – Functional Specifications

http://saturn.aerohive.com/view.php?fDocumentId=3985

- Approved – Unit Test Plans

http://saturn.aerohive.com/view.php?fDocumentId=4084

- CLI Review-Accept

http://saturn.aerohive.com/view.php?fDocumentId=4001

# Product Pass Criterion

Meet all objects in marketing requirement or function spec which may include key function objectives, capacity objectives,performance objectives and so on.

# Test Bed/Topo Design

Topo 1

Topo2

Topo3

Topo4

Topo5

# TestCase

<Good organization should be able to reflect your test strategy>

Case table format:

| CaseID | FeatureName_TestType_<Customization string>_Number |  |  |
| --- | --- | --- | --- |
| Priority | Accept/High/Middle/Low | Automation Flag | Yes/NA |
| Topology | TopoID or Simple Topo, the minimum devices set of the current case Should add test tools’ description if there are some special requirements for it |  |  |
| Description | Objective for this test case |  |  |
| PlatformDependence | Supported platforms |  |  |
| Pre-condition | Initial configuration or pre-test configuration |  |  |
| TestProcedure | Test steps and procedure which should use the following format strictly, otherwise automation team may encounter issues while translating case to xml file) 1. step1 description “cli cmd” if you have, result 1 2. step2 description “cli cmd” if you have, result 2 3. step3 description “cli cmd” if you have, result 3 |  |  |
| ExpectResult | Result 1: The expect test result for step 1 Result 2: The expect test result for step 2 Result 3: The expect test result for step 3 The expect test result, assume there is no bug or defect, should include the necessary debug info and log |  |  |
| TestResult | The real test result, list all bugs or defects and their influence |  |  |
| Comment | Other special things should be attention |  |  |

<comments about case table:

1: Priority has four levels: Accept,High,Middle,Low. “Accept” cases will be executed in acceptance testing, any one of these fails, will cause denial of feature testing.For “Accept” cases, should be less than 10% in total cases number. For “Accept” and “High” cases, should be less than 30% in total cases number.

2: Automation flag, yes means it is able to be automated; NA means it is not able to be automated.

3:spaces or special characters should not be used as part of case ID

4: FeatureName should be full name of a specific feature, such as “HighDensity”, rather than “HD”, or “SpectrumAnalysis” rather than “SA”

5: TestType should be one of “Solution”,”Function”,”Stress”,”Longevity”,”Performance”,”Capacity”,”Compatibility”, ”Negative”, “Other”, ”Cli”,”GUI”, “Typical issue”.>

## Solution

### 6.1.1. USB is Backup Mode

#### WanRedundancy_Solution_UsbIsBackup_01

| CaseID | WanRedundancy_Solution_UsbIsBackup_01 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when no interface available |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. No eth port is connected 2. No usb modem is plugged 3. No wifi wan is enabled |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Check wan state, as well as monitor state, result 2 3. Check default route in main route table, result 3 4. Run ping internet process on client to check its accessibility, result 4 5. Visit http on client, result 5 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: wan state is “no wan”, and both monitor state and default monitor state of each interface are down AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wanD - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Wed Jan 23 03:52:41 2013 last_event: BACKUP_WAN_DOWN last_evt_time: Wed Jan 23 03:52:41 2013 Result 3: no default route AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 4: ping internet will be reponsed with mgt0 Result 5: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_02

| CaseID | WanRedundancy_Solution_UsbIsBackup_02 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is only eth0 available, and both track wan and track default gateway for eth0 are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config track wan group for eth0, result 2 3. Check wan state, as well as monitor state, result 3 4. Check default route in main route table, result 4 5. Run ping internet process on client to check its accessibility, result 5 6. Visit http on client, result 6 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: Eth0 is configured with track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 3: Active wan is eth0, and both monitor state and default monitor state of eth0 are Up AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Interface: eth0; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Jan 23 18:00:47 2013 last_event: BACKUP_WAN_DOWN last_evt_time: Wed Jan 23 18:00:47 2013 Result 4: Default route is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 00 eth0 Result 5: ping internet successfully on client Result 6: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_03

| CaseID | WanRedundancy_Solution_UsbIsBackup_03 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is only eth0 available, and track wan for eth0 is down while track default gateway is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config track wan group for eth0, result 2 3. Track wan fail on eth0, result 3 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 4. Check wan state, as well as monitor state, result 4 5. Check default route in main route table, result 5 6. Run ping internet process on client to check its accessibility, result 6 7. Visit http on client, result 7 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: Eth0 is configured with track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 3: Track wan on eth0 unsuccessfully AH-8c8040#sh lo b ¦ i eth0 2013-01-23 19:56:24 err ah_brd: WFOIFM: Failed to launch arping cmd [/bin/ping -q -c 1 -I eth0 -w 2 1.1.1.1 2>&1 > /tmp/brd_wan_ifmon_ping_3069] 2013-01-23 19:56:22 err ah_brd: WFOIFM: Failed to launch arping cmd [/bin/ping -q -c 1 -I eth0 -w 2 1.1.1.1 2>&1 > /tmp/brd_wan_ifmon_ping_2885] 2013-01-23 19:56:22 debug ah_brd: [brd_wanmon]: WFOIFM: Arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.31.254 2>&1 > /tmp/brd_wan_ifmon_arping_3670] executed 2013-01-23 19:56:21 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Going to test 1 targets for eth0 ... 2013-01-23 19:56:21 debug ah_brd: [brd_wanmon]: WFOIFM: Interface eth0 allows ARP (ifr_flags = 0x1043) 2013-01-23 19:56:21 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa9b1ffe for eth0 2013-01-23 19:56:21 debug ah_brd: [brd_wanmon]: WFOIFM: send event AH_EVENT_WAN_IFMON_IF_STATE_CHANGE success. ifmon_type: 0, ifmon_cfg_name: DFT_GW_TRACK_IP, kifname: eth0, if_status: 1, if_status_pre: 0 2013-01-23 19:56:21 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth0: 1 targets tested, result: PASS 2013-01-23 19:56:20 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Going to test 1 targets for eth0 ... 2013-01-23 19:56:20 debug ah_brd: [brd_wanmon]: WFOIFM: Interface eth0 allows ARP (ifr_flags = 0x1043) 2013-01-23 19:56:20 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Determined current gw IP as 0xa9b1ffe for eth0 2013-01-23 19:56:20 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-eth0] thread begin to do test. 2013-01-23 19:56:20 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL Result 4: wan state is “no wan”, monitor state of eth0 is Down, default monitor state of eth0 is Up AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: down Interface: eth0; State: unknown [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: NOWAN prev_state_time: Wed Jan 23 19:46:24 2013 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Jan 23 19:46:24 2013 Result 5: Default route is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 6: ping internet successfully on client Result 7: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_04

| CaseID | WanRedundancy_Solution_UsbIsBackup_04 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is only eth0 available, and both track wan and track default gateway for eth0 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config track wan group for eth0, result 2 3. Track wan fail on eth0, result 3 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 4. Track default gateway fail on eth0, result 4 no in eth0 dhcp client 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: Eth0 is configured with track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 1.1.1.1 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 3: Track wan on eth0 unsuccessfully AH-8c8040#sh lo b 2013-01-23 20:02:18 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-eth0] thread wait last_test_time: 1358971338, waittime: 1358971339 2013-01-23 20:02:18 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-23 20:02:18 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-eth0] thread begin to do test. 2013-01-23 20:02:17 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-usbnet0] thread wait last_test_time: 1358971337, waittime: 1358971342 2013-01-23 20:02:17 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. 2013-01-23 20:02:17 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-usbnet0] thread begin to do test. 2013-01-23 20:02:17 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-eth0] thread wait last_test_time: 1358971337, waittime: 1358971338 2013-01-23 20:02:17 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-23 20:02:17 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-eth0] thread begin to do test. 2013-01-23 20:02:16 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread wait last_test_time: 1358971336, waittime: 1358971342 2013-01-23 20:02:16 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. 2013-01-23 20:02:16 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread begin to do test. Result 4: Track default gateway on eth0 unsuccessfully AH-8c8040#sh lo b 2013-01-23 20:02:18 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-eth0] thread wait last_test_time: 1358971338, waittime: 1358971339 2013-01-23 20:02:18 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-23 20:02:18 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-eth0] thread begin to do test. 2013-01-23 20:02:17 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-usbnet0] thread wait last_test_time: 1358971337, waittime: 1358971342 2013-01-23 20:02:17 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. 2013-01-23 20:02:17 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-usbnet0] thread begin to do test. 2013-01-23 20:02:17 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-eth0] thread wait last_test_time: 1358971337, waittime: 1358971338 2013-01-23 20:02:17 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-23 20:02:17 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-eth0] thread begin to do test. 2013-01-23 20:02:16 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread wait last_test_time: 1358971336, waittime: 1358971342 2013-01-23 20:02:16 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. 2013-01-23 20:02:16 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread begin to do test. Result 5: wan state is “no wan”, and both monitor state and default monitor state of eth0 are Down AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wanU - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: down Interface: eth0; State: unknown [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Wed Jan 23 20:00:22 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Wed Jan 23 20:00:22 2013 Result 6: no default gw available AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 7: ping internet will be reponsed with mgt0 Result 8: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_05

| CaseID | WanRedundancy_Solution_UsbIsBackup_05 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0 and usb available, and both track wan and track default gateway for eth0 are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config track wan group each interface, result 3 4. Check wan state, as well as monitor state, result 4 5. Check default route in main route table, result 5 6. Run ping internet process on client to check its accessibility, result 6 7. Visit http on client, result 7 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Active wan is eth0, and both monitor state and default monitor state of eth0 are Up AH-8c8040#sh wan in ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: usbnet0 kifname: usbnet0 …. priority: 600 is_wan: 1 is_default_wan: 0 … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 5: Default route is via eth0, and there is non-default route via usbnet with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 6: ping internet successfully on client Result 7: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_06

| CaseID | WanRedundancy_Solution_UsbIsBackup_06 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0 and usb available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e220 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e220 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x1003 APN: 3gnet APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 5: wan failover to eth0, both monitor state and default monitor state of usbnet are Up Result 6: Default route is via usbnet, and there is non-default route via eth0 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_07

| CaseID | WanRedundancy_Solution_UsbIsBackup_07 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0 and usb available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | - Get a client associated to BR with open auth, result 1 - Config usb modem as always-connect mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 5. Track wan fail on usbnet, result 5 track-wan use-for-wan-usbnet0 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 5: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-26 19:34:13 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” Result 7: Default route is via eth0, and there is non-default route via usbnet with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_08

| CaseID | WanRedundancy_Solution_UsbIsBackup_08 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0 and usb available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config track wan group each interface, result 3 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 4. Track wan fail on eth0, result 4 no in eth0 dhcp client 5. Track default gateway fail on eth0, result 5 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 5: Track default gateway on eth0 unsuccessfully AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: wan failover to eth0, both monitor state and default monitor state of usbnet are Up AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknow Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: unknow Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 19:44:54 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 19:44:54 2013 Result 7: Default route is via usbnet, and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_09

| CaseID | WanRedundancy_Solution_UsbIsBackup_09 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0 and usb available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is down and track default gateway for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on eth0, result 4 _deb brd wanmon no in eth0 dhcp client 5. Track wan fail on usbnet, result 5 track-wan use-for-wan-usbnet0 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on eth0 are unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 5: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-26 19:34:13 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknow Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: unknow Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Sat Jan 26 19:46:07 2013 last_event: BACKUP_WAN_DOWN last_evt_time: Sat Jan 26 19:46:07 2013 Result 7: default gw with highest priority is via usbnet0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_10

| CaseID | WanRedundancy_Solution_UsbIsBackup_10 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0 and usb available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for usbnet are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on eth0, result 4 _deb brd wanmon no in eth0 dhcp client 5. Both track wan and track default gateway fail on usbnet, result 5 no in usbnet0 dhcp client 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on eth0 are unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 5: Track wanand track default gateway on usbnet unsuccessfully, and monitor state of usbnet is unknow AH-8c8040#sh lo bu …. 1970-01-01 00:02:23 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. 1970-01-01 00:02:27 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: down Interface: eth0; State: unknown [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: down Interface: eth0; State: unknown [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Thu Jan 1 00:00:11 1970 last_event: NOWAN last_evt_time: Thu Jan 1 00:00:11 1970 Result 7: no default gw available AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_11

| CaseID | WanRedundancy_Solution_UsbIsBackup_11 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth1 available, and both track wan and track default gateway for eth0 are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth1 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Check wan state, as well as monitor state, result 4 5. Check default route in main route table, result 5 6. Run ping internet process on client to check its accessibility, result 6 7. Visit http on client, result 7 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Active wan is eth0, and both monitor state and default monitor state of eth0 are Up AH-8c8040#sh wan in ifname: usbnet0 kifname: wanif_type: 1 wifidx: 1 kifidx: 0 priority: 600 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:11 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifmon_up_count: 0 ifmon_up_time: Thu Jan 1 00:00:00 1970 ifmon_down_count: 0 ifmon_down_time: Thu Jan 1 00:00:00 1970 dft_ifmon_up_count: 0 dft_ifmon_up_time: Thu Jan 1 00:00:00 1970 dft_ifmon_down_count: 0 dft_ifmon_down_time: Thu Jan 1 00:00:00 1970 nat_policy_id: ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:29 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifmon_up_count: 2 ifmon_up_time: Wed Jan 23 22:22:39 2013 ifmon_down_count: 206 ifmon_down_time: Wed Jan 23 20:00:15 2013 dft_ifmon_up_count: 14 dft_ifmon_up_time: Wed Jan 23 22:22:50 2013 dft_ifmon_down_count: 0 dft_ifmon_down_time: Thu Jan 1 00:00:00 1970 nat_policy_id: ifname: eth1 kifname: eth1 wanif_type: 0 wifidx: 2 kifidx: 6 priority: 10 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 2 lkup_time: Wed Jan 23 22:15:48 2013 lkdn_count: 1 lkdn_time: Wed Jan 23 22:15:47 2013 ifmon_up_count: 1 ifmon_up_time: Wed Jan 23 22:20:44 2013 ifmon_down_count: 16 ifmon_down_time: Wed Jan 23 22:20:26 2013 dft_ifmon_up_count: 3 dft_ifmon_up_time: Wed Jan 23 22:18:16 2013 dft_ifmon_down_count: 0 dft_ifmon_down_time: Wed Jan 23 21:59:46 2013 nat_policy_id: AH-8c8040# sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Jan 23 22:22:50 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Jan 23 22:22:50 2013 Result 5: Default route is via eth0, and there is non-default route via eth1 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 Result 6: ping internet successfully on client Result 7: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_12

| CaseID | WanRedundancy_Solution_UsbIsBackup_12 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth1 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for eth1 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth1 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c8040# sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=disabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ifname: usbnet0 kifname: wanif_type: 1 wifidx: 1 kifidx: 0 priority: 600 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:11 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifmon_up_count: 0 ifmon_up_time: Thu Jan 1 00:00:00 1970 ifmon_down_count: 0 ifmon_down_time: Thu Jan 1 00:00:00 1970 dft_ifmon_up_count: 0 dft_ifmon_up_time: Thu Jan 1 00:00:00 1970 dft_ifmon_down_count: 0 dft_ifmon_down_time: Thu Jan 1 00:00:00 1970 nat_policy_id: ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:29 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifmon_up_count: 2 ifmon_up_time: Wed Jan 23 22:22:39 2013 ifmon_down_count: 206 ifmon_down_time: Wed Jan 23 20:00:15 2013 dft_ifmon_up_count: 14 dft_ifmon_up_time: Wed Jan 23 22:22:50 2013 dft_ifmon_down_count: 0 dft_ifmon_down_time: Thu Jan 1 00:00:00 1970 nat_policy_id: ifname: eth1 kifname: eth1 wanif_type: 0 wifidx: 2 kifidx: 6 priority: 10 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 2 lkup_time: Wed Jan 23 22:15:48 2013 lkdn_count: 1 lkdn_time: Wed Jan 23 22:15:47 2013 ifmon_up_count: 1 ifmon_up_time: Wed Jan 23 22:20:44 2013 ifmon_down_count: 16 ifmon_down_time: Wed Jan 23 22:20:26 2013 dft_ifmon_up_count: 3 dft_ifmon_up_time: Wed Jan 23 22:18:16 2013 dft_ifmon_down_count: 0 dft_ifmon_down_time: Wed Jan 23 21:59:46 2013 nat_policy_id: Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh log buf ¦ i wanmon 2013-01-24 18:56:14 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via eth1: RTT min/avg/max = 37.625999/37.625999/37.625999 2013-01-24 18:56:14 debug ah_brd: [brd_wanmon]: WFOIFM: Ping 8.8.8.8 via eth1: 0% packet loss 2013-01-24 18:56:14 debug ah_brd: [brd_wanmon]: WFOIFM: Ping cmd [/bin/ping -q -c 1 -I eth1 -w 2 8.8.8.8 2>&1 > /tmp/brd_wan_ifmon_ping_3188] executed 2013-01-24 18:56:13 debug ah_brd: [brd_wanmon]: WFOIFM: Rest track-wan [use-for-wan-eth0], kifname: eth0. 2013-01-24 18:56:13 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Going to test 1 targets for eth0 ... 2013-01-24 18:56:13 debug ah_brd: [brd_wanmon]: WFOIFM: Interface eth0 allows ARP (ifr_flags = 0x1043) 2013-01-24 18:56:13 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Determined current gw IP as 0xa9b1ffe for eth0 2013-01-24 18:56:13 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-eth0] thread begin to do test. 2013-01-24 18:56:13 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-eth0] thread wait last_test_time: 1359053767, waittime: 1359053768 2013-01-24 18:56:13 debug ah_brd: [brd_wanmon]: WFOIFM: send event AH_EVENT_WAN_IFMON_IF_STATE_CHANGE success. ifmon_type: 1, ifmon_cfg_name: use-for-wan- eth0, kifname: eth0, if_status: 2, if_status_pre: 0 2013-01-24 18:56:13 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#show track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 () [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: wan failover to eth1, both monitor state and default monitor state of eth1 are Up AH-8c8040#sh wan in ifname: usbnet0 kifname: wanif_type: 1 wifidx: 1 kifidx: 0 priority: 600 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:11 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifmon_up_count: 0 ifmon_up_time: Thu Jan 1 00:00:00 1970 ifmon_down_count: 0 ifmon_down_time: Thu Jan 1 00:00:00 1970 dft_ifmon_up_count: 0 dft_ifmon_up_time: Thu Jan 1 00:00:00 1970 dft_ifmon_down_count: 0 dft_ifmon_down_time: Thu Jan 1 00:00:00 1970 nat_policy_id: ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 2 lkup_time: Wed Jan 23 23:30:16 2013 lkdn_count: 1 lkdn_time: Wed Jan 23 22:51:22 2013 ifmon_up_count: 4 ifmon_up_time: Wed Jan 23 23:30:45 2013 ifmon_down_count: 240 ifmon_down_time: Wed Jan 23 23:40:19 2013 dft_ifmon_up_count: 15 dft_ifmon_up_time: Wed Jan 23 23:30:59 2013 dft_ifmon_down_count: 0 dft_ifmon_down_time: Thu Jan 1 00:00:00 1970 nat_policy_id: ifname: eth1 kifname: eth1 wanif_type: 0 wifidx: 2 kifidx: 6 priority: 10 is_wan: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 2 lkup_time: Wed Jan 23 22:15:48 2013 lkdn_count: 1 lkdn_time: Wed Jan 23 22:15:47 2013 ifmon_up_count: 1 ifmon_up_time: Wed Jan 23 22:20:44 2013 ifmon_down_count: 16 ifmon_down_time: Wed Jan 23 22:20:26 2013 dft_ifmon_up_count: 3 dft_ifmon_up_time: Wed Jan 23 22:18:16 2013 dft_ifmon_down_count: 0 dft_ifmon_down_time: Wed Jan 23 21:59:46 2013 nat_policy_id: - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#show wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Jan 23 23:42:20 2013 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Jan 23 23:42:20 2013 Result 6: Default route is via eth1, and there is non-default route via eth0 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_13

| CaseID | WanRedundancy_Solution_UsbIsBackup_13 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth1 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for eth1 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth1 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 5. Track wan fail on eth1, result 5 track-wan use-for-wan-eth1 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ifname: usbnet0 kifname: wanif_type: 1 wifidx: 1 kifidx: 0 priority: 600 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:11 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifmon_up_count: 0 ifmon_up_time: Thu Jan 1 00:00:00 1970 ifmon_down_count: 0 ifmon_down_time: Thu Jan 1 00:00:00 1970 dft_ifmon_up_count: 0 dft_ifmon_up_time: Thu Jan 1 00:00:00 1970 dft_ifmon_down_count: 0 dft_ifmon_down_time: Thu Jan 1 00:00:00 1970 nat_policy_id: ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 3 lkup_time: Thu Jan 24 01:09:10 2013 lkdn_count: 2 lkdn_time: Thu Jan 24 01:07:40 2013 ifmon_up_count: 17 ifmon_up_time: Thu Jan 24 18:54:55 2013 ifmon_down_count: 5676 ifmon_down_time: Thu Jan 24 19:06:41 2013 dft_ifmon_up_count: 154 dft_ifmon_up_time: Thu Jan 24 19:05:48 2013 dft_ifmon_down_count: 6 dft_ifmon_down_time: Thu Jan 24 19:05:18 2013 nat_policy_id: ifname: eth1 kifname: eth1 wanif_type: 0 wifidx: 2 kifidx: 6 priority: 10 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 6 lkup_time: Thu Jan 24 18:53:02 2013 lkdn_count: 5 lkdn_time: Thu Jan 24 18:53:01 2013 ifmon_up_count: 714 ifmon_up_time: Thu Jan 24 19:04:20 2013 ifmon_down_count: 1333 ifmon_down_time: Thu Jan 24 19:04:06 2013 dft_ifmon_up_count: 15 dft_ifmon_up_time: Thu Jan 24 18:53:15 2013 dft_ifmon_down_count: 0 dft_ifmon_down_time: Wed Jan 23 21:59:46 2013 nat_policy_id: Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh log buf ¦ i wanmom 2013-01-24 19:17:22 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-usbnet0] thread wait last_test_time: 1359055042, waittime: 1359055047 2013-01-24 19:17:22 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. 2013-01-24 19:17:22 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-usbnet0] thread begin to do test. 2013-01-24 19:17:22 debug ah_brd: [brd_wanmon]: WFOIFM: Arping OK: RTT avg = 1.864000 (out of 1 counts) 2013-01-24 19:17:22 debug ah_brd: [brd_wanmon]: WFOIFM: Arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.31.254 2>&1 > /tmp/brd_wan_ifmon_arping_1928] executed 2013-01-24 19:17:21 debug ah_brd: [brd_wanmon]: WFOIFM: Rest track-wan [use-for-wan-eth0], kifname: eth0. 2013-01-24 19:17:21 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Going to test 1 targets for eth0 ... 2013-01-24 19:17:21 debug ah_brd: [brd_wanmon]: WFOIFM: Interface eth0 allows ARP (ifr_flags = 0x1043) 2013-01-24 19:17:21 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Determined current gw IP as 0xa9b1ffe for eth0 2013-01-24 19:17:21 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-eth0] thread begin to do test. 2013-01-24 19:17:21 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-eth0] thread wait last_test_time: 1359055035, waittime: 1359055036 2013-01-24 19:17:21 debug ah_brd: [brd_wanmon]: WFOIFM: send event AH_EVENT_WAN_IFMON_IF_STATE_CHANGE success. ifmon_type: 1, ifmon_cfg_name: use-for-wan- eth0, kifname: eth0, if_status: 2, if_status_pre: 0 2013-01-24 19:17:21 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL 2013-01-24 19:17:19 debug ah_brd: [brd_wanmon]: WFOIFM: Arping OK: RTT avg = 3.482000 (out of 1 counts) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on eth1 unsuccessfully, and monitor state of eth1 is Down AH-8c8040#sh log buf ¦ i wanmom 2013-01-24 19:17:25 debug ah_brd: [brd_wanmon]: WFOIFM: Arping OK: RTT avg = 1.969000 (out of 1 counts) 2013-01-24 19:17:25 debug ah_brd: [brd_wanmon]: WFOIFM: Arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.31.254 2>&1 > /tmp/brd_wan_ifmon_arping_3326] executed 2013-01-24 19:17:24 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Going to test 1 targets for eth1 ... 2013-01-24 19:17:24 debug ah_brd: [brd_wanmon]: WFOIFM: Interface eth1 allows ARP (ifr_flags = 0x1043) 2013-01-24 19:17:24 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Determined current gw IP as 0xc0a80a01 for eth1 2013-01-24 19:17:24 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [use-for-wan-eth1] thread begin to do test. 2013-01-24 19:17:24 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth1: 1 targets tested, result: FAIL - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover to eth1 AH-8c8040#sh wan in ifname: usbnet0 kifname: wanif_type: 1 wifidx: 1 kifidx: 0 priority: 600 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:11 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifmon_up_count: 0 ifmon_up_time: Thu Jan 1 00:00:00 1970 ifmon_down_count: 0 ifmon_down_time: Thu Jan 1 00:00:00 1970 dft_ifmon_up_count: 0 dft_ifmon_up_time: Thu Jan 1 00:00:00 1970 dft_ifmon_down_count: 0 dft_ifmon_down_time: Thu Jan 1 00:00:00 1970 nat_policy_id: ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 3 lkup_time: Thu Jan 24 01:09:10 2013 lkdn_count: 2 lkdn_time: Thu Jan 24 01:07:40 2013 ifmon_up_count: 17 ifmon_up_time: Thu Jan 24 18:54:55 2013 ifmon_down_count: 5744 ifmon_down_time: Thu Jan 24 19:20:22 2013 dft_ifmon_up_count: 155 dft_ifmon_up_time: Thu Jan 24 19:13:23 2013 dft_ifmon_down_count: 6 dft_ifmon_down_time: Thu Jan 24 19:05:18 2013 nat_policy_id: ifname: eth1 kifname: eth1 wanif_type: 0 wifidx: 2 kifidx: 6 priority: 10 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 6 lkup_time: Thu Jan 24 18:53:02 2013 lkdn_count: 5 lkdn_time: Thu Jan 24 18:53:01 2013 ifmon_up_count: 714 ifmon_up_time: Thu Jan 24 19:04:20 2013 ifmon_down_count: 1356 ifmon_down_time: Thu Jan 24 19:20:31 2013 dft_ifmon_up_count: 15 dft_ifmon_up_time: Thu Jan 24 18:53:15 2013 dft_ifmon_down_count: 0 dft_ifmon_down_time: Wed Jan 23 21:59:46 2013 nat_policy_id:- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: FAILOVER prev_state_time: Thu Jan 24 19:22:11 2013 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Thu Jan 24 19:22:11 2013 Result 7: Default route is via eth0, and there is non-default route via eth1 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_14

| CaseID | WanRedundancy_Solution_UsbIsBackup_14 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth1 available, both track wan and track default gateway for eth0 are down, while track wan for eth1 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth1 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 5. Track default gateway fail on eth0, result 5 no in eth0 dhcp client 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; AH-8c8040#sh wan in ifname: usbnet0 kifname: wanif_type: 1 wifidx: 1 kifidx: 0 priority: 600 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:11 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifmon_up_count: 0 ifmon_up_time: Thu Jan 1 00:00:00 1970 ifmon_down_count: 0 ifmon_down_time: Thu Jan 1 00:00:00 1970 dft_ifmon_up_count: 0 dft_ifmon_up_time: Thu Jan 1 00:00:00 1970 dft_ifmon_down_count: 0 dft_ifmon_down_time: Thu Jan 1 00:00:00 1970 nat_policy_id: ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 3 lkup_time: Thu Jan 24 01:09:10 2013 lkdn_count: 2 lkdn_time: Thu Jan 24 01:07:40 2013 ifmon_up_count: 17 ifmon_up_time: Thu Jan 24 18:54:55 2013 ifmon_down_count: 5676 ifmon_down_time: Thu Jan 24 19:06:41 2013 dft_ifmon_up_count: 154 dft_ifmon_up_time: Thu Jan 24 19:05:48 2013 dft_ifmon_down_count: 6 dft_ifmon_down_time: Thu Jan 24 19:05:18 2013 nat_policy_id: ifname: eth1 kifname: eth1 wanif_type: 0 wifidx: 2 kifidx: 6 priority: 10 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 6 lkup_time: Thu Jan 24 18:53:02 2013 lkdn_count: 5 lkdn_time: Thu Jan 24 18:53:01 2013 ifmon_up_count: 714 ifmon_up_time: Thu Jan 24 19:04:20 2013 ifmon_down_count: 1333 ifmon_down_time: Thu Jan 24 19:04:06 2013 dft_ifmon_up_count: 15 dft_ifmon_up_time: Thu Jan 24 18:53:15 2013 dft_ifmon_down_count: 0 dft_ifmon_down_time: Wed Jan 23 21:59:46 2013 nat_policy_id: Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo b ¦ i wanmon …… 2013-01-24 20:03:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL …… 2013-01-24 20:03:30 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth1: 1 targets tested, result: PASS -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#show track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track default gateway on eth0 unsuccessfully AH-8c8040#sh lo bu ¦ i DFT_GW_TRACK_IP …… 2013-01-24 20:13:35 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …… -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#show track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknow Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan failover to eth1, both monitor state and default monitor state of eth1 are Up AH-8c8040#sh wan in ifname: usbnet0 kifname: wanif_type: 1 wifidx: 1 kifidx: 0 priority: 600 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:11 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifmon_up_count: 0 ifmon_up_time: Thu Jan 1 00:00:00 1970 ifmon_down_count: 0 ifmon_down_time: Thu Jan 1 00:00:00 1970 dft_ifmon_up_count: 0 dft_ifmon_up_time: Thu Jan 1 00:00:00 1970 dft_ifmon_down_count: 0 dft_ifmon_down_time: Thu Jan 1 00:00:00 1970 nat_policy_id: ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 2 lkup_time: Wed Jan 23 23:30:16 2013 lkdn_count: 1 lkdn_time: Wed Jan 23 22:51:22 2013 ifmon_up_count: 4 ifmon_up_time: Wed Jan 23 23:30:45 2013 ifmon_down_count: 388 ifmon_down_time: Thu Jan 24 00:10:07 2013 dft_ifmon_up_count: 18 dft_ifmon_up_time: Wed Jan 23 23:59:30 2013 dft_ifmon_down_count: 0 dft_ifmon_down_time: Thu Jan 1 00:00:00 1970 nat_policy_id: ifname: eth1 kifname: eth1 wanif_type: 0 wifidx: 2 kifidx: 6 priority: 10 is_wan: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 2 lkup_time: Wed Jan 23 22:15:48 2013 lkdn_count: 1 lkdn_time: Wed Jan 23 22:15:47 2013 ifmon_up_count: 4 ifmon_up_time: Thu Jan 24 00:11:39 2013 ifmon_down_count: 150 ifmon_down_time: Thu Jan 24 00:11:20 2013 dft_ifmon_up_count: 3 dft_ifmon_up_time: Wed Jan 23 22:18:16 2013 dft_ifmon_down_count: 0 dft_ifmon_down_time: Wed Jan 23 21:59:46 2013 nat_policy_id: -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Thu Jan 24 00:11:39 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Thu Jan 24 00:11:39 2013 Result 7: Default route is via eth1, and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_15

| CaseID | WanRedundancy_Solution_UsbIsBackup_15 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth1 available, both track wan and track default gateway for eth0 are down, while track wan for eth1 is down and track default gateway for eth1 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth1 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on eth0, result 4 _deb brd wanmon no in eth0 dhcp client 5. Track wan fail on eth1, result 5 track-wan use-for-wan-eth1 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ifname: usbnet0 kifname: wanif_type: 1 wifidx: 1 kifidx: 0 priority: 600 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:11 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifmon_up_count: 0 ifmon_up_time: Thu Jan 1 00:00:00 1970 ifmon_down_count: 0 ifmon_down_time: Thu Jan 1 00:00:00 1970 dft_ifmon_up_count: 0 dft_ifmon_up_time: Thu Jan 1 00:00:00 1970 dft_ifmon_down_count: 0 dft_ifmon_down_time: Thu Jan 1 00:00:00 1970 nat_policy_id: ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 3 lkup_time: Thu Jan 24 01:09:10 2013 lkdn_count: 2 lkdn_time: Thu Jan 24 01:07:40 2013 ifmon_up_count: 20 ifmon_up_time: Thu Jan 24 20:56:44 2013 ifmon_down_count: 5985 ifmon_down_time: Thu Jan 24 20:56:31 2013 dft_ifmon_up_count: 162 dft_ifmon_up_time: Thu Jan 24 20:19:42 2013 dft_ifmon_down_count: 8 dft_ifmon_down_time: Thu Jan 24 19:37:51 2013 nat_policy_id: ifname: eth1 kifname: eth1 wanif_type: 0 wifidx: 2 kifidx: 6 priority: 10 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 6 lkup_time: Thu Jan 24 18:53:02 2013 lkdn_count: 5 lkdn_time: Thu Jan 24 18:53:01 2013 ifmon_up_count: 716 ifmon_up_time: Thu Jan 24 20:56:51 2013 ifmon_down_count: 1515 ifmon_down_time: Thu Jan 24 20:56:37 2013 dft_ifmon_up_count: 16 dft_ifmon_up_time: Thu Jan 24 19:35:05 2013 dft_ifmon_down_count: 0 dft_ifmon_down_time: Wed Jan 23 21:59:46 2013 nat_policy_id: Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 1.1.1.1 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 1.1.1.1 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on eth0 are unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ¦ i wanmon ….. 2013-01-24 21:32:38 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. ……. 2013-01-24 21:36:30 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …… -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on eth1 unsuccessfully, and monitor state of eth1 is Down AH-8c8040#sh lo bu ….. 2013-01-24 21:40:10 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth1: 1 targets tested, result: FAIL ……. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ifname: usbnet0 kifname: wanif_type: 1 wifidx: 1 kifidx: 0 priority: 600 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:11 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifmon_up_count: 0 ifmon_up_time: Thu Jan 1 00:00:00 1970 ifmon_down_count: 0 ifmon_down_time: Thu Jan 1 00:00:00 1970 dft_ifmon_up_count: 0 dft_ifmon_up_time: Thu Jan 1 00:00:00 1970 dft_ifmon_down_count: 0 dft_ifmon_down_time: Thu Jan 1 00:00:00 1970 nat_policy_id: ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 3 lkup_time: Thu Jan 24 01:09:10 2013 lkdn_count: 2 lkdn_time: Thu Jan 24 01:07:40 2013 ifmon_up_count: 20 ifmon_up_time: Thu Jan 24 20:56:44 2013 ifmon_down_count: 5986 ifmon_down_time: Thu Jan 24 21:18:56 2013 dft_ifmon_up_count: 162 dft_ifmon_up_time: Thu Jan 24 20:19:42 2013 dft_ifmon_down_count: 8 dft_ifmon_down_time: Thu Jan 24 19:37:51 2013 nat_policy_id: ifname: eth1 kifname: eth1 wanif_type: 0 wifidx: 2 kifidx: 6 priority: 10 is_wan: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 6 lkup_time: Thu Jan 24 18:53:02 2013 lkdn_count: 5 lkdn_time: Thu Jan 24 18:53:01 2013 ifmon_up_count: 716 ifmon_up_time: Thu Jan 24 20:56:51 2013 ifmon_down_count: 1540 ifmon_down_time: Thu Jan 24 21:44:30 2013 dft_ifmon_up_count: 16 dft_ifmon_up_time: Thu Jan 24 19:35:05 2013 dft_ifmon_down_count: 0 dft_ifmon_down_time: Wed Jan 23 21:59:46 2013 nat_policy_id: -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: NOWAN prev_state_time: Thu Jan 24 00:35:48 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Thu Jan 24 00:35:48 2013 Result 7: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_16

| CaseID | WanRedundancy_Solution_UsbIsBackup_16 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth1 available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for eth1 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth1 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on eth0, result 4 _deb brd wanmon no in eth0 dhcp client 5. Both track wan and track default gateway fail on eth1, result 5 no in eth0 dhcp client 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c8040# sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth kifname: eth1 .... priority: 10 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on eth0 are unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu 2013-01-24 21:58:18 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-24 21:59:23 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan and Track default gateway on eth1 are unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu 2013-01-24 21:58:18 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth1: interface is not connected. 2013-01-24 22:05:18 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. 2013-01-24 22:05:18 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: down Interface: eth0; State: unknown Interface: eth1; State: unknown [use-for-wan-eth1] WAN Monitor State: unknown Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan state is “no wan” AH-8c8040#sh wan in …… ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 0 …. ifname: eth1 kifname: eth1 wanif_type: 0 wifidx: 2 kifidx: 6 priority: 10 is_wan: 1 is_default_wan: 0 ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Thu Jan 24 00:38:57 2013 last_event: BACKUP_WAN_DOWN last_evt_time: Thu Jan 24 00:38:57 2013 Result 7: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_17

| CaseID | WanRedundancy_Solution_UsbIsBackup_17 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth2 available, and both track wan and track default gateway for eth0 are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth2 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Check wan state, as well as monitor state, result 4 5. Check default route in main route table, result 5 6. Run ping internet process on client to check its accessibility, result 6 7. Visit http on client, result 7 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth2 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth kifname: eth2 .... priority: 20 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth2 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Active wan is eth0, and both monitor state and default monitor state of eth0 are Up AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth kifname: eth2 .... priority: 20 is_wan: 1 is_default_wan: 0 …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Thu Jan 24 22:20:07 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Thu Jan 24 22:20:07 2013 Result 5: Default route is via eth0, and there is non-default route via eth2 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.20.1 0.0.0.0 UG 20 0 0 eth2 Result 6: ping internet successfully on client Result 7: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_18

| CaseID | WanRedundancy_Solution_UsbIsBackup_18 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth2 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for eth2 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth2 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth2 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth kifname: eth2 .... priority: 20 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth2 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo b ¦ i wanmon …… 2013-01-24 22:43:59 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL …… -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: wan failover to eth2, both monitor state and default monitor state of eth2 are Up AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 0 …. ifname: eth kifname: eth2 .... priority: 20 is_wan: 1 is_default_wan: 1 …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: FAILOVER prev_state_time: Thu Jan 24 23:05:50 2013 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Thu Jan 24 23:05:50 2013 Result 6: Default route is via eth2, and there is non-default route via eth0 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.20.1 0.0.0.0 UG 20 0 0 eth2 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_19

| CaseID | WanRedundancy_Solution_UsbIsBackup_19 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth2 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for eth2 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth2 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 5. Track wan fail on eth2, result 5 track-wan use-for-wan-eth0 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth2 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth2 is wan mode AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth kifname: eth2 .... priority: 20 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth2 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo b ¦ i wanmon …… 2013-01-24 22:43:59 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL …… -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on eth2 unsuccessfully, and monitor state of eth2 is Down AH-8c8040#sh lo bu ¦ i wanmon 2013-01-24 23:36:29 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth2]: Interface eth2: 1 targets tested, result: FAIL -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: down Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth kifname: eth2 .... priority: 20 is_wan: 1 is_default_wan: 0 …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Thu Jan 24 23:39:55 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Thu Jan 24 23:39:55 2013 Result 7: Default route is via eth0, and there is non-default route via eth2 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.20.1 0.0.0.0 UG 10020 0 0 eth2 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_20

| CaseID | WanRedundancy_Solution_UsbIsBackup_20 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth2 available, both track wan and track default gateway for eth0 are down, while track wan for eth2 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth2 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 5. Track default gateway fail on eth0, result 5 no in eth0 dhcp client 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth2 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth kifname: eth2 .... priority: 20 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth2 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-24 23:46:57 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 () [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track default gateway on eth0 unsuccessfully AH-8c8040#sh lo bu ¦ i DFT_GW_TRACK_IP …… 2013-01-24 23:57:33 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won’t failover to eth2, both monitor state and default monitor state of eth2 are Up AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 0 …. ifname: eth kifname: eth2 .... priority: 20 is_wan: 1 is_default_wan: 1 …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State:unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Default route is via eth2, and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.20.1 0.0.0.0 UG 20 0 0 eth2 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_21

| CaseID | WanRedundancy_Solution_UsbIsBackup_21 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth2 available, both track wan and track default gateway for eth0 are down, while track wan for eth2 is down and track default gateway for eth2 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth2 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on eth0, result 4 _deb brd wanmon no in eth0 dhcp client 5. Track wan fail on eth2, result 5 track-wan use-for-wan-eth2 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth2 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth kifname: eth2 .... priority: 20 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth2 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on eth0 are unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-24 23:46:57 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-24 23:57:33 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 () [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on eth2 unsuccessfully, and monitor state of eth2 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-24 23:46:57 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: down Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 () [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 0 …. ifname: eth kifname: eth2 .... priority: 20 is_wan: 1 is_default_wan: 1 …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: NOWAN prev_state_time: Fri Jan 25 00:17:59 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Jan 25 00:17:59 2013 Result 7: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.20.1 0.0.0.0 UG 10020 0 0 eth2 Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_22

| CaseID | WanRedundancy_Solution_UsbIsBackup_22 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth2 available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for eth2 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth2 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on eth0, result 4 _deb brd wanmon no in eth0 dhcp client 5. Both track wan and track default gateway fail on eth2, result 5 no in eth0 dhcp client 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth2 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth kifname: eth2 .... priority: 20 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth2 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on eth0 are unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-24 23:46:57 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-24 23:57:33 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State:unknown Interface: eth0 () [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on eth2 unsuccessfully, and monitor state of eth2 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-24 23:46:57 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth2: interface is not connected. 2013-01-24 23:57:33 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. 2013-01-24 23:57:33 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State:unknown [use-for-wan-eth2] WAN Monitor State: unknown Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 () [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 0 …. ifname: eth kifname: eth2 .... priority: 20 is_wan: 1 is_default_wan: 0 …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Fri Jan 25 00:22:33 2013 last_event: BACKUP_WAN_DOWN last_evt_time: Fri Jan 25 00:22:33 2013 Result 7: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_23

| CaseID | WanRedundancy_Solution_UsbIsBackup_23 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth3 available, and both track wan and track default gateway for eth0 are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth3 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Check wan state, as well as monitor state, result 4 5. Check default route in main route table, result 5 6. Run ping internet process on client to check its accessibility, result 6 7. Visit http on client, result 7 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth3 kifname: eth3 …. priority: 30 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Active wan is eth0, and both monitor state and default monitor state of eth0 are Up AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth kifname: eth3 .... priority: 30 is_wan: 1 is_default_wan: 0 …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Thu Jan 24 22:20:07 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Thu Jan 24 22:20:07 2013 Result 5: Default route is via eth0, and there is non-default route via eth3 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.20.1 0.0.0.0 UG 30 0 0 eth3 Result 6: ping internet successfully on client Result 7: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_24

| CaseID | WanRedundancy_Solution_UsbIsBackup_24 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth3 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for eth3 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth3 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth3 kifname: eth3 …. priority: 30 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo b ¦ i wanmon …… 2013-01-24 22:43:59 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL …… -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: wan failover to eth3, both monitor state and default monitor state of eth3 are Up AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 0 …. ifname: eth kifname: eth2 .... priority: 20 is_wan: 1 is_default_wan: 1 …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: FAILOVER prev_state_time: Thu Jan 24 23:05:50 2013 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Thu Jan 24 23:05:50 2013 Result 6: Default route is via eth3, and there is non-default route via eth0 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.30.1 0.0.0.0 UG 30 0 0 eth3 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 7: ping internet successfully on client Result 8: Http visit on client successful |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_25

| CaseID | WanRedundancy_Solution_UsbIsBackup_25 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth3 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for eth3 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth3 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 5. Track wan fail on eth3, result 5 track-wan use-for-wan-eth3 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth3 kifname: eth3 …. priority: 30 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo b ¦ i wanmon …… 2013-01-24 22:43:59 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL …… -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on eth3 unsuccessfully, and monitor state of eth3 is Down AH-8c8040#sh lo b ¦ i wanmon …… 2013-01-24 22:43:59 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth3: 1 targets tested, result: FAIL …… -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: down Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth3 kifname: eth3 …. priority: 30 is_wan: 1 is_default_wan: 0 …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - ]AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Jan 25 01:42:00 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Jan 25 01:42:00 2013 Result 7: Default route is via eth0, and there is non-default route via eth3 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.30.1 0.0.0.0 UG 10030 0 0 eth3 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_26

| CaseID | WanRedundancy_Solution_UsbIsBackup_26 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth3 available, both track wan and track default gateway for eth0 are down, while track wan for eth3 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth3 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 5. Track default gateway fail on eth0, result 5 no in eth0 dhcp client 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth3 kifname: eth3 …. priority: 30 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-24 23:46:57 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 () [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track default gateway on eth0 unsuccessfully AH-8c8040#sh lo bu ¦ i DFT_GW_TRACK_IP …… 2013-01-24 23:57:33 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan failover to eth3, both monitor state and default monitor state of eth3 are Up AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 0 …. ifname: eth3 kifname: eth3 …. priority: 30 is_wan: 1 is_default_wan: 1 …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Jan 25 01:51:03 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Fri Jan 25 01:51:03 2013 Result 7: Default route is via eth3, and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.30.1 0.0.0.0 UG 30 0 0 eth3 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_27

| CaseID | WanRedundancy_Solution_UsbIsBackup_27 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth3 available, both track wan and track default gateway for eth0 are down, while track wan for eth3 is down and track default gateway for eth3 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth3 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on eth0, result 4 _deb brd wanmon no in eth0 dhcp client 5. Track wan fail on eth3, result 5 track-wan use-for-wan-eth3 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth3 kifname: eth3 …. priority: 30 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on eth0 are unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-24 23:46:57 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-24 23:57:33 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State:unknown Interface: eth0 () [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: : Track wan and Track default gateway on eth3 are unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-24 23:46:57 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth3: interface is not connected. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: down Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State:unknown Interface: eth0 () [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 0 …. ifname: eth kifname: eth3 .... priority: 30 is_wan: 1 is_default_wan: 1 …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: NOWAN prev_state_time: Fri Jan 25 00:17:59 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Jan 25 00:17:59 2013 Result 7: default gw with highest priority is via eth3 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.20.1 0.0.0.0 UG 10020 0 0 eth3 Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_28

| CaseID | WanRedundancy_Solution_UsbIsBackup_28 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth3 available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for eth3 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth3 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on eth0, result 4 _deb brd wanmon no in eth0 dhcp client 5. Both track wan and track default gateway fail on eth3, result 5 no in eth3 dhcp client 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth4 shutdown interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth3 kifname: eth3 …. priority: 30 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on eth0 are unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-24 23:46:57 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-24 23:57:33 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State:unknown Interface: eth0 () [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on eth3 unsuccessfully, and monitor state of eth3 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-24 23:46:57 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth3: interface is not connected. 2013-01-24 23:57:33 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. 2013-01-24 23:57:33 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: unknown [use-for-wan-eth3] WAN Monitor State: unknown Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 () [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 0 …. ifname: eth kifname: eth3 .... priority: 30 is_wan: 1 is_default_wan: 0 …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Fri Jan 25 00:17:59 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Jan 25 00:17:59 2013 Result 7: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_29

| CaseID | WanRedundancy_Solution_UsbIsBackup_29 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth4 available, and both track wan and track default gateway for eth0 are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth4 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Check wan state, as well as monitor state, result 4 5. Check default route in main route table, result 5 6. Run ping internet process on client to check its accessibility, result 6 7. Visit http on client, result 7 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode wan interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth4 kifname: eth4 …. priority: 40 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Active wan is eth0, and both monitor state and default monitor state of eth0 are Up AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth kifname: eth4 .... priority: 40 is_wan: 1 is_default_wan: 0 …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Thu Jan 24 22:20:07 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Thu Jan 24 22:20:07 2013 Result 5: Default route is via eth0, and there is non-default route via eth4 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.40.1 0.0.0.0 UG 40 0 0 eth4 Result 6: ping internet successfully on client Result 7: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_30

| CaseID | WanRedundancy_Solution_UsbIsBackup_30 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth4 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for eth4 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth4 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | R config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode wan interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable\ Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth4 kifname: eth4 …. priority: 40 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ¦ i wanmon …… 2013-01-25 03:29:55 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: wan failover to eth4, both monitor state and default monitor state of eth4 are Up AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 wanif_type: 0 wifidx: 0 kifidx: 3 priority: 1 is_wan: 1 is_default_wan: 0 …. ifname: eth kifname: eth4 .... priority: 40 is_wan: 1 is_default_wan: 1 …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: FAILOVER prev_state_time: Thu Jan 24 23:05:50 2013 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Thu Jan 24 23:05:50 2013 Result 6: Default route is via eth4, and there is non-default route via eth0 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 192.168.40.1 0.0.0.0 UG 40 0 0 eth4 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_31

| CaseID | WanRedundancy_Solution_UsbIsBackup_31 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth4 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for eth4 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth4 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 5. Track wan fail on eth4, result 5 track-wan use-for-wan-eth0 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | R config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode wan interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth4 kifname: eth4 …. priority: 40 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ¦ i wanmon …… 2013-01-25 03:29:55 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on eth4 unsuccessfully, and monitor state of eth4 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-25 18:11:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth4]: Interface eth4: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “CONNNECT” AH-8c8040#sh wan in ifname: eth4 kifname: eth4 …. priority: 40 is_wan: 1 is_default_wan: 0 …. ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 1 …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: NOWAN prev_state_time: Fri Jan 25 18:23:27 2013 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Fri Jan 25 18:23:27 2013 Result 7: Default route is via eth0, and there is non-default route via eth4 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.40.1 0.0.0.0 UG 10040 0 0 eth4 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_32

| CaseID | WanRedundancy_Solution_UsbIsBackup_32 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth4 available, both track wan and track default gateway for eth0 are down, while track wan for eth4 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth4 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 5. Track default gateway fail on eth0, result 5 no in eth0 dhcp client 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | R config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode wan interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth4 kifname: eth4 …. priority: 40 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ¦ i wanmon …… 2013-01-25 03:29:55 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track default gateway on eth0 unsuccessfully AH-8c8040#sh lo bu 2013-01-25 18:34:29 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan failover to eth4, both monitor state and default monitor state of eth4 are Up AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 0 …. ifname: eth4 kifname: eth4 …. priority: 40 is_wan: 1 is_default_wan: 1 …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Jan 25 18:30:07 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Jan 25 18:30:07 2013 Result 7: Default route is via eth4, and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 192.168.40.1 0.0.0.0 UG 40 0 0 eth4 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_33

| CaseID | WanRedundancy_Solution_UsbIsBackup_33 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth4 available, both track wan and track default gateway for eth0 are down, while track wan for eth4 is down and track default gateway for eth4 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth4 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on eth0, result 4 _deb brd wanmon no in eth0 dhcp client 5. Track wan fail on eth4, result 5 track-wan use-for-wan-eth0 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | R config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode wan interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth4 kifname: eth4 …. priority: 40 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on eth0 are unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-25 18:51:58 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-25 18:40:04 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on eth4 unsuccessfully, and monitor state of eth4 is Down AH-8c8040#sh lo bu ¦ i wanmon ….. 2013-01-25 18:44:45 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth4]: Interface eth4: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 0 …. ifname: eth4 kifname: eth4 …. priority: 40 is_wan: 1 is_default_wan: 1 …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: NOWAN prev_state_time: Fri Jan 25 18:46:28 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Jan 25 18:46:28 2013 Result 7: default gw with highest priority is via eth4 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 192.168.40.1 0.0.0.0 UG 10040 0 0 eth4 Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_34

| CaseID | WanRedundancy_Solution_UsbIsBackup_34 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth4 available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for eth4 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth4 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on eth0, result 4 _deb brd wanmon no in eth0 dhcp client 5. Both track wan and track default gateway fail on eth4, result 5 no in eth4 dhcp client 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | R config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode wan interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth4 kifname: eth4 …. priority: 40 is_wan: 1 is_default_wan: 0 …. Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on eth0 are unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-25 18:51:58 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-25 18:40:04 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on eth4 unsuccessfully, and monitor state of eth4 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-25 18:51:58 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth4: interface is not connected. 2013-01-25 18:40:04 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: down Interface: eth0; State: unknown Interface: eth4; State: unknown [use-for-wan-eth4] WAN Monitor State: unknown Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ….. ifname: eth0 kifname: eth0 …. priority: 1 is_wan: 1 is_default_wan: 1 …. ifname: eth4 kifname: eth4 …. priority: 40 is_wan: 1 is_default_wan: 0 …. AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Fri Jan 25 18:53:23 2013 last_event: BACKUP_WAN_DOWN last_evt_time: Fri Jan 25 18:53:23 2013 Result 7: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_35

| CaseID | WanRedundancy_Solution_UsbIsBackup_35 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 4 |  |  |
| Description | WAN behavior when there is eth0 and wifi0 available, and both track wan and track default gateway for eth0 are up |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config wifi0 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Check wan state, as well as monitor state, result 4 5. Check default route in main route table, result 5 6. Run ping internet process on client to check its accessibility, result 6 7. Visit http on client, result 7 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: wifi0 is wan mode and connected with ISP AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Wifi0 0019:778c:8054 wan U 5 - radio_ng0 - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in wifi0 ¦ i mode Mode=wan; Radio disabled=no; Phymode=11ng; Short preamble mode=enabled; Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface wifi0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable Result 4: Active wan is eth0, and both monitor state and default monitor state of eth0 are Up AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: wifi0.2; State: up [use-for-wan-wifi0] WAN Monitor State: up Interface: wifi0 (wifi0) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Jan 30 00:38:44 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Jan 30 00:38:44 2013 Result 5: Default route is via eth0, and there is non-default route via wifi0 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.50.0 0.0.0.0 255.255.255.0 U 0 0 0 wifi0.2 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.50.1 0.0.0.0 UG 500 0 0 wifi0.2 Result 6: ping internet successfully on client Result 7: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_36

| CaseID | WanRedundancy_Solution_UsbIsBackup_36 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 4 |  |  |
| Description | WAN behavior when there is eth0 and wifi0 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for wifi0 is up |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config wifi0 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: wifi0 is wan mode and connected with ISP AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Wifi0 0019:778c:8054 wan U 5 - radio_ng0 - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in wifi0 ¦ i mode Mode=wan; Radio disabled=no; Phymode=11ng; Short preamble mode=enabled; Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface wifi0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-30 00:49:00 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: wifi0.2; State: up [use-for-wan-wifi0] WAN Monitor State: up Interface: wifi0 (wifi0) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) Result 5: wan failover to wifi0, both monitor state and default monitor state of wifi0 are Up AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: wifi0.2; State: up [use-for-wan-wifi0] WAN Monitor State: up Interface: wifi0 (wifi0) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Jan 30 01:14:44 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Jan 30 01:14:44 2013 Result 6: Default route is via wifi0, and there is non-default route via eth0 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.50.0 0.0.0.0 255.255.255.0 U 0 0 0 wifi0.2 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.50.1 0.0.0.0 UG 500 0 0 wifi0.2 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_37

| CaseID | WanRedundancy_Solution_UsbIsBackup_37 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 4 |  |  |
| Description | WAN behavior when there is eth0 and wifi0 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for wifi0 is down |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config wifi0 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 5. Track wan fail on wifi0, result 5 track-wan use-for-wan-wifi0 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: wifi0 is wan mode and connected with ISP AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Wifi0 0019:778c:8054 wan U 5 - radio_ng0 - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in wifi0 ¦ i mode Mode=wan; Radio disabled=no; Phymode=11ng; Short preamble mode=enabled; Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface wifi0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-30 00:49:00 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: wifi0.2; State: up [use-for-wan-wifi0] WAN Monitor State: up Interface: wifi0 (wifi0) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) Result 5: Track wan on wifi0 unsuccessfully, and monitor state of wifi0 is Down AH-8c8040#sh lo bu …. 2013-01-30 00:49:00 debug ah_brd: [brd_wanmon]: WFOIFM: [test for use-for-wan-wifi0]: Interface wifi0: 1 targets tested, result: FAIL .…. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: wifi0.2; State: up [use-for-wan-wifi0] WAN Monitor State: down Interface: wifi0 (wifi0) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Jan 30 01:14:44 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Jan 30 01:14:44 2013 Result 7: Default route is via eth0, and there is non-default route via wifi0 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.50.0 0.0.0.0 255.255.255.0 U 0 0 0 wifi0.2 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.50.1 0.0.0.0 UG 10500 0 0 wifi0.2 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_38

| CaseID | WanRedundancy_Solution_UsbIsBackup_38 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 4 |  |  |
| Description | WAN behavior when there is eth0 and wifi0 available, both track wan and track default gateway for eth0 are down, while track wan for wifi0 is up |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config wifi0 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 5. Track default gateway fail on eth0, result 5 no in eth0 dhcp client 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: wifi0 is wan mode and connected with ISP AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Wifi0 0019:778c:8054 wan U 5 - radio_ng0 - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in wifi0 ¦ i mode Mode=wan; Radio disabled=no; Phymode=11ng; Short preamble mode=enabled; Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface wifi0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable Result 4: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-30 00:49:00 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: wifi0.2; State: up [use-for-wan-wifi0] WAN Monitor State: up Interface: wifi0 (wifi0) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) Result 5: Track default gateway on eth0 unsuccessfully AH-8c8040#sh lo bu …. 2013-01-30 01:33:09 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xc0a83201 for wifi0.2 2013-01-30 01:33:09 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: wifi0.2; State: up [use-for-wan-wifi0] WAN Monitor State: up Interface: wifi0 (wifi0) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) Result 6: wan failover to wifi0, both monitor state and default monitor state of wifi0 are Up AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: wifi0.2; State: up [use-for-wan-wifi0] WAN Monitor State: up Interface: wifi0 (wifi0) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Jan 30 01:14:44 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Jan 30 01:14:44 2013 Result 7: Default route is via wifi0, and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.50.0 0.0.0.0 255.255.255.0 U 0 0 0 wifi0.2 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.50.1 0.0.0.0 UG 500 0 0 wifi0.2 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_39

| CaseID | WanRedundancy_Solution_UsbIsBackup_39 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 4 |  |  |
| Description | WAN behavior when there is eth0 and wifi0 available, both track wan and track default gateway for eth0 are down, while track wan for wifi0 is down and track default gateway for wifi0 is up |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config wifi0 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on eth0, result 4 _deb brd wanmon no in eth0 dhcp client 5. Track wan fail on wifi0, result 5 track-wan use-for-wan-wifi0 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: wifi0 is wan mode and connected with ISP AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Wifi0 0019:778c:8054 wan U 5 - radio_ng0 - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in wifi0 ¦ i mode Mode=wan; Radio disabled=no; Phymode=11ng; Short preamble mode=enabled; Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface wifi0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable Result 4: Track wan and Track default gateway on eth0 are unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu 2013-01-30 01:35:43 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-30 01:33:09 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: wifi0.2; State: up [use-for-wan-wifi0] WAN Monitor State: up Interface: wifi0 (wifi0) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) Result 5: Track wan on wifi0 unsuccessfully, and monitor state of wifi0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-30 00:49:00 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: wifi0.2; State: up [use-for-wan-wifi0] WAN Monitor State: down Interface: wifi0 (wifi0) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Jan 30 01:14:44 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Jan 30 01:14:44 2013 Result 7: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.50.0 0.0.0.0 255.255.255.0 U 0 0 0 wifi0.2 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.50.1 0.0.0.0 UG 10500 0 0 wifi0.2 Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_40

| CaseID | WanRedundancy_Solution_UsbIsBackup_40 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0 and wifi0 available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for wifi0 are down |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config wifi0 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on eth0, result 4 _deb brd wanmon no in eth0 dhcp client 5. Track wan fail on wifi0, result 5 no in w0 ssid ssgao-br100 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: wifi0 is wan mode and connected with ISP AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Wifi0 0019:778c:8054 wan U 5 - radio_ng0 - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in wifi0 ¦ i mode Mode=wan; Radio disabled=no; Phymode=11ng; Short preamble mode=enabled; Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface wifi0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable Result 4: Track wan and track default gateway on eth0 are unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu 2013-01-30 01:35:43 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-30 01:33:09 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: wifi0.2; State: up [use-for-wan-wifi0] WAN Monitor State: up Interface: wifi0 (wifi0) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) Result 5: Track wan and track default gatewayon wifi0 unsuccessfully, and monitor state of wifi0 is Down AH-8c8040#sh lo bu …. 2013-01-30 01:40:30 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-wifi0: interface is not connected. …… -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: down Interface: eth0; State: unknown Interface: wifi0.2; State: unknown [use-for-wan-wifi0] WAN Monitor State: unknown Interface: wifi0 (wifi0) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Wed Jan 30 01:42:09 2013 last_event: BACKUP_WAN_DOWN last_evt_time: Wed Jan 30 01:42:09 2013 Result 7: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_41

| CaseID | WanRedundancy_Solution_UsbIsBackup_41 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, and both track wan and track default gateway for eth0 are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e220 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Active wan is eth0, and both monitor state and default monitor state of eth0 are Up AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Sat Jan 26 20:22:01 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 20:22:01 2013 Result 6: Default route is via eth0, and there are non-default route via usbnet and eth1 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_42

| CaseID | WanRedundancy_Solution_UsbIsBackup_42 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway successfully on eth0, result 6 7. Track wan successfully on usbnet, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and monitor state ofeth0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track default gateway on eth0 successfully, and default monitor state of eth0 is Up AH-8c8040#AH-8c8040#sh lo bu 2013-01-26 23:33:52 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth0: 1 targets tested, result: PASS - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 7: Track wan on usbnet successfully, and monitor state of usbnet is Up AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-26 23:33:49 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: PASS …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 8: wan failover to usbnet AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth1 kifname: eth1 …… priority: 10 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: Default route is via usbnet, and there are non-default route via eth0 and eth1 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_43

| CaseID | WanRedundancy_Solution_UsbIsBackup_43 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is down and track wan for eth1 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway successfully on eth0, result 6 7. Track wan fail on usbnet, result 7 track-wan use-for-wan-usbnet0 ip 1.1.1.1 8. Track wan successfully on eth1, result 8 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track default gateway on eth0 successfully, and default monitor state of eth0 is Up AH-8c8040#AH-8c8040#sh lo bu 2013-01-26 23:33:52 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth0: 1 targets tested, result: PASS - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 7: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-26 19:34:13 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: Track wan on eth1 successfully, and monitor state of eth1 is Up AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth1: 1 targets tested, result: PASS ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 9: wan failover to eth1 AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth1 kifname: eth1 …… priority: 10 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 10: Default route is via eth1, and there are non-default route via eth0 and usbnet with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_44

| CaseID | WanRedundancy_Solution_UsbIsBackup_44 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is down and track wan for eth1 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway successfully on eth0, result 6 7. Track wan fail on usbnet, result 7 track-wan use-for-wan-usbnet0 ip 1.1.1.1 8. Track wan fail on eth1, result 8 track-wan use-for-wan-eth1 ip 1.1.1.1 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track default gateway on eth0 successfully, and default monitor state of eth0 is Up AH-8c8040#AH-8c8040#sh lo bu … 2013-01-26 23:33:52 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth0: 1 targets tested, result: PASS … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 7: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-26 23:33:49 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State:down Interface: usbnet0 (ppp0) Result 8: Track wan on eth1 unsuccessfully, and monitor state of eth1 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth1: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 9: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 1 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth1 kifname: eth1 …… priority: 10 is_wan: 1 is_default_wan: 0 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 10: Default route is via eth0, and there are non-default route via usbnet and eth1 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_45

| CaseID | WanRedundancy_Solution_UsbIsBackup_45 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway fail on eth0, result 6 no in eth0 dhcp client 7. Track wan successfully on usbnet, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and monitor state of wifi0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-27 00:01:44 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …… - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State:unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 7: Track wan on usbnet successfully, and monitor state of usbnet is Up AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-26 23:33:49 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: PASS …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 8: wan failover to usbnet AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth1 kifname: eth1 …… priority: 10 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: Default route is via usbnet, and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_46

| CaseID | WanRedundancy_Solution_UsbIsBackup_46 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is down and track wan for eth1 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, and track default gateway fail on eth0, result 6 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet, result 6 track-wan use-for-wan-usbnet0 ip 1.1.1.1 7. Track wan successfully on eth1, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-26 23:33:49 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State:down Interface: usbnet0 (ppp0) Result 7: Track wan on eth1 successfully, and monitor state of eth1 is Up AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth1: 1 targets tested, result: PASS ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: wan failover to eth1 AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth1 kifname: eth1 …… priority: 10 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: Default route is via eth1, and and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_47

| CaseID | WanRedundancy_Solution_UsbIsBackup_47 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is down and track default gateway for usbnet is up, and track wan for eth1 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, and track default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet, result 6 track-wan use-for-wan-usbnet0 ip 1.1.1.1 7. Track wan fail on eth1, result 7 track-wan use-for-wan-eth01 ip 1.1.1.1 7. Check wan state, as well as monitor state, result 7 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-26 23:33:49 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State:down Interface: usbnet0 (ppp0) Result 7: Track wan on eth1 unsuccessfully, and monitor state of eth1 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-27 00:52:44 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth1: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State:down Interface: usbnet0 (ppp0) Result 8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth1 kifname: eth1 …… priority: 10 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: default gw with highest priority is via eth1 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_48

| CaseID | WanRedundancy_Solution_UsbIsBackup_48 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for usbnet are down, and track wan for eth1 is down and track default gateway for eth1 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan and default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan and default gateway fail on usbnet, result 6 no usbm modem-id huawei-e220 apn 7. Track wan fail on eth1, result 7 track-wan use-for-wan-eth1 ip 1.1.1.1 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, andtrack default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet unsuccessfully, andtrack default gateway on usbnet unsuccessfully, and default monitor state of usbnet is unknown AH-8c8040#sh lo bu …. 2013-01-28 19:55:13 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State:unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth1 unsuccessfully, and monitor state of eth1 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-27 00:52:44 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth1: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State:unknown Interface: usbnet0 (ppp0) Result 8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth1 kifname: eth1 …… priority: 10 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: default gw with highest priority is via eth1 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_49

| CaseID | WanRedundancy_Solution_UsbIsBackup_49 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for usbnet are down, and both track wan and track default gateway for eth1 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan and default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan and default gateway fail on usbnet, result 6 no usbm modem-id huawei-e220 apn 7. Track wan and default gateway fail on eth1, result 7 no in eth1 dhcp client 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unkonwn Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet unsuccessfully, andtrack default gateway on usbnet unsuccessfully, and default monitor state of usbnet is unknown AH-8c8040#sh lo bu …. 2013-01-28 19:55:13 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State:unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth1 unsuccessfully, and track default gateway on eth1 unsuccessfully, and default monitor state of eth1 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth1: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: unknown [use-for-wan-eth1] WAN Monitor State: unknown Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unkonwn Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 9: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth1 kifname: eth1 …… priority: 10 is_wan: 1 is_default_wan: 0 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 10: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 11: ping internet will be reponsed with mgt0 Result 12: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_50

| CaseID | WanRedundancy_Solution_UsbIsBackup_50 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, and both track wan and track default gateway for eth0 are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth2 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth2 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Active wan is eth0, and both monitor state and default monitor state of eth0 are Up AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Sat Jan 26 20:22:01 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 20:22:01 2013 Result 6: Default route is via eth0, and there are non-default route via usbnet and eth2 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 20 0 0 eth2 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_51

| CaseID | WanRedundancy_Solution_UsbIsBackup_51 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth2 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway successfully on eth0, result 6 7. Track wan successfully on usbnet, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth2 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track default gateway on eth0 successfully, and default monitor state of eth0 is Up AH-8c8040#AH-8c8040#sh lo bu 2013-01-26 23:33:52 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth0: 1 targets tested, result: PASS - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 7: Track wan on usbnet successfully, and monitor state of usbnet is Up AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-26 23:33:49 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: PASS …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 8: wan failover to usbnet AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth2 kifname: eth2 …… priority: 20 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: Default route is via usbnet, and there are non-default route via eth0 and eth2 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.20.1 0.0.0.0 UG 20 0 0 eth2 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_52

| CaseID | WanRedundancy_Solution_UsbIsBackup_52 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is down and track wan for eth2 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth2 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway successfully on eth0, result 6 7. Track wan fail on usbnet, result 7 track-wan use-for-wan-eth2 ip 1.1.1.1 8. Track wan successfully on eth2, result 8 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth2 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track default gateway on eth0 successfully, and default monitor state of eth0 is Up AH-8c8040#AH-8c8040#sh lo bu 2013-01-26 23:33:52 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth0: 1 targets tested, result: PASS - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2 WAN Monitor State: up Interface: eth2 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 7: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-26 19:34:13 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: Track wan on eth2 successfully, and monitor state of eth2 is Up AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth2: 1 targets tested, result: PASS ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 9: wan failover to eth2 AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth2 kifname: eth2 …… priority: 20 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 10: Default route is via eth2, and there are non-default route via eth0 and usbnet with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.20.1 0.0.0.0 UG 20 0 0 eth2 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_53

| CaseID | WanRedundancy_Solution_UsbIsBackup_53 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is down and track wan for eth2 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth2 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway successfully on eth0, result 6 7. Track wan fail on usbnet, result 7 track-wan use-for-wan-usbnet0 ip 1.1.1.1 8. Track wan fail on eth2, result 8 track-wan use-for-wan-eth2 ip 1.1.1.1 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth2 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and monitor state of wifi0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track default gateway on eth0 successfully, and default monitor state of eth0 is Up AH-8c8040#AH-8c8040#sh lo bu … 2013-01-26 23:33:52 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth0: 1 targets tested, result: PASS … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 7: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-26 23:33:49 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State:down Interface: usbnet0 (ppp0) Result 8: Track wan on eth2 unsuccessfully, and monitor state of eth2 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth2: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: down Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 9: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 1 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth2 kifname: eth2 …… priority: 20 is_wan: 1 is_default_wan: 0 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 10: Default route is via eth0, and there are non-default route via usbnet and eth2 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.20.1 0.0.0.0 UG 10020 0 0 eth2 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_54

| CaseID | WanRedundancy_Solution_UsbIsBackup_54 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth2 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway fail on eth0, result 6 no in eth0 dhcp client 7. Track wan successfully on usbnet, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth2 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and monitor state of wifi0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-27 00:01:44 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …… - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State:unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 7: Track wan on usbnet successfully, and monitor state of usbnet is Up AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-26 23:33:49 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: PASS …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 8: wan failover to usbnet AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth2 kifname: eth2 …… priority: 20 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: Default route is via usbnet, and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.20.1 0.0.0.0 UG 20 0 0 eth2 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_55

| CaseID | WanRedundancy_Solution_UsbIsBackup_55 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is down and track wan for eth2 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth2 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0 andtrack default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet, result 6 track-wan use-for-wan-usbnet0 ip 1.1.1.1 7. Track wan successfully on eth2, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth2 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-26 23:33:49 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State:down Interface: usbnet0 (ppp0) Result 7: Track wan on eth2 successfully, and monitor state of eth2 is Up AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth2: 1 targets tested, result: PASS ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: wan failover to eth2 AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth2 kifname: eth2 …… priority: 20 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: Default route is via eth2, and and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.20.1 0.0.0.0 UG 20 0 0 eth2 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_56

| CaseID | WanRedundancy_Solution_UsbIsBackup_56 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is down and track default gateway for usbnet is up, and track wan for eth2 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth2 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0,and track default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet, result 6 track-wan use-for-wan-usbnet0 ip 1.1.1.1 7. Track wan fail on eth2, result 7 track-wan use-for-wan-eth2 ip 1.1.1.1 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth2 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-26 23:33:49 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State:down Interface: usbnet0 (ppp0) Result 7: Track wan on eth2 unsuccessfully, and monitor state of eth2 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-27 00:52:44 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth2: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: down Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State:down Interface: usbnet0 (ppp0) Result 8: wan will failover AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth2 kifname: eth2 …… priority: 20 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.20.1 0.0.0.0 UG 10020 0 0 eth2 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 10: Png internet will be reponsed with mgt0 Result 11:Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_57

| CaseID | WanRedundancy_Solution_UsbIsBackup_57 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for usbnet are down, and track wan for eth2 is down and track default gateway for eth2 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth2 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan on eth0 and default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet andtrack default gateway fail on usbnet, result6 no usbm modem-id huawei-e1752 apn 7. Track wan fail on eth2, result 7 track-wan use-for-wan-eth2 ip 1.1.1.1 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth2 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet unsuccessfully, andtrack default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-28 19:55:13 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State:unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth2 unsuccessfully, and monitor state of eth2 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-27 00:52:44 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth2: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up Interface: ppp0; State: unknown [use-for-wan-eth2] WAN Monitor State: down Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth2 kifname: eth2 …… priority: 20 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.20.1 0.0.0.0 UG 10020 0 0 eth2 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_58

| CaseID | WanRedundancy_Solution_UsbIsBackup_58 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for usbnet are down, and both track wan and track default gateway for eth2 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth2 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0 andtrack default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet andtrack default gateway fail on usbnet, result 6 no usbm modem-id huawei-e1752 apn 7. Track wan fail on eth1 andtrack default gateway fail on eth1, result 7 no in eth2 dhcp client 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth2 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet unsuccessfully, andtrack default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-28 19:55:13 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State:unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 unsuccessfully, andtrack default gateway on eth2 unsuccessfully, and default monitor state of eth2 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth2: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: unknown [use-for-wan-eth2] WAN Monitor State: unknown Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth2 kifname: eth2 …… priority: 20 is_wan: 1 is_default_wan: 0 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#show wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: FAILOVER prev_state_time: Mon Jan 28 23:33:11 2013 last_event: NOWAN last_evt_time: Mon Jan 28 23:33:56 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_59

| CaseID | WanRedundancy_Solution_UsbIsBackup_59 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, and both track wan and track default gateway for eth0 are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth3 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Active wan is eth0, and both monitor state and default monitor state of eth0 are Up AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Sat Jan 26 20:22:01 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 20:22:01 2013 Result 6: Default route is via eth0, and there are non-default route via usbnet and eth3 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.30.1 0.0.0.0 UG 30 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_60

| CaseID | WanRedundancy_Solution_UsbIsBackup_60 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth3 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway successfully on eth0, result 6 7. Track wan successfully on usbnet, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track default gateway on eth0 successfully, and default monitor state of eth0 is Up AH-8c8040#AH-8c8040#sh lo bu 2013-01-26 23:33:52 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth0: 1 targets tested, result: PASS - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 7: Track wan on usbnet successfully, and monitor state of usbnet is Up AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-26 23:33:49 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: PASS …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 8: wan failover to usbnet AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth3 kifname: eth3 …… priority: 30 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: Default route is via usbnet, and there are non-default route via eth0 and eth3 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.30.1 0.0.0.0 UG 30 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_61

| CaseID | WanRedundancy_Solution_UsbIsBackup_61 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is down and track wan for eth3 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth3 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway successfully on eth0, result 6 7. Track wan fail on usbnet, result 7 track-wan use-for-wan-usbnet0 ip 1.1.1.1 8. Track wan successfully on eth3, result 8 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and monitor state of wifi0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track default gateway on eth0 successfully, and default monitor state of eth0 is Up AH-8c8040#AH-8c8040#sh lo bu 2013-01-26 23:33:52 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth0: 1 targets tested, result: PASS - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 7: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-26 19:34:13 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: Track wan on eth3 successfully, and monitor state of eth3 is Up AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth3: 1 targets tested, result: PASS ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 9: wan failover to eth3 AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth3 kifname: eth3 …… priority: 30 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 10: Default route is via eth3, and there are non-default route via eth0 and usbnet with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.30.1 0.0.0.0 UG 30 0 0 eth3 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_62

| CaseID | WanRedundancy_Solution_UsbIsBackup_62 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is down and track wan for eth3 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth3 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway successfully on eth0, result 6 7. Track wan fail on usbnet, result 7 track-wan use-for-wan-usbnet0 ip 1.1.1.1 8. Track wan fail on eth3, result 8 track-wan use-for-wan-eth3 ip 1.1.1.1 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track default gateway on eth0 successfully, and default monitor state of eth0 is Up AH-8c8040#AH-8c8040#sh lo bu 2013-01-26 23:33:52 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth0: 1 targets tested, result: PASS - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 7: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-26 19:34:13 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: Track wan on eth3 unsuccessfully, and monitor state of eth3 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth3: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: down Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 9: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 1 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth3 kifname: eth3 …… priority: 30 is_wan: 1 is_default_wan: 0 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 10: Default route is via eth0, and there are non-default route via usbnet and eth3 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.30.1 0.0.0.0 UG 10030 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_63

| CaseID | WanRedundancy_Solution_UsbIsBackup_63 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth3 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway fail on eth0, result 6 no in eth0 dhcp client 7. Track wan successfully on usbnet, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and monitor state of wifi0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-27 00:01:44 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …… - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State:unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 7: Track wan on usbnet successfully, and monitor state of usbnet is Up AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-26 23:33:49 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: PASS …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 8: wan failover to usbnet AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth3 kifname: eth3 …… priority: 30 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: Default route is via usbnet, and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.20.1 0.0.0.0 UG 30 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_64

| CaseID | WanRedundancy_Solution_UsbIsBackup_64 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is down and track wan for eth3 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth3 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0 and Track default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet, result 6 track-wan use-for-wan-usbnet0 ip 1.1.1.1 7. Track wan successfully on eth3, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-26 19:34:13 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Track wan on eth3 successfully, and monitor state of eth3 is Up AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth3: 1 targets tested, result: PASS ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3 State: up Interface: ppp0; State: up [use-for-wan-eth3 WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: wan failover to eth3 AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth3 kifname: eth3 …… priority: 30 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: Default route is via eth3, and and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.30.1 0.0.0.0 UG 30 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_65

| CaseID | WanRedundancy_Solution_UsbIsBackup_65 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is down and track default gateway for usbnet is up, and track wan for eth3 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth3 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0 and track default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet, result 6 track-wan use-for-wan-usbnet0 ip 1.1.1.1 7. Track wan fail on eth3, result 7 track-wan use-for-wan-eth3 ip 1.1.1.1 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-26 19:34:13 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Track wan on eth3 unsuccessfully, and monitor state of eth3 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth3: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: down Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: wan will failover. AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth3 kifname: eth3 …… priority: 30 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.20.1 0.0.0.0 UG 10030 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_66

| CaseID | WanRedundancy_Solution_UsbIsBackup_66 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for usbnet are down, and track wan for eth3 is down and track default gateway for eth3 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth3 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0 and track default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet andtrack default gateway fail on usbnet, result 6 no usbm modem-id huawei-e1752 apn 7. Track wan fail on eth3, result 7 track-wan use-for-wan-eth3 ip 1.1.1.1 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet unsuccessfully, andtrack default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-28 19:55:13 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State:unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth3 unsuccessfully, and monitor state of eth3 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-27 00:52:44 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth3: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up Interface: ppp0; State: unknown [use-for-wan-eth3] WAN Monitor State: down Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan will failover AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth3 kifname: eth3 …… priority:30 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.30.1 0.0.0.0 UG 10030 0 0 eth3 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_67

| CaseID | WanRedundancy_Solution_UsbIsBackup_67 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for usbnet are down, and both track wan and track default gateway for eth3 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth3 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0 and track default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet andtrack default gateway fail on usbnet, result 6 no usbm modem-id huawei-e1752 apn 7. Track wan fail on eth0 and track default gateway fail on eth0, result 7 no in eth3 dhcp client 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet unsuccessfully, andtrack default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-28 19:55:13 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State:unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth3 unsuccessfully, andtrack default gateway on eth3 unsuccessfully, and default monitor state of eth3 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth3: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: unknown [use-for-wan-eth3] WAN Monitor State: unknown Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth3 kifname: eth3 …… priority: 30 is_wan: 1 is_default_wan: 0 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#show wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: FAILOVER prev_state_time: Mon Jan 28 23:33:11 2013 last_event: NOWAN last_evt_time: Mon Jan 28 23:33:56 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_68

| CaseID | WanRedundancy_Solution_UsbIsBackup_68 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, and both track wan and track default gateway for eth0 are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth4 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Active wan is eth0, and both monitor state and default monitor state of eth0 are Up AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Sat Jan 26 20:22:01 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 20:22:01 2013 Result 6: Default route is via eth0, and there are non-default route via usbnet and eth4 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.40.1 0.0.0.0 UG 40 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_69

| CaseID | WanRedundancy_Solution_UsbIsBackup_69 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth4 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway successfully on eth0, result 6 7. Track wan successfully on usbnet, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and monitor state of wifi0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track default gateway on eth0 successfully, and default monitor state of eth0 is Up AH-8c8040#AH-8c8040#sh lo bu 2013-01-26 23:33:52 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth0: 1 targets tested, result: PASS - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 7: Track wan on usbnet successfully, and monitor state of usbnet is Up AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-26 23:33:49 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: PASS …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 8: wan failover to usbnet AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth4 kifname: eth4 …… priority: 40 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: Default route is via usbnet, and there are non-default route via eth0 and eth4 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.40.1 0.0.0.0 UG 30 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_70

| CaseID | WanRedundancy_Solution_UsbIsBackup_70 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is down and track wan for eth4 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth4 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway successfully on eth0, result 6 7. Track wan fail on usbnet, result 7 track-wan use-for-wan-usbnet0 ip 1.1.1.1 8. Track wan successfully on eth4, result 8 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and monitor state of wifi0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track default gateway on eth0 successfully, and default monitor state of eth0 is Up AH-8c8040#AH-8c8040#sh lo bu 2013-01-26 23:33:52 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth0: 1 targets tested, result: PASS - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 7: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-26 19:34:13 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: Track wan on eth4 successfully, and monitor state of eth4 is Up AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth4: 1 targets tested, result: PASS ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 9: wan failover to eth4 AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth4 kifname: eth4 …… priority: 40 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 10: Default route is via eth4, and there are non-default route via eth0 and usbnet with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.30.1 0.0.0.0 UG 40 0 0 eth4 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_71

| CaseID | WanRedundancy_Solution_UsbIsBackup_71 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is down and track wan for eth4 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth4 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway successfully on eth0, result 6 7. Track wan fail on usbnet, result 7 track-wan use-for-wan-usbnet0 ip 1.1.1.1 8. Track wan fail on eth4, result 8 track-wan use-for-wan-eth4 ip 1.1.1.1 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and monitor state of wifi0 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track default gateway on eth0 successfully, and default monitor state of eth0 is Up AH-8c8040#AH-8c8040#sh lo bu 2013-01-26 23:33:52 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth0: 1 targets tested, result: PASS - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 7: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-26 19:34:13 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: Track wan on eth4 unsuccessfully, and monitor state of eth4 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth4: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 9: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 1 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth4 kifname: eth4 …… priority: 40 is_wan: 1 is_default_wan: 0 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 10: Default route is via eth0, and there are non-default route via usbnet and eth4 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.40.1 0.0.0.0 UG 10040 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_72

| CaseID | WanRedundancy_Solution_UsbIsBackup_72 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth4 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0 and track default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan successfully on usbnet, result 6 7. Check wan state, as well as monitor state, result 7 8. Check default route in main route table, result 8 9. Run ping internet process on client to check its accessibility, result 9 10. Visit http on client, result 10 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet successfully, and monitor state of usbnet is Up AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-26 23:33:49 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: PASS …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 7: wan failover to usbnet AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth4 kifname: eth4 …… priority: 40 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 8: Default route is via usbnet, and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.20.1 0.0.0.0 UG 40 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 9: ping internet successfully on client Result 10: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_73

| CaseID | WanRedundancy_Solution_UsbIsBackup_73 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is down and track wan for eth4 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth4 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0 and track default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet, result 6 track-wan use-for-wan-usbnet0 ip 1.1.1.1 7. Track wan successfully on eth4, result 7 8. Check wan state, as well as monitor state, result8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-26 19:34:13 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Track wan on eth4 successfully, and monitor state of eth4 is Up AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth4: 1 targets tested, result: PASS ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4 State: up Interface: ppp0; State: up [use-for-wan-eth4 WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: wan failover to eth4 AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth4 kifname: eth4 …… priority: 40 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: Default route is via eth4, and and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.40.1 0.0.0.0 UG 40 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_74

| CaseID | WanRedundancy_Solution_UsbIsBackup_74 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is down and track default gateway for usbnet is up, and track wan for eth4 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth4 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0,and track default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet, result 6 track-wan use-for-wan-usbnet0 ip 1.1.1.1 7. Track wan fail on eth4, result 7 track-wan use-for-wan-eth4 ip 1.1.1.1 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-26 19:34:13 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Track wan on eth4 unsuccessfully, and monitor state of eth4 is Down AH-8c8040#sh lo bu ….. 2013-01-26 19:12:58 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth4: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth4 kifname: eth4 …… priority: 40 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.40.1 0.0.0.0 UG 10040 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_75

| CaseID | WanRedundancy_Solution_UsbIsBackup_75 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for usbnet are down, and track wan for eth4 is down and track default gateway for eth4 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth4 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0,and track default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet andtrack default gateway fail on usbnet, result 6 no usbm modem-id huawei-e1752 apn 7. Track wan fail on eth4, result 7 track-wan use-for-wan-eth4 ip 1.1.1.1 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet unsuccessfully, andtrack default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-28 19:55:13 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State:unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth4 unsuccessfully, and monitor state of eth4 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-27 00:52:44 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth4: 1 targets tested, result: FAIL ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up Interface: ppp0; State: unknown [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth4 kifname: eth4 …… priority:40 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Sat Jan 26 23:07:57 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Sat Jan 26 23:07:57 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.40.1 0.0.0.0 UG 10040 0 0 eth4 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_76

| CaseID | WanRedundancy_Solution_UsbIsBackup_76 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for usbnet are down, and both track wan and track default gateway for eth4 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth4 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, and track default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet andtrack default gateway fail on usbnet, result 6 no usbm modem-id huawei-e1752 apn 7. Track wan fail on eth4, and track default gateway fail on eth4, result 7 no in eth4 dhcp client 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=600 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet unsuccessfully, andtrack default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-28 19:55:13 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State:unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth4 unsuccessfully, and track default gateway on eth4 unsuccessfully, and default monitor state of eth4 is Down AH-8c8040#sh lo bu … 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth4: interface is not connected. 2013-01-26 19:37:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: unknown [use-for-wan-eth4] WAN Monitor State: unknown Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan in ifname: eth0 kifname: eth0 ….. priority: 1 is_wan: 1 is_default_wan: 0 ….. ifname: usbnet0 kifname: ppp0 ….. priority: 600 is_wan: 1 is_default_wan: 0 …… ifname: eth4 kifname: eth4 …… priority: 40 is_wan: 1 is_default_wan: 0 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#show wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: FAILOVER prev_state_time: Mon Jan 28 23:33:11 2013 last_event: NOWAN last_evt_time: Mon Jan 28 23:33:56 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_77

| CaseID | WanRedundancy_Solution_UsbIsBackup_77 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0 available, and both track wan and track default gateway for eth0 are up |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config wifi0 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: wifi0 is wan mode and connected with ISP Result 4: Each interface is configured with a track wan group Result 5: Active wan is eth0, and both monitor state and default monitor state of eth0 are Up Result 6: Default route is via eth0, and there are non-default route via usbnet and wifi0 with corresponding priority Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_78

| CaseID | WanRedundancy_Solution_UsbIsBackup_78 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is up |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config wifi0 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, result 5 6. Track default gateway successfully on eth0, result 6 7. Track wan successfully on usbnet, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: wifi0 is wan mode and connected with ISP Result 4: Each interface is configured with a track wan group Result 5: Track wan on eth0 unsuccessfully, and monitor state of wifi0 is Down Result 6: Track default gateway on eth0 successfully, and default monitor state of eth0 is Up Result 7: Track wan on usbnet successfully, and monitor state of usbnet is Up Result 8: wan failover to usbnet Result 9: Default route is via usbnet, and there are non-default route via eth0 and wifi0 with corresponding priority Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_79

| CaseID | WanRedundancy_Solution_UsbIsBackup_79 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is down and track wan for wifi0 is up |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config wifi0 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway successfully on eth0, result 6 7. Track wan fail on usbnet, result 7 track-wan use-for-wan-usbnet0 ip 1.1.1.1 8. Track wan successfully on wifi0, result 8 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: wifi0 is wan mode and connected with ISP Result 4: Each interface is configured with a track wan group Result 5: Track wan on eth0 unsuccessfully, and monitor state of wifi0 is Down Result 6: Track default gateway on eth0 successfully, and default monitor state of eth0 is Up Result 7: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down Result 8: Track wan on wifi0 successfully, and monitor state of wifi0 is Up Result 9: wan failover to wifi0 Result 10: Default route is via wifi0, and there are non-default route via eth0 and usbnet with corresponding priority Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_80

| CaseID | WanRedundancy_Solution_UsbIsBackup_80 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is down and track wan for wifi0 is down |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config wifi0 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0, and track default gateway successfully on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet, result 6 track-wan use-for-wan-usbnet0 ip 1.1.1.1 7. Track wan fail on wifi0, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 11 11. Visit http on client, result 11 |  |  |
| ExpectResult | Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: wifi0 is wan mode and connected with ISP Result 4: Each interface is configured with a track wan group Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 successfully, and default monitor state of eth0 is Up Result 6: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down Result 7: Track wan on wifi0 unsuccessfully, and monitor state of wifi0 is Down Result 8: wan won't failover, and wan state is “no wan” Result 9: Default route is via eth0, and there are non-default route via usbnet and wifi0 with corresponding priority Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_81

| CaseID | WanRedundancy_Solution_UsbIsBackup_81 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is up |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config wifi0 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0 and track default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan successfully on usbnet, result 6 7. Check wan state, as well as monitor state, result 7 8. Check default route in main route table, result 8 9. Run ping internet process on client to check its accessibility, result 9 10. Visit http on client, result 10 |  |  |
| ExpectResult | Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: wifi0 is wan mode and connected with ISP Result 4: Each interface is configured with a track wan group Result 5: Track wan on eth0 unsuccessfully, and monitor state of wifi0 is Down Result 6: Track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down Result 7: Track wan on usbnet successfully, and monitor state of usbnet is Up Result 8: wan failover to usbnet Result 9: Default route is via usbnet, and there is no default route via eth0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_82

| CaseID | WanRedundancy_Solution_UsbIsBackup_82 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is down and track wan for wifi0 is up |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config wifi0 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0 and track default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet, result 6 track-wan use-for-wan-eth4 ip 1.1.1.1 7. Track wan successfully on wifi0, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: wifi0 is wan mode and connected with ISP Result 4: Each interface is configured with a track wan group Result 5: Track wan on eth0 unsuccessfully,and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down Result 6: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down Result 7: Track wan on wifi0 successfully, and monitor state of wifi0 is Up Result 8: wan failover to wifi0 Result 9: Default route is via wifi0, and and there is no default route via eth0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_83

| CaseID | WanRedundancy_Solution_UsbIsBackup_83 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is down and track default gateway for usbnet is up, and track wan for wifi0 is down |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config wifi0 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0 and track default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet, result 6 track-wan use-for-wan-usbnet0 ip 1.1.1.1 7. Track wan fail on wifi0, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 , 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: wifi0 is wan mode and connected with ISP Result 4: Each interface is configured with a track wan group Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down Result 6: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down Result 7: Track wan on wifi0 unsuccessfully, and monitor state of wifi0 is Down Result 8: wan won't failover, and wan state is “no wan” Result 9: default gw with highest priority is via eth0 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_84

| CaseID | WanRedundancy_Solution_UsbIsBackup_84 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0 available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for usbnet are down, and track wan for wifi0 is down and track default gateway for wifi0 is up |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config wifi0 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0 and default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet, result 6 track-wan use-for-wan-usbnet0 ip 1.1.1.1 7. Track wan fail on wifi0, result 7 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: wifi0 is wan mode and connected with ISP Result 4: Each interface is configured with a track wan group Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down Result 6: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down Result 7: Track wan on wifi0 unsuccessfully, and monitor state of wifi0 is Down Result 8: wan won't failover, and wan state is “no wan” Result 9: default gw with highest priority is via eth0 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_85

| CaseID | WanRedundancy_Solution_UsbIsBackup_85 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0 available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for usbnet are down, and both track wan and track default gateway for wifi0 are down |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config wifi0 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on eth0 and track default gateway fail on eth0, result 5 _deb brd wanmon no in eth0 dhcp client 6. Track wan fail on usbnet andtrack default gateway fail on usbnet, result 6 7. Track wan fail on wifi0 andtrack default gateway fail on wifi0, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: wifi0 is wan mode and connected with ISP Result 4: Each interface is configured with a track wan group Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down Result 6: Track wan on usbnet unsuccessfully, andtrack default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down Result 8: Track wan on wifi0 unsuccessfully, and monitor state of wifi0 is Down Result 9: wan won't failover, and wan state is “no wan” Result 10: default gw with highest priority is via eth0 Result 11: ping internet will be reponsed with mgt0 Result 12: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### 6.1.2. USB is Primary Mode

#### WanRedundancy_Solution_UsbIsBackup_01

| CaseID | WanRedundancy_Solution_UsbIsBackup_01 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is only usb0 available, and both track wan and track default gateway for usb0 are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Usb0 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config track wan group for usb0, result 2 3. Check wan state, as well as monitor state, result 3 4. Check default route in main route table, result 4 5. Run ping internet process on client to check its accessibility, result 5 6. Visit http on client, result 6 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: Usb0 is configured with track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 3: Active wan is usb0, and both monitor state and default monitor state of usb0 are Up AH-8c8040#sh wan in ifname: usbnet0 kifname: ppp0 …. priority: 0 is_wan: 1 is_default_wan: 1 ….. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan D - - - - - Ppp0 0000:0000:0000 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: ppp0; State: up [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: FAILOVER prev_state_time: Tue Jan 29 02:50:41 2013 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Tue Jan 29 02:50:41 2013 Result 4: Default route is via usb0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 0 0 0 ppp0 Result 5: ping internet successfully on client Result 6: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_02

| CaseID | WanRedundancy_Solution_UsbIsBackup_02 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is only usb0 available, and track wan for usb0 is down while track default gateway is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Usb0 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config track wan group for usb0, result 2 3. Track wan fail on usb0, result 3 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 4. Check wan state, as well as monitor state, result 4 5. Check default route in main route table, result 5 6. Run ping internet process on client to check its accessibility, result 6 7. Visit http on client, result 7 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: Usb0 is configured with track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 3: Track wan on usb0 unsuccessfully AH-8c8040#sh lo bu ….. 2013-01-29 02:55:34 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. Result 4: wan state is “no wan”, monitor state of usb0 is Down, default monitor state of usb0 is Up AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: ppp0; State: up [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Tue Jan 29 02:54:03 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 02:54:03 2013 Result 5: Default route is via usb0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 Result 6: ping internet successfully on client Result 7: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_03

| CaseID | WanRedundancy_Solution_UsbIsBackup_03 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is only usb0 available, and both track wan and track default gateway for usb0 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Usb0 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config track wan group for usb0, result 2 3. Track wan fail on usb0, result 3 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 4.Track default gateway fail on usb0, result 4 no usbm modem-id Huawei_e1752 apn 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: Usb0 is configured with track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 3: Track wan on usb0 unsuccessfully AH-8c8040#sh lo bu ….. 2013-01-29 02:55:34 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. Result 4: Track default gateway on usb0 unsuccessfully AH-8c8040#sh lo bu …. 2013-01-29 03:03:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. Result 5: wan state is “no wan”, and both monitor state and default monitor state of eth0 are Down AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Tue Jan 29 03:05:12 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 03:05:12 2013 Result 6: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 7: ping internet will be reponsed with mgt0 Result 8: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_04

| CaseID | WanRedundancy_Solution_UsbIsBackup_04 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0 and usb available, and both track wan and track default gateway for usb0 are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. usb0 is modem is plugged initially 2. Eth0 connected with Switch initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config track wan group each interface, result 3 4. Check wan state, as well as monitor state, result 4 5. Check default route in main route table, result 5 6. Run ping internet process on client to check its accessibility, result 6 7. Visit http on client, result 7 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Active wan is usb0, and both monitor state and default monitor state of usb0 are Up AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 5: Default route is via usb0, and there is non-default route via eth0 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 0 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 Result 6: ping internet successfully on client Result 7: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_05

| CaseID | WanRedundancy_Solution_UsbIsBackup_05 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0 and usb available, track wan for usbnet0 is down and track default gateway for usbnet0 is up, while track wan for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on usb0, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usb0 unsuccessfully, and monitor state of usb0 is Down AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 5: wan failover to usbnet, both monitor state and default monitor state of eth0 are Up AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 18:55:42 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 18:55:42 2013 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Default route is via usbnet, and there is non-default route via eth0 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_06

| CaseID | WanRedundancy_Solution_UsbIsBackup_06 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0 and usb available, track wan for usb0 is down and track default gateway for usb0 is up, while track wan for eth0 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on usb0, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Track wan fail on eth0, result 5 track-wan use-for-wan-eth0 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usb0 unsuccessfully, and monitor state of usb0 is Down AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 5: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:05:59 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: wan will failover AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 19:04:59 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:04:59 2013 Result 7: Default route is via eth0, and there is non-default route via usbnet with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_07

| CaseID | WanRedundancy_Solution_UsbIsBackup_07 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0 and usb available, both track wan and track default gateway for usb0 are down, while track wan for eth0 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on usb0, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Track default gateway fail on usb0, result 5 no usbm modem-id Huawei_e1752 apn 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usb0 unsuccessfully, and monitor state of usb0 is Down AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 5: Track default gateway on usb0 unsuccessfully AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan failover to eth0, both monitor state and default monitor state of eth0 are Up AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 19:39:28 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:40:34 2013 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: unknown [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Default route is via usbnet, and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_08

| CaseID | WanRedundancy_Solution_UsbIsBackup_08 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0 and usb available, both track wan and track default gateway for usb0 are down, while track wan for eth0 is down and track default gateway for eth0 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on usbnet, result 4 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 5. Track wan fail on eth0, result 5 track-wan use-for-wan-eth0 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on usbnet are unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:48:33 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Tue Jan 29 19:48:16 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:49:22 2013 Result 7: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_09

| CaseID | WanRedundancy_Solution_UsbIsBackup_09 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0 and usb available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for usbnet are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on usb0, result 4 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 5. Both track wan and track default gateway fail on eth0, result 5 no in eth0 dhcp client 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on usbnet are unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan and Track default gateway on eth0 are unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:55:28 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-29 19:55:24 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: down Interface: eth0; State: unknown [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Tue Jan 29 19:55:58 2013 last_event: NOWAN last_evt_time: Tue Jan 29 19:57:04 2013 Result 7: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_10

| CaseID | WanRedundancy_Solution_UsbIsBackup_10 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usb0 and eth1 available, and both track wan and track default gateway for usb0 are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. usb0 is connected with Switch initially 2. Eth1 is connected with Switch initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth1 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Check wan state, as well as monitor state, result 4 5. Check default route in main route table, result 5 6. Run ping internet process on client to check its accessibility, result 6 7. Visit http on client, result 7 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode wan interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Active wan is usbnet0, and both monitor state and default monitor state of usbnet0 are Up AH-8c8040#sh in ¦ i wan Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 5: Default route is via usbnet, and there is non-default route via eth1 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 0 0 0 ppp0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 Result 6: ping internet successfully on client Result 7: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_11

| CaseID | WanRedundancy_Solution_UsbIsBackup_11 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth1 available, track wan for usbnet is down and track default gateway for usbnet is up, while track wan for eth1 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Usbnet is connected with Switch initially 2. Eth1 is connected with Switch initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth1 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on usbnet0, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode wan interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 5: wan failover to eth1, both monitor state and default monitor state of eth1 are Up AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 18:55:42 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 18:55:42 2013 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Default route is via eth1, and there is non-default route via usbnet with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_12

| CaseID | WanRedundancy_Solution_UsbIsBackup_12 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth1 available, track wan for usbnet is down and track default gateway for usbnet is up, while track wan for eth1 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Usbnet is connected with Switch initially 2. Eth1 is connected with Switch initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth1 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on usbnet, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Track wan fail on eth1, result 5 track-wan use-for-wan-eth1 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode wan interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usbnet0 unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 5: Track wan on eth1 unsuccessfully, and monitor state of eth1 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:05:59 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth1: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 19:04:59 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:04:59 2013 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Default route is via eth0, and there is non-default route via eth1 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_13

| CaseID | WanRedundancy_Solution_UsbIsBackup_13 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth1 available, both track wan and track default gateway for usbnet are down, while track wan for eth1 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Usbnet is connected with Switch initially 2. Eth1 is connected with Switch initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth1 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on usbnet, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Track default gateway fail on usbnet, result 5 no usbm modem-id Huawei_e1752 apn 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode wan interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 5: Track default gateway on usbnet unsuccessfully AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan failover to eth1, both monitor state and default monitor state of eth1 are Up AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 19:39:28 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:40:34 2013 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth1; State: up Interface: ppp0; State: unknown [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Default route is via eth1, and there is no default route via usbnet AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_14

| CaseID | WanRedundancy_Solution_UsbIsBackup_14 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth1 available, both track wan and track default gateway for usbnet are down, while track wan for eth1 is down and track default gateway for eth1 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. Eth1 is connected with Switch initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth1 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on usbnet, result 4 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 5. Track wan fail on eth1, result 5 track-wan use-for-wan-eth1 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode wan interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on usbnet are unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on eth1 unsuccessfully, and monitor state of eth1 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:48:33 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Tue Jan 29 19:48:16 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:49:22 2013 Result 7: default gw with highest priority is via eth1 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_15

| CaseID | WanRedundancy_Solution_UsbIsBackup_15 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth1 available, both track wan and track default gateway for usbnet are down, while both track wan and track default gateway for eth1 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. Eth1 is connected with Switch initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth1 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on usbnet, result 4 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 5. Both track wan and track default gateway fail on eth1, result 5 no in eth1 dhcp client 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode wan interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on usbnet are unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan and Track default gateway on eth1 are unsuccessfully, and monitor state of eth1 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:55:28 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth1: interface is not connected. 2013-01-29 19:55:24 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: down Interface: eth1; State: unknown [use-for-wan-eth1] WAN Monitor State: unknown Interface: eth1 (eth1) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Tue Jan 29 19:55:58 2013 last_event: NOWAN last_evt_time: Tue Jan 29 19:57:04 2013 Result 7: default gw with highest priority is via usbnet AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_16

| CaseID | WanRedundancy_Solution_UsbIsBackup_16 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth2 available, and both track wan and track default gateway for usbnet are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Usbnet is connected with Switch initially 2. Eth2 is connected with Switch initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth2 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Check wan state, as well as monitor state, result 4 5. Check default route in main route table, result 5 6. Run ping internet process on client to check its accessibility, result 6 7. Visit http on client, result 7 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth2 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Active wan is usbnet, and both monitor state and default monitor state of usbnet are Up AH-8c8040#sh in ¦ i wan Eth2 0019:778c:8043 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 5: Default route is via usbnet, and there is non-default route via eth2 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 0 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 20 0 0 eth2 Result 6: ping internet successfully on client Result 7: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_17

| CaseID | WanRedundancy_Solution_UsbIsBackup_17 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth2 available, track wan for usbnet is down and track default gateway for usbnet is up, while track wan for eth2 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. eth2 is connected with Switch initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth2 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on usbnet, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth2 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 5: wan failover to eth2, both monitor state and default monitor state of eth2 are Up AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 18:55:42 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 18:55:42 2013 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Default route is via eth2, and there is non-default route via usbnet with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.20.1 0.0.0.0 UG 20 0 0 eth2 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_18

| CaseID | WanRedundancy_Solution_UsbIsBackup_18 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth2 available, track wan for usbnet is down and track default gateway for usbnet is up, while track wan for eth2 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Usbnet is connected with Switch initially 2. eth2 is connected with Switch initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth2 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on eth0, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Track wan fail on eth2, result 5 track-wan use-for-wan-eth2 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth2 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 5: Track wan on eth2 unsuccessfully, and monitor state of eth2 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:05:59 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth2: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: down Interface: eth2 (eth2) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 19:04:59 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:04:59 2013 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: down Interface: eth2 (eth2) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Default route is via usbnet, and there is non-default route via eth2 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 0.0.0.0 192.168.20.1 0.0.0.0 UG 10020 0 0 eth2 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_19

| CaseID | WanRedundancy_Solution_UsbIsBackup_19 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth2 available, both track wan and track default gateway for usbnet are down, while track wan for eth2 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Usbnet is connected with Switch initially 2. eth2 is connected with Switch initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth2 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on usbnet, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Track default gateway fail on usbnet result 5 no usbm modem-id Huawei_e1752 apn 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth2 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 5: Track default gateway on usbnet unsuccessfully AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan failover to eth2, both monitor state and default monitor state of eth2 are Up AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 19:39:28 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:40:34 2013 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth2; State: up Interface: ppp0; State: unknown [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Default route is via eth2, and there is no default route via usbnet AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.20.1 0.0.0.0 UG 20 0 0 eth2 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_20

| CaseID | WanRedundancy_Solution_UsbIsBackup_20 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth2 available, both track wan and track default gateway for usbnet are down, while track wan for eth2 is down and track default gateway for eth2 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. eth2 is connected with Switch initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth2 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on usbnet, result 4 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 5. Track wan fail on eth2, result 5 track-wan use-for-wan-eth2 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth2 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on usbnet are unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on eth2 unsuccessfully, and monitor state of eth2 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:48:33 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth2: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: down Interface: eth2 (eth2) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Tue Jan 29 19:48:16 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:49:22 2013 Result 7: default gw with highest priority is via eth2 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.20.1 0.0.0.0 UG 10020 0 0 eth2 Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_21

| CaseID | WanRedundancy_Solution_UsbIsBackup_21 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth2 available, both track wan and track default gateway for usbnet are down, while both track wan and track default gateway for eth2 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1.Get a client associated to BR with open auth, result 1 2.Config eth2 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on usbnet, result 4 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 5. Both track wan and track default gateway fail on eth2, result 4 no in eth2 dhcp client 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth2 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on usbnet are unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan and Track default gateway on eth2 are unsuccessfully, and monitor state of eth2 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:55:28 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth2: interface is not connected. 2013-01-29 19:55:24 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: down Interface: eth2; State: unknown [use-for-wan-eth2] WAN Monitor State: unknown Interface: eth2 (eth2) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Tue Jan 29 19:55:58 2013 last_event: NOWAN last_evt_time: Tue Jan 29 19:57:04 2013 Result 7: default gw with highest priority is via usbnet AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_22

| CaseID | WanRedundancy_Solution_UsbIsBackup_22 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth3 available, and both track wan and track default gateway for usbnet are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth3 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Check wan state, as well as monitor state, result 4 5. Check default route in main route table, result 5 6. Run ping internet process on client to check its accessibility, result 6 7. Visit http on client, result 7 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth3 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Active wan is usbnet, and both monitor state and default monitor state of usbnet are Up AH-8c8040#sh in ¦ i wan Eth3 0019:778c:8044 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 5: Default route is via usbnet, and there is non-default route via eth3 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 0 0 0 ppp0 0.0.0.0 192.168.30.1 0.0.0.0 UG 30 0 0 eth3 Result 6: ping internet successfully on client Result 7: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_23

| CaseID | WanRedundancy_Solution_UsbIsBackup_23 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0 and eth3 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for eth3 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth3 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on usbnet, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth3 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 5: wan failover to eth3, both monitor state and default monitor state of eth3 are Up AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 18:55:42 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 18:55:42 2013 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Default route is via eth3, and there is non-default route via usbnet with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.30.1 0.0.0.0 UG 30 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 Result 7: ping internet successfully on client Result 8: Http visit on client successful |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_24

| CaseID | WanRedundancy_Solution_UsbIsBackup_24 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth3 available, track wan for usbnet is down and track default gateway for usbnet is up, while track wan for eth3 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth3 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on usbnet, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Track wan fail on eth3, result 5 track-wan use-for-wan-eth3 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth3 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 5: Track wan on eth3 unsuccessfully, and monitor state of eth3 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:05:59 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth3: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: down Interface: eth3 (eth3) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 19:04:59 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:04:59 2013 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: down Interface: eth3 (eth3) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Default route is via eth0, and there is non-default route via eth3 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 0.0.0.0 192.168.30.1 0.0.0.0 UG 10030 0 0 eth3 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_25

| CaseID | WanRedundancy_Solution_UsbIsBackup_25 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth3 available, both track wan and track default gateway for usbnet are down, while track wan for eth3 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth3 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on usbnet, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Track default gateway fail on usbnet, result 5 no usbm modem-id Huawei_e1752 apn 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth3 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 5: Track default gateway on usbnet unsuccessfully AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan failover to eth3, both monitor state and default monitor state of eth3 are Up AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 19:39:28 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:40:34 2013 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth3; State: up Interface: ppp0; State: unknown [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Default route is via eth3, and there is no default route via usbnet AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.30.1 0.0.0.0 UG 30 0 0 eth3 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_26

| CaseID | WanRedundancy_Solution_UsbIsBackup_26 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth3 available, both track wan and track default gateway for usbnet are down, while track wan for eth3 is down and track default gateway for eth3 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth3 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on usbnet, result 4 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 5. Track wan fail on eth3, result 5 track-wan use-for-wan-eth3 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth3 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on usbnet are unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on eth3 unsuccessfully, and monitor state of eth3 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:48:33 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth3: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: down Interface: eth3 (eth3) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Tue Jan 29 19:48:16 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:49:22 2013 Result 7: default gw with highest priority is via eth3 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.30.1 0.0.0.0 UG 10030 0 0 eth3 Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_27

| CaseID | WanRedundancy_Solution_UsbIsBackup_27 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth3 available, both track wan and track default gateway for usbnet are down, while both track wan and track default gateway for eth3 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth3 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on usbnet, result 4 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 5. Both track wan and track default gateway fail on eth3, result 5 no in eth3 dhcp client 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth3 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on usbnet are unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan and Track default gateway on eth3 are unsuccessfully, and monitor state of eth3 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:55:28 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth3: interface is not connected. 2013-01-29 19:55:24 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: down Interface: eth3; State: unknown [use-for-wan-eth3] WAN Monitor State: unknown Interface: eth3 (eth3) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Tue Jan 29 19:55:58 2013 last_event: NOWAN last_evt_time: Tue Jan 29 19:57:04 2013 Result 7: default gw with highest priority is via usbnet AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_28

| CaseID | WanRedundancy_Solution_UsbIsBackup_28 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth4 available, and both track wan and track default gateway for usbnet are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth4 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Check wan state, as well as monitor state, result 4 5. Check default route in main route table, result 5 6. Run ping internet process on client to check its accessibility, result 6 7. Visit http on client, result 7 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth4 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth4 0019:778c:8045 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Active wan is usbnet, and both monitor state and default monitor state of usbnet are Up AH-8c8040#sh in ¦ i wan Eth4 0019:778c:8045 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 5: Default route is via usbnet, and there is non-default route via eth4 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 0 0 0 ppp0 0.0.0.0 192.168.30.1 0.0.0.0 UG 40 0 0 eth4 Result 6: ping internet successfully on client Result 7: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_29

| CaseID | WanRedundancy_Solution_UsbIsBackup_29 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth4 available, track wan for usbnet is down and track default gateway for usbnet is up, while track wan for eth4 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth4 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on usbnet, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth4 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth4 0019:778c:8045 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 5: wan failover to eth4, both monitor state and default monitor state of eth4 are Up AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 18:55:42 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 18:55:42 2013 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Default route is via eth4, and there is non-default route via usbnet with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.40.1 0.0.0.0 UG 40 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_30

| CaseID | WanRedundancy_Solution_UsbIsBackup_30 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth4 available, track wan for usbnet is down and track default gateway for usbnet is up, while track wan for eth4 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth4 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on usbnet, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Track wan fail on eth4, result 5 track-wan use-for-wan-eth4 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth4 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth4 0019:778c:8045 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 5: Track wan on eth4 unsuccessfully, and monitor state of eth4 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:05:59 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth4: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 19:04:59 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:04:59 2013 Result 7: Default route is via eth0, and there is non-default route via eth4 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 0.0.0.0 192.168.40.1 0.0.0.0 UG 10040 0 0 eth4 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_31

| CaseID | WanRedundancy_Solution_UsbIsBackup_31 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth4 available, both track wan and track default gateway for usbnet are down, while track wan for eth4 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth4 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on usbnet, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Track default gateway fail on usbnet, result 5 no usbm modem-id Huawei_e1752 apn 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth4 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth4 0019:778c:8045 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track default gateway on usbnet unsuccessfully AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: wan failover to eth4, both monitor state and default monitor state of eth4 are Up AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 19:39:28 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:40:34 2013 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth4; State: up Interface: ppp0; State: unknown [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Default route is via eth4, and there is no default route via usbnet AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.40.1 0.0.0.0 UG 40 0 0 eth4 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_32

| CaseID | WanRedundancy_Solution_UsbIsBackup_32 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth4 available, both track wan and track default gateway for usbnet are down, while track wan for eth4 is down and track default gateway for eth4 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth4 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on usnbet, result 4 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 5. Track wan fail on eth4, result 5 track-wan use-for-wan-eth4 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth4 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth4 0019:778c:8045 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on usbnet are unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on eth4 unsuccessfully, and monitor state of eth4 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:48:33 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth4: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Tue Jan 29 19:48:16 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:49:22 2013 Result 7: default gw with highest priority is via eth4 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.40.1 0.0.0.0 UG 10040 0 0 eth4 Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_33

| CaseID | WanRedundancy_Solution_UsbIsBackup_33 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is usbnet and eth4 available, both track wan and track default gateway for usbnet are down, while both track wan and track default gateway for eth4 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth4 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on usbnet, result 4 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 5. Both track wan and track default gateway fail on eth4, result 5 no in eth4 dhcp client 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth4 is wan mode AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Eth4 0019:778c:8045 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on usbnet are unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan and Track default gateway on eth4 are unsuccessfully, and monitor state of eth4 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:55:28 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth4: interface is not connected. 2013-01-29 19:55:24 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: down Interface: eth4; State: unknown [use-for-wan-eth4] WAN Monitor State: unknown Interface: eth4 (eth4) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Tue Jan 29 19:48:16 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:49:22 2013 Result 7: default gw with highest priority is via usbnet AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_34

| CaseID | WanRedundancy_Solution_UsbIsBackup_34 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 4 |  |  |
| Description | WAN behavior when there is usbnet and wifi0 available, and both track wan and track default gateway for usbnet are up |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config wifi0 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Check wan state, as well as monitor state, result 4 5. Check default route in main route table, result 5 6. Run ping internet process on client to check its accessibility, result 6 7. Visit http on client, result 7 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface eth0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: wifi0 is wan mode and connected with ISP AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Wifi0 0019:778c:8054 wan U 5 - radio_ng0 - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in wifi0 ¦ i mode Mode=wan; Radio disabled=no; Phymode=11ng; Short preamble mode=enabled; Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface wifi0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Active wan is usbnet, and both monitor state and default monitor state of usbnet are Up AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: usbnet0; State: up Interface: wifi0.2; State: up [use-for-wan-wifi0] WAN Monitor State: up Interface: wifi0 (wifi0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (usbnet0) -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Jan 30 00:38:44 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Jan 30 00:38:44 2013 Result 5: Default route is via usbnet, and there is non-default route via wifi0 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 192.168.50.0 0.0.0.0 255.255.255.0 U 0 0 0 wifi0.2 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 1 0 0 ppp0 0.0.0.0 192.168.50.1 0.0.0.0 UG 500 0 0 wifi0.2 Result 6: ping internet successfully on client Result 7: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_35

| CaseID | WanRedundancy_Solution_UsbIsBackup_35 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 4 |  |  |
| Description | WAN behavior when there is usbnet and wifi0 available, track wan for usbnet is down and track default gateway for usbnet is up, while track wan for wifi0 is up |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config wifi0 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on usbnet, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface eth0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: wifi0 is wan mode and connected with ISP AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Wifi0 0019:778c:8054 wan U 5 - radio_ng0 - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in wifi0 ¦ i mode Mode=wan; Radio disabled=no; Phymode=11ng; Short preamble mode=enabled; Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface wifi0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 5: wan failover to wifi0, both monitor state and default monitor state of wifi0 are Up AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 18:55:42 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 18:55:42 2013 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: wifi0; State: up Interface: ppp0; State: up [use-for-wan-wifi0] WAN Monitor State: up Interface: wifi0 (wifi0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Default route is via wifi0, and there is non-default route via usbnet with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.50.1 0.0.0.0 UG 500 0 0 wifi0.2 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_36

| CaseID | WanRedundancy_Solution_UsbIsBackup_36 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 4 |  |  |
| Description | WAN behavior when there is usbnet and wifi0 available, track wan for usbnet is down and track default gateway for usbnet is up, while track wan for wifi0 is down |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config wifi0 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on usbnet, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Track wan fail on wifi0, result 5 track-wan use-for-wan-wifi0 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface eth0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: wifi0 is wan mode and connected with ISP AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Wifi0 0019:778c:8054 wan U 5 - radio_ng0 - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in wifi0 ¦ i mode Mode=wan; Radio disabled=no; Phymode=11ng; Short preamble mode=enabled; Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface wifi0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface:wifi0; State: up Interface: ppp0; State: up [use-for-wan-wifi0] WAN Monitor State: up Interface: wifi0 (wifi0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 5: Track wan on wifi0 unsuccessfully, and monitor state of wifi0 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:05:59 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface wifi0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: wifi0; State: up Interface: ppp0; State: up [use-for-wan-wifi0] WAN Monitor State: down Interface: wifi0 (wifi0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 19:04:59 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:04:59 2013 Result 7: Default route is via usbnet, and there is non-default route via wifi0 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 0.0.0.0 192.168.50.1 0.0.0.0 UG 10500 0 0 wifi0.2 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_37

| CaseID | WanRedundancy_Solution_UsbIsBackup_37 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 4 |  |  |
| Description | WAN behavior when there is usbnet and wifi0 available, both track wan and track default gateway for usbnet are down, while track wan for wifi0 is up |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config wifi0 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Track wan fail on usbnet, result 4 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 5. Track default gateway fail on usbnet, result 5 no usbm modem-id Huawei_e1752 apn 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface eth0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: wifi0 is wan mode and connected with ISP AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Wifi0 0019:778c:8054 wan U 5 - radio_ng0 - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in wifi0 ¦ i mode Mode=wan; Radio disabled=no; Phymode=11ng; Short preamble mode=enabled; Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface wifi0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 18:57:31 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface:wifi0; State: up Interface: ppp0; State: up [use-for-wan-wifi0] WAN Monitor State: up Interface: wifi0 (wifi0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 5: Track default gateway on usbnet unsuccessfully AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: wifi0; State: up [use-for-wan-wifi0] WAN Monitor State: up Interface: wifi0 (wifi0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan failover to wifi0, both monitor state and default monitor state of wifi0 are Up AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 19:39:28 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:40:34 2013 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: wifi0; State: up Interface: ppp0; State: unknown [use-for-wan-wifi0] WAN Monitor State: up Interface: wifi0 (wifi0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Default route is via wifi0, and there is no default route via usbnet AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.50.1 0.0.0.0 UG 500 0 0 wifi0.2 Result 8: ping internet successfully on client Result 9: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_38

| CaseID | WanRedundancy_Solution_UsbIsBackup_38 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 4 |  |  |
| Description | WAN behavior when there is usbnet and wifi0 available, both track wan and track default gateway for usbnet are down, while track wan for wifi0 is down and track default gateway for wifi0 is up |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config wifi0 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on usbnet, result 4 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 5. track wan fail on wifi0, result 5 track-wan use-for-wan-wifi0 ip 1.1.1.1 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface eth0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: wifi0 is wan mode and connected with ISP AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Wifi0 0019:778c:8054 wan U 5 - radio_ng0 - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in wifi0 ¦ i mode Mode=wan; Radio disabled=no; Phymode=11ng; Short preamble mode=enabled; Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface wifi0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on usbnet are unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: wifi0; State: up [use-for-wan-wifi0] WAN Monitor State: up Interface: wifi0 (wifi0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on wifi0 are unsuccessfully, and monitor state of wifi0 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:48:33 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface wifi0: 1 targets tested, result: FAIL … - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: wifi0; State: up [use-for-wan-wifi0] WAN Monitor State: down Interface: wifi0 (wifi0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Tue Jan 29 19:48:16 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:49:22 2013 Result 7: default gw with highest priority is via usbnet AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 192.168.50.1 0.0.0.0 UG 10500 0 0 wifi0.2 Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_39

| CaseID | WanRedundancy_Solution_UsbIsBackup_39 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is usbnet and wifi0 available, both track wan and track default gateway for usbnet are down, while both track wan and track default gateway for wifi0 are down |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. usbnet is connected with Switch initially 2. wifi0 has been configured as wan mode and connected with ISP ssid |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config wifi0 as wan mode, result 2 3. Config track wan group each interface, result 3 4. Both track wan and track default gateway fail on usbnet, result 4 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 5. Both track wan and track default gateway fail on wifi0, result 4 no in w0 ssid ssgao-br100 6. Check wan state, as well as monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Visit http on client, result 9 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth0 shutdown interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth1 shutdown interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth2 shutdown interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth3 shutdown interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface eth0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: wifi0 is wan mode and connected with ISP AH-8c8040#sh in ¦ i wan Ppp0 0000:0000:0000 wan U - - - - - Wifi0 0019:778c:8054 wan U 5 - radio_ng0 - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usbnet0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in wifi0 ¦ i mode Mode=wan; Radio disabled=no; Phymode=11ng; Short preamble mode=enabled; Result 3: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-wifi0 track-wan use-for-wan-wifi0 ip 8.8.8.8 track-wan use-for-wan-wifi0 interval 1 track-wan use-for-wan-wifi0 interface wifi0 track-wan use-for-wan-wifi0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: Track wan and Track default gateway on usbnet are unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 19:40:00 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: wifi0; State: up [use-for-wan-wifi0] WAN Monitor State: up Interface: wifi0 (wifi0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan and Track default gateway on wifi0 are unsuccessfully, and monitor state of wifi0 is Down AH-8c8040#sh lo bu …. 2013-01-29 19:55:28 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-wifi0: interface is not connected. 2013-01-29 19:55:24 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: down Interface: wifi0; State: unknown [use-for-wan-wifi0] WAN Monitor State: unknown Interface: eth4 (wifi0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Tue Jan 29 19:48:16 2013 last_event: PRIMARY_WAN_DOWN last_evt_time: Tue Jan 29 19:49:22 2013 Result 7: default gw with highest priority is via usbnet AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 8: ping internet will be reponsed with mgt0 Result 9: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_40

| CaseID | WanRedundancy_Solution_UsbIsBackup_40 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, and both track wan and track default gateway for eth0 are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Active wan is usb0, and both monitor state and default monitor state of usb0 are Up AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Tue Jan 29 22:06:16 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:06:16 2013 -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Default route is via usb0, and there are non-default route via eth0 and eth1 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 0 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_41

| CaseID | WanRedundancy_Solution_UsbIsBackup_41 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usb0, result 5 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 6. Track default gateway successfully on usb0, result 6 7. Track wan successfully on eth0, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usb0 unsuccessfully, and monitor state of usb0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-29 22:27:23 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Track default gateway on usb0 successfully, and default monitor state of usb0 is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa404040 for ppp0 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth1: 1 targets tested, result: PASS ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 successfully, and monitor state of eth0 is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:45:19 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: PASS …… -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: wan failover to eth0 AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: Default route is via eth0, and there are non-default route via usb0 and eth1 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_42

| CaseID | WanRedundancy_Solution_UsbIsBackup_42 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is down and track wan for eth1 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usb0, result 5 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 6. Track default gateway successfully on usb0, result 6 7. Track wan fail on eth0, result 7 track-wan use-for-wan-eth0 ip 1.1.1.1 8. Track wan successfully on eth1, result 8 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usb0 unsuccessfully, and monitor state of usb0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-29 22:27:23 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Track default gateway on eth0 successfully, and default monitor state of eth0 is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa404040 for ppp0 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth1: 1 targets tested, result: PASS ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:51:32 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: Track wan on eth1 successfully, and monitor state of eth1 is Up AH-8c8040#sh lo bu …. 2013-01-29 22:53:12 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth1: 1 targets tested, result: PASS …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 9: wan failover to eth0 AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 10: Default route is via eth1, and there are non-default route via eth0 and usbnet with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_43

| CaseID | WanRedundancy_Solution_UsbIsBackup_43 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, track wan for eth0 is down and track default gateway for eth0 is up, while track wan for usbnet is down and track wan for eth1 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usb0, result 5 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 6. Track default gateway successfully on usb0, result 6 7. Track wan fail on eth0, result 7 track-wan use-for-wan-eth0 ip 1.1.1.1 8. Track wan fail on eth1, result 8 track-wan use-for-wan-eth1 ip 1.1.1.1 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usb0 unsuccessfully, and monitor state of usb0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-29 22:27:23 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Track default gateway on usb0 successfully, and default monitor state of usb0 is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa404040 for ppp0 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth1: 1 targets tested, result: PASS ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:51:32 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: Track wan on eth1 unsuccessfully, and monitor state of eth1 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:57:35 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth1: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 9: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 10: Default route is via usbnet, and there are non-default route via eth0 and eth1 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_44

| CaseID | WanRedundancy_Solution_UsbIsBackup_44 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usb0, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway fail on usb0, result 6 no usbm modem-id Huawei_e1752 apn 7. Track wan successfully on eth0 result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usb0 unsuccessfully, and monitor state of usb0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-29 22:27:23 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Track default gateway on usb0 unsuccessfully, and default monitor state of usb0 is unknow AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 successfully, and monitor state of eth0 is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:45:19 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: PASS …… -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan failover to eth0 AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: Default route is via eth0, and there is no default route via usb0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_45

| CaseID | WanRedundancy_Solution_UsbIsBackup_45 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is down and track wan for eth1 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usb0 and track default gateway fail on usb0, result 5 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 6. Track wan fail on eth0, result 6 track-wan use-for-wan-eth0 ip 1.1.1.1 7. Track wan successfully on eth1, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is unknown AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:51:32 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result7: Track wan on eth1 successfully, and monitor state of eth1 is Up AH-8c8040#sh lo bu …. 2013-01-29 22:53:12 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth1: 1 targets tested, result: PASS …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan failover to eth1 AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: Default route is via eth1, and and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_46

| CaseID | WanRedundancy_Solution_UsbIsBackup_46 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is down and track default gateway for usbnet is up, and track wan for eth1 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usb0,and track default gateway fail on usb0, result 5 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 7. Track wan fail on eth0, result 7 track-wan use-for-wan-eth0 ip 1.1.1.1 8. Track wan fail on eth1, result 8 track-wan use-for-wan-eth1 ip 1.1.1.1 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usb0 unsuccessfully, and track default gateway on usb0 unsuccessfully, and default monitor state of usb0 is Down AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:51:32 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: Track wan on eth1 unsuccessfully, and monitor state of eth1 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:57:35 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth1: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 9: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 10: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 Result 11: ping internet will be reponsed with mgt0 Result 12: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_47

| CaseID | WanRedundancy_Solution_UsbIsBackup_47 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for usbnet are down, and track wan for eth1 is down and track default gateway for eth1 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usb0, and track default gateway fail on usb0, result 5 _deb brd wanmon no usbm modem-id huawei_e1752 apn 6. Track wan fail on eth0, and track default gateway fail on eth0, result 6 no in eth0 dhcp client 7. Track wan fail on eth1, result 7 track-wan use-for-wan-eth1 ip 1.1.1.1 8. Check wan state, as well as monitor state, result8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usb0 unsuccessfully, and track default gateway on usb0 unsuccessfully, and default monitor state of usb0 is unknown AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is unknown AH-8c8040#sh lo bu …. 2013-01-29 23:25:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-29 23:26:09 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth1 unsuccessfully, and monitor state of eth1 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:57:35 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth1: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_48

| CaseID | WanRedundancy_Solution_UsbIsBackup_48 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth1 available, both track wan and track default gateway for eth0 are down, while both track wan and track default gateway for usbnet are down, and both track wan and track default gateway for eth1 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth1 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usb0, and track default gateway fail on usb0, result 5 _deb brd wanmon no usbm modem-id huawei_e1752 apn 6. Track wan fail on eth0, and track default gateway fail on eth0, result 6 no in eth0 dhcp client 7. Track wan fail on eth1, and track default gateway fail on eth1, result 7 no in eth1 dhcp client 8. Check wan state, as well as monitor state, result8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode wan interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth1 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth1 0019:778c:8042 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 1 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usb0 unsuccessfully, and track default gateway on usb0 unsuccessfully, and default monitor state of usb0 is unknown AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is unknown AH-8c8040#sh lo bu …. 2013-01-29 23:25:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-29 23:26:09 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 5: Track wan on eth1 unsuccessfully, and track default gateway on eth1 unsuccessfully, and default monitor state of eth1 is unknown AH-8c8040#sh lo bu …. 2013-01-29 23:25:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth1: interface is not connected. 2013-01-29 23:26:09 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: unknown [use-for-wan-eth1] WAN Monitor State: unknown Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 9: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Tue Jan 29 23:33:42 2013 last_event: BACKUP_WAN_DOWN last_evt_time: Tue Jan 29 23:33:42 2013 Result 10: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 11: ping internet will be reponsed with mgt0 Result 12: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_49

| CaseID | WanRedundancy_Solution_UsbIsBackup_49 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, and both track wan and track default gateway for usbnet are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth2 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Active wan is usbnet, and both monitor state and default monitor state of usbnet are Up AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Tue Jan 29 22:06:16 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:06:16 2013 -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Default route is via usb0, and there are non-default route via usbnet and eth2 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 0.0.0.0 10.64.64.64 0.0.0.0 UG 0 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.20.1 0.0.0.0 UG 20 0 0 eth2 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_50

| CaseID | WanRedundancy_Solution_UsbIsBackup_50 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, track wan for usbnet is down and track default gateway for eth0 is up, while track wan for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth2 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet, result 5 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 6. Track default gateway successfully on usbnet, result 6 7. Track wan successfully on usbnet, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-29 22:27:23 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Track default gateway on usbnet successfully, and default monitor state of usbnet is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa404040 for ppp0 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth2: 1 targets tested, result: PASS ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 successfully, and monitor state of eth0 is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:45:19 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: PASS …… -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: wan failover to usbnet AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: Default route is via usbnet, and there are non-default route via eth0 and eth2 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.20.1 0.0.0.0 UG 20 0 0 eth2 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_51

| CaseID | WanRedundancy_Solution_UsbIsBackup_51 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, track wan for usbnet is down and track default gateway for usbnet is up, while track wan for eth0 is down and track wan for eth2 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth2 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet, result 5 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 6. Track default gateway successfully on usbnet, result 6 7. Track wan fail on eth0, result 7 track-wan use-for-wan-eth0 ip 1.1.1.1 8. Track wan successfully on eth2, result 8 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-29 22:27:23 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Track default gateway on usbnet successfully, and default monitor state of usbnet is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa404040 for ppp0 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth2: 1 targets tested, result: PASS ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:51:32 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: Track wan on eth2 successfully, and monitor state of eth2 is Up AH-8c8040#sh lo bu …. 2013-01-29 22:53:12 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth2: 1 targets tested, result: PASS …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 9: wan failover to eth2 AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 10: Default route is via eth2, and there are non-default route via eth0 and usbnet with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 0.0.0.0 192.168.20.1 0.0.0.0 UG 20 0 0 eth2 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_52

| CaseID | WanRedundancy_Solution_UsbIsBackup_52 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, track wan for usbnet is down and track default gateway for eth0 is up, while track wan for eth0 is down and track wan for eth2 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth2 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet, result 5 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 6. Track default gateway successfully on usbnet, result 6 7. Track wan fail on usbnet, result 7 track-wan use-for-wan-eth0 ip 1.1.1.1 8. Track wan fail on eth2, result 8 track-wan use-for-wan-eth2 ip 1.1.1.1 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-29 22:27:23 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Track default gateway on usbnet successfully, and default monitor state of usbnet is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa404040 for ppp0 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth2: 1 targets tested, result: PASS ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:51:32 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: Track wan on eth2 unsuccessfully, and monitor state of eth2 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:57:35 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth2: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: down Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 9: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 10: Default route is via eth0, and there are non-default route via usbnet and eth2 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.20.1 0.0.0.0 UG 10020 0 0 eth2 Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_53

| CaseID | WanRedundancy_Solution_UsbIsBackup_53 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, both track wan and track default gateway for usbnet are down, while track wan for eth0 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth2 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet, result 5 _deb brd wanmon track-wan use-for-wan-eth0 ip 1.1.1.1 6. Track default gateway fail on usbnet, result 6 no usbm modem-id Huawei_e1752 apn 7. Track wan successfully on usbnet, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-29 22:27:23 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up Interface: ppp0; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 successfully, and monitor state of eth0 is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:45:19 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: PASS …… -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan failover to usbnet AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: Default route is via usbnet, and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.20.1 0.0.0.0 UG 20 0 0 eth2 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_54

| CaseID | WanRedundancy_Solution_UsbIsBackup_54 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is down and track wan for eth2 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth2 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail and track default gateway fail on usbnet, result 5 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 6. Track wan fail on eth0, result 6 track-wan use-for-wan-eth0 ip 1.1.1.1 7. Track wan successfully on eth2, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:51:32 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth2 successfully, and monitor state of eth2 is Up AH-8c8040#sh lo bu …. 2013-01-29 22:53:12 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth2: 1 targets tested, result: PASS …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan failover to eth2 AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: Default route is via eth2, and and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 0.0.0.0 192.168.20.1 0.0.0.0 UG 20 0 0 eth2 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_55

| CaseID | WanRedundancy_Solution_UsbIsBackup_55 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, both track wan and track default gateway for usbnet are down, while track wan for eth0 is down and track default gateway for eth0 is up, and track wan for eth2 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth2 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet and track default gateway fail on usbnet, result 5 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 6. Track wan fail on eth0, result 6 track-wan use-for-wan-eth0 ip 1.1.1.1 7. Track wan fail on eth2, result 7 track-wan use-for-wan-eth2 ip 1.1.1.1 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:51:32 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth2 unsuccessfully, and monitor state of eth2 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:57:35 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth2: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: down Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.20.1 0.0.0.0 UG 10020 0 0 eth2 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_56

| CaseID | WanRedundancy_Solution_UsbIsBackup_56 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, both track wan and track default gateway for usbnet are down, while both track wan and track default gateway for eth0 are down, and track wan for eth2 is down and track default gateway for eth2 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1.Get a client associated to BR with open auth, result 1 2.Config usb modem as always-connect mode, result 2 3.Config eth2 as wan mode, result 3 4.Config track wan group each interface, result 4 5. Track wan fail on usbnet and track default gateway fail on usbnet, result 5 _deb brd wanmon no usbm modem-id huawei_e1752 apn 6. Track wan fail on eth0 and track default gateway fail on eth0, result 6 no in eth0 dhcp client 7. Track wan fail on eth2, result 7 track-wan use-for-wan-eth1 ip 1.1.1.1 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:25:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-29 23:26:09 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth2 unsuccessfully, and monitor state of eth2 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:57:35 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth2: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: down Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.20.0 0.0.0.0 255.255.255.0 U 0 0 0 eth2 0.0.0.0 192.168.20.1 0.0.0.0 UG 10020 0 0 eth2 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_57

| CaseID | WanRedundancy_Solution_UsbIsBackup_57 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth2 available, both track wan and track default gateway for usbnet are down, while both track wan and track default gateway for eth0 are down, and both track wan and track default gateway for eth2 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth2 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet and track default gateway fail on usbnet, result 5 _deb brd wanmon no usbm modem-id huawei_e1752 apn 6. Track wan fail on eth0 and track default gateway fail on eth0, result 6 no in eth0 dhcp client 7. Track wan fail on eth2 and track default gateway fail on eth2, result 7 no in eth2 dhcp client 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode wan interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth2 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth2 0019:778c:8043 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth2 track-wan use-for-wan-eth2 ip 8.8.8.8 track-wan use-for-wan-eth2 interval 1 track-wan use-for-wan-eth2 interface eth1 track-wan use-for-wan-eth2 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu …. 2013-01-29 23:25:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-29 23:26:09 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: up [use-for-wan-eth2] WAN Monitor State: up Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth2 unsuccessfully, and track default gateway on eth2 unsuccessfully, and default monitor state of eth2 is Down AH-8c8040#sh lo bu …. 2013-01-29 23:25:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth2: interface is not connected. 2013-01-29 23:26:09 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth2; State: unknown [use-for-wan-eth2] WAN Monitor State: unknown Interface: eth2 (eth2) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_58

| CaseID | WanRedundancy_Solution_UsbIsBackup_58 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, and both track wan and track default gateway for usbnet are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth3 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Active wan is usbnet, and both monitor state and default monitor state of usbnet are Up AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Tue Jan 29 22:06:16 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:06:16 2013 -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Default route is via usb0, and there are non-default route via usbnet and eth3 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 0 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.30.1 0.0.0.0 UG 30 0 0 eth3 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_59

| CaseID | WanRedundancy_Solution_UsbIsBackup_59 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, track wan for usbnet is down and track default gateway for eth0 is up, while track wan for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth3 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet, result 5 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 6. Track default gateway successfully on usbnet, result 6 7. Track wan successfully on usbnet, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-29 22:27:23 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Track default gateway on usbnet successfully, and default monitor state of usbnet is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa404040 for ppp0 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth3: 1 targets tested, result: PASS ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 successfully, and monitor state of eth0 is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:45:19 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: PASS …… -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: wan failover to usbnet AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: Default route is via usbnet, and there are non-default route via eth0 and eth3 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.30.1 0.0.0.0 UG 30 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_60

| CaseID | WanRedundancy_Solution_UsbIsBackup_60 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, track wan for usbnet is down and track default gateway for usbnet is up, while track wan for eth0 is down and track wan for eth3 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth3 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet, result 5 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 6. Track default gateway successfully on usbnet, result 6 7. Track wan fail on eth0, result 7 track-wan use-for-wan-eth0 ip 1.1.1.1 8. Track wan successfully on eth3, result 8 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and monitor state of wifi0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-29 22:27:23 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Track default gateway on usbnet successfully, and default monitor state of usbnet is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa404040 for ppp0 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth3: 1 targets tested, result: PASS ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:51:32 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: Track wan on eth3 successfully, and monitor state of eth3 is Up AH-8c8040#sh lo bu …. 2013-01-29 22:53:12 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth3: 1 targets tested, result: PASS …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 9: wan failover to eth3 AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 10: Default route is via eth3, and there are non-default route via eth0 and usbnet with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 192.168.30.1 0.0.0.0 UG 30 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_61

| CaseID | WanRedundancy_Solution_UsbIsBackup_61 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, track wan for usbnet is down and track default gateway for eth0 is up, while track wan for eth0 is down and track wan for eth3 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth2 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth3 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet, result 5 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 6. Track default gateway successfully on eth0, result 6 7. Track wan fail on eth0, result 7 track-wan use-for-wan-eth0 ip 1.1.1.1 8. Track wan fail on eth3, result 8 track-wan use-for-wan-eth3 ip 1.1.1.1 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-29 22:27:23 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Track default gateway on usbnet successfully, and default monitor state of usbnet is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa404040 for ppp0 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth3: 1 targets tested, result: PASS ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:51:32 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: Track wan on eth3 unsuccessfully, and monitor state of eth3 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:57:35 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth3: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: down Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 9: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 10: Default route is via eth0, and there are non-default route via usbnet and eth3 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.30.1 0.0.0.0 UG 10030 0 0 eth3 Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_62

| CaseID | WanRedundancy_Solution_UsbIsBackup_62 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, both track wan and track default gateway for usbnet are down, while track wan for eth0 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth3 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet, result 5 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 6. Track default gateway fail on usbnet, result 6 no usbm modem-id Huawei_e1752 apn 7. Track wan successfully on usbnet, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-29 22:27:23 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 successfully, and monitor state of eth0 is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:45:19 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: PASS …… -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan failover to usbnet AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: Default route is via eth0, and there is no default route via usbnet AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.30.1 0.0.0.0 UG 30 0 0 eth3 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_63

| CaseID | WanRedundancy_Solution_UsbIsBackup_63 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is down and track wan for eth3 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth3 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail and track default gateway fail on usbnet, result 5 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 6. Track wan fail on eth0, result 6 track-wan use-for-wan-eth0 ip 1.1.1.1 7. Track wan successfully on eth3, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:51:32 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth3 successfully, and monitor state of eth3 is Up AH-8c8040#sh lo bu …. 2013-01-29 22:53:12 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth3: 1 targets tested, result: PASS …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan failover to eth3 AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: Default route is via eth3, and and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 192.168.30.1 0.0.0.0 UG 30 0 0 eth3 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_64

| CaseID | WanRedundancy_Solution_UsbIsBackup_64 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, both track wan and track default gateway for usbnet are down, while track wan for eth0 is down and track default gateway for eth0 is up, and track wan for eth3 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth3 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet and track default gateway fail on usbnet, result 5 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 6. Track wan fail on eth0, result 6 track-wan use-for-wan-eth0 ip 1.1.1.1 7. Track wan fail on eth3, result 7 track-wan use-for-wan-eth3 ip 1.1.1.1 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:51:32 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth3 unsuccessfully, and monitor state of eth3 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:57:35 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth3: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: down Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.30.1 0.0.0.0 UG 10030 0 0 eth3 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_65

| CaseID | WanRedundancy_Solution_UsbIsBackup_65 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, both track wan and track default gateway for usbnet are down, while both track wan and track default gateway for eth0 are down, and track wan for eth3 is down and track default gateway for eth3 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1.Get a client associated to BR with open auth, result 1 2.Config usb modem as always-connect mode, result 2 3.Config eth3 as wan mode, result 3 4.Config track wan group each interface, result 4 5. Track wan fail on usbnet and track default gateway fail on usbnet, result 5 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 6. Track wan fail on eth0 and track default gateway fail on eth0, result 6 no in eth0 dhcp client 7. Track wan fail on eth3, result 7 track-wan use-for-wan-eth3 ip 1.1.1.1 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:25:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-29 23:26:09 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth3 unsuccessfully, and monitor state of eth3 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:57:35 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth3: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: down Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: default gw with highest priority is via eth3 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 192.168.30.1 0.0.0.0 UG 10030 0 0 eth3 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_66

| CaseID | WanRedundancy_Solution_UsbIsBackup_66 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth3 available, both track wan and track default gateway for usbnet are down, while both track wan and track default gateway for eth0 are down, and both track wan and track default gateway for eth3 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth3 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet and track default gateway fail on usbnet, result 5 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 6. Track wan fail on eth0 and track default gateway fail on eth0, result 6 no in eth0 dhcp client 7. Track wan fail on eth3 and track default gateway fail on eth3, result 7 no in eth3 dhcp client 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode wan interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth3 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth3 0019:778c:8044 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 1 track-wan use-for-wan-eth3 interface eth1 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu …. 2013-01-29 23:25:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-29 23:26:09 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth3 unsuccessfully, and track default gateway on eth3 unsuccessfully, and default monitor state of eth3 is Down AH-8c8040#sh lo bu …. 2013-01-29 23:25:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth3: interface is not connected. 2013-01-29 23:26:09 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: unknown [use-for-wan-eth3] WAN Monitor State: unknown Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_67

| CaseID | WanRedundancy_Solution_UsbIsBackup_67 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, and both track wan and track default gateway for usbnet are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth4 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Active wan is usbnet, and both monitor state and default monitor state of usbnet are Up AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Tue Jan 29 22:06:16 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:06:16 2013 -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 6: Default route is via usb0, and there are non-default route via usbnet and eth4 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 0 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.40.1 0.0.0.0 UG 40 0 0 eth4 Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_68

| CaseID | WanRedundancy_Solution_UsbIsBackup_68 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, track wan for usbnet is down and track default gateway for eth0 is up, while track wan for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth4 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet, result 5 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 6. Track default gateway successfully on usbnet, result 6 7. Track wan successfully on usbnet, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-29 22:27:23 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Track default gateway on usbnet successfully, and default monitor state of usbnet is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa404040 for ppp0 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth3: 1 targets tested, result: PASS ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 successfully, and monitor state of eth0 is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:45:19 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: PASS …… -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: wan failover to usbnet AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: Default route is via usbnet, and there are non-default route via eth0 and eth4 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.40.1 0.0.0.0 UG 40 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_69

| CaseID | WanRedundancy_Solution_UsbIsBackup_69 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, track wan for usbnet is down and track default gateway for usbnet is up, while track wan for eth0 is down and track wan for eth4 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth4 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet, result 5 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 6. Track default gateway successfully on usbnet, result 6 7. Track wan fail on eth0, result 7 track-wan use-for-wan-eth0 ip 1.1.1.1 8. Track wan successfully on eth4, result 8 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and monitor state of wifi0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-29 22:27:23 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Track default gateway on usbnet successfully, and default monitor state of usbnet is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa404040 for ppp0 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth3: 1 targets tested, result: PASS ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:51:32 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: Track wan on eth4 successfully, and monitor state of eth4 is Up AH-8c8040#sh lo bu …. 2013-01-29 22:53:12 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth3: 1 targets tested, result: PASS …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 9: wan failover to eth4 AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 10: Default route is via eth4, and there are non-default route via eth0 and usbnet with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 192.168.40.1 0.0.0.0 UG 40 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_70

| CaseID | WanRedundancy_Solution_UsbIsBackup_70 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, track wan for usbnet is down and track default gateway for eth0 is up, while track wan for eth0 is down and track wan for eth4 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth4 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet, result 5 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 6. Track default gateway successfully on eth0, result 6 7. Track wan fail on eth0, result 7 track-wan use-for-wan-eth0 ip 1.1.1.1 8. Track wan fail on eth4, result 8 track-wan use-for-wan-eth4 ip 1.1.1.1 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and monitor state of wifi0 is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-29 22:27:23 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Track default gateway on usbnet successfully, and default monitor state of usbnet is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa404040 for ppp0 2013-01-29 22:42:29 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth3: 1 targets tested, result: PASS ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:51:32 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 8: Track wan on eth4 unsuccessfully, and monitor state of eth4 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:57:35 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth4: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 9: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 10: Default route is via eth0, and there are non-default route via usbnet and eth4 with corresponding priority AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.40.1 0.0.0.0 UG 10040 0 0 eth4 Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_71

| CaseID | WanRedundancy_Solution_UsbIsBackup_71 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, both track wan and track default gateway for usbnet are down, while track wan for eth0 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth4 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet, result 5 _deb brd wanmon track-wan use-for-wan-usbnet0 ip 1.1.1.1 6. Track default gateway fail on usbnet, result 6 no usbm modem-id Huawei_e1752 apn 7. Track wan successfully on usbnet, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down AH-8c8040#sh lo bu ¦ i wanmon …. 2013-01-29 22:27:23 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-usbnet0]: Interface ppp0: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) Result 6: Track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth0 successfully, and monitor state of eth0 is Up AH-8c8040#sh lo bu ….. 2013-01-29 22:45:19 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: PASS …… -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan failover to usbnet AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: Default route is via usbnet, and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.155.31.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.40.1 0.0.0.0 UG 40 0 0 eth4 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_72

| CaseID | WanRedundancy_Solution_UsbIsBackup_72 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is down and track wan for eth4 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth4 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail and track default gateway fail on usbnet, result 5 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 6. Track wan fail on eth0, result 6 track-wan use-for-wan-eth0 ip 1.1.1.1 7. Track wan successfully on eth4, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:51:32 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth4 successfully, and monitor state of eth4 is Up AH-8c8040#sh lo bu …. 2013-01-29 22:53:12 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth4: 1 targets tested, result: PASS …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan failover to eth4 AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: Default route is via eth4, and and there is no default route via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 192.168.40.1 0.0.0.0 UG 40 0 0 eth4 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_73

| CaseID | WanRedundancy_Solution_UsbIsBackup_73 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, both track wan and track default gateway for usbnet are down, while track wan for eth0 is down and track default gateway for eth0 is up, and track wan for eth4 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth4 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet and track default gateway fail on usbnet, result 5 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 6. Track wan fail on eth0, result 6 track-wan use-for-wan-eth0 ip 1.1.1.1 7. Track wan fail on eth4, result 7 track-wan use-for-wan-eth4 ip 1.1.1.1 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:51:32 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 1 targets tested, result: FAIL ….. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth4 unsuccessfully, and monitor state of eth4 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:57:35 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth4: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.31.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.155.31.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.40.1 0.0.0.0 UG 10040 0 0 eth4 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_74

| CaseID | WanRedundancy_Solution_UsbIsBackup_74 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, both track wan and track default gateway for usbnet are down, while both track wan and track default gateway for eth0 are down, and track wan for eth4 is down and track default gateway for eth4 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1.Get a client associated to BR with open auth, result 1 2.Config usb modem as always-connect mode, result 2 3.Config eth4 as wan mode, result 3 4.Config track wan group each interface, result 4 5. Track wan fail on usbnet and track default gateway fail on usbnet, result 5 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 6. Track wan fail on eth0 and track default gateway fail on eth0, result 6 no in eth0 dhcp client 7. Track wan fail on eth4, result 7 track-wan use-for-wan-eth0 ip 1.1.1.1 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:25:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-29 23:26:09 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth4 unsuccessfully, and monitor state of eth4 is Down AH-8c8040#sh lo bu ….. 2013-01-29 22:57:35 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth1]: Interface eth4: 1 targets tested, result: FAIL …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.40.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 192.168.40.1 0.0.0.0 UG 10040 0 0 eth4 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_75

| CaseID | WanRedundancy_Solution_UsbIsBackup_75 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and eth4 available, both track wan and track default gateway for usbnet are down, while both track wan and track default gateway for eth0 are down, and both track wan and track default gateway for eth4 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config eth4 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet and track default gateway fail on usbnet, result 5 _deb brd wanmon no usbm modem-id Huawei_e1752 apn 6. Track wan fail on eth0 and track default gateway fail on eth0, result 6 no in eth0 dhcp client 7. Track wan fail on eth4 and track default gateway fail on eth4, result 7 no in eth4 dhcp client 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | BR config, AH-8c8040#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode wan interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client no interface eth0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.202 usbmodem mode primary-wan usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: USB modem is always-connect mode AH-8c8040#sh usbmodem modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: eth4 is wan mode AH-8c8040#sh in ¦ i wan Eth0 0019:778c:8040 wan U - - - - - Eth4 0019:778c:8045 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in eth4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh in usb0 ¦ i mode Mode=wan; NAT support=enabled; WAN Priority=0 Result 4: Each interface is configured with a track wan group AH-8c8040#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 1 track-wan use-for-wan-eth4 interface eth1 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 1 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 1 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 5: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down AH-8c8040#sh lo bu …. 2013-01-29 23:05:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-usbnet0: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 6: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down AH-8c8040#sh lo bu …. 2013-01-29 23:25:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth0: interface is not connected. 2013-01-29 23:26:09 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 7: Track wan on eth4 unsuccessfully, and track default gateway on eth4 unsuccessfully, and default monitor state of eth4 is Down AH-8c8040#sh lo bu …. 2013-01-29 23:25:02 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for use-for-wan-eth4: interface is not connected. 2013-01-29 23:26:09 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface is not connected. …. -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c8040#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: unknown [use-for-wan-eth4] WAN Monitor State: unknown Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 8: wan won't failover, and wan state is “no wan” AH-8c8040#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Jan 29 22:26:22 2013 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Jan 29 22:26:22 2013 Result 9: default gw with highest priority is via eth0 AH-8c8040#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Notes: No “0.0.0.0” route here Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_76

| CaseID | WanRedundancy_Solution_UsbIsBackup_76 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0 available, and both track wan and track default gateway for usbnet are up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config wifi0 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Check wan state, as well as monitor state, result 5 6. Check default route in main route table, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Visit http on client, result 8 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: USB modem is always-connect mode Result 3: wifi0 is wan mode Result 4: Each interface is configured with a track wan group Result 5: Active wan is usbnet, and both monitor state and default monitor state of usbnet are Up Result 6: Default route is via usb0, and there are non-default route via usbnet and wifi0 with corresponding priority Result 7: ping internet successfully on client Result 8: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_77

| CaseID | WanRedundancy_Solution_UsbIsBackup_77 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0 available, track wan for usbnet is down and track default gateway for eth0 is up, while track wan for usbnet is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config wifi0 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet, result 5 6. Track default gateway successfully on usbnet, result 6 7. Track wan successfully on usbnet, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: USB modem is always-connect mode Result 3: wifi0 is wan mode Result 4: Each interface is configured with a track wan group Result 5: Track wan on usbnet unsuccessfully, and monitor state of wifi0 is Down Result 6: Track default gateway on usbnet successfully, and default monitor state of usbnet is Up Result 7: Track wan on usbnet successfully, and monitor state of usbnet is Up Result 8: wan failover to usbnet Result 9: Default route is via usbnet, and there are non-default route via eth0 and wifi0 with corresponding priority Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_78

| CaseID | WanRedundancy_Solution_UsbIsBackup_78 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0available, track wan for usbnet is down and track default gateway for usbnet is up, while track wan for eth0 is down and track wan for wifi0 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config wifi0 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet, result 5 6. Track default gateway successfully on usbnet, result 6 7. Track wan fail on eth0, result 7 8. Track wan successfully on wifi0, result 8 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: USB modem is always-connect mode Result 3: wifi0 is wan mode Result 4: Each interface is configured with a track wan group Result 5: Track wan on usbnet unsuccessfully, and monitor state of wifi0 is Down Result 6: Track default gateway on usbnet successfully, and default monitor state of usbnet is Up Result 7: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down Result 8: Track wan on wifi0 successfully, and monitor state of wifi0 is Up Result 9: wan failover to wifi0 Result 10: Default route is via wifi0, and there are non-default route via eth0 and usbnet with corresponding priority Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_79

| CaseID | WanRedundancy_Solution_UsbIsBackup_79 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0 available, track wan for usbnet is down and track default gateway for eth0 is up, while track wan for eth0 is down and track wan for wifi0 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config wifi0 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet, result 5 6. Track default gateway successfully on eth0, result 6 7. Track wan fail on usbnet, result 7 8. Track wan fail on wifi0, result 8 9. Check wan state, as well as monitor state, result 9 10. Check default route in main route table, result 10 11. Run ping internet process on client to check its accessibility, result 11 12. Visit http on client, result 12 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: USB modem is always-connect mode Result 3: wifi0 is wan mode Result 4: Each interface is configured with a track wan group Result 5: Track wan on usbnet unsuccessfully, and monitor state of wifi0 is Down Result 6: Track default gateway on usbnet successfully, and default monitor state of usbnet is Up Result 7: Track wan on eth0 unsuccessfully, and monitor state of eth0 is Down Result 8: Track wan on wifi0 unsuccessfully, and monitor state of wifi0 is Down Result 9: wan won't failover, and wan state is “no wan” Result 10: Default route is via eth0, and there are non-default route via usbnet and eth4 with corresponding priority Result 11: ping internet successfully on client Result 12: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_80

| CaseID | WanRedundancy_Solution_UsbIsBackup_80 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0 available, both track wan and track default gateway for usbnet are down, while track wan for eth0 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config wifi0 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet, result 5 6. Track default gateway fail on usbnet, result 6 7. Track wan successfully on usbnet, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: USB modem is always-connect mode Result 3: wifi0 is wan mode Result 4: Each interface is configured with a track wan group Result 5: Track wan on usbnet unsuccessfully, and monitor state of wifi0 is Down Result 6: Track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down Result 7: Track wan on eth0 successfully, and monitor state of eth0 is Up Result 8: wan failover to usbnet Result 9: Default route is via usbnet, and there is no default route via eth0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_81

| CaseID | WanRedundancy_Solution_UsbIsBackup_81 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0 available, both track wan and track default gateway for eth0 are down, while track wan for usbnet is down and track wan for wifi0 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config wifi0 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail and track default gateway fail on usbnet, result 5 6. Track wan fail on eth0, result 6 7. Track wan successfully on wifi0, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: USB modem is always-connect mode Result 3: wifi0 is wan mode Result 4: Each interface is configured with a track wan group Result 5: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down Result 6: Track wan on usbnet unsuccessfully, and monitor state of usbnet is Down Result 7: Track wan on wifi0 successfully, and monitor state of wifi0 is Up Result 8: wan failover to wifi0 Result 9: Default route is via wifi0, and and there is no default route via eth0 Result 10: ping internet successfully on client Result 11: Http visit on client successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_82

| CaseID | WanRedundancy_Solution_UsbIsBackup_82 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0 available, both track wan and track default gateway for usbnet are down, while track wan for eth0 is down and track default gateway for eth0 is up, and track wan for wifi0 is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config wifi0 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet and track default gateway fail on usbnet, result 5 6. Track wan fail on eth0, result 6 7. Track wan fail on wifi0, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: USB modem is always-connect mode Result 3: wifi0 is wan mode Result 4: Each interface is configured with a track wan group Result 5: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down Result 6: Track wan on usbnet unsuccessfully, and monitor state of eth0 is Down Result 7: Track wan on wifi0 unsuccessfully, and monitor state of wifi0 is Down Result 8: wan won't failover, and wan state is “no wan” Result 9: default gw with highest priority is via eth0 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_83

| CaseID | WanRedundancy_Solution_UsbIsBackup_83 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0 available, both track wan and track default gateway for usbnet are down, while both track wan and track default gateway for eth0 are down, and track wan for wifi0 is down and track default gateway for wifi0 is up |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 is connected with Switch initially |  |  |
| TestProcedure | 1.Get a client associated to BR with open auth, result 1 2.Config usb modem as always-connect mode, result 2 3.Config wifi0as wan mode, result 3 4.Config track wan group each interface, result 4 5. Track wan fail on usbnet and track default gateway fail on usbnet, result 5 6. Track wan fail on eth0 and track default gateway fail on eth0, result 6 7. Track wan fail on wifi0, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: USB modem is always-connect mode Result 3: wifi0 is wan mode Result 4: Each interface is configured with a track wan group Result 5: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down Result 6: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down Result 7: Track wan on wifi0unsuccessfully, and monitor state of wifi0 is Down Result 8: wan won't failover, and wan state is “no wan” Result 9: default gw with highest priority is via eth0 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit won't be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Solution_UsbIsBackup_84

| CaseID | WanRedundancy_Solution_UsbIsBackup_84 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN behavior when there is eth0, usb and wifi0 available, both track wan and track default gateway for usbnet are down, while both track wan and track default gateway for eth0 are down, and both track wan and track default gateway for wifi0 are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connect mode, result 2 3. Config wifi0 as wan mode, result 3 4. Config track wan group each interface, result 4 5. Track wan fail on usbnet and track default gateway fail on usbnet, result 5 6. Track wan fail on eth0 and track default gateway fail on eth0, result 6 7. Track wan fail on wifi0 and track default gateway fail on wifi0, result 7 8. Check wan state, as well as monitor state, result 8 9. Check default route in main route table, result 9 10. Run ping internet process on client to check its accessibility, result 10 11. Visit http on client, result 11 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: USB modem is always-connect mode Result 3: eth4 is wan mode Result 4: Each interface is configured with a track wan group Result 5: Track wan on usbnet unsuccessfully, and track default gateway on usbnet unsuccessfully, and default monitor state of usbnet is Down Result 6: Track wan on eth0 unsuccessfully, and track default gateway on eth0 unsuccessfully, and default monitor state of eth0 is Down Result 7: Track wan on eth4 unsuccessfully, and track default gateway on eth4 unsuccessfully, and default monitor state of eth4 is Down Result 8: wan won't failover, and wan state is “no wan” Result 9: default gw with highest priority is via eth0 Result 10: ping internet will be reponsed with mgt0 Result 11: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

## Function Test Case

### 6.2.1. Interface

#### WanRedundancy_Function_Interface_01

| CaseID | WanRedundancy_Function_Interface_01 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | Eth0 as WAN interface |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially |  |  |
| TestProcedure | 1. Config eth0 as WAN and highest priority, result 1 2. Config track wan group for eth0, result 2 3. Check wan monitor state of eth0, result 3 4. Check default route in main route table, result 4 5. Check the corresponding DNS config, result 5 6. Get a client associated to BR with open auth, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Check NAT prodedure via eth0 for the traffic of client, result 8 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 usbmodem modem-id sierra_308 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 bonjour-gateway realm building01_floor01_hive0 Result 1: Eth0 is WAN mode, with highest priority AH-8c66c0#sh in e0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth0 Eth0 4018:b18c:66c0 wan U - - - - - Result 2: Eth0 is configured with track wan group AH-8c66c0#sh ru ¦ i use-for-wan-eth0 track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable Result 3: WAN monitor state of eth0 is Up AH-8c66c0#sh track-wan [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0]SH WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 4: Default route is via eth0 AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 Result 5: There is DNS server for eth0 AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 6: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0192.168.85.10 1 195M 130M -18(59) open none 00:00:16 1 Yes 0 11ng disabled 20MHz No data collecting... Result 7: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 8: There is NAT for traffic of client |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Interface_02

| CaseID | WanRedundancy_Function_Interface_02 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | Eth1 as WAN interface |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Config eth1 as WAN and highest priority, result 1 in e1 mode wan in e1 mode wan p 1 in e0 mode wan p 2 2. Config track wan group for eth1, result 2 3. Check wan monitor state of eth1, result 3 4. Check default route in main route table, result 4 5. Check the corresponding DNS config, result 5 6. Get a client associated to BR with open auth, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Check NAT prodedure via eth1 for the traffic of client, result 8 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 usbmodem modem-id sierra_308 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 bonjour-gateway realm building01_floor01_hive0 Result 1: eth1 is WAN mode, with highest priority AH-8c66c0#sh in e1 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 wan D - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 2 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: wifidx: 1 kifidx: 0 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Mon Nov 5 10:10:35 2012 lkdn_count: 1 lkdn_time: Mon Nov 5 10:33:24 2012 ifname: œs$œs$ kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Tue Nov 6 07:28:23 2012 lkdn_count: 0 lkdn_time: Tue Nov 6 07:28:23 2012 Result 2: eth1 is configured with track wan group AH-8c66c0#sh ru ¦ i use-for-wan-eth1 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable Result 3: WAN monitor state of eth1 is Up AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 4: Default route is via eth1 AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 1 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 2 0 0 eth0 Result 5: There is DNS server for eth1 AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 6: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.11 1 1M 156M -22(57) open none 00:50:07 1 Yes 0 11ng disabled 20MHz No Good Result 7: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 8: There is NAT for traffic of client |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Interface_03

| CaseID | WanRedundancy_Function_Interface_03 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | Eth4 as WAN interface |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Config eth4 as WAN and highest priority, result 1 in e2 mode wan in e2 mode wan p 1 in e0 mode wan p 2 2. Config track wan group for eth4, result 2 3. Check wan monitor state of eth4, result 3 4. Check default route in main route table, result 4 5. Check the corresponding DNS config, result 5 6. Get a client associated to BR with open auth, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Check NAT prodedure via eth4 for the traffic of client, result 8 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 usbmodem modem-id sierra_308 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 bonjour-gateway realm building01_floor01_hive0 Result 1: eth4 is WAN mode, with highest priority AH-8c66c0#sh in e2 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth4 Eth4 4018:b18c:66c3 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 2 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Tue Nov 6 08:11:55 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: wifidx: 1 kifidx: 0 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:10 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth4 wifidx: 2 kifidx: 8 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:21 1970 Result 2: eth4 is configured with track wan group AH-8c66c0#sh ru ¦ i use-for-wan-eth4 track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 5 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable Result 3: WAN monitor state of eth4 is Up AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth1] WAN Monitor State: unknown Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 4: Default route is via eth4 AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 192.168.10.1 0.0.0.0 UG 1 0 0 eth4 0.0.0.0 10.155.30.254 0.0.0.0 UG 2 0 0 eth0 Result 5: There is DNS server for eth4 AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 6: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.11 1 1M 156M -22(57) open none 00:50:07 1 Yes 0 11ng disabled 20MHz No Good Result 7: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 8: There is NAT for traffic of client |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Interface_04

| CaseID | WanRedundancy_Function_Interface_04 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | Eth3 as WAN interface |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth3 is connected with Switch initially |  |  |
| TestProcedure | 1. Config eth3 as WAN and highest priority, result 1 in e3 mode wan in e3 mode wan p 1 in e0 mode wan p 2 2. Config track wan group for eth3, result 2 3. Check wan monitor state of eth3, result 3 4. Check default route in main route table, result 4 5. Check the corresponding DNS config, result 5 6. Get a client associated to BR with open auth, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Check NAT prodedure via eth3 for the traffic of client, result 8 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 usbmodem modem-id sierra_308 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 bonjour-gateway realm building01_floor01_hive0 Result 1: eth3 is WAN mode, with highest priority AH-8c66c0#sh in e3 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth3 Eth3 4018:b18c:66c3 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 2 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Tue Nov 6 08:11:55 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: wifidx: 1 kifidx: 0 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:10 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth3 wifidx: 2 kifidx: 8 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:21 1970 Result 2: eth3 is configured with track wan group AH-8c66c0#sh ru ¦ i use-for-wan-eth3 track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 5 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable Result 3: WAN monitor state of eth3 is Up AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth1] WAN Monitor State: unknown Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 4: Default route is via eth3 AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 192.168.10.1 0.0.0.0 UG 1 0 0 eth3 0.0.0.0 10.155.30.254 0.0.0.0 UG 2 0 0 eth0 Result 5: There is DNS server for eth3 AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 6: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.11 1 1M 156M -22(57) open none 00:50:07 1 Yes 0 11ng disabled 20MHz No Good Result 7: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 8: There is NAT for traffic of client |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Interface_05

| CaseID | WanRedundancy_Function_Interface_05 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | Eth4 as WAN interface |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth4 is connected with Switch initially |  |  |
| TestProcedure | 1. Config eth4 as WAN and highest priority, result 1 in e4 mode wan in e4 mode wan p 1 in e0 mode wan p 2 2. Config track wan group for eth4, result 2 3. Check wan monitor state of eth4, result 3 4. Check default route in main route table, result 4 5. Check the corresponding DNS config, result 5 6. Get a client associated to BR with open auth, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Check NAT prodedure via eth4 for the traffic of client, result 8 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 usbmodem modem-id sierra_308 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 bonjour-gateway realm building01_floor01_hive0 Result 1: eth4 is WAN mode, with highest priority AH-8c66c0#sh in e4 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth4 Eth4 4018:b18c:66c3 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 2 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Tue Nov 6 08:11:55 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: wifidx: 1 kifidx: 0 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:10 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth4 wifidx: 2 kifidx: 8 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:21 1970 Result 2: eth4 is configured with track wan group AH-8c66c0#sh ru ¦ i use-for-wan-eth4 track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 5 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable Result 3: WAN monitor state of eth4 is Up AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth1] WAN Monitor State: unknown Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) Result 4: Default route is via eth4 AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 192.168.10.1 0.0.0.0 UG 1 0 0 eth4 0.0.0.0 10.155.30.254 0.0.0.0 UG 2 0 0 eth0 Result 5: There is DNS server for eth4 AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 6: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.11 1 1M 156M -22(57) open none 00:50:07 1 Yes 0 11ng disabled 20MHz No Good Result 7: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 8: There is NAT for traffic of client |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Interface_06

| CaseID | WanRedundancy_Function_Interface_06 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | usbnet as WAN interface |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. usb modem is plugged with BR initially 2. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Config usbnet as WAN and highest priority, result 1 usbm mode a usbmodem modem-id sierra_308 apn 3gnet in usbnet0 mode wan p 1 in e0 mode wan p 2 2. Config track wan group for usbnet, result 2 3. Check wan monitor state of usbnet, result 3 4. Check default route in main route table, result 4 5. Check the corresponding DNS config, result 5 6. Get a client associated to BR with open auth, result 6 7. Run ping internet process on client to check its accessibility, result 7 8. Check NAT prodedure via usbnet for the traffic of client, result 8 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 usbmodem modem-id sierra_308 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 bonjour-gateway realm building01_floor01_hive0 Result 1: usbnet WAN is with highest priority AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 2 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 18 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Tue Nov 6 09:32:24 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 2: usbnet is configured with track wan group AH-8c66c0#sh ru ¦ i use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 3: WAN monitor state of usbnet is Up AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) Result 4: Default route is via usbnet AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 1 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 2 0 0 eth0 Result 5: There is DNS server for usbnet AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 6: Client gets association with BR AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.11 1 1M 156M -22(57) open none 00:50:07 1 Yes 0 11ng disabled 20MHz No Good Result 7: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 8: There is NAT for traffic of client |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Interface_07

| CaseID | WanRedundancy_Function_Interface_07 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 4 |  |  |
| Description | Wifi0 as WAN interface |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. There is ISP SSID available |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config wifi0 as WAN and highest priority, result 2 3. On client, let BR connect with an ISP ssid, result 3 4. Config track wan group for usbnet, result 4 5. Check the WAN monitor status of wifi0, result 5 6. Check default route in main route table, result 6 7. Check the corresponding DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Check NAT prodedure via eth0 for the traffic of client, result 9 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: wifi client interface is with highest priority Result 3: BR is associated with an ISP ssid Result 4: wifi0 is configured with track wan group Result 5: WAN monitor state of wifi client interface is Up Result 6: Default route is via wifi client interface Result 7: There is DNS server for wifi client interface Result 8: Ping successfully on client Result 9: There is NAT for traffic of client |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Interface_08

| CaseID | WanRedundancy_Function_Interface_08 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 4 |  |  |
| Description | Wifi1 as WAN interface |  |  |
| PlatformDependence | BRAP330, BRAP350 |  |  |
| Pre-condition | 1. There is ISP SSID available |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config wifi1 as WAN and highest priority, result 2 3. On client, let BR connect with an ISP ssid, result 3 4. Config track wan group for usbnet, result 4 5. Check the WAN monitor status of wifi1, result 5 6. Check default route in main route table, result 6 7. Check the corresponding DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Check NAT prodedure via eth0 for the traffic of client, result 9 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: wifi client interface is with highest priority Result 3: BR is associated with an ISP ssid Result 4: wifi1 is configured with track wan group Result 5: WAN monitor state of wifi client interface is Up Result 6: Default route is via wifi client interface Result 7: There is DNS server for wifi client interface Result 8: Ping successfully on client Result 9: There is NAT for traffic of client |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Interface_09

| CaseID | WanRedundancy_Function_Interface_09 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | VPN tunnel on multi interfaces |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth1 as wan mode, result 2 3. Config track wan group for each interface, result 3 4. Config vpn tunnel for eth0/ eth1, result 4 5. Check wan monitor state of eth0/ eth1, result 5 6. Check status of each vpn tunnel, result 6 7. Run traffic on client via tunnel, result 7 8. Track fail on eth0, wan failover to eth1, result 8 9. Check vpn tunnel status, result 9 10. Run traffic on client via tunnel, result 10 11. Restore track on eth0, wan failback to eth0, result 5 12. Check vpn tunnel status, result 6 13. Run traffic on client via tunnel, result 7 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: eth1 is wan mode Result 3: Track wan group is configured for each wan Result 4: VPN tunnel is configured for eth0 & eth1 Result 5: eth0 is active wan Result 6: Active VPN tunnel is via eth0 Result 7: Traffic on client is carried by the tunnel via eth0 Result 8: eth1 is active wan Result 9: Active VPN tunnel is via eth1 Result 10: Traffic on client is carried by the tunnel via eth1 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### 6.2.2. Priority

#### WanRedundancy_Function_Priority_01

| CaseID | WanRedundancy_Function_Priority_01 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN failover with eth1 as default priority when usb modem is always-connect mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 2. Config eth1 as wan mode, result 2 in e1 mode wan 3. Config track wan group for each interface, result 3 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable 4. Config usb modem as always-connect mode, result 4 usbm mode a usbmodem modem-id sierra_308 apn 3gnet 5. Check wan monitor state of each interface, result 5 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on eth0, wan failover to eth1, check wan monitor state, result 9 track-wan use-for-wan-eth0 ip 9.9.9.9 10. Repeat Step 6, result 10 11. Repeat Step 7, result 11 12. Maintain ping process on client, result 8 13. Track fail on eth1, wan failover to usbnet, repeat Step 9~ 12, result 12 track-wan use-for-wan-eth1 ip 9.9.9.9 14. Restore track on eth1, check wan monitor state and default route, result 13 no track-wan use-for-wan-eth1 ip 9.9.9.9 15. Restore track on eth0, check wan monitor state and default route, result 14 no track-wan use-for-wan-eth0 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan D - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.10 1 195M 1M -24(55) open none 00:02:15 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c66c0#sh in e1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 wan U - - - - - Result 3: Each interface is configured with a track wan group AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 wan U - - - - - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: USB modem is always-connect mode AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 5: eth0 is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:25 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 20 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 3 lkup_time: Wed Nov 7 03:04:42 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 10 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 7 03:03:49 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:20 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 Result 6: Default route is via eth0, and there are non-default routes via eth1/ usbnet with corresponding priority AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U eth1 192.168.10.5 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.27.104.129 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 7: There is DNS server for eth0 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 8: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 9: eth1 is active wan AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:29:19 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:29:19 2012 Result 10: Default route is via eth1, and there are non-default routes via eth0/ usbnet with corresponding priority AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 Result 11: There is DNS server for eth1 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 12: Active WAN is usbnet, default route is via usbnet, DNS is corresponding to usbnet, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:59:40 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:59:40 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - !!! Still with issue, bug 20545 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 13: Active WAN is eth1, default route is via eth1, DNS is corresponding to eth1, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:29:19 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:29:19 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 14: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_02

| CaseID | WanRedundancy_Function_Priority_02 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN failback with higher priority interface when usb modem is always-connect mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 2. Config eth1 as wan mode, result 2 in e1 mode wan 3. Config track wan group for each interface, result 3 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable 4. Config usb modem as always-connect mode, result 4 usbm mode a usbmodem modem-id sierra_308 apn 3gnet 5. Check wan monitor state of each interface, result 5 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on eth0, wan failover to eth1, check wan monitor state, result 9 track-wan use-for-wan-eth0 ip 9.9.9.9 10. Repeat Step 6, result 10 11. Repeat Step 7, result 11 12. Maintain ping process on client, result 8 13. Track fail on eth1, wan failover to usbnet, repeat Step 9~ 12, result 12 track-wan use-for-wan-eth1 ip 9.9.9.9 14. Restore track on both eth0 and eth1, check wan monitor state and default route, result 13 no track-wan use-for-wan-eth1 ip 9.9.9.9 no track-wan use-for-wan-eth0 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan D - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.10 1 195M 1M -24(55) open none 00:02:15 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c66c0#sh in e1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 wan U - - - - - Result 3: Each interface is configured with a track wan group AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 wan U - - - - - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: USB modem is always-connect mode AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 5: eth0 is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:25 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 20 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 3 lkup_time: Wed Nov 7 03:04:42 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 10 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 7 03:03:49 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:20 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 Result 6: Default route is via eth0, and there are non-default routes via eth1/ usbnet with corresponding priority AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U eth1 192.168.10.5 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.27.104.129 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 7: There is DNS server for eth0 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 8: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 9: eth1 is active wan AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:29:19 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:29:19 2012 Result 10: Default route is via eth1, and there are non-default routes via eth0/ usbnet with corresponding priority AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 Result 11: There is DNS server for eth1 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 12: Active WAN is usbnet, default route is via usbnet, DNS is corresponding to usbnet, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:59:40 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:59:40 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - !!! Still with issue, bug 20545 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 13: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_03

| CaseID | WanRedundancy_Function_Priority_03 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN failover with eth4 as default priority when usb modem is always-connect mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth4 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 2. Config eth4 as wan mode, result 2 in e2 mode wan 3. Config track wan group for each interface, result 3 track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 5 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable 4. Config usb modem as always-connect mode, result 4 usbm mode a usbmodem modem-id sierra_308 apn 3gnet 5. Check wan monitor state of each interface, result 5 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on eth0, wan failover to eth4, check wan monitor state, result 9 track-wan use-for-wan-eth0 ip 9.9.9.9 10. Repeat Step 6, result 10 11. Repeat Step 7, result 11 12. Maintain ping process on client, result 8 13. Track fail on eth4, wan failover to usbnet, repeat Step 9~ 12, result 12 track-wan use-for-wan-eth4 ip 9.9.9.9 14. Restore track on eth4, check wan monitor state and default route, result 13 no track-wan use-for-wan-eth4 ip 9.9.9.9 15. Restore track on eth0, check wan monitor state and default route, result 14 no track-wan use-for-wan-eth0 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 wan D - - - - - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.10 1 195M 1M -24(55) open none 00:02:15 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth4 is wan mode AH-8c66c0#sh in e2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth4 Eth4 4018:b18c:66c2 wan U - - - - - Result 3: Each interface is configured with a track wan group AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 wan D - - - - - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 5 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: USB modem is always-connect mode AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 5: eth0 is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:25 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 20 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 3 lkup_time: Wed Nov 7 03:04:42 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth4 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 20 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 7 03:03:49 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:20 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 Result 6: Default route is via eth0, and there are non-default routes via eth4/ usbnet with corresponding priority AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U eth4 192.168.10.5 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.27.104.129 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 20 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 7: There is DNS server for eth0 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 8: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 9: eth4 is active wan AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:29:19 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:29:19 2012 Result 10: Default route is via eth4, and there are non-default routes via eth0/ usbnet with corresponding priority AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 192.168.10.1 0.0.0.0 UG 20 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 Result 11: There is DNS server for eth4 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 12: Active WAN is usbnet, default route is via usbnet, DNS is corresponding to usbnet, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:59:40 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:59:40 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10020 0 0 eth4 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - !!! Still with issue, bug 20545 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 13: Active WAN is eth4, default route is via eth4, DNS is corresponding to eth4, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:29:19 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:29:19 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 192.168.10.1 0.0.0.0 UG 20 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 14: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 20 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_04

| CaseID | WanRedundancy_Function_Priority_04 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN failover with eth3 as default priority when usb modem is always-connect mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth3 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | - Get a client associated to BR with open auth, result 1 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 2. Config eth3 as wan mode, result 2 in e3 mode wan 3. Config track wan group for each interface, result 3 track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 5 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable 4. Config usb modem as always-connect mode, result 4 usbm mode a usbmodem modem-id sierra_308 apn 3gnet 5. Check wan monitor state of each interface, result 5 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on eth0, wan failover to eth3, check wan monitor state, result 9 track-wan use-for-wan-eth0 ip 9.9.9.9 10. Repeat Step 6, result 10 11. Repeat Step 7, result 11 12. Maintain ping process on client, result 8 13. Track fail on eth3, wan failover to usbnet, repeat Step 9~ 12, result 12 track-wan use-for-wan-eth3 ip 9.9.9.9 14. Restore track on eth3, check wan monitor state and default route, result 13 no track-wan use-for-wan-eth3 ip 9.9.9.9 15. Restore track on eth0, check wan monitor state and default route, result 14 no track-wan use-for-wan-eth0 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 wan U - - - - - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 6 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 6 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0 192.168.85.10 6 195M 175.5M -29(50) open none 00:00:09 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth3 is wan mode AH-8c66c0#sh in e3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth3 Eth3 4018:b18c:66c2 wan U - - - - - Result 3: Each interface is configured with a track wan group AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 wan U - - - - - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 6 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 6 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 5 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: USB modem is always-connect mode AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 5: eth0 is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:25 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 20 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 3 lkup_time: Wed Nov 7 03:04:42 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth3 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 30 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 7 03:03:49 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:20 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 Result 6: Default route is via eth0, and there are non-default routes via eth3/ usbnet with corresponding priority AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U eth3 192.168.10.5 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.27.104.129 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 30 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 7: There is DNS server for eth0 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 8: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 9: eth3 is active wan AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:29:19 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:29:19 2012 Result 10: Default route is via eth3, and there are non-default routes via eth0/ usbnet with corresponding priority AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 192.168.10.1 0.0.0.0 UG 30 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 Result 11: There is DNS server for eth3 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 12: Active WAN is usbnet, default route is via usbnet, DNS is corresponding to usbnet, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: down Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:59:40 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:59:40 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10030 0 0 eth3 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - !!! Still with issue, bug 20545 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 13: Active WAN is eth3, default route is via eth3, DNS is corresponding to eth3, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:29:19 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:29:19 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 192.168.10.1 0.0.0.0 UG 30 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 14: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 30 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_05

| CaseID | WanRedundancy_Function_Priority_05 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN failover with eth4 as default priority when usb modem is always-connect mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth4 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | - Get a client associated to BR with open auth, result 1 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 2. Config eth4 as wan mode, result 2 in e4 mode wan 3. Config track wan group for each interface, result 3 track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 5 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable 4. Config usb modem as always-connect mode, result 4 usbm mode a usbmodem modem-id sierra_308 apn 3gnet 5. Check wan monitor state of each interface, result 5 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on eth0, wan failover to eth4, check wan monitor state, result 9 track-wan use-for-wan-eth0 ip 9.9.9.9 10. Repeat Step 6, result 10 11. Repeat Step 7, result 11 12. Maintain ping process on client, result 8 13. Track fail on eth4, wan failover to usbnet, repeat Step 9~ 12, result 12 track-wan use-for-wan-eth4 ip 9.9.9.9 14. Restore track on eth4, check wan monitor state and default route, result 13 no track-wan use-for-wan-eth4 ip 9.9.9.9 15. Restore track on eth0, check wan monitor state and default route, result 14 no track-wan use-for-wan-eth0 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 6 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 6 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0 192.168.85.10 6 195M 175.5M -29(50) open none 00:00:09 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth4 is wan mode AH-8c66c0#sh in e4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth4 Eth4 4018:b18c:66c2 wan U - - - - - Result 3: Each interface is configured with a track wan group AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 6 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 6 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 5 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: USB modem is always-connect mode AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 5: eth0 is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:25 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 20 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 3 lkup_time: Wed Nov 7 03:04:42 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth4 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 40 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 7 03:03:49 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:20 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 Result 6: Default route is via eth0, and there are non-default routes via eth4/ usbnet with corresponding priority AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U eth4 192.168.10.5 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.27.104.129 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 40 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 7: There is DNS server for eth0 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 8: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 9: eth4 is active wan AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:29:19 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:29:19 2012 Result 10: Default route is via eth4, and there are non-default routes via eth0/ usbnet with corresponding priority AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 192.168.10.1 0.0.0.0 UG 40 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 Result 11: There is DNS server for eth4 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 12: Active WAN is usbnet, default route is via usbnet, DNS is corresponding to usbnet, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:59:40 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:59:40 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10040 0 0 eth4 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - !!! Still with issue, bug 20545 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 13: Active WAN is eth4, default route is via eth4, DNS is corresponding to eth4, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:29:19 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:29:19 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 192.168.10.1 0.0.0.0 UG 40 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 14: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 40 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_06

| CaseID | WanRedundancy_Function_Priority_06 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 4 |  |  |
| Description | WAN failover with wifi0 as default priority when usb modem is always-connect mode |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 has been configured as wan mode and connected with ISP ssid 5. wan monitor state through wifi0 is up |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config wifi0 as wan mode, result 2 3. Check the state of wan via wifi0, result 3 4. Config track wan group for each interface, result 4 5. Config usb modem as always-connect mode, result 5 6. Check wan monitor state of each interface, result 6 7. Check default route in main route table, result 7 8. Check the active DNS config, result 8 9. Run ping internet process on client to check its accessibility, result 9 10. Track fail on eth0, wan failover to wifi0, check wan monitor state, result 10 11. Repeat Step 7, result 11 12. Repeat Step 8, result 12 13. Maintain ping process on client, result 9 14. Track fail on wifi0, wan failover to usbnet, repeat Step 10~ 13, result 13 15. Restore track on wifi0, check wan monitor state and default route, result 14 16. Restore track on eth0, check wan monitor state and default route, result 15 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: wifi0 is wan mode Result 3: wifi0 get association with ISP ssid Result 4: Each interface is configured with a track wan group Result 5: USB modem is always-connect mode Result 6: eth0 is active wan Result 7: Default route is via eth0, and there are non-default routes via wifi0/ usbnet with corresponding priority Result 8: There is DNS server for eth0 Result 9: Ping successfully on client Result 10: wifi0 is active wan Result 11: Default route is via wifi0, and there are non-default routes via eth0/ usbnet with corresponding priority Result 12: There is DNS server for wifi0 Result 13: Active WAN is usbnet, default route is via usbnet, DNS is corresponding to usbnet, and ping successfully on client Result 14: Active WAN is wifi0, default route is via wifi0, DNS is corresponding to wifi0, and ping successfully on client Result 15: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_07

| CaseID | WanRedundancy_Function_Priority_07 |  |  |
| --- | --- | --- | --- |
| Priority | Accept | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN failover with eth1 as default priority when usb modem is prim-wan mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | - Get a client associated to BR with open auth, result 1 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 - Config eth1 as wan mode, result 2 in e1 mode wan - Config track wan group for each interface, result 3 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable - Config usb modem as prim-wan mode, result 4 usbm mode p 5. Check wan monitor state of each interface, result 5 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on usbnet, wan failover to eth0, check wan monitor state, result 9 track-wan use-for-wan-usbnet0 ip 9.9.9.9 10. Repeat Step 6, result 10 11. Repeat Step 7, result 11 12. Maintain ping process on client, result 8 13. Track fail on eth0, wan failover to eth1, repeat Step 9~ 12, result 12 track-wan use-for-wan-eth0 ip 9.9.9.9 14. Restore track on eth0, check wan monitor state and default route, result 13 no track-wan use-for-wan-eth0 ip 9.9.9.9 15. Restore track on usbnet, check wan monitor state and default route, result 14 no track-wan use-for-wan-usbnet0 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 wan U - - - - - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0 192.168.85.10 6 195M 175.5M -29(50) open none 00:00:09 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c66c0#sh in e1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 wan U - - - - - Result 3: Each interface is configured with a track wan group AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 wan U - - - - - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: USB modem is prim-wan mode AH-8c66c0#sh usbmodem modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 5: usbnet is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 18 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Fri Nov 9 05:58:31 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 10 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Fri Nov 9 06:05:29 2012 lkdn_count: 0 lkdn_time: Fri Nov 9 06:05:02 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 9 06:05:50 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 06:05:50 2012 Result 6: Default route is via usbnet, and there are non-default routes via eth0/ eth1 with corresponding priority AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U eth1 192.168.10.5 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.46.165.22 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 0 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 Result 7: There is DNS server for usbnet AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 8: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 9: eth0 is active wan AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 9 06:14:54 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 06:14:54 2012 Result 10: Default route is via eth0, and there are non-default routes via eth1/ usbnet with corresponding priority AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 11: There is DNS server for eth0 AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 12: Active WAN is eth1, default route is via eth1, DNS is corresponding to eth1, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: FAILOVER prev_state_time: Fri Nov 9 06:18:32 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Fri Nov 9 06:18:32 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 13: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 9 06:14:54 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 06:14:54 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 14: Active WAN is usbnet, default route is via usbnet, DNS is corresponding to usbnet, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 9 06:05:50 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 06:05:50 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 0 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_08

| CaseID | WanRedundancy_Function_Priority_08 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN failback with higher priority interface when usb modem is prim-wan mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 2. Config eth1 as wan mode, result 2 in e1 mode wan 3. Config track wan group for each interface, result 3 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable 4. Config usb modem as prim-wan mode, result 4 usbm mode p 5. Check wan monitor state of each interface, result 5 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on usbnet, wan failover to eth0, check wan monitor state, result 9 track-wan use-for-wan-usbnet0 ip 9.9.9.9 10. Repeat Step 6, result 10 11. Repeat Step 7, result 11 12. Maintain ping process on client, result 8 13. Track fail on eth0, wan failover to eth1, repeat Step 9~ 12, result 12 track-wan use-for-wan-eth0 ip 9.9.9.9 14. Restore track on both eth0 and usbnet, check wan monitor state and default route, result 13 no track-wan use-for-wan-eth0 ip 9.9.9.9 no track-wan use-for-wan-usbnet0 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 wan U - - - - - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 2477:0312:d9c0 192.168.85.10 6 195M 175.5M -29(50) open none 00:00:09 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c66c0#sh in e1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 wan U - - - - - Result 3: Each interface is configured with a track wan group AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 wan U - - - - - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: USB modem is prim-wan mode AH-8c66c0#sh usbmodem modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 5: usbnet is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 18 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Fri Nov 9 05:58:31 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 10 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Fri Nov 9 06:05:29 2012 lkdn_count: 0 lkdn_time: Fri Nov 9 06:05:02 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 9 06:05:50 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 06:05:50 2012 Result 6: Default route is via usbnet, and there are non-default routes via eth0/ eth1 with corresponding priority AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U eth1 192.168.10.5 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.46.165.22 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 0 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 Result 7: There is DNS server for usbnet AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 8: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 9: eth0 is active wan AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 9 06:14:54 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 06:14:54 2012 Result 10: Default route is via eth0, and there are non-default routes via eth1/ usbnet with corresponding priority AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 Result 11: There is DNS server for eth0 AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 12: Active WAN is eth1, default route is via eth1, DNS is corresponding to eth1, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: FAILOVER prev_state_time: Fri Nov 9 06:18:32 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Fri Nov 9 06:18:32 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 13: Active WAN is usbnet, default route is via usbnet, DNS is corresponding to usbnet, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 9 06:05:50 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 06:05:50 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 0 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_09

| CaseID | WanRedundancy_Function_Priority_09 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN failover with eth1 as customized priority when usb modem is always-connect mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 2. Config eth1 as wan mode, result 2 in e1 mode wan 3. Config track wan group for each interface, result 3 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable 4. Config usb modem as always-connect mode, result 4 usbm mode a usbmodem modem-id sierra_308 apn 3gnet 5. Config wan priority as customized sequence (eth1> usbnet> eth0), and check wan state of each interface, result 5 in e0 mode wan p 1000 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on eth1, wan failover to usbnet, check wan monitor state, result 9 track-wan use-for-wan-eth1 ip 9.9.9.9 10. Repeat Step 6, result 10 11. Repeat Step 7, result 11 12. Maintain ping process on client, result 8 13. Track fail on usbnet, wan failover to eth0, repeat Step 9~ 12, result 12 track-wan use-for-wan-usbnet0 ip 9.9.9.9 14. Restore track on usbnet, check wan monitor state and default route, result 13 no track-wan use-for-wan-usbnet0 ip 9.9.9.9 15. Restore track on eth1, check wan monitor state and default route, result 14 no track-wan use-for-wan-eth1 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 wan U - - - - - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.10 1 195M 1M -24(55) open none 00:02:15 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c66c0#sh in e1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 wan U - - - - - Result 3: Each interface is configured with a track wan group AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 wan U - - - - - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: USB modem is always-connect mode AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 5: wan priority is as customized, and eth1 is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1000 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 18 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Fri Nov 9 08:48:33 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth1 wifidx: 2 kifidx: 8 wanif_type: 0 priority: 10 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:21 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 9 08:53:24 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 08:53:24 2012 Result 6: Default route is via eth1, and there are non-default routes via eth0/ usbnet with corresponding priority AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U eth1 192.168.10.5 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.27.104.129 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh i ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 Result 7: There is DNS server for eth1 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 8: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 9: usbnet is active wan AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Nov 9 09:12:16 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 09:12:16 2012 Result 10: Default route is via usbnet, and there are non-default routes via eth0/ eth1 with corresponding priority AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 Result 11: There is DNS server for usbnet AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 12: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Nov 9 09:40:57 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 09:40:57 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 13: Active WAN is usbnet, default route is via usbnet, DNS is corresponding to usbnet, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Nov 9 09:12:16 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 09:12:16 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 14: Active WAN is eth1, default route is via eth1, DNS is corresponding to eth1, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 9 08:53:24 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 08:53:24 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh i ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_10

| CaseID | WanRedundancy_Function_Priority_10 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN failback with higher priority interface when usb modem is always-connect mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 2. Config eth1 as wan mode, result 2 in e1 mode wan 3. Config track wan group for each interface, result 3 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable 4. Config usb modem as always-connect mode, result 4 usbm mode a usbmodem modem-id sierra_308 apn 3gnet 5. Config wan priority as customized sequence (eth1> usbnet> eth0), and check wan state of each interface, result 5 in e0 mode wan p 1000 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on eth1, wan failover to usbnet, check wan monitor state, result 9 track-wan use-for-wan-eth1 ip 9.9.9.9 10. Repeat Step 6, result 10 11. Repeat Step 7, result 11 12. Maintain ping process on client, result 8 13. Track fail on usbnet, wan failover to eth0, repeat Step 9~ 12, result 12 track-wan use-for-wan-usbnet0 ip 9.9.9.9 14. Restore track on both eth1 and usbnet, check wan monitor state and default route, result 13 no track-wan use-for-wan-usbnet0 ip 9.9.9.9 no track-wan use-for-wan-eth1 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 wan U - - - - - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.10 1 195M 1M -24(55) open none 00:02:15 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c66c0#sh in e1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 wan U - - - - - Result 3: Each interface is configured with a track wan group AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 wan U - - - - - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: USB modem is always-connect mode AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 5: eth1 is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1000 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 18 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Fri Nov 9 08:48:33 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth1 wifidx: 2 kifidx: 8 wanif_type: 0 priority: 10 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:21 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 9 08:53:24 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 08:53:24 2012 Result 6: Default route is via eth1, and there are non-default routes via eth0/ usbnet with corresponding priority AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U eth1 192.168.10.5 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.27.104.129 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh i ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 Result 7: There is DNS server for eth1 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 8: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 9: usbnet is active wan AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Nov 9 09:12:16 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 09:12:16 2012 Result 10: Default route is via usbnet, and there are non-default routes via eth0/ eth1 with corresponding priority AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 Result 11: There is DNS server for usbnet AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 12: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Nov 9 09:40:57 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 09:40:57 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 13: Active WAN is eth1, default route is via eth1, DNS is corresponding to eth1, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 9 08:53:24 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 08:53:24 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh i ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_11

| CaseID | WanRedundancy_Function_Priority_11 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN failover with eth4 as customized priority when usb modem is always-connect mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth4 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 2. Config eth4 as wan mode, result 2 in e2 mode wan 3. Config track wan group for each interface, result 3 track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 5 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable 4. Config usb modem as always-connect mode, result 4 usbm mode a usbmodem modem-id sierra_308 apn 3gnet 5. Config wan priority as customized sequence (eth4> usbnet> eth0), and check wan state of each interface, result 5 in e0 mode wan p 1000 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on eth4, wan failover to usbnet, check wan monitor state, result 9 track-wan use-for-wan-eth4 ip 9.9.9.9 10. Repeat Step 6, result 10 11. Repeat Step 7, result 11 12. Maintain ping process on client, result 8 13. Track fail on usbnet, wan failover to eth0, repeat Step 9~ 12, result 12 track-wan use-for-wan-usbnet0 ip 9.9.9.9 14. Restore track on usbnet, check wan monitor state and default route, result 13 no track-wan use-for-wan-usbnet0 ip 9.9.9.9 15. Restore track on eth4, check wan monitor state and default route, result 14 no track-wan use-for-wan-eth4 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 wan D - - - - - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.10 1 195M 1M -24(55) open none 00:02:15 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth4 is wan mode AH-8c66c0#sh in e2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth4 Eth4 4018:b18c:66c2 wan U - - - - - Result 3: Each interface is configured with a track wan group AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 wan D - - - - - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 5 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: USB modem is always-connect mode AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 5: wan priority is as customized, and eth4 is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1000 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 18 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Fri Nov 9 08:48:33 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth4 wifidx: 2 kifidx: 8 wanif_type: 0 priority: 20 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:21 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 9 08:53:24 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 08:53:24 2012 Result 6: Default route is via eth4, and there are non-default routes via eth0/ usbnet with corresponding priority AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U eth4 192.168.10.5 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.27.104.129 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh i ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 192.168.10.1 0.0.0.0 UG 20 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 Result 7: There is DNS server for eth4 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 8: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 9: usbnet is active wan AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Nov 9 09:12:16 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 09:12:16 2012 Result 10: Default route is via usbnet, and there are non-default routes via eth0/ eth4 with corresponding priority AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10020 0 0 eth4 Result 11: There is DNS server for usbnet AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 12: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Nov 9 09:40:57 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 09:40:57 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10020 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 13: Active WAN is usbnet, default route is via usbnet, DNS is corresponding to usbnet, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Nov 9 09:12:16 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 09:12:16 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10020 0 0 eth4 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 14: Active WAN is eth4, default route is via eth4, DNS is corresponding to eth4, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 9 08:53:24 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 08:53:24 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh i ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 192.168.10.1 0.0.0.0 UG 20 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_12

| CaseID | WanRedundancy_Function_Priority_12 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN failover with eth3 as customized priority when usb modem is always-connect mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth3 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 2. Config eth3 as wan mode, result 2 in e3 mode wan 3. Config track wan group for each interface, result 3 track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 5 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable 4. Config usb modem as always-connect mode, result 4 usbm mode a usbmodem modem-id sierra_308 apn 3gnet 5. Config wan priority as customized sequence (eth3> usbnet> eth0), and check wan state of each interface, result 5 in e0 mode wan p 1000 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on eth3, wan failover to usbnet, check wan monitor state, result 9 track-wan use-for-wan-eth3 ip 9.9.9.9 10. Repeat Step 6, result 10 11. Repeat Step 7, result 11 12. Maintain ping process on client, result 8 13. Track fail on usbnet, wan failover to eth0, repeat Step 9~ 12, result 12 track-wan use-for-wan-usbnet0 ip 9.9.9.9 14. Restore track on usbnet, check wan monitor state and default route, result 13 no track-wan use-for-wan-usbnet0 ip 9.9.9.9 15. Restore track on eth3, check wan monitor state and default route, result 14 no track-wan use-for-wan-eth3 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 wan U - - - - - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 6 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 6 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.10 1 195M 1M -24(55) open none 00:02:15 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth3 is wan mode AH-8c66c0#sh in e3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth3 Eth3 4018:b18c:66c2 wan U - - - - - Result 3: Each interface is configured with a track wan group AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 wan U - - - - - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 6 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 6 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 5 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: USB modem is always-connect mode AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 5: wan priority is as customized, and eth3 is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1000 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 18 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Fri Nov 9 08:48:33 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth3 wifidx: 2 kifidx: 8 wanif_type: 0 priority: 30 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:21 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 9 08:53:24 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 08:53:24 2012 Result 6: Default route is via eth3, and there are non-default routes via eth0/ usbnet with corresponding priority AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U eth3 192.168.10.5 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.27.104.129 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh i ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 192.168.10.1 0.0.0.0 UG 30 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 Result 7: There is DNS server for eth3 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 8: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 9: usbnet is active wan AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: down Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Nov 9 09:12:16 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 09:12:16 2012 Result 10: Default route is via usbnet, and there are non-default routes via eth0/ eth3 with corresponding priority AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10030 0 0 eth3 Result 11: There is DNS server for usbnet AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 12: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: down Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Nov 9 09:40:57 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 09:40:57 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10030 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 13: Active WAN is usbnet, default route is via usbnet, DNS is corresponding to usbnet, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: down Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Nov 9 09:12:16 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 09:12:16 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10030 0 0 eth3 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 14: Active WAN is eth3, default route is via eth3, DNS is corresponding to eth3, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth3; State: up Interface: ppp0; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (eth3) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 9 08:53:24 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 08:53:24 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh i ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 192.168.10.1 0.0.0.0 UG 30 0 0 eth3 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_13

| CaseID | WanRedundancy_Function_Priority_13 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN failover with eth4 as customized priority when usb modem is always-connect mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth4 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 2. Config eth4 as wan mode, result 2 in e4 mode wan 3. Config track wan group for each interface, result 3 track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 5 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable 4. Config usb modem as always-connect mode, result 4 usbm mode a usbmodem modem-id sierra_308 apn 3gnet 5. Config wan priority as customized sequence (eth4> usbnet> eth0), and check wan state of each interface, result 5 in e0 mode wan p 1000 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on eth4, wan failover to usbnet, check wan monitor state, result 9 track-wan use-for-wan-eth4 ip 9.9.9.9 10. Repeat Step 6, result 10 11. Repeat Step 7, result 11 12. Maintain ping process on client, result 8 13. Track fail on usbnet, wan failover to eth0, repeat Step 9~ 12, result 12 track-wan use-for-wan-usbnet0 ip 9.9.9.9 14. Restore track on usbnet, check wan monitor state and default route, result 13 no track-wan use-for-wan-usbnet0 ip 9.9.9.9 15. Restore track on eth4, check wan monitor state and default route, result 14 no track-wan use-for-wan-eth4 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 6 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 6 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.10 1 195M 1M -24(55) open none 00:02:15 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth4 is wan mode AH-8c66c0#sh in e4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth4 Eth4 4018:b18c:66c2 wan U - - - - - Result 3: Each interface is configured with a track wan group AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 wan U - - - - - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 6 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 6 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 5 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: USB modem is always-connect mode AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 5: wan priority is as customized, and eth4 is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1000 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 18 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Fri Nov 9 08:48:33 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth4 wifidx: 2 kifidx: 8 wanif_type: 0 priority: 40 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:21 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 9 08:53:24 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 08:53:24 2012 Result 6: Default route is via eth4, and there are non-default routes via eth0/ usbnet with corresponding priority AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U eth4 192.168.10.5 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.27.104.129 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh i ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 192.168.10.1 0.0.0.0 UG 40 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 Result 7: There is DNS server for eth4 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 8: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 9: usbnet is active wan AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Nov 9 09:12:16 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 09:12:16 2012 Result 10: Default route is via usbnet, and there are non-default routes via eth0/ eth4 with corresponding priority AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10040 0 0 eth4 Result 11: There is DNS server for usbnet AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 12: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Nov 9 09:40:57 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 09:40:57 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10040 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 10600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 13: Active WAN is usbnet, default route is via usbnet, DNS is corresponding to usbnet, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: down Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Nov 9 09:12:16 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 09:12:16 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10040 0 0 eth4 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 14: Active WAN is eth4, default route is via eth4, DNS is corresponding to eth4, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth4; State: up Interface: ppp0; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (eth4) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 9 08:53:24 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 9 08:53:24 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh i ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 192.168.10.1 0.0.0.0 UG 40 0 0 eth4 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1000 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_14

| CaseID | WanRedundancy_Function_Priority_14 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 4 |  |  |
| Description | WAN failover with wifi0 as customized priority when usb modem is always-connect mode |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 has been configured as wan mode and connected with ISP ssid 5. wan monitor state through wifi0 is up |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config wifi0 as wan mode, result 2 3. Config track wan group for each interface, result 3 4. Config usb modem as always-connect mode, result 4 5. Config wan priority as customized sequence (wifi0> usbnet> eth0), and check wan state of each interface, result 5 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on wifi0, wan failover to usbnet, check wan monitor state, result 9 10. Repeat Step 6, result 10 11. Repeat Step 7, result 11 12. Maintain ping process on client, result 8 13. Track fail on usbnet, wan failover to eth0, repeat Step 9~ 12, result 12 14. Restore track on usbnet, check wan monitor state and default route, result 13 15. Restore track on wifi0, check wan monitor state and default route, result 14 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: wifi0 is wan mode Result 3: Each interface is configured with a track wan group Result 4: USB modem is always-connect mode Result 5: wan priority is as customized, and wifi0 is active wan Result 6: Default route is via wifi0, and there are non-default routes via eth0/ usbnet with corresponding priority Result 7: There is DNS server for wifi0 Result 8: Ping successfully on client Result 9: usbnet is active wan Result 10: Default route is via usbnet, and there are non-default routes via eth0/ wifi0 with corresponding priority Result 11: There is DNS server for usbnet Result 12: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client Result 13: Active WAN is usbnet, default route is via usbnet, DNS is corresponding to usbnet, and ping successfully on client Result 14: Active WAN is wifi0, default route is via wifi0, DNS is corresponding to wifi0, and ping successfully on client |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_15

| CaseID | WanRedundancy_Function_Priority_15 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN failover with eth1 as customized priority when usb modem is prim-wan mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth1 as wan mode, result 2 3. Config track wan group for each interface, result 3 4. Config usb modem as prim-wan mode, result 4 5. Config wan priority as customized sequence (eth0> eth1> usbnet), and check wan state of each interface, result 5 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on eth0, wan failover to eth1, check wan monitor state, result 9 10. Repeat Step 6, result 10 11. Repeat Step 7, result 11 12. Maintain ping process on client, result 8 13. Track fail on eth1, wan failover to usbnet, repeat Step 9~ 12, result 12 14. Restore track on eth1, check wan monitor state and default route, result 13 15. Restore track on eth0, check wan monitor state and default route, result 14 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: eth1 is wan mode Result 3: Each interface is configured with a track wan group Result 4: USB modem is prim-wan mode Result 5: wan priority is as customized, and eth0 is active wan Result 6: Default route is via eth0, and there are non-default routes via eth1/ usbnet with corresponding priority Result 7: There is DNS server for eth0 Result 8: Ping successfully on client Result 9: eth1 is active wan Result 10: Default route is via eth1, and there are non-default routes via eth0/ usbnet with corresponding priority Result 11: There is DNS server for eth1 Result 12: Active WAN is usbnet, default route is via usbnet, DNS is corresponding to usbnet, and ping successfully on client Result 13: Active WAN is eth1, default route is via eth1, DNS is corresponding to eth1, and ping successfully on client Result 14: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_16

| CaseID | WanRedundancy_Function_Priority_16 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN failback with higher priority interface when usb modem is prim-wan mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config eth1 as wan mode, result 2 3. Config track wan group for each interface, result 3 4. Config usb modem as prim-wan mode, result 4 5. Config wan priority as customized sequence (eth0> eth1> usbnet), and check wan state of each interface, result 5 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on eth0, wan failover to eth1, check wan monitor state, result 9 10. Repeat Step 6, result 10 11. Repeat Step 7, result 11 12. Maintain ping process on client, result 8 13. Track fail on eth1, wan failover to usbnet, repeat Step 9~ 12, result 12 14. Restore track on both eth0 and eth1, check wan monitor state and default route, result 13 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: eth1 is wan mode Result 3: Each interface is configured with a track wan group Result 4: USB modem is prim-wan mode Result 5: wan priority is as customized, and eth0 is active wan Result 6: Default route is via eth0, and there are non-default routes via eth1/ usbnet with corresponding priority Result 7: There is DNS server for eth0 Result 8: Ping successfully on client Result 9: eth1 is active wan Result 10: Default route is via eth1, and there are non-default routes via eth0/ usbnet with corresponding priority Result 11: There is DNS server for eth1 Result 12: Active WAN is usbnet, default route is via usbnet, DNS is corresponding to usbnet, and ping successfully on client Result 13: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_17

| CaseID | WanRedundancy_Function_Priority_17 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Priority of WAN when delta config usb modem mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 2. Config track wan group for each interface, result 2 in e1 mode wan track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable 3. Config usb modem as always-connect mode, result 3 usbm mode a usbmodem modem-id sierra_308 apn 3gnet 4. Check wan interface priority, result 4 5. Check wan monitor state of each interface, result 5 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Change usb modem to prim-wan mode, result 9 usbm mode p 10. Repeat Step 4~ 8, result 10 11. Change usb modem mode back to always-connect, result 3 usbm mode a usbmodem modem-id sierra_308 apn 3gnet 12. Repeat Step 4~ 8, result 11 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan D - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.10 1 195M 1M -24(55) open none 00:02:15 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: Each interface is configured with a track wan group AH-8c66c0#sh in e1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 wan U - - - - - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 3: USB modem is always-connect mode AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 4: WAN priority is as default sequence AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:25 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 20 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 3 lkup_time: Wed Nov 7 03:04:42 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 10 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 7 03:03:49 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:20 1970 Result 5: Active WAN is eth0 AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 Result 6: Default route is via eth0, and there are non-default routes via eth1/ usbnet with corresponding priority AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U eth1 192.168.10.5 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.27.104.129 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 7: There is DNS server for eth0 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 8: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 9: USB modem is prim-wan mode AH-8c66c0#sh usbmodem modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 10: usbnet is the interface with highest priority, Active wan is usbnet, default route is via usbnet, the current DNS is corresponding to usbnet, Ping successfully on client AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Mon Nov 12 11:02:17 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 18 wanif_type: 1 priority: 0 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Mon Nov 12 11:02:24 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: eth1 kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 10 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:28 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:22 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Mon Nov 12 11:02:35 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Mon Nov 12 11:02:35 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 0 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 11: eth0 is the interface with highest priority, Active wan is eth0, default route is via eth0, the current DNS is corresponding to eth0, Ping successfully on client AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:25 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 20 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 3 lkup_time: Wed Nov 7 03:04:42 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 10 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 7 03:03:49 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:20 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server from DHCP: Domain name suffix: QA Primary : 10.155.3.222 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_18

| CaseID | WanRedundancy_Function_Priority_18 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Priority of route when configure static route with "ip route net …" and customized metric |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Check default wan priority sequence, result 1 in e1 mode wan 2. Check default route in main route table, result 2 3. Config static route via eth1 with cli, no in e1 dhcp client in e1 ip 192.168.10.5/24 ip route net 0.0.0.0 0.0.0.0 gateway 192.168.10.1 metric 5 4. Check the static route in main route table, result 3 5. Customize wan priority of eth1 as below, result 4 in e1 mode wan p 100 6. Repeat Step 4, result 3 7. Restore wan priority config, check wan priority sequence, result 1 no in e1 mode wan p 8. Repeat Step 4, result 3 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: WAN priority sequence is eth0> eth1 AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:35 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: usbnet0 kifname: wifidx: 1 kifidx: 0 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:11 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: eth1 kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 10 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Tue Nov 13 01:45:00 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:21 1970 Result 2: Default route is via eth0 AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 Result 3: Static route has the customized priority, AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 5 0 0 eth1 Result 4: WAN priority sequence is updated, AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:35 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: usbnet0 kifname: wifidx: 1 kifidx: 0 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:11 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: eth1 kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 100 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Tue Nov 13 01:45:00 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:21 1970 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Priority_19

| CaseID | WanRedundancy_Function_Priority_19 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Priority of route when configure static route with "ip route default gateway …" and customized metric |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially |  |  |
| TestProcedure | 1. Check default wan priority sequence, result 1 in e1 mode wan 2. Check default route in main route table, result 2 3. Config static route via eth1 with cli, no in e1 dhcp client in e1 ip 192.168.10.5/24 ip route default gateway 192.168.10.1 metric 5 4. Check the static route in main route table, result 3 5. Customize wan priority of eth1 as below, result 4 in e1 mode wan p 100 6. Repeat Step 4, result 3 7. Restore wan priority config, check wan priority sequence, result 1 no in e1 mode wan p 8. Repeat Step 4, result 3 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: WAN priority sequence is eth0> eth1 AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:35 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: usbnet0 kifname: wifidx: 1 kifidx: 0 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:11 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: eth1 kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 10 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Tue Nov 13 01:45:00 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:21 1970 Result 2: Default route is via eth0 AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 Result 3: Static route has the customized priority, AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 5 0 0 eth1 Result 4: WAN priority sequence is updated, AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:35 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: usbnet0 kifname: wifidx: 1 kifidx: 0 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:11 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: eth1 kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 100 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Tue Nov 13 01:45:00 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:21 1970 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### 6.2.3. Track IP

#### WanRedundancy_Function_TrackIP_01

| CaseID | WanRedundancy_Function_TrackIP_01 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | N/A |
| Topology | Topo 3 |  |  |
| Description | No default route change when no internet access via default wan, no track wan group configured for eth0 and usb modem is always-connect mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. No track wan group configured/ enabled for eth0, result 1 no track-wan use-for-wan-eth0 enable 2. Config usb modem as always-connect mode, result 2 usbm mode a usbmodem modem-id sierra_308 apn 3gnet 3. Check wan interface priority, result 3 4. Check wan monitor state of each interface, result 4 5. Check default route in main route table, result 5 6. No internet access with eth0, check wan state and default route, result 6 Unplug the north bound cable of the switch that BR is connecting to 7. Restore internet access with eth0, check wan state and default route, result 7 Plug the north bound cable of the switch that BR is connecting to |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: No track wan group enabled for eth0 AH-8c66c0#sh ru ¦ i tr track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 2: USB modem is always-connect mode AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: WAN priority is as default sequence AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:28 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 18 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Tue Nov 13 06:16:55 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 Result 4: Active WAN is eth0 AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Tue Nov 13 06:34:22 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Nov 13 06:34:22 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.32.35.219 wan - 0000:0000:0000 U Result 5: Default route is via eth0, and there is non-default route via usbnet with corresponding priority, ping internet successfully AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#ping 8.8.8.8 PING 8.8.8.8 (8.8.8.8) 56(84) bytes of data. 64 bytes from 8.8.8.8: icmp_seq=1 ttl=45 time=33.4 ms 64 bytes from 8.8.8.8: icmp_seq=2 ttl=45 time=33.4 ms 64 bytes from 8.8.8.8: icmp_seq=3 ttl=45 time=35.2 ms 64 bytes from 8.8.8.8: icmp_seq=4 ttl=45 time=34.4 ms 64 bytes from 8.8.8.8: icmp_seq=5 ttl=45 time=35.0 ms --- 8.8.8.8 ping statistics --- 5 packets transmitted, 5 received, 0% packet loss, time 4004ms rtt min/avg/max/mdev = 33.440/34.344/35.273/0.782 ms Result 6: wan won't failover, default gw of eth0 is up but fail to access internet, and no change with default route, ping internet unsuccessfully AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: down Interface: ppp0; State: up [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Tue Nov 13 06:34:22 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Nov 13 06:34:22 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#ping 8.8.8.8 PING 8.8.8.8 (8.8.8.8) 56(84) bytes of data. --- 8.8.8.8 ping statistics --- 5 packets transmitted, 0 received, 100% packet loss, time 4002ms Result 7: Active WAN is eth0, and default route is via eth0, and there is non-default route via usbnet, ping internet successfully AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Tue Nov 13 06:34:22 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Nov 13 06:34:22 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#ping 8.8.8.8 PING 8.8.8.8 (8.8.8.8) 56(84) bytes of data. 64 bytes from 8.8.8.8: icmp_seq=1 ttl=45 time=33.4 ms 64 bytes from 8.8.8.8: icmp_seq=2 ttl=45 time=33.4 ms 64 bytes from 8.8.8.8: icmp_seq=3 ttl=45 time=35.2 ms 64 bytes from 8.8.8.8: icmp_seq=4 ttl=45 time=34.4 ms 64 bytes from 8.8.8.8: icmp_seq=5 ttl=45 time=35.0 ms --- 8.8.8.8 ping statistics --- 5 packets transmitted, 5 received, 0% packet loss, time 4004ms rtt min/avg/max/mdev = 33.440/34.344/35.273/0.782 ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_TrackIP_02

| CaseID | WanRedundancy_Function_TrackIP_02 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Default route change when no internet access via default wan, no track wan group configured for usbnet0 and usb modem is always-connect mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. No track wan group configured for usbnet0, result 1 no track-wan use-for-wan-usbnet0 enable 2. Config usb modem as always-connect mode, result 2 usbm mode a usbmodem modem-id sierra_308 apn 3gnet 3. Check wan interface priority, result 3 4. Check wan monitor state of each interface, result 4 5. Check default route in main route table, result 5 6. Track wan fail on eth0, check wan state and default route, result 6 track-wan use-for-wan-eth0 ip 9.9.9.9 7. Restore track wan oneth0, check wan state and default route, result 7 no track-wan use-for-wan-eth0 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: No track wan group configured for usbnet0 AH-8c66c0#sh ru ¦ i tr track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 Result 2: USB modem is always-connect mode AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: WAN priority is as default sequence AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:28 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 18 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Tue Nov 13 06:16:55 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 Result 4: Active WAN is eth0 AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Tue Nov 13 06:34:22 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Nov 13 06:34:22 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.32.35.219 wan - 0000:0000:0000 U Result 5: Default route is via eth0, and there is non-default route via usbnet with corresponding priority, ping internet successfully AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#ping 8.8.8.8 PING 8.8.8.8 (8.8.8.8) 56(84) bytes of data. 64 bytes from 8.8.8.8: icmp_seq=1 ttl=45 time=33.4 ms 64 bytes from 8.8.8.8: icmp_seq=2 ttl=45 time=33.4 ms 64 bytes from 8.8.8.8: icmp_seq=3 ttl=45 time=35.2 ms 64 bytes from 8.8.8.8: icmp_seq=4 ttl=45 time=34.4 ms 64 bytes from 8.8.8.8: icmp_seq=5 ttl=45 time=35.0 ms --- 8.8.8.8 ping statistics --- 5 packets transmitted, 5 received, 0% packet loss, time 4004ms rtt min/avg/max/mdev = 33.440/34.344/35.273/0.782 ms Result 6: wan failover to usbnet0, and there is update with default route correspondingly, ping internet successfully AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: FAILOVER prev_state_time: Tue Nov 13 06:59:34 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Tue Nov 13 06:59:34 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#ping 8.8.8.8 PING 8.8.8.8 (8.8.8.8) 56(84) bytes of data. 64 bytes from 8.8.8.8: icmp_seq=1 ttl=45 time=98.4 ms 64 bytes from 8.8.8.8: icmp_seq=2 ttl=45 time=97.0 ms 64 bytes from 8.8.8.8: icmp_seq=3 ttl=45 time=96.0 ms 64 bytes from 8.8.8.8: icmp_seq=4 ttl=45 time=104 ms 64 bytes from 8.8.8.8: icmp_seq=5 ttl=45 time=103 ms --- 8.8.8.8 ping statistics --- 5 packets transmitted, 5 received, 0% packet loss, time 4004ms rtt min/avg/max/mdev = 96.002/99.692/104.013/3.232 ms Result 7: Active WAN is eth0, and default route is via eth0, and there is non-default route via usbnet, ping internet successfully AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Tue Nov 13 06:34:22 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Nov 13 06:34:22 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#ping 8.8.8.8 PING 8.8.8.8 (8.8.8.8) 56(84) bytes of data. 64 bytes from 8.8.8.8: icmp_seq=1 ttl=45 time=33.4 ms 64 bytes from 8.8.8.8: icmp_seq=2 ttl=45 time=33.4 ms 64 bytes from 8.8.8.8: icmp_seq=3 ttl=45 time=35.2 ms 64 bytes from 8.8.8.8: icmp_seq=4 ttl=45 time=34.4 ms 64 bytes from 8.8.8.8: icmp_seq=5 ttl=45 time=35.0 ms --- 8.8.8.8 ping statistics --- 5 packets transmitted, 5 received, 0% packet loss, time 4004ms rtt min/avg/max/mdev = 33.440/34.344/35.273/0.782 ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_TrackIP_03

| CaseID | WanRedundancy_Function_TrackIP_03 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Default route change when there is no track wan group configured for eth0 and usb modem is prim-wan mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. No track wan group configured for eth0, result 1 no track-wan use-for-wan-eth0 enable 2. Config usb modem as prim-wan mode, result 2 usbm mode p 3. Check wan interface priority, result 3 4. Check wan monitor state of each interface, result 4 5. Check default route in main route table, result 5 6. Track wan fail on usbnet0, check wan state and default route, result 6 track-wan use-for-wan-usbnet0 ip 9.9.9.9 7. Restore track wan onusbnet0, check wan state and default route, result 7 no track-wan use-for-wan-usbnet0 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: No track wan group configured for eth0 AH-8c66c0#sh ru ¦ i tr track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 2: USB modem is prim-wan mode AH-8c66c0#sh usbmodem modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: primary-wan ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 3: WAN priority is as default sequence AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:28 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 18 wanif_type: 1 priority: 0 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Tue Nov 13 06:16:55 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 Result 4: Active WAN is usbnet AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Tue Nov 13 07:13:15 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Tue Nov 13 07:13:15 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.32.35.219 wan - 0000:0000:0000 U Result 5: Default route is via usbnet, and there is non-default route via eth0 with corresponding priority, ping internet successfully AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 0 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#ping 8.8.8.8 PING 8.8.8.8 (8.8.8.8) 56(84) bytes of data. 64 bytes from 8.8.8.8: icmp_seq=1 ttl=45 time=98.4 ms 64 bytes from 8.8.8.8: icmp_seq=2 ttl=45 time=97.0 ms 64 bytes from 8.8.8.8: icmp_seq=3 ttl=45 time=96.0 ms 64 bytes from 8.8.8.8: icmp_seq=4 ttl=45 time=104 ms 64 bytes from 8.8.8.8: icmp_seq=5 ttl=45 time=103 ms --- 8.8.8.8 ping statistics --- 5 packets transmitted, 5 received, 0% packet loss, time 4004ms rtt min/avg/max/mdev = 96.002/99.692/104.013/3.232 ms Result 6: wan failover to eth0, and there is update with default route correspondingly, ping internet successfully AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-usbnet0] WAN Monitor State: down Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: FAILOVER prev_state_time: Tue Nov 13 07:15:54 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Tue Nov 13 07:15:54 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 10000 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#ping 8.8.8.8 PING 8.8.8.8 (8.8.8.8) 56(84) bytes of data. 64 bytes from 8.8.8.8: icmp_seq=1 ttl=45 time=33.9 ms 64 bytes from 8.8.8.8: icmp_seq=2 ttl=45 time=34.5 ms 64 bytes from 8.8.8.8: icmp_seq=3 ttl=45 time=35.3 ms 64 bytes from 8.8.8.8: icmp_seq=4 ttl=45 time=33.6 ms 64 bytes from 8.8.8.8: icmp_seq=5 ttl=45 time=34.1 ms --- 8.8.8.8 ping statistics --- 5 packets transmitted, 5 received, 0% packet loss, time 4002ms rtt min/avg/max/mdev = 33.688/34.323/35.319/0.580 ms Result 7: Active WAN is usbnet, and default route is via usbnet, and there is non-default route via eth0, ping internet successfully AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Tue Nov 13 07:13:15 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Tue Nov 13 07:13:15 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 0 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#ping 8.8.8.8 PING 8.8.8.8 (8.8.8.8) 56(84) bytes of data. 64 bytes from 8.8.8.8: icmp_seq=1 ttl=45 time=98.4 ms 64 bytes from 8.8.8.8: icmp_seq=2 ttl=45 time=97.0 ms 64 bytes from 8.8.8.8: icmp_seq=3 ttl=45 time=96.0 ms 64 bytes from 8.8.8.8: icmp_seq=4 ttl=45 time=104 ms 64 bytes from 8.8.8.8: icmp_seq=5 ttl=45 time=103 ms --- 8.8.8.8 ping statistics --- 5 packets transmitted, 5 received, 0% packet loss, time 4004ms rtt min/avg/max/mdev = 96.002/99.692/104.013/3.232 ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_TrackIP_04

| CaseID | WanRedundancy_Function_TrackIP_04 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Track with "or" multi-dst-logic |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Config track wan group with “or” multi-dst-logic, result 1 2. Config track wan group with ip address, result 2 3. Config track wan group for each interface, result 3 4. Config usb modem as always-connect mode, result 4 usbm mode a usbmodem modem-id sierra_308 apn 3gnet 5. Check wan interface priority, result 5 6. Check wan monitor state of each interface, result 6 7. Check default route in main route table, result 7 8. Fail to track one ip address in track wan group with eth0, wan failover to usbnet, result 8 track-wan use-for-wan-eth0 ip 9.9.9.9 _deb brd wanmon 9. Repeat Step 7, result 9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: multi-dst-logic is “or” for track wan group There is no multi-dst-logic config in running config AH-8c66c0#sh ru ¦ i tr track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 2: track wan group is configured with ip address AH-8c66c0#sh ru ¦ i 8.8.8.8 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 ip 8.8.8.8 Result 3: track wan group is configured for each interface AH-8c66c0#sh ru ¦ i tr track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: USB modem is always-connect mode AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 5: priority of eth0 is the highest AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:28 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 18 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Tue Nov 13 06:16:55 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 Result 6: Active wan is eth0 AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Tue Nov 13 06:34:22 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Nov 13 06:34:22 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.32.35.219 wan - 0000:0000:0000 U Result 7: Default route is via eth0, and there is non-default route via usbnet with corresponding priority AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 8: wan failover to usbnet although once there is track failure with one object AH-8c66c0#sh lo b ¦ i fail 2012-11-13 07:39:45 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 2 targets tested, result: FAIL 2012-11-13 07:39:45 err ah_brd: WFOIFM: Failed to launch arping cmd [/bin/ping -q -c 1 -I eth0 -w 2 9.9.9.9 2>&1 > /tmp/brd_wan_ ifmon_ping_2199] - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Nov 13 07:39:00 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Nov 13 07:39:00 2012 Result 9: Default route is via usbnet, and there is non-default route via eth0 AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_TrackIP_05

| CaseID | WanRedundancy_Function_TrackIP_05 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Track with "and" multi-dst-logic |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Config track wan group with “and” multi-dst-logic, result 1 track-wan use-for-wan-eth0 multi-dst-logic and 2. Config track wan group with ip address, result 2 3. Config track wan group for each interface, result 3 4. Config usb modem as always-connect mode, result 4 usbm mode a usbmodem modem-id sierra_308 apn 3gnet 5. Check wan interface priority, result 5 6. Check wan monitor state of each interface, result 6 7. Check default route in main route table, result 7 8. Fail to track some ip address in track wan group with eth0, check wan state and default gateway, result 8 track-wan use-for-wan-eth0 ip 9.9.9.9 track-wan use-for-wan-eth0 ip 7.7.7.7 _deb brd wanmon 9. Remove the workable object in track wan group with eth0, check wan state and default gateway, result 9 no track-wan use-for-wan-eth0 ip 8.8.8.8 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: multi-dst-logic is “and” for track wan group AH-8c66c0#sh ru ¦ i use-for-wan-eth0 track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 ip 9.9.9.9 track-wan use-for-wan-eth0 multi-dst-logic and track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable Result 2: track wan group is configured with ip address AH-8c66c0#sh ru ¦ i 8.8.8.8 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 ip 8.8.8.8 Result 3: track wan group is configured for each interface AH-8c66c0#sh ru ¦ i tr track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 multi-dst-logic and track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: USB modem is always-connect mode AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 5: priority of eth0 is the highest AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:28 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 18 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Tue Nov 13 06:16:55 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 Result 6: Active wan is eth0 AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Tue Nov 13 06:34:22 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Nov 13 06:34:22 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.32.35.219 wan - 0000:0000:0000 U Result 7: Default route is via eth0, and there is non-default route via usbnet with corresponding priority AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 8: wan won't failover, and active wan is still eth0, default gateway is still via eth0 AH-8c66c0#sh lo b ¦ i "3 targets tested" 2012-11-13 07:56:29 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 3 targets tested, result: PASS - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Tue Nov 13 06:34:22 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Nov 13 06:34:22 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 9: wan failover to usbnet, and default gateway is via usbnet AH-8c66c0#sh lo b ¦ i fail 2012-11-13 07:58:36 debug ah_brd: [brd_wanmon]: WFOIFM: [use-for-wan-eth0]: Interface eth0: 2 targets tested, result: FAIL 2012-11-13 07:58:36 err ah_brd: WFOIFM: Failed to launch arping cmd [/bin/ping -q -c 1 -I eth0 -w 2 7.7.7.7 2>&1 > /tmp/brd_wan_ ifmon_ping_1073] 2012-11-13 07:58:34 err ah_brd: WFOIFM: Failed to launch arping cmd [/bin/ping -q -c 1 -I eth0 -w 2 7.7.7.7 2>&1 > /tmp/brd_wan_ ifmon_ping_3949] 2012-11-13 07:58:32 err ah_brd: WFOIFM: Failed to launch arping cmd [/bin/ping -q -c 1 -I eth0 -w 2 7.7.7.7 2>&1 > /tmp/brd_wan_ ifmon_ping_408] 2012-11-13 07:58:30 err ah_brd: WFOIFM: Failed to launch arping cmd [/bin/ping -q -c 1 -I eth0 -w 2 9.9.9.9 2>&1 > /tmp/brd_wan_ ifmon_ping_1247] 2012-11-13 07:58:28 err ah_brd: WFOIFM: Failed to launch arping cmd [/bin/ping -q -c 1 -I eth0 -w 2 9.9.9.9 2>&1 > /tmp/brd_wan_ ifmon_ping_1372] 2012-11-13 07:58:26 err ah_brd: WFOIFM: Failed to launch arping cmd [/bin/ping -q -c 1 -I eth0 -w 2 9.9.9.9 2>&1 > /tmp/brd_wan_ ifmon_ping_3263] - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: ppp0; State: up [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Nov 13 07:39:00 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Nov 13 07:39:00 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_TrackIP_06

| CaseID | WanRedundancy_Function_TrackIP_06 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Default track when no track-wan configured, as there is no IP/ gateway on the interface |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is the only wan interface 2. Shutdown eth0 or unplug eth0 cable 3. Set debug toggle, “_sh brd wanmon” 4. There is ssid configured on wifi interface |  |  |
| TestProcedure | 1. Check config of track-wan, result 1 2. Check IP/ gateway of eth0, result 2 3. Check default track process in log buffer, result 3 4. Check default track state, result 4 5. Get a client associated, result 5 6. On client, try to visit http, result 6 |  |  |
| ExpectResult | BR config, zhxu-8c66c0#sh ru …… interface eth0 mode wan interface eth0 mode wan priority 4 interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 hive zhxu-vhm01 interface usbnet0 mode wan priority 6 interface wifi0 ssid zhxu-br interface eth2 pse priority 1 hostname zhxu-8c66c0 interface mgt0 ip 172.18.252.129 255.255.255.128 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 172.18.252.129 interface mgt0 dhcp-server options dns1 172.18.252.129 interface mgt0 dhcp-server options custom 229 ip 172.18.252.129 interface mgt0 dhcp-server options custom 230 ip 172.18.252.129 interface mgt0 dhcp-server options custom 231 ip 172.18.252.129 interface mgt0 dhcp-server ip-pool 172.18.252.130 172.18.252.254 interface mgt0 dhcp-server enable interface mgt0 dns-server enable …… Result 1: There is no track-wan configured zhxu-8c66c0#sh ru ¦ i tra zhxu-8c66c0# Result 2: There is no IP/ gateway on eth0 zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 0.0.0.0 wan - 4018:b18c:66c0 D mgt0 172.18.252.129 - 1 4018:b18c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh in e0 dhcp cl HM=HiveManager; SLS=system log server; DHCP client: Enabled Timeout for applying static IP or default IP: 20 secs State: discovery; Get IP address 0.0.0.0 from the server 0.0.0.0; Relay agent 0.0.0.0 Lease time: 0 seconds; Duration time: 0 seconds Get options from server: Netmask (option number 1): 0.0.0.0 Router (option number 3): DNS server (option number 6): Log server (option number 7): DNS domain (option number 15): NTP server (option number 42): HM string (custom option 225): HM IP (custom option 226): SLS string (custom option 227): SLS IP (custom option 228): PPSK server IP (custom option 229): RADIUS server IP (custom option 230): RADIUS accounting server IP (custom option 231): - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.252.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Result 3: There is periodical probe to check the IP/ gateway of eth0 zhxu-8c66c0#sh lo b ¦ i wanmon …… 2013-04-10 00:55:25 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread wait last_test_time: 1365580525, waittime: 1365580531 2013-04-10 00:55:25 debug ah_brd: [brd_wanmon]: WFOIFM: default track-wan status change: 2 to 2 2013-04-10 00:55:25 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface (eth0 : wifidx=0) is not connected. 2013-04-10 00:55:25 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread begin to do test. 2013-04-10 00:55:19 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread wait last_test_time: 1365580519, waittime: 1365580525 2013-04-10 00:55:19 debug ah_brd: [brd_wanmon]: WFOIFM: default track-wan status change: 2 to 2 2013-04-10 00:55:19 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface (eth0 : wifidx=0) is not connected. 2013-04-10 00:55:19 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread begin to do test. 2013-04-10 00:55:13 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread wait last_test_time: 1365580513, waittime: 1365580519 2013-04-10 00:55:13 debug ah_brd: [brd_wanmon]: WFOIFM: default track-wan status change: 2 to 2 2013-04-10 00:55:13 debug ah_brd: [brd_wanmon]: WFOIFM: [Mark as unknown]: Skip test for DFT_GW_TRACK_IP: interface (eth0 : wifidx=0) is not connected. 2013-04-10 00:55:13 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread begin to do test. …… Result 4: Default track state is “down” zhxu-8c66c0#sh track- [Default Track IP] ------------------------------- Result: down Interface: eth0; State: unknown Result 5: Client get association zhxu-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; PMF = PMF state Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State PMF -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 7cd1:c37a:8eea 172.18.252.130 1 1M 1M -48(49) wpa2-psk aes ccmp 00:00:24 1 Yes 0 11ng disabled 20MHz No data collecting... No Result 6: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_TrackIP_07

| CaseID | WanRedundancy_Function_TrackIP_07 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Default track when no track-wan configured, as there is valid IP/ gateway on the interface |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is the only wan interface 2. Set debug toggle, “_sh brd wanmon” 3. There is ssid configured on wifi interface |  |  |
| TestProcedure | 1. Check config of track-wan, result 1 2. Check IP/ gateway of eth0, result 2 3. Check default track process in log buffer, result 3 4. Check default track state, result 4 5. Get a client associated, result 5 6. On client, try to visit http, result 6 |  |  |
| ExpectResult | BR config, zhxu-8c66c0#sh ru …… interface eth0 mode wan interface eth0 mode wan priority 4 interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 hive zhxu-vhm01 interface usbnet0 mode wan priority 6 interface wifi0 ssid zhxu-br interface eth2 pse priority 1 hostname zhxu-8c66c0 interface mgt0 ip 172.18.252.129 255.255.255.128 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 172.18.252.129 interface mgt0 dhcp-server options dns1 172.18.252.129 interface mgt0 dhcp-server options custom 229 ip 172.18.252.129 interface mgt0 dhcp-server options custom 230 ip 172.18.252.129 interface mgt0 dhcp-server options custom 231 ip 172.18.252.129 interface mgt0 dhcp-server ip-pool 172.18.252.130 172.18.252.254 interface mgt0 dhcp-server enable interface mgt0 dns-server enable …… Result 1: There is no track-wan configured zhxu-8c66c0#sh ru ¦ i tra zhxu-8c66c0# Result 2: There is valid IP/ gateway on eth0 zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.8 wan - 4018:b18c:66c0 U mgt0 172.18.252.129 - 1 4018:b18c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh in e0 dhcp cl HM=HiveManager; SLS=system log server; DHCP client: Enabled Timeout for applying static IP or default IP: 20 secs State: Network configuration received; Get IP address 10.155.30.8 from the server 10.155.3.220; Relay agent 10.155.30.254 Lease time: 172800 seconds; Duration time: 105 seconds Get options from server: Netmask (option number 1): 255.255.255.0 Router (option number 3): 10.155.30.254 DNS server (option number 6): 10.155.3.222 Log server (option number 7): DNS domain (option number 15): QA NTP server (option number 42): HM string (custom option 225): HM IP (custom option 226): SLS string (custom option 227): SLS IP (custom option 228): PPSK server IP (custom option 229): RADIUS server IP (custom option 230): RADIUS accounting server IP (custom option 231): - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.252.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 4 0 0 eth0 Result 3: There is periodical probe to check the IP/ gateway of eth0 zhxu-8c66c0#sh lo b ¦ i wanmon …… 2013-04-10 01:46:54 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread wait last_test_time: 1365583614, waittime: 1365583620 2013-04-10 01:46:54 debug ah_brd: [brd_wanmon]: WFOIFM: default track-wan status change: 1 to 1 2013-04-10 01:46:54 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa9b1efe for eth0 2013-04-10 01:46:54 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread begin to do test. 2013-04-10 01:46:48 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread wait last_test_time: 1365583608, waittime: 1365583614 2013-04-10 01:46:48 debug ah_brd: [brd_wanmon]: WFOIFM: default track-wan status change: 1 to 1 2013-04-10 01:46:48 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa9b1efe for eth0 2013-04-10 01:46:48 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread begin to do test. 2013-04-10 01:46:42 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread wait last_test_time: 1365583602, waittime: 1365583608 2013-04-10 01:46:42 debug ah_brd: [brd_wanmon]: WFOIFM: default track-wan status change: 1 to 1 2013-04-10 01:46:42 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa9b1efe for eth0 2013-04-10 01:46:42 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread begin to do test. …… Result 4: Default track state is “up” zhxu-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Result 5: Client get association zhxu-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; PMF = PMF state Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State PMF -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 7cd1:c37a:8eea 172.18.252.130 1 1M 1M -48(49) wpa2-psk aes ccmp 00:00:24 1 Yes 0 11ng disabled 20MHz No data collecting... No Result 6: Http visit successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_TrackIP_08

| CaseID | WanRedundancy_Function_TrackIP_08 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Default track when no track-wan configured, as there is invalid IP or gateway on the interface |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is the only wan interface 2. Set debug toggle, “_sh brd wanmon” 3. There is ssid configured on wifi interface |  |  |
| TestProcedure | 1. Check config of track-wan, result 1 2. Config static ip of eth0 with invalid gateway, result 2 interface eth0 ip 10.155.30.8/24 ip route default gateway 10.155.30.250 # 10.155.30.250 is inaccessible 3. Check default track process in log buffer, result 3 4. Check default track state, result 4 5. Get a client associated, result 5 6. On client, try to visit http, result 6 |  |  |
| ExpectResult | BR config, zhxu-8c66c0#sh ru …… interface eth0 mode wan interface eth0 mode wan priority 4 interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 hive zhxu-vhm01 interface usbnet0 mode wan priority 6 interface wifi0 ssid zhxu-br interface eth2 pse priority 1 hostname zhxu-8c66c0 interface mgt0 ip 172.18.252.129 255.255.255.128 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 172.18.252.129 interface mgt0 dhcp-server options dns1 172.18.252.129 interface mgt0 dhcp-server options custom 229 ip 172.18.252.129 interface mgt0 dhcp-server options custom 230 ip 172.18.252.129 interface mgt0 dhcp-server options custom 231 ip 172.18.252.129 interface mgt0 dhcp-server ip-pool 172.18.252.130 172.18.252.254 interface mgt0 dhcp-server enable interface mgt0 dns-server enable …… Result 1: There is no track-wan configured zhxu-8c66c0#sh ru ¦ i tra zhxu-8c66c0# Result 2: There is IP/ gateway on eth0 zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.8 wan - 4018:b18c:66c0 U mgt0 172.18.252.129 - 1 4018:b18c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.252.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.250 0.0.0.0 UG 4 0 0 eth0 Result 3: There is periodical probe to check the IP/ gateway of eth0 zhxu-8c66c0#sh lo b ¦ i wanmon …… 2013-04-10 01:46:54 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread wait last_test_time: 1365583614, waittime: 1365583620 2013-04-10 01:46:54 debug ah_brd: [brd_wanmon]: WFOIFM: default track-wan status change: 1 to 1 2013-04-10 01:46:54 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa9b1efe for eth0 2013-04-10 01:46:54 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread begin to do test. 2013-04-10 01:46:48 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread wait last_test_time: 1365583608, waittime: 1365583614 2013-04-10 01:46:48 debug ah_brd: [brd_wanmon]: WFOIFM: default track-wan status change: 1 to 1 2013-04-10 01:46:48 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa9b1efe for eth0 2013-04-10 01:46:48 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread begin to do test. 2013-04-10 01:46:42 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread wait last_test_time: 1365583602, waittime: 1365583608 2013-04-10 01:46:42 debug ah_brd: [brd_wanmon]: WFOIFM: default track-wan status change: 1 to 1 2013-04-10 01:46:42 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa9b1efe for eth0 2013-04-10 01:46:42 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread begin to do test. …… Result 4: Default track state is “up” zhxu-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Result 5: Client get association zhxu-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; PMF = PMF state Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State PMF -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 7cd1:c37a:8eea 172.18.252.130 1 1M 1M -48(49) wpa2-psk aes ccmp 00:00:24 1 Yes 0 11ng disabled 20MHz No data collecting... No Result 6: Http visit unsuccessfully, but redirection does not happen |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_TrackIP_09

| CaseID | WanRedundancy_Function_TrackIP_09 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Default track when track-wan with target as default gateway which is accessible |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is the only wan interface 2. Set debug toggle, “_sh brd wanmon” 3. There is ssid configured on wifi interface |  |  |
| TestProcedure | 1. Config track-wan for eth0 with target as default gateway, result 1 track-wan track-eth0 track-wan track-eth0 default-gateway track-wan track-eth0 interface eth0 track-wan track-eth0 enable 2. Check IP/ gateway of eth0, result 2 3. Check default track process in log buffer, result 3 4. Check default track state, result 4 5. Get a client associated, result 5 6. On client, try to visit http, result 6 |  |  |
| ExpectResult | BR config, zhxu-8c66c0#sh ru …… interface eth0 mode wan interface eth0 mode wan priority 4 interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 hive zhxu-vhm01 interface usbnet0 mode wan priority 6 interface wifi0 ssid zhxu-br interface eth2 pse priority 1 hostname zhxu-8c66c0 interface mgt0 ip 172.18.252.129 255.255.255.128 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 172.18.252.129 interface mgt0 dhcp-server options dns1 172.18.252.129 interface mgt0 dhcp-server options custom 229 ip 172.18.252.129 interface mgt0 dhcp-server options custom 230 ip 172.18.252.129 interface mgt0 dhcp-server options custom 231 ip 172.18.252.129 interface mgt0 dhcp-server ip-pool 172.18.252.130 172.18.252.254 interface mgt0 dhcp-server enable interface mgt0 dns-server enable …… Result 1: There is track-wan configured zhxu-8c66c0#sh ru ¦ i tra track-wan track-eth0 track-wan track-eth0 default-gateway track-wan track-eth0 interface eth0 track-wan track-eth0 enable Result 2: There is valid IP/ gateway on eth0 zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.114 wan - 4018:b18c:66c0 U mgt0 172.18.252.129 - 1 4018:b18c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh in e0 dhcp cl HM=HiveManager; SLS=system log server; DHCP client: Enabled Timeout for applying static IP or default IP: 20 secs State: Network configuration received; Get IP address 10.155.30.114 from the server 10.155.3.220; Relay agent 10.155.30.254 Lease time: 172800 seconds; Duration time: 310 seconds Get options from server: Netmask (option number 1): 255.255.255.0 Router (option number 3): 10.155.30.254 DNS server (option number 6): 10.155.3.222 Log server (option number 7): DNS domain (option number 15): QA NTP server (option number 42): HM string (custom option 225): HM IP (custom option 226): SLS string (custom option 227): SLS IP (custom option 228): PPSK server IP (custom option 229): RADIUS server IP (custom option 230): RADIUS accounting server IP (custom option 231): - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.252.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 4 0 0 eth0 Result 3: There is arping and icmp ping process on eth0 zhxu-8c66c0#sh lo b ¦ i DFT_GW_TRACK_IP 2013-04-10 04:00:24 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Arping OK: RTT avg = 13.034000 (out of 1 counts) group_name[DFT_GW_TRACK_IP] try_seq [1] 2013-04-10 04:00:24 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.254 2>&1 > /tmp/brd_wan_ifmon_arping_2265] executed group_name[DFT_GW_TRACK_IP] try_seq [1] 2013-04-10 04:00:23 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Launching arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.254 2>&1 > /tmp/brd_wan_ifmon_arping_2265] group_name[DFT_GW_TRACK_IP] try_seq [1] 2013-04-10 04:00:21 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Arping OK: RTT avg = 2.695000 (out of 1 counts) group_name[DFT_GW_TRACK_IP] try_seq [0] 2013-04-10 04:00:21 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.254 2>&1 > /tmp/brd_wan_ifmon_arping_3855] executed group_name[DFT_GW_TRACK_IP] try_seq [0] 2013-04-10 04:00:20 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Launching arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.254 2>&1 > /tmp/brd_wan_ifmon_arping_3855] group_name[DFT_GW_TRACK_IP] try_seq [0] 2013-04-10 04:00:20 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Going to test 1 targets for eth0 ... 2013-04-10 04:00:20 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa9b1efe for eth0 2013-04-10 04:00:20 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread begin to do test. 2013-04-10 04:00:20 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread wait last_test_time: 1365591611, waittime: 1365591617 2013-04-10 04:00:20 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth0: 1 targets tested, result: PASS ...... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh lo b ¦ i track-eth0 2013-04-10 04:08:42 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Launching arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.254 2>&1 > /tmp/brd_wan_ifmon_arping_2419] group_name[track-eth0] try_seq [1] 2013-04-10 04:08:40 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Arping OK: RTT avg = 2.245000 (out of 1 counts) group_name[track-eth0] try_seq [0] 2013-04-10 04:08:40 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.254 2>&1 > /tmp/brd_wan_ifmon_arping_1360] executed group_name[track-eth0] try_seq [0] 2013-04-10 04:08:39 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Launching arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.254 2>&1 > /tmp/brd_wan_ifmon_arping_1360] group_name[track-eth0] try_seq [0] 2013-04-10 04:08:39 debug ah_brd: [brd_wanmon]: WFOIFM: [track-eth0]: Going to test 1 targets for eth0 ... 2013-04-10 04:08:39 debug ah_brd: [brd_wanmon]: WFOIFM: [track-eth0]: Determined current gw IP as 0xa9b1efe for eth0 2013-04-10 04:08:39 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [track-eth0] thread begin to do test. 2013-04-10 04:08:39 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [track-eth0] thread wait last_test_time: 1365592110, waittime: 1365592116 2013-04-10 04:08:39 debug ah_brd: [brd_wanmon]: WFOIFM: [track-eth0]: Interface eth0: 1 targets tested, result: PASS …… Result 4: Default track state is “up” zhxu-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up [track-eth0] WAN Monitor State: up Interface: eth0 (eth0) Result 5: Client get association zhxu-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; PMF = PMF state Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State PMF -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 7cd1:c37a:8eea 172.18.252.130 1 1M 1M -48(49) wpa2-psk aes ccmp 00:00:24 1 Yes 0 11ng disabled 20MHz No data collecting... No Result 6: Http visit successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_TrackIP_10

| CaseID | WanRedundancy_Function_TrackIP_10 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Default track when track-wan with target as default gateway which is inaccessible |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is the only wan interface 2. Set debug toggle, “_sh brd wanmon” 3. There is ssid configured on wifi interface |  |  |
| TestProcedure | 1. Config track-wan for eth0 with target as default gateway, result 1 track-wan track-eth0 track-wan track-eth0 default-gateway track-wan track-eth0 interface eth0 track-wan track-eth0 enable 2. Config static ip and an inaccessible gateway on eth0, result 2 interface eth0 ip 10.155.30.8/24 ip route default gateway 10.155.30.250 # 10.155.30.250 is inaccessible 3. Check default track process in log buffer, result 3 4. Check default track state, result 4 5. Get a client associated, result 5 6. On client, try to visit http, result 6 |  |  |
| ExpectResult | BR config, zhxu-8c66c0#sh ru …… interface eth0 mode wan interface eth0 mode wan priority 4 interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 hive zhxu-vhm01 interface usbnet0 mode wan priority 6 interface wifi0 ssid zhxu-br interface eth2 pse priority 1 hostname zhxu-8c66c0 interface mgt0 ip 172.18.252.129 255.255.255.128 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 172.18.252.129 interface mgt0 dhcp-server options dns1 172.18.252.129 interface mgt0 dhcp-server options custom 229 ip 172.18.252.129 interface mgt0 dhcp-server options custom 230 ip 172.18.252.129 interface mgt0 dhcp-server options custom 231 ip 172.18.252.129 interface mgt0 dhcp-server ip-pool 172.18.252.130 172.18.252.254 interface mgt0 dhcp-server enable interface mgt0 dns-server enable …… Result 1: There is track-wan configured zhxu-8c66c0#sh ru ¦ i tra track-wan track-eth0 track-wan track-eth0 default-gateway track-wan track-eth0 interface eth0 track-wan track-eth0 enable Result 2: There is valid IP/ gateway on eth0 zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.8 wan - 4018:b18c:66c0 U mgt0 172.18.252.129 - 1 4018:b18c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.252.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.250 0.0.0.0 UG 4 0 0 eth0 Result 3: There is arping and icmp ping process on eth0 zhxu-8c66c0#sh lo b ¦ i DFT_GW_TRACK_IP 2013-04-10 04:17:44 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Launching ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_ping_3360] group_name[DFT_GW_TRACK_IP] try_seq [0] 2013-04-10 04:17:44 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Failed to launch arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_arping_2810] group_name[DFT_GW_TRACK_IP] try_seq [2] 2013-04-10 04:17:44 debug [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Failed to launch arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_arping_2810] group_name[DFT_GW_TRACK_IP] try_seq [2] retry_count [1] errstr [No such device] …… 2013-04-10 03:29:11 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Launching ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_ping_1344] group_name[DFT_GW_TRACK_IP] try_seq [0] 2013-04-10 03:29:13 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Failed to launch ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_ping_1344] group_name[DFT_GW_TRACK_IP] try_seq [0] 2013-04-10 03:29:13 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Launching ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_ping_1907] group_name[DFT_GW_TRACK_IP] try_seq [1] 2013-04-10 03:29:15 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Failed to launch ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_ping_1907] group_name[DFT_GW_TRACK_IP] try_seq [1] 2013-04-10 03:29:15 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Launching ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_ping_2801] group_name[DFT_GW_TRACK_IP] try_seq [2] 2013-04-10 03:29:17 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Failed to launch ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_ping_2801] group_name[DFT_GW_TRACK_IP] try_seq [2] 2013-04-10 03:29:17 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth0: 1 targets tested, result: FAIL 2013-04-10 03:29:17 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread wait last_test_time: 1365589736, waittime: 1365589742 2013-04-10 03:29:17 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread begin to do test. 2013-04-10 03:29:17 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa9b1efa for eth0 2013-04-10 03:29:17 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Going to test 1 targets for eth0 ... ...... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh lo b ¦ i track-eth0 ...... 2013-04-10 19:05:50 debug ah_brd: [brd_wanmon]: WFOIFM: [track-eth0]: Interface eth0: 1 targets tested, result: FAIL 2013-04-10 19:05:50 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Failed to launch ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_ping_2563] group_name[track-eth0] try_seq [2] 2013-04-10 19:05:48 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Launching ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_ping_2563] group_name[track-eth0] try_seq [2] …… 2013-04-10 19:05:40 debug [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Failed to launch arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_arping_1253] group_name[track-eth0] try_seq [2] retry_count [5] errstr [No such file or directory] 2013-04-10 19:05:39 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Launching arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_arping_1253] group_name[track-eth0] try_seq [2] 2013-04-10 19:05:29 debug ah_brd: [brd_wanmon]: WFOIFM: [track-eth0]: Going to test 1 targets for eth0 ... 2013-04-10 19:05:29 debug ah_brd: [brd_wanmon]: WFOIFM: [track-eth0]: Determined current gw IP as 0xa9b1efa for eth0 2013-04-10 19:05:29 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [track-eth0] thread begin to do test. 2013-04-10 19:05:29 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [track-eth0] thread wait last_test_time: 1365645908, waittime: 1365645914 ...... Result 4: Default track state is “down” zhxu-8c66c0#sh track- [Default Track IP] ------------------------------- Result: down Interface: eth0; State: down [track-eth0] WAN Monitor State: down Interface: eth0 (eth0) Result 5: Client get association zhxu-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; PMF = PMF state Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State PMF -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 7cd1:c37a:8eea 172.18.252.130 1 1M 1M -48(49) wpa2-psk aes ccmp 00:00:24 1 Yes 0 11ng disabled 20MHz No data collecting... No Result 6: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_TrackIP_11

| CaseID | WanRedundancy_Function_TrackIP_11 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Default track when track-wan with target as customized ip while gateway is accessible |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is the only wan interface 2. Set debug toggle, “_sh brd wanmon” 3. There is ssid configured on wifi interface |  |  |
| TestProcedure | 1. Config track-wan for eth0 with target as “8.8.8.8”, result 1 track-wan track-eth0 track-wan track-eth0 ip 8.8.8.8 track-wan track-eth0 interface eth0 track-wan track-eth0 enable 2. Check IP/ gateway of eth0, result 2 3. Check default track process in log buffer, result 3 4. Check default track state, result 4 5. Get a client associated, result 5 6. On client, try to visit http, result 6 |  |  |
| ExpectResult | BR config, zhxu-8c66c0#sh ru …… interface eth0 mode wan interface eth0 mode wan priority 4 interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 hive zhxu-vhm01 interface usbnet0 mode wan priority 6 interface wifi0 ssid zhxu-br interface eth2 pse priority 1 hostname zhxu-8c66c0 interface mgt0 ip 172.18.252.129 255.255.255.128 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 172.18.252.129 interface mgt0 dhcp-server options dns1 172.18.252.129 interface mgt0 dhcp-server options custom 229 ip 172.18.252.129 interface mgt0 dhcp-server options custom 230 ip 172.18.252.129 interface mgt0 dhcp-server options custom 231 ip 172.18.252.129 interface mgt0 dhcp-server ip-pool 172.18.252.130 172.18.252.254 interface mgt0 dhcp-server enable interface mgt0 dns-server enable …… Result 1: There is track-wan configured zhxu-8c66c0#sh ru ¦ i tra track-wan track-eth0 track-wan track-eth0 ip 8.8.8.8 track-wan track-eth0 interface eth0 track-wan track-eth0 enable Result 2: There is valid IP/ gateway on eth0 zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.151 wan - 4018:b18c:66c0 U mgt0 172.18.252.129 - 1 4018:b18c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh in e0 dhcp cl HM=HiveManager; SLS=system log server; DHCP client: Enabled Timeout for applying static IP or default IP: 20 secs State: Network configuration received; Get IP address 10.155.30.151 from the server 10.155.3.220; Relay agent 10.155.30.254 Lease time: 172800 seconds; Duration time: 310 seconds Get options from server: Netmask (option number 1): 255.255.255.0 Router (option number 3): 10.155.30.254 DNS server (option number 6): 10.155.3.222 Log server (option number 7): DNS domain (option number 15): QA NTP server (option number 42): HM string (custom option 225): HM IP (custom option 226): SLS string (custom option 227): SLS IP (custom option 228): PPSK server IP (custom option 229): RADIUS server IP (custom option 230): RADIUS accounting server IP (custom option 231): - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.252.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 4 0 0 eth0 Result 3: There is arping and icmp ping process on eth0 zhxu-8c66c0#sh lo b ¦ i DFT_GW_TRACK_IP ...... 2013-04-10 19:31:31 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread wait last_test_time: 1365647482, waittime: 1365647488 2013-04-10 19:31:31 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth0: 1 targets tested, result: PASS 2013-04-10 19:31:29 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Arping OK: RTT avg = 3.024000 (out of 1 counts) group_name[DFT_GW_TRACK_IP] try_seq [2] 2013-04-10 19:31:29 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.254 2>&1 > /tmp/brd_wan_ifmon_arping_868] executed group_name[DFT_GW_TRACK_IP] try_seq [2] 2013-04-10 19:31:28 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Launching arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.254 2>&1 > /tmp/brd_wan_ifmon_arping_868] group_name[DFT_GW_TRACK_IP] try_seq [2] …… 2013-04-10 19:31:22 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Going to test 1 targets for eth0 ... 2013-04-10 19:31:22 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa9b1efe for eth0 2013-04-10 19:31:22 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread begin to do test. 2013-04-10 19:31:22 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread wait last_test_time: 1365647473, waittime: 1365647479 ...... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh lo b ¦ i track-eth0 ...... 2013-04-10 19:37:27 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [track-eth0] thread wait last_test_time: 1365647841, waittime: 1365647847 2013-04-10 19:37:27 debug ah_brd: [brd_wanmon]: WFOIFM: [track-eth0]: Interface eth0: 1 targets tested, result: PASS 2013-04-10 19:37:25 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 8.8.8.8 2>&1 > /tmp/brd_wan_ifmon_ping_211] executed group_name[track-eth0] try_seq [2] 2013-04-10 19:37:25 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Launching ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 8.8.8.8 2>&1 > /tmp/brd_wan_ifmon_ping_211] group_name[track-eth0] try_seq [2] …… 2013-04-10 19:37:21 debug ah_brd: [brd_wanmon]: WFOIFM: [track-eth0]: Going to test 1 targets for eth0 ... 2013-04-10 19:37:21 debug ah_brd: [brd_wanmon]: WFOIFM: [track-eth0]: Determined current gw IP as 0xa9b1efe for eth0 2013-04-10 19:37:21 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [track-eth0] thread begin to do test. 2013-04-10 19:37:15 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [track-eth0] thread wait last_test_time: 1365647828, waittime: 1365647834 ...... Result 4: Default track state is “up” zhxu-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up [track-eth0] WAN Monitor State: up Interface: eth0 (eth0) Result 5: Client get association zhxu-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; PMF = PMF state Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State PMF -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 7cd1:c37a:8eea 172.18.252.130 1 1M 1M -48(49) wpa2-psk aes ccmp 00:00:24 1 Yes 0 11ng disabled 20MHz No data collecting... No Result 6: Http visit successfully |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_TrackIP_12

| CaseID | WanRedundancy_Function_TrackIP_12 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Default track when track-wan with target as customized ip while gateway is inaccessible |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is the only wan interface 2. Set debug toggle, “_sh brd wanmon” 3. There is ssid configured on wifi interface |  |  |
| TestProcedure | 1. Config track-wan for eth0 with target as default gateway, result 1 track-wan track-eth0 track-wan track-eth0 ip 8.8.8.8 track-wan track-eth0 interface eth0 track-wan track-eth0 enable 2. Config static ip and an inaccessible gateway on eth0, result 2 interface eth0 ip 10.155.30.8/24 ip route default gateway 10.155.30.250 # 10.155.30.250 is inaccessible 3. Check default track process in log buffer, result 3 4. Check default track state, result 4 5. Get a client associated, result 5 6. On client, try to visit http, result 6 |  |  |
| ExpectResult | BR config, zhxu-8c66c0#sh ru …… interface eth0 mode wan interface eth0 mode wan priority 4 interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth2 mode bridge-access interface eth2 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 hive zhxu-vhm01 interface usbnet0 mode wan priority 6 interface wifi0 ssid zhxu-br interface eth2 pse priority 1 hostname zhxu-8c66c0 interface mgt0 ip 172.18.252.129 255.255.255.128 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 172.18.252.129 interface mgt0 dhcp-server options dns1 172.18.252.129 interface mgt0 dhcp-server options custom 229 ip 172.18.252.129 interface mgt0 dhcp-server options custom 230 ip 172.18.252.129 interface mgt0 dhcp-server options custom 231 ip 172.18.252.129 interface mgt0 dhcp-server ip-pool 172.18.252.130 172.18.252.254 interface mgt0 dhcp-server enable interface mgt0 dns-server enable …… Result 1: There is track-wan configured zhxu-8c66c0#sh ru ¦ i tra track-wan track-eth0 track-wan track-eth0 ip 8.8.8.8 track-wan track-eth0 interface eth0 track-wan track-eth0 enable Result 2: There is valid IP/ gateway on eth0 zhxu-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.8 wan - 4018:b18c:66c0 U mgt0 172.18.252.129 - 1 4018:b18c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 172.18.252.128 0.0.0.0 255.255.255.128 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.250 0.0.0.0 UG 4 0 0 eth0 Result 3: There is arping and icmp ping process on eth0 zhxu-8c66c0#sh lo b ¦ i DFT_GW_TRACK_IP 2013-04-10 04:17:44 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Launching ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_ping_3360] group_name[DFT_GW_TRACK_IP] try_seq [0] 2013-04-10 04:17:44 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Failed to launch arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_arping_2810] group_name[DFT_GW_TRACK_IP] try_seq [2] 2013-04-10 04:17:44 debug [brd_wanmon]: WFOIFM: ah_wan_ifmon_arping: Failed to launch arping cmd [/usr/bin/arping -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_arping_2810] group_name[DFT_GW_TRACK_IP] try_seq [2] retry_count [1] errstr [No such device] …… 2013-04-10 03:29:11 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Launching ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_ping_1344] group_name[DFT_GW_TRACK_IP] try_seq [0] 2013-04-10 03:29:13 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Failed to launch ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_ping_1344] group_name[DFT_GW_TRACK_IP] try_seq [0] 2013-04-10 03:29:13 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Launching ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_ping_1907] group_name[DFT_GW_TRACK_IP] try_seq [1] 2013-04-10 03:29:15 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Failed to launch ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_ping_1907] group_name[DFT_GW_TRACK_IP] try_seq [1] 2013-04-10 03:29:15 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Launching ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_ping_2801] group_name[DFT_GW_TRACK_IP] try_seq [2] 2013-04-10 03:29:17 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Failed to launch ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 10.155.30.250 2>&1 > /tmp/brd_wan_ifmon_ping_2801] group_name[DFT_GW_TRACK_IP] try_seq [2] 2013-04-10 03:29:17 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Interface eth0: 1 targets tested, result: FAIL 2013-04-10 03:29:17 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread wait last_test_time: 1365589736, waittime: 1365589742 2013-04-10 03:29:17 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [DFT_GW_TRACK_IP] thread begin to do test. 2013-04-10 03:29:17 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Determined current gw IP as 0xa9b1efa for eth0 2013-04-10 03:29:17 debug ah_brd: [brd_wanmon]: WFOIFM: [DFT_GW_TRACK_IP]: Going to test 1 targets for eth0 ... ...... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-8c66c0#sh lo b ¦ i track-eth0 ...... 2013-04-10 19:46:56 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [track-eth0] thread wait last_test_time: 1365648410, waittime: 1365648416 2013-04-10 19:46:56 debug ah_brd: [brd_wanmon]: WFOIFM: [track-eth0]: Interface eth0: 1 targets tested, result: FAIL 2013-04-10 19:46:56 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Failed to launch ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 8.8.8.8 2>&1 > /tmp/brd_wan_ifmon_ping_1086] group_name[track-eth0] try_seq [2] 2013-04-10 19:46:54 debug ah_brd: [brd_wanmon]: WFOIFM: ah_wan_ifmon_ping: Launching ping cmd [/bin/ping -q -c 1 -I eth0 -w 2 8.8.8.8 2>&1 > /tmp/brd_wan_ifmon_ping_1086] group_name[track-eth0] try_seq [2] …… 2013-04-10 19:46:50 debug ah_brd: [brd_wanmon]: WFOIFM: [track-eth0]: Going to test 1 targets for eth0 ... 2013-04-10 19:46:50 debug ah_brd: [brd_wanmon]: WFOIFM: [track-eth0]: Determined current gw IP as 0xa9b1efa for eth0 2013-04-10 19:46:50 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [track-eth0] thread begin to do test. 2013-04-10 19:46:50 debug ah_brd: [brd_wanmon]: WFOIFM: track ip [track-eth0] thread wait last_test_time: 1365648404, waittime: 1365648410 ...... Result 4: Default track state is “down” zhxu-8c66c0#sh track- [Default Track IP] ------------------------------- Result: down Interface: eth0; State: down [track-eth0] WAN Monitor State: down Interface: eth0 (eth0) Result 5: Client get association zhxu-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; PMF = PMF state Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State PMF -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 7cd1:c37a:8eea 172.18.252.130 1 1M 1M -48(49) wpa2-psk aes ccmp 00:00:24 1 Yes 0 11ng disabled 20MHz No data collecting... No Result 6: Http visit will be redirected to HiveUI |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### 6.2.4. Failover

#### WanRedundancy_Function_Failover_01

| CaseID | WanRedundancy_Function_Failover_01 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 2 |  |  |
| Description | Failover to usbnet when usb is on-demand mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 2. Config eth1 as wan mode, result 2 in e1 mode wan 3. Config track wan group for each interface, result 3 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable 4. Config usb modem as on-demand mode, result 4 no usbm mode usbmodem modem-id sierra_308 apn 3gnet 5. Config wan priority as customized sequence (eth0> usbnet> eth1), and check wan state of each interface, result 5 in e1 mode wan p 1000 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on eth0, and track on eth1 still succeed, check wan monitor state, result 9 track-wan use-for-wan-eth0 ip 9.9.9.9 10. Repeat Step 6, result 10 11. Repeat Step 7, result 11 12. Maintain ping process on client, result 8 13. Restore track on eth0, check wan monitor state and default route, result 12 no track-wan use-for-wan-eth0 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 wan U - - - - - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.10 1 195M 1M -24(55) open none 00:02:15 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c66c0#sh in e1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 wan U - - - - - Result 3: Each interface is configured with a track wan group AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 wan U - - - - - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: USB modem is on-demand mode AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: on-demand ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 5: wan priority is as customized, and eth0 is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:25 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 20 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 3 lkup_time: Wed Nov 7 03:04:42 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 1000 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 7 03:03:49 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:20 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 Result 6: Default route is via eth0, and there are non-default routes via eth1/ usbnet with corresponding priority AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U eth1 192.168.10.5 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 1000 0 0 eth1 Result 7: There is DNS server for eth0 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 8: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 9: usb modem start to connect mobile network, and wan failover to usbnet after usb modem is connected AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:59:40 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:59:40 2012 Result 10: Default route is via usbnet, and there are non-default routes via eth0/ eth1 with corresponding priority AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.11.12.13 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 192.168.10.1 0.0.0.0 UG 1000 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 Result 11: There is DNS server for usbnet !!! Still with issue, bug 20545 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 12: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 1000 0 0 eth1 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Failover_02

| CaseID | WanRedundancy_Function_Failover_02 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Failover behavior when fail track with both 1st and 2nd priority interfaces |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially 3. Disable usb modem no usbmodem enable |  |  |
| TestProcedure | 1. Config eth1 as wan mode, result 1 in e1 mode wan 2. Config track wan group for each interface, result 2 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable 3. Track fail on eth1, check wan state of each interface, result 3 track-wan use-for-wan-eth1 ip 9.9.9.9 4. Check default route in main route table, result 4 5. Track fail on eth0, as well as on eth1, check wan monitor state, result 5 track-wan use-for-wan-eth0 ip 9.9.9.9 6. Repeat Step 4, result 4 7. Restore track on eth1, check wan state and default gateway, result 6 no track-wan use-for-wan-eth1 ip 9.9.9.9 8. Restore track on eth0, check wan state and default gateway, result 7 no track-wan use-for-wan-eth0 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable no usbmodem enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: eth1 is wan mode AH-8c66c0#sh in e1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 wan U - - - - - Result 2: Each interface is configured with a track wan group AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 3: wan priority is as default, and eth0 is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:25 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 20 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 3 lkup_time: Wed Nov 7 03:04:42 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 10 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 7 03:03:49 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:20 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 Result 4: Default route is via eth0, and there is non-default routes via eth1 with corresponding priority AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 100100 0 eth1 Result 5: wan won't failover to eth1 AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: FAILOVER prev_state_time: Tue Nov 13 09:31:02 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Tue Nov 13 09:31:02 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 Result 6: wan failover to eth1, and default route is via eth1 AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Nov 13 09:34:04 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Nov 13 09:34:04 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 Result 7: wan failback to eth0, and default route is via eth0 AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Failover_03

| CaseID | WanRedundancy_Function_Failover_03 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Failover behavior when fail track with 1st priority interface, and 2nd priority interface is down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is disconnected with Switch initially 3. Disable usb modem no usbmodem enable |  |  |
| TestProcedure | 1. Config eth1 as wan mode, result 1 in e1 mode wan 2. Config track wan group for each interface, result 2 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable 3. Track wan group works well on eth0 initially, check wan state of each interface, result 3 4. Shutdown eth1, check default route in main route table, result 4 in e1sh 5. Track fail on eth0, check wan monitor state, result 5 track-wan use-for-wan-eth0 ip 9.9.9.9 6. Repeat Step 4, result 4 7. Restore track on eth0, check wan state and default gateway, result 6 no track-wan use-for-wan-eth0 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable no usbmodem enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: eth1 is wan mode AH-8c66c0#sh in e1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 wan U - - - - - Result 2: Each interface is configured with a track wan group AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 3: wan priority is as default, and eth0 is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:25 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 20 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 3 lkup_time: Wed Nov 7 03:04:42 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 10 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 7 03:03:49 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:20 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 Result 4: Default route is via eth0 AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 Result 5: wan state is “NOWAN” and won't failover to eth1 AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: unknown [use-for-wan-eth1] WAN Monitor State: unknown Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: NOWAN prev_state_time: Tue Nov 13 09:49:52 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Tue Nov 13 09:49:52 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 Result 6: wan failback to eth0, and default route is via eth0 AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: unknown [use-for-wan-eth1] WAN Monitor State: unknown Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Failover_04

| CaseID | WanRedundancy_Function_Failover_04 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Failover behavior when 1st priority interface is down, and fail track with 2nd priority interface |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is disconnected with Switch initially 2. Eth1 is connected with Switch initially 3. Disable usb modem no usbmodem enable |  |  |
| TestProcedure | 1. Config eth1 as wan mode, result 1 in e1 mode wan 2. Config track wan group for each interface, result 2 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable 3. Track wan group works well on eth1 initially, check wan state of each interface, result 3 4. Check default route in main route table, result 4 5. Track fail on eth1, check wan monitor state, result 5 track-wan use-for-wan-eth1 ip 9.9.9.9 6. Shutdown eth0, check wan state and default gateway, result 6 in e0sh 7. Restore track on eth1, check wan state and default gateway, result 7 no track-wan use-for-wan-eth1 ip 9.9.9.9 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable no usbmodem enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: eth1 is wan mode AH-8c66c0#sh in e1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 wan U - - - - - Result 2: Each interface is configured with a track wan group AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 3: wan priority is as default, and eth1 is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:25 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 20 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 3 lkup_time: Wed Nov 7 03:04:42 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 10 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 7 03:03:49 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:20 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 Result 4: Default route is via eth0 AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 Result 5: wan monitor state of eth1 is down AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 Result 6: wan still failover to eth1 AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 0.0.0.0 wan - 4018:b18c:66c0 D eth1 192.168.10.5 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Nov 13 10:06:49 2012 last_event: BACKUP_WAN_DOWN last_evt_time: Tue Nov 13 10:06:49 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 Result 7: active wan is eth1, and default route is via eth1 AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Tue Nov 13 10:08:50 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Tue Nov 13 10:08:50 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_Failover_05

| CaseID | WanRedundancy_Function_Failover_05 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | Failover behavior when both 1st and 2nd priority interfaces are down |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is disconnected with Switch initially 3. Disable usb modem no usbmodem enable |  |  |
| TestProcedure | 1. Config eth1 as wan mode, result 1 in e1 mode wan 2. Config track wan group for each interface, result 2 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable 3. Shutdown eth1,check wan state of each interface, result 3 in e1sh 4. Check default route in main route table, result 4 5. Shutdown eth0, check wan monitor state, result 5 in e0sh 6. Repeat Step 4, result 6 7. Restore eth0, check wan state and default gateway, result 7 no in e0sh |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable no usbmodem enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: eth1 is wan mode AH-8c66c0#sh in e1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 wan U - - - - - Result 2: Each interface is configured with a track wan group AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 3: wan priority is as default, and eth0 is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:25 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 20 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 3 lkup_time: Wed Nov 7 03:04:42 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 10 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 7 03:03:49 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:20 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: unknown [use-for-wan-eth1] WAN Monitor State: unknown Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: BACKUP_WAN_DOWN last_evt_time: Wed Nov 7 03:09:56 2012 Result 4: Default route is via eth0, and there is no route via eth1 AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.118 wan - 4018:b18c:66c0 U eth1 0.0.0.0 wan - 4018:b18c:66c2 D mgt0 192.168.85.1 - 1 4018:b18c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 Result 5: wan state is “NOWAN” and won't failover to eth1 AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: down Interface: eth0; State: unknown Interface: eth1; State: unknown [use-for-wan-eth1] WAN Monitor State: unknown Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: NOWAN prev_state: NOWAN prev_state_time: Wed Nov 14 01:17:20 2012 last_event: BACKUP_WAN_DOWN last_evt_time: Wed Nov 14 01:17:20 2012 Result 6: No default route either via eth0 or eth1 AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 0.0.0.0 wan - 4018:b18c:66c0 D eth1 0.0.0.0 wan - 4018:b18c:66c2 D mgt0 192.168.85.1 - 1 4018:b18c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo Result 7: active wan is eth0, and default route is via eth0 AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: unknown [use-for-wan-eth1] WAN Monitor State: unknown Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: BACKUP_WAN_DOWN last_evt_time: Wed Nov 7 03:09:56 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.118 wan - 4018:b18c:66c0 U eth1 0.0.0.0 wan - 4018:b18c:66c2 D mgt0 192.168.85.1 - 1 4018:b18c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.155.3.222 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### 6.2.5. DNS

#### WanRedundancy_Function_DNS_01

| CaseID | WanRedundancy_Function_DNS_01 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | N/A |
| Topology | Topo 3 |  |  |
| Description | When the DNS of active wan is down, will query the DNS corresponding to other wan |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config track wan group for each interface, result 2 3. Config usb modem as always-connect mode, result 3 4. Check wan monitor state of each interface, result 4 5. Check default route in main route table, result 5 6. Check the active DNS config, result 6 7. Run http visit on client, result 7 8. Make DNS of eth0 unavailable, result 8 9. Repeat Step 7, result 9 10. Restore DNS of eth0, result 10 11. Repeat Step 7, result 7 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: Each interface is configured with a track wan group Result 3: USB modem is always-connect mode Result 4: eth0 is active wan Result 5: Default route is via eth0, and there are non-default routes via eth1/ usbnet with corresponding priority Result 6: There is DNS server for eth0, and it is available Result 7: visit http successfully on client via DNS of eth0 Result 8: DNS of eth0 is unavailable Result 9: visit http successfully on client via DNS of usbnet Result 10: DNS of eth0 is available |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### 6.2.6. PPPoE

#### WanRedundancy_Function_PPPoE_01

| CaseID | WanRedundancy_Function_PPPoE_01 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 5 |  |  |
| Description | PPPoE works well on eth0 |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. PPPoE server is available |  |  |
| TestProcedure | 1. Config eth0 as WAN and highest priority, result 1 in e0 mode wan 2. Config eth0 as PPPoE enabled, result 2 interface eth0 pppoe username wcai password aerohive interface eth0 pppoe enable 3. Config track wan group for eth0, result 3 4. Check wan monitor state of eth0, result 4 5. Check default route in main route table, result 5 6. Get a client associated to BR with open auth, result 6 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 7. Run ping internet process on client to check its accessibility, result 7 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: Eth0 is WAN mode, with highest priority AH-8c66c0#sh in e0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth0 Eth0 4018:b18c:66c2 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 18 wanif_type: 0 priority: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 2 lkup_time: Wed Nov 14 06:24:47 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: usbnet0 kifname: wifidx: 1 kifidx: 0 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:11 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 Result 2: PPPoE is configured on eth0 AH-8c66c0#sh in e0 pp PPPoE status: enabled Username: wcai Password: *** Authentication: ANY Connection status: connected Local IP: 10.5.16.44 Remote IP: 10.5.16.1 DNS server: 10.155.3.250 DNS server: 10.155.32.190 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: ifname: ppp1 kifname: ppp1 wifidx: 0 kifidx: 18 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 2 lkup_time: Wed Nov 14 06:24:47 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: usbnet0 kifname: wifidx: 1 kifidx: 0 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 0 lkup_time: Thu Jan 1 00:00:11 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 Result 3: Eth0 is configured with track wan group AH-8c66c0#sh ru ¦ i tr track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: WAN monitor state of eth0 is Up AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: ppp1; State: up [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (ppp1) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 14 07:13:26 2012 last_event: BACKUP_WAN_DOWN last_evt_time: Wed Nov 14 07:13:26 2012 Result 5: Default route is via eth0 AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 0.0.0.0 wan - 4018:b18c:66c0 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp1 10.5.16.49 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.5.16.1 0.0.0.0 255.255.255.255 UH 0 0 0 ppp1 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.5.16.1 0.0.0.0 UG 1 0 0 ppp1 Result 6: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan D - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.10 1 195M 1M -24(55) open none 00:02:15 1 Yes 0 11ng disabled 20MHz No data collecting... Result 7: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_PPPoE_02

| CaseID | WanRedundancy_Function_PPPoE_02 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 5 |  |  |
| Description | PPPoE works well on eth1 |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth1 is connected with Switch initially 2. PPPoE server is available |  |  |
| TestProcedure | 1. Config eth1 as WAN as default priority, result 1 in e1 mode wan 2. Config eth1 as PPPoE enabled, result 2 interface eth1 pppoe username wcai password aerohive interface eth1 pppoe enable 3. Config track wan group for eth1, result 3 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable 4. Check wan monitor state of eth1, result 4 5. Check default route in main route table, result 5 6. Get a client associated to BR with open auth, result 6 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 7. Run ping internet process on client to check its accessibility, result 7 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: eth1 is WAN mode, with highest priority AH-8c66c0#sh in e1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: ifname: eth1 kifname: eth1 wifidx: 0 kifidx: 25 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 8 lkup_time: Wed Nov 14 07:26:52 2012 lkdn_count: 5 lkdn_time: Wed Nov 14 07:19:53 2012 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 26 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 2 lkup_time: Wed Nov 14 07:27:06 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: eth1 kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 10 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 14 07:19:51 2012 lkdn_count: 0 lkdn_time: Wed Nov 14 07:18:02 2012 Result 2: PPPoE is configured on eth1 AH-8c66c0#sh in e1 pp PPPoE status: enabled Username: wcai Password: *** Authentication: ANY Connection status: connected Local IP: 10.5.16.50 Remote IP: 10.5.16.1 DNS server: 10.155.3.250 DNS server: 10.155.32.190 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: ifname: ppp1 kifname: ppp1 wifidx: 0 kifidx: 25 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 8 lkup_time: Wed Nov 14 07:26:52 2012 lkdn_count: 5 lkdn_time: Wed Nov 14 07:19:53 2012 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 26 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 2 lkup_time: Wed Nov 14 07:27:06 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: eth1 kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 10 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 14 07:19:51 2012 lkdn_count: 0 lkdn_time: Wed Nov 14 07:18:02 2012 Result 3: eth1 is configured with track wan group AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: WAN monitor state of eth1 is Up AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth1; State: unknown Interface: ppp1; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (ppp1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 14 07:38:20 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 14 07:39:37 2012 Result 5: Default route is via eth1 AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 0.0.0.0 wan - 4018:b18c:66c0 D eth1 0.0.0.0 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp1 10.5.16.53 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.5.16.1 0.0.0.0 255.255.255.255 UH 0 0 0 ppp1 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.5.16.1 0.0.0.0 UG 1 0 0 ppp1 Result 6: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan D - - - - - Eth1 4018:b18c:66c2 wan D - - - hive0 - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.10 1 195M 1M -24(55) open none 00:02:15 1 Yes 0 11ng disabled 20MHz No data collecting... Result 7: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_PPPoE_03

| CaseID | WanRedundancy_Function_PPPoE_03 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 5 |  |  |
| Description | PPPoE works well on eth4 |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth4 is connected with Switch initially 2. PPPoE server is available |  |  |
| TestProcedure | 1. Config eth4 as WAN as default priority, result 1 in e2 mode wan 2. Config eth4 as PPPoE enabled, result 2 interface eth4 pppoe username wcai password aerohive interface eth4 pppoe enable 3. Config track wan group for eth4, result 3 track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 5 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable 4. Check wan monitor state of eth4, result 4 5. Check default route in main route table, result 5 6. Get a client associated to BR with open auth, result 6 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 7. Run ping internet process on client to check its accessibility, result 7 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: eth4 is WAN mode, with highest priority AH-8c66c0#sh in e2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth4 Eth4 4018:b18c:66c2 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: ifname: eth4 kifname: eth4 wifidx: 0 kifidx: 25 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 8 lkup_time: Wed Nov 14 07:26:52 2012 lkdn_count: 5 lkdn_time: Wed Nov 14 07:19:53 2012 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 26 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 2 lkup_time: Wed Nov 14 07:27:06 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: eth4 kifname: eth4 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 20 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 14 07:19:51 2012 lkdn_count: 0 lkdn_time: Wed Nov 14 07:18:02 2012 Result 2: PPPoE is configured on eth4 AH-8c66c0#sh in e2 pp PPPoE status: enabled Username: wcai Password: *** Authentication: ANY Connection status: connected Local IP: 10.5.16.50 Remote IP: 10.5.16.1 DNS server: 10.155.3.250 DNS server: 10.155.32.190 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: ifname: ppp1 kifname: ppp1 wifidx: 0 kifidx: 25 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 8 lkup_time: Wed Nov 14 07:26:52 2012 lkdn_count: 5 lkdn_time: Wed Nov 14 07:19:53 2012 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 26 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 2 lkup_time: Wed Nov 14 07:27:06 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: eth4 kifname: eth4 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 20 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 14 07:19:51 2012 lkdn_count: 0 lkdn_time: Wed Nov 14 07:18:02 2012 Result 3: eth4 is configured with track wan group AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 5 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: WAN monitor state of eth4 is Up AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: unknown Interface: ppp1; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (ppp1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 14 07:38:20 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 14 07:39:37 2012 Result 5: Default route is via eth4 AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 0.0.0.0 wan - 4018:b18c:66c0 D eth4 0.0.0.0 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp1 10.5.16.53 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.5.16.1 0.0.0.0 255.255.255.255 UH 0 0 0 ppp1 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.5.16.1 0.0.0.0 UG 1 0 0 ppp1 Result 6: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan D - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 wan D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.10 1 195M 1M -24(55) open none 00:02:15 1 Yes 0 11ng disabled 20MHz No data collecting... Result 7: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_PPPoE_04

| CaseID | WanRedundancy_Function_PPPoE_04 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 5 |  |  |
| Description | PPPoE works well on eth3 |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth3 is connected with Switch initially 2. PPPoE server is available |  |  |
| TestProcedure | 1. Config eth3 as WAN as default priority, result 1 in e3 mode wan 2. Config eth3 as PPPoE enabled, result 2 interface eth3 pppoe username wcai password aerohive interface eth3 pppoe enable 3. Config track wan group for eth3, result 3 track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 5 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable 4. Check wan monitor state of eth3, result 4 5. Check default route in main route table, result 5 6. Get a client associated to BR with open auth, result 6 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 7. Run ping internet process on client to check its accessibility, result 7 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: eth3 is WAN mode, with highest priority AH-8c66c0#sh in e3 ¦ i mode Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth3 Eth3 4018:b18c:66c2 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: ifname: eth3 kifname: eth3 wifidx: 0 kifidx: 25 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 8 lkup_time: Wed Nov 14 07:26:52 2012 lkdn_count: 5 lkdn_time: Wed Nov 14 07:19:53 2012 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 26 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 2 lkup_time: Wed Nov 14 07:27:06 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: eth3 kifname: eth3 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 30 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 14 07:19:51 2012 lkdn_count: 0 lkdn_time: Wed Nov 14 07:18:02 2012 Result 2: PPPoE is configured on eth3 AH-8c66c0#sh in e3 pp PPPoE status: enabled Username: wcai Password: *** Authentication: ANY Connection status: connected Local IP: 10.5.16.50 Remote IP: 10.5.16.1 DNS server: 10.155.3.250 DNS server: 10.155.32.190 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: ifname: ppp1 kifname: ppp1 wifidx: 0 kifidx: 25 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 8 lkup_time: Wed Nov 14 07:26:52 2012 lkdn_count: 5 lkdn_time: Wed Nov 14 07:19:53 2012 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 26 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 2 lkup_time: Wed Nov 14 07:27:06 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: eth3 kifname: eth3 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 30 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 14 07:19:51 2012 lkdn_count: 0 lkdn_time: Wed Nov 14 07:18:02 2012 Result 3: eth3 is configured with track wan group AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth3 track-wan use-for-wan-eth3 ip 8.8.8.8 track-wan use-for-wan-eth3 interval 5 track-wan use-for-wan-eth3 interface eth3 track-wan use-for-wan-eth3 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: WAN monitor state of eth3 is Up AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth3; State: unknown Interface: ppp1; State: up [use-for-wan-eth3] WAN Monitor State: up Interface: eth3 (ppp1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 14 07:38:20 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 14 07:39:37 2012 Result 5: Default route is via eth3 AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 0.0.0.0 wan - 4018:b18c:66c0 D eth3 0.0.0.0 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp1 10.5.16.53 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.5.16.1 0.0.0.0 255.255.255.255 UH 0 0 0 ppp1 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.5.16.1 0.0.0.0 UG 1 0 0 ppp1 Result 6: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan D - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 wan D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.10 1 195M 1M -24(55) open none 00:02:15 1 Yes 0 11ng disabled 20MHz No data collecting... Result 7: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_PPPoE_05

| CaseID | WanRedundancy_Function_PPPoE_05 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 5 |  |  |
| Description | PPPoE works well on eth4 |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition | 1. Eth4 is connected with Switch initially 2. PPPoE server is available |  |  |
| TestProcedure | 1. Config eth4 as WAN as defaultpriority, result 1 in e4 mode wan 2. Config eth4 as PPPoE enabled, result 2 interface eth4 pppoe username wcai password aerohive interface eth4 pppoe enable 3. Config track wan group for eth4, result 3 track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 5 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable 4. Check wan monitor state of eth4, result 4 5. Check default route in main route table, result 5 6. Get a client associated to BR with open auth, result 6 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 7. Run ping internet process on client to check its accessibility, result 7 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: eth4 is WAN mode, with highest priority AH-8c66c0#sh in e4 ¦ i mode Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth4 Eth4 4018:b18c:66c2 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: ifname: eth4 kifname: eth4 wifidx: 0 kifidx: 25 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 8 lkup_time: Wed Nov 14 07:26:52 2012 lkdn_count: 5 lkdn_time: Wed Nov 14 07:19:53 2012 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 26 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 2 lkup_time: Wed Nov 14 07:27:06 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: eth4 kifname: eth4 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 40 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 14 07:19:51 2012 lkdn_count: 0 lkdn_time: Wed Nov 14 07:18:02 2012 Result 2: PPPoE is configured on eth4 AH-8c66c0#sh in e4 pp PPPoE status: enabled Username: wcai Password: *** Authentication: ANY Connection status: connected Local IP: 10.5.16.50 Remote IP: 10.5.16.1 DNS server: 10.155.3.250 DNS server: 10.155.32.190 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: ifname: ppp1 kifname: ppp1 wifidx: 0 kifidx: 25 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 8 lkup_time: Wed Nov 14 07:26:52 2012 lkdn_count: 5 lkdn_time: Wed Nov 14 07:19:53 2012 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 26 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 2 lkup_time: Wed Nov 14 07:27:06 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 ifname: eth4 kifname: eth4 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 40 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 14 07:19:51 2012 lkdn_count: 0 lkdn_time: Wed Nov 14 07:18:02 2012 Result 3: eth4 is configured with track wan group AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth4 track-wan use-for-wan-eth4 ip 8.8.8.8 track-wan use-for-wan-eth4 interval 5 track-wan use-for-wan-eth4 interface eth4 track-wan use-for-wan-eth4 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: WAN monitor state of eth4 is Up AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: unknown Interface: eth4; State: unknown Interface: ppp1; State: up [use-for-wan-eth4] WAN Monitor State: up Interface: eth4 (ppp1) [use-for-wan-eth0] WAN Monitor State: unknown Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: unknown Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 14 07:38:20 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 14 07:39:37 2012 Result 5: Default route is via eth4 AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 0.0.0.0 wan - 4018:b18c:66c0 D eth4 0.0.0.0 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp1 10.5.16.53 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.5.16.1 0.0.0.0 255.255.255.255 UH 0 0 0 ppp1 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.5.16.1 0.0.0.0 UG 1 0 0 ppp1 Result 6: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan D - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 wan D - - - hive0 - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.10 1 195M 1M -24(55) open none 00:02:15 1 Yes 0 11ng disabled 20MHz No data collecting... Result 7: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_PPPoE_06

| CaseID | WanRedundancy_Function_PPPoE_06 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 5 |  |  |
| Description | Failover behavior between ppp1 (eth0 with PPPoE) and ppp0 (usb is backup mode) |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. PPPoE server is available 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as always-connected mode, result 2 3. Config eth0 as WAN and highest priority, result 3 4. Config eth0 as PPPoE enabled, result 4 5. Config track wan group for eth0, result 5 6. Check wan monitor state of eth0, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on eth0, and track on usbnet0 still succeed, check wan monitor state, result 9 10. Repeat Step 7, result 10 11. Maintain ping process on client, result 8 12. Restore track on eth0, check wan monitor state, result 6 13. Repeat Step 7, result 7 14. Maintain ping process on client, result 8 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: usb modem is always-connected mode Result 3: Eth0 is WAN mode, with highest priority Result 4: PPPoE is configured on eth0 Result 5: Eth0 is configured with track wan group Result 6: WAN monitor state of eth0 is Up Result 7: Default route via ppp1 has the highest priority Result 8: Ping successfully on client Result 9: wan failover to usbnet0 Result 10: Default route via ppp0 has the highest priority |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_PPPoE_07

| CaseID | WanRedundancy_Function_PPPoE_07 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 5 |  |  |
| Description | Failover behavior between ppp1 (eth0 with PPPoE) and ppp0 (usb is primary mode) |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. PPPoE server is available 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config usb modem as prim-wan mode, result 2 3. Config eth0 as WAN, result 3 4. Config eth0 as PPPoE enabled, result 4 5. Config track wan group for wan interfaces, result 5 6. Check wan monitor state, result 6 7. Check default route in main route table, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on usbnet0, and track on eth0 still succeed, check wan monitor state, result 9 10. Repeat Step 7, result 10 11. Maintain ping process on client, result 8 12. Restore track on usbnet0, check wan monitor state, result 6 13. Repeat Step 7, result 7 14. Maintain ping process on client, result 8 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: usb modem is prim-wan mode Result 3: Eth0 is WAN mode Result 4: PPPoE is configured on eth0 Result 5: wan interfaces are configured with track wan group Result 6: WAN monitor state of usbnet0 and eth0 is Up Result 7: Default route via ppp0 has the highest priority Result 8: Ping successfully on client Result 9: wan failover to eth0 Result 10: Default route via ppp1 has the highest priority |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### 6.2.7. LED

#### WanRedundancy_Function_LED_01

| CaseID | WanRedundancy_Function_LED_01 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | LED state while wan failover between interfaces, usb is backup mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Config usb modem as always-connect mode, result 1 2. Config eth1 as wan mode, result 2 3. Check wan interface priority, result 3 4. Check wan monitor state of each interface, result 4 5. Config CAPWAP server for BR, echo the capwap connection state, result 5 6. Check LED state when both wan monitor state of eth0 and capwap connection are up, result 6 7. Shutdown eth1, as well as usbnet0, check state of interfaces, result 7 8. Track fail on eth0, check LED state, result 8 9. Restore both eth1 and usbnet0, check LED state, result 9 10. Track fail on eth1, check LED state, result 10 11. Track fail on usbnet0, check LED state, result 11 12. Restore track on usbnet0, check LED state, result 10 13. Restore track on eth1, check LED state, result 9 14. Restore track on eth0, check LED state, result 12 |  |  |
| ExpectResult | Result 1: USB modem is always-connect mode Result 2: eth1 is wan mode Result 3: WAN priority sequence is eth0> eth1> usbnet Result 4: Active wan is eth0 Result 5: CAPWAP state is “Connected” Result 6: LED state is “White”&“no blinking” Result 7: eth0 is the only available wan interface Result 8: LED state is “Amber”&“no blinking”, wan monitor state of eth0 is down Result 9: LED state is “White”&“Blinking”, wan monitor state of eth1 is up Result 10: LED state is “White”&“Blinking”, wan monitor state of usbnet0 is up Result 11: LED state is “Amber”&“no blinking”, wan monitor state is down Result 12: LED state is “White”&“no blinking”, wan monitor state of eth0 is up |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

#### WanRedundancy_Function_LED_02

| CaseID | WanRedundancy_Function_LED_02 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | LED state while wan failover between interfaces, usb is prim-wan mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Config usb modem as prim-wan mode, result 1 2. Config eth1 as wan mode, result 2 3. Check wan interface priority, result 3 4. Check wan monitor state of each interface, result 4 5. Config CAPWAP server for BR, echo the capwap connection state, result 5 6. Check LED state when both wan monitor state of usbnet0 and capwap connection are up, result 6 7. Shutdown eth0, as well as eth1, check state of interfaces, result 7 8. Track fail on usbnet0, check LED state, result 8 9. Restore both eth0 and eth1, check LED state, result 9 10. Track fail on eth0, check LED state, result 10 11. Track fail on eth1, check LED state, result 11 12. Restore track on eth1, check LED state, result 10 13. Restore track on eth0, check LED state, result 9 14. Restore track on usbnet0, check LED state, result 12 |  |  |
| ExpectResult | Result 1: USB modem is prim-wan mode Result 2: eth1 is wan mode Result 3: WAN priority sequence is usbnet>eth0> eth1> Result 4: Active wan is usbnet0 Result 5: CAPWAP state is “Connected” Result 6: LED state is “White”&“no blinking” Result 7: usbnet0 is the only available wan interface Result 8: LED state is “Amber”&“no blinking”, wan monitor state of usbnet0 is down Result 9: LED state is “White”&“Blinking”, wan monitor state of eth0 is up Result 10: LED state is “White”&“Blinking”, wan monitor state of eth1 is up Result 11: LED state is “Amber”&“no blinking”, wan monitor state is down Result 12: LED state is “White”&“no blinking”, wan monitor state of usbnet0 is up |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

## Stress Test Case

### WanRedundancy_Stress_01

| CaseID | WanRedundancy_Stress_01 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 3 |  |  |
| Description | WAN failover between eth0/ eth1/ usbnet for 100 times |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. Eth1 is connected with Switch initially 3. usb modem is plugged initially 4. There is signal of mobile network available for usb modem |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 ssid zhxu-br200-wifi0 in w0 ssid zhxu-br200-wifi0 2. Config eth1 as wan mode, result 2 in e1 mode wan 3. Config track wan group for each interface, result 3 track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable 4. Config usb modem as always-connect mode, result 4 usbm mode a usbmodem modem-id sierra_308 apn 3gnet 5. Check wan monitor state of each interface, result 5 6. Check default route in main route table, result 6 7. Check the active DNS config, result 7 8. Run ping internet process on client to check its accessibility, result 8 9. Track fail on eth0, wan failover to eth1, check wan monitor state, result 9 track-wan use-for-wan-eth0 ip 9.9.9.9 10. Repeat Step 6, result 10 11. Repeat Step 7, result 11 12. Maintain ping process on client, result 8 13. Track fail on eth1, wan failover to usbnet, repeat Step 9~ 12, result 12 track-wan use-for-wan-eth1 ip 9.9.9.9 14. Restore track on eth1, check wan monitor state and default route, result 13 no track-wan use-for-wan-eth1 ip 9.9.9.9 15. Restore track on eth0, check wan monitor state and default route, result 14 no track-wan use-for-wan-eth0 ip 9.9.9.9 16. Repeat Step 9~ 15 for 100 times, result 15 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: Client gets association with BR AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan D - - - - - Eth1 4018:b18c:66c2 access D - - - hive0 - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh st Chan=channel number; Pow=Power in dBm; A-Mode=Authentication mode; Cipher=Encryption mode; A-Time=Associated time; Auth=Authenticated; UPID=User profile Identifier; Phymode=Physical mode; Ifname=wifi0.1, Ifindex=17, SSID=zhxu-br200-wifi0: Mac Addr IP Addr Chan Tx Rate Rx Rate Pow(SNR) A-Mode Cipher A-Time VLAN Auth UPID Phymode SM-PS Chan-width Release Station-State -------------- --------------- ---- ------- ------- --- ---------- -------- -------- ---- ---- ---- ------- --------- ---------- ------- 0024:d7f4:26a8 192.168.85.10 1 195M 1M -24(55) open none 00:02:15 1 Yes 0 11ng disabled 20MHz No data collecting... Result 2: eth1 is wan mode AH-8c66c0#sh in e1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 wan U - - - - - Result 3: Each interface is configured with a track wan group AH-8c66c0#sh in State=Operational state; Chan=Channel; Radio=Radio profile; U=up; D=down; Name MAC addr Mode State Chan VLAN Radio Hive SSID ------- -------------- -------- ----- ---- ---- ---------- ---------- --------- Mgt0 4018:b18c:66c0 - U - 1 - hive0 - Eth0 4018:b18c:66c0 wan U - - - - - Eth1 4018:b18c:66c2 wan U - - - - - Eth4 4018:b18c:66c3 access D - - - hive0 - Eth3 4018:b18c:66c4 access D - - - hive0 - Eth4 4018:b18c:66c5 access D - - - hive0 - Ppp0 0000:0000:0000 wan U - - - - - Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 1 - radio_ng0 hive0 zhxu-br... - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ru ¦ i track track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 4: USB modem is always-connect mode AH-8c66c0#sh usbm modem-id sierra_308 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: sierra_308 USB Id(s): Vendor Id: 0xf3d, Product Id: 0x68a3 Vendor Id: 0x1199, Product Id: 0x68a3 APN: 3gnet Dialup Username: ISP@CINGULARGPRS.COM Dialup Password: CINGULAR1 Dialup Number: ATD*99***1# <End of USB modem configuration> Result 5: eth0 is active wan AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:25 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 20 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 3 lkup_time: Wed Nov 7 03:04:42 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: kifname: eth1 wifidx: 2 kifidx: 6 wanif_type: 0 priority: 10 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Wed Nov 7 03:03:49 2012 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:20 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 Result 6: Default route is via eth0, and there are non-default routes via eth1/ usbnet with corresponding priority AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U eth1 192.168.10.5 wan - 4018:b18c:66c2 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.27.104.129 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 Result 7: There is DNS server for eth0 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 8: Ping successfully on client C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 9: eth1 is active wan AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:29:19 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:29:19 2012 Result 10: Default route is via eth1, and there are non-default routes via eth0/ usbnet with corresponding priority AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 Result 11: There is DNS server for eth1 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 Result 12: Active WAN is usbnet, default route is via usbnet, DNS is corresponding to usbnet, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: down Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:59:40 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:59:40 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10010 0 0 eth1 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - !!! Still with issue, bug 20545 AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 13: Active WAN is eth1, default route is via eth1, DNS is corresponding to eth1, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: down Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Wed Nov 7 05:29:19 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Wed Nov 7 05:29:19 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 0.0.0.0 10.155.30.254 0.0.0.0 UG 10001 0 0 eth0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 14: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client AH-8c66c0#sh track- [Default Track IP] ------------------------------- Result: up Interface: eth0; State: up Interface: eth1; State: up Interface: ppp0; State: up [use-for-wan-eth1] WAN Monitor State: up Interface: eth1 (eth1) [use-for-wan-eth0] WAN Monitor State: up Interface: eth0 (eth0) [use-for-wan-usbnet0] WAN Monitor State: up Interface: usbnet0 (ppp0) - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Wed Nov 7 03:09:56 2012 last_event: PRIMARY_WAN_CONNECTED last_evt_time: Wed Nov 7 03:09:56 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 192.168.10.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 192.168.10.1 0.0.0.0 UG 10 0 0 eth1 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh dns DNS server configuration: Domain name suffix: Primary : 10.155.3.250 Secondary : 0.0.0.0 Tertiary : 0.0.0.0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - C:\Windows\system32>ping www.bing.com Pinging a134.dsw3.akamai.net [165.254.156.120] with 32 bytes of data: Reply from 165.254.156.120: bytes=32 time=227ms TTL=50 Reply from 165.254.156.120: bytes=32 time=321ms TTL=50 Reply from 165.254.156.120: bytes=32 time=249ms TTL=50 Reply from 165.254.156.120: bytes=32 time=244ms TTL=50 Ping statistics for 165.254.156.120: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 227ms, Maximum = 321ms, Average = 260ms Result 15: Similar result as Result 9~ 14, and failover/ failback without failure |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_Stress_02

| CaseID | WanRedundancy_Stress_02 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 4 |  |  |
| Description | WAN failover between eth0/ wifi0/ usbnet for 100 times |  |  |
| PlatformDependence | BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. Eth0 is connected with Switch initially 2. usb modem is plugged initially 3. There is signal of mobile network available for usb modem 4. wifi0 has been configured as wan mode and connected with ISP ssid 5. wan monitor state through wifi0 is up |  |  |
| TestProcedure | 1. Get a client associated to BR with open auth, result 1 2. Config wifi0 as wan mode, result 2 3. Check the state of wan via wifi0, result 3 4. Config track wan group for each interface, result 4 5. Config usb modem as always-connect mode, result 5 6. Check wan monitor state of each interface, result 6 7. Check default route in main route table, result 7 8. Check the active DNS config, result 8 9. Run ping internet process on client to check its accessibility, result 9 10. Track fail on eth0, wan failover to wifi0, check wan monitor state, result 10 11. Repeat Step 7, result 11 12. Repeat Step 8, result 12 13. Maintain ping process on client, result 9 14. Track fail on wifi0, wan failover to usbnet, repeat Step 10~ 13, result 13 15. Restore track on wifi0, check wan monitor state and default route, result 14 16. Restore track on eth0, check wan monitor state and default route, result 15 17. Repeat Step 10~ 16 for 100 times, result 16 |  |  |
| ExpectResult | Result 1: Client gets association with BR Result 2: wifi0 is wan mode Result 3: wifi0 get association with ISP ssid Result 4: Each interface is configured with a track wan group Result 5: USB modem is always-connect mode Result 6: eth0 is active wan Result 7: Default route is via eth0, and there are non-default routes via wifi0/ usbnet with corresponding priority Result 8: There is DNS server for eth0 Result 9: Ping successfully on client Result 10: wifi0 is active wan Result 11: Default route is via wifi0, and there are non-default routes via eth0/ usbnet with corresponding priority Result 12: There is DNS server for wifi0 Result 13: Active WAN is usbnet, default route is via usbnet, DNS is corresponding to usbnet, and ping successfully on client Result 14: Active WAN is wifi0, default route is via wifi0, DNS is corresponding to wifi0, and ping successfully on client Result 15: Active WAN is eth0, default route is via eth0, DNS is corresponding to eth0, and ping successfully on client Result 16: Similar result as Result 6~ 15, and failover/ failback without failure |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

## Longevity Test Case

## Performance Test Case

## Capacity Test Case

## Compatibility Test Case

### WanRedundancy_Compatibility_01

| CaseID | WanRedundancy_Compatibility_01 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | HOS upgrade from previous release, with track ip configured |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. BR is running with img which is prior to Edinburgh |  |  |
| TestProcedure | 1. Check the version of BR release, result 1 2. There is track-ip configured, result 2 3. Upgrade BR to Edinburgh release, result 3 4. Check the track-wan config, result 4 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track test-for-wan ip 8.8.8.8 track test-for-wan default-gateway track test-for-wan multi-dst-logic and track test-for-wan use-for-wan-testing capwap client server name 10.155.31.205 usbmodem mode primary-wan Result 1: BR is running with img which is prior to Edinburgh AH-8c66c0#sh ver Aerohive Networks Inc. Copyright (C) 2006-2012 Version: HiveOS 5.1r4 release build0986 Build time: Sat Nov 17 18:57:40 UTC 2012 Build cookie: 111712054110 Platform: BR200-WP Bootloader ver: v1.0.2.10 TPM ver: v1.2.35.8 Uptime: 0 weeks, 0 days, 0 hours, 15 minutes, 14 seconds Result 2: There is track ip configured for wan failover AH-8c66c0#sh ru ¦ i tr track test-for-wan ip 8.8.8.8 track test-for-wan default-gateway track test-for-wan multi-dst-logic and track test-for-wan use-for-wan-testing Result 3: BR is upgraded to Edinburgh release AH-8c66c0#sh ver Aerohive Networks Inc. Copyright (C) 2006-2012 Version: HiveOS 6.0r1 release build0935 Build time: Sat Nov 17 21:09:08 UTC 2012 Build cookie: 111712070055 Platform: BR200-WP Bootloader ver: v1.0.2.10 TPM ver: v1.2.35.8 Uptime: 0 weeks, 0 days, 0 hours, 0 minutes, 33 seconds Result 4: There is track-wan group configured for each interface whose config is the same as track-ip AH-8c66c0#sh ru ¦ i tr track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 default-gateway track-wan use-for-wan-eth0 multi-dst-logic and track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 default-gateway track-wan use-for-wan-usbnet0 multi-dst-logic and track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_Compatibility_02

| CaseID | WanRedundancy_Compatibility_02 |  |  |
| --- | --- | --- | --- |
| Priority | High | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | HOS upgrade from previous release, without track ip configured |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. BR is running with img which is prior to Edinburgh |  |  |
| TestProcedure | 1. Check the version of BR release, result 1 2. There is no track-ip configured, result 2 3. Upgrade BR to Edinburgh release, result 3 4. Check if there is track-wan config, result 2 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.205 Result 1: BR is running with img which is prior to Edinburgh AH-8c66c0#sh ver Aerohive Networks Inc. Copyright (C) 2006-2012 Version: HiveOS 5.1r4 release build0986 Build time: Sat Nov 17 18:57:40 UTC 2012 Build cookie: 111712054110 Platform: BR200-WP Bootloader ver: v1.0.2.10 TPM ver: v1.2.35.8 Uptime: 0 weeks, 0 days, 0 hours, 15 minutes, 14 seconds Result 2: There is no track ip configured for wan failover AH-8c66c0#sh ru ¦ i tr AH-8c66c0# Result 3: BR is upgraded to Edinburgh release AH-8c66c0#sh ver Aerohive Networks Inc. Copyright (C) 2006-2012 Version: HiveOS 6.0r1 release build0935 Build time: Sat Nov 17 21:09:08 UTC 2012 Build cookie: 111712070055 Platform: BR200-WP Bootloader ver: v1.0.2.10 TPM ver: v1.2.35.8 Uptime: 0 weeks, 0 days, 0 hours, 0 minutes, 33 seconds |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_Compatibility_03

| CaseID | WanRedundancy_Compatibility_03 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | HOS upgrade from previous release, usb is backup wan mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. BR is running with img which is prior to Edinburgh |  |  |
| TestProcedure | 1. Check the version of BR release, result 1 2. Config usb modem as backup wan mode, result 2 3. Upgrade BR to Edinburgh release, result 3 4. Check the mode of usb modem, result 2 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.205 Result 1: BR is running with img which is prior to Edinburgh AH-8c66c0#sh ver Aerohive Networks Inc. Copyright (C) 2006-2012 Version: HiveOS 5.1r4 release build0986 Build time: Sat Nov 17 18:57:40 UTC 2012 Build cookie: 111712054110 Platform: BR200-WP Bootloader ver: v1.0.2.10 TPM ver: v1.2.35.8 Uptime: 0 weeks, 0 days, 0 hours, 15 minutes, 14 seconds Result 2: usb modem is configured as on-demand mode AH-8c66c0#sh usbm ¦ i mode: Mode: on-demand Result 3: BR is upgraded to Edinburgh release AH-8c66c0#sh ver Aerohive Networks Inc. Copyright (C) 2006-2012 Version: HiveOS 6.0r1 release build0935 Build time: Sat Nov 17 21:09:08 UTC 2012 Build cookie: 111712070055 Platform: BR200-WP Bootloader ver: v1.0.2.10 TPM ver: v1.2.35.8 Uptime: 0 weeks, 0 days, 0 hours, 0 minutes, 33 seconds |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_Compatibility_04

| CaseID | WanRedundancy_Compatibility_04 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | HOS upgrade from previous release, usb is primary wan mode |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition | 1. BR is running with img which is prior to Edinburgh |  |  |
| TestProcedure | 1. Check the version of BR release, result 1 2. Config usb modem as primary wan mode, result 2 3. Upgrade BR to Edinburgh release, result 3 4. Check the mode of usb modem, result 2 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru ssid zhxu-br200-wifi0 interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 ssid zhxu-br200-wifi0 interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track test-for-wan ip 8.8.8.8 track test-for-wan default-gateway track test-for-wan multi-dst-logic and track test-for-wan use-for-wan-testing capwap client server name 10.155.31.205 usbmodem mode primary-wan Result 1: BR is running with img which is prior to Edinburgh AH-8c66c0#sh ver Aerohive Networks Inc. Copyright (C) 2006-2012 Version: HiveOS 5.1r4 release build0986 Build time: Sat Nov 17 18:57:40 UTC 2012 Build cookie: 111712054110 Platform: BR200-WP Bootloader ver: v1.0.2.10 TPM ver: v1.2.35.8 Uptime: 0 weeks, 0 days, 0 hours, 15 minutes, 14 seconds Result 2: usb modem is configured as primary-wan mode AH-8c66c0#sh usbm ¦ i mode: Mode: primary-wan Result 3: BR is upgraded to Edinburgh release AH-8c66c0#sh ver Aerohive Networks Inc. Copyright (C) 2006-2012 Version: HiveOS 6.0r1 release build0935 Build time: Sat Nov 17 21:09:08 UTC 2012 Build cookie: 111712070055 Platform: BR200-WP Bootloader ver: v1.0.2.10 TPM ver: v1.2.35.8 Uptime: 0 weeks, 0 days, 0 hours, 0 minutes, 33 seconds |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

## Negative Test Case

## Other Test Case

## CLI Management

CLI that this feature invokes,

[no] interface <ethx | usbnetx | wifix> mode wan priority <number>

Show wan interface

Show wan failover

[no] track-wan <string>

[no] track-wan <string> ip <ip_addr>

[no] track-wan <string> default-gateway

[no] track-wan <string> multi-dst-logic {or|and}

[no] track-wan <string> enable

[no] track-wan <string> interval [numbers]

[no] track-wan <string> timeout [numbers]

[no] track-wan <string> retry [numbers]

[no] track-wan <string> interface <ethx|usbnetx|wifix>

show track-wan

### WanRedundancy_CliMgt_01

| CaseID | WanRedundancy_CliMgt_01 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface ethx mode wan; show wan interface ethx |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default mode of each interface, result 1 2. Try to change the wan mode from eth0, result 2 no in e0 mode 3. Check default mode of eth1, result 3 4. Config mode of eth1 as wan, result 4 interface eth1 mode wan 5. Save config, then reboot BR 6. Check the mode of eth1, result 4 7. Restore the mode of eth1, result 3 interface eth1 mode bridge-access 8. Save config, then reboot BR 9. Check the mode of eth1, result 3 10. Repeat Step 3~ 9 for eth4, result 5 in e2 mode wan 11. Repeat Step 3~ 9 for eth3, result 6 in e3 mode wan 12. Repeat Step 3~ 9 for eth4, result 7 in e4 mode wan |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable bonjour-gateway realm building01_floor01_hive0 Result 1: eth0 is wan mode by default AH-8c66c0#sh l3 i ¦ i eth0 eth0 10.155.30.114 wan - 4018:b18c:66c0 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in e0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; Result 2: Fail to change the mode of eth0 no in e0 mode Result 3: eth1 is Access mode AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 access D - - - hive0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in e1 ¦ i mode Mode=bridge-access; Mac learning= enabled; Admin state=enabled; Result 4: eth1 is wan mode AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 wan D - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in e1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= enabled; Admin state=enabled; Result 5: Similar as Result 3~ 4 Default mode of eth4, AH-8c66c0#sh in ¦ i eth4 Eth4 4018:b18c:66c3 access D - - - hive0 - AH-8c66c0#sh in e2 ¦ i mode Mode=bridge-access; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Change mode of eth4 to wan, AH-8c66c0#sh in ¦ i eth4 Eth4 4018:b18c:66c3 wan D - - - - - AH-8c66c0#sh in e2 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; Result 6: Similar as Result 3~ 4 Default mode of eth3, AH-8c66c0#sh in ¦ i eth3 Eth34018:b18c:66c3 access D - - - hive0 - AH-8c66c0#sh in e3 ¦ i mode Mode=bridge-access; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Change mode of eth3 to wan, AH-8c66c0#sh in ¦ i eth3 Eth3 4018:b18c:66c3 wan D - - - - - AH-8c66c0#sh in e3 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; Result 7: Similar as Result 3~ 4 Default mode of eth4, AH-8c66c0#sh in ¦ i eth4 Eth44018:b18c:66c3 access D - - - hive0 - AH-8c66c0#sh in e4 ¦ i mode Mode=bridge-access; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Change mode of eth4 to wan, AH-8c66c0#sh in ¦ i eth4 Eth4 4018:b18c:66c3 wan D - - - - - AH-8c66c0#sh in e4 ¦ i mode Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_02

| CaseID | WanRedundancy_CliMgt_02 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface ethx mode wan; show wan interface ethx |  |  |
| PlatformDependence | BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default mode of each interface, result 1 2. Try to change the wan mode from eth0, result 2 no in e0 mode 3. Check default mode of eth1, result 3 4. Config mode of eth1 as wan, result 4 interface eth1 mode wan 5. Save config, then reboot BR 6. Check the mode of eth1, result 4 7. Restore the mode of eth1, result 3 interface eth1 mode bridge-access 8. Save config, then reboot BR 9. Check the mode of eth1, result 3 |  |  |
| ExpectResult | BR config, zhxu-4e3640#sh ru interface eth0 mode wan interface eth1 mode bridge-access hostname zhxu-4e3640 no interface mgt0 dhcp client config version 3 config rollback enable snmp location change_me@ no capwap client dtls negotiation enable capwap client HTTP proxy name 10.155.60.114 port 808 application reporting collection-period 3600 report-period 3600 Result 1: eth0 is wan mode by default zhxu-4e3640#sh in ¦ i eth0 Eth0 4018:b14e:3640 wan U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-4e3640#sh in e0 ¦ i mode Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; Result 2: Fail to change the mode of eth0 Result 3: eth1 is lan mode zhxu-4e3640#sh in ¦ i eth1 Eth1 4018:b14e:3641 access D - - - hive0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-4e3640#sh in e1 ¦ i mode Mode=bridge-access; Mac learning= disabled; Admin state=enabled; Result 4: eth1 is wan mode zhxu-4e3640#sh in ¦ i eth1 Eth1 4018:b14e:3641 wan D - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-4e3640#sh in e1 ¦ i mode Mode=wan; WAN Priority=10; Mac learning= disabled; Admin state=enabled; |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_03

| CaseID | WanRedundancy_CliMgt_03 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface wifi0 mode wan; show wan interface wifi0 |  |  |
| PlatformDependence | BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default mode of wifi0, result 1 2. Config mode of wifi0 as wan, result 2 interface wifi0 mode wan 3. Save config, then reboot BR 4. Check the mode of wifi0, result 2 5. Restore the mode of wifi0, result 1 no interface wifi0 mode wan 6. Save config, then reboot BR 7. Check the mode of wifi0, result 1 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable bonjour-gateway realm building01_floor01_hive0 Result 1: wifi0 is access mode AH-8c66c0#sh in ¦ i wifi0 Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access D 1 - radio_ng0 hive0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in w0 ¦ i mode Mode=access; Radio disabled=no; Result 2: wifi0 is wan mode AH-8c66c0#sh in ¦ i wifi0 Wifi0 4018:b18c:66d4 wan U 6 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 6 - radio_ng0 hive0 - Wifi0.2 4018:b18c:66d5 client D 6 - radio_ng0 hive0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in w0 ¦ i mode Mode=wan; Radio disabled=no; |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_04

| CaseID | WanRedundancy_CliMgt_04 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface wifix mode wan; show wan interface wifix |  |  |
| PlatformDependence | BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default mode of wifi0, result 1 2. Config mode of wifi0 as wan, result 2 interface wifi0 mode wan 3. Save config, then reboot BR 4. Check the mode of wifi0, result 2 5. Restore the mode of wifi0, result 1 no interface wifi0 mode wan 6. Save config, then reboot BR 7. Check the mode of wifi0, result 1 8. Repeat Step 1~ 7 for wifi1, result 3 |  |  |
| ExpectResult | BR config, zhxu-4e3640#sh ru interface eth0 mode wan interface eth1 mode bridge-access hostname zhxu-4e3640 no interface mgt0 dhcp client config version 3 config rollback enable snmp location change_me@ no capwap client dtls negotiation enable capwap client HTTP proxy name 10.155.60.114 port 808 application reporting collection-period 3600 report-period 3600 Result 1: wifi0 is access mode zhxu-4e3640#sh in ¦ i wifi0 Wifi0 4018:b14e:3650 access U 1 - radio_ng0 - - Wifi0.1 4018:b14e:3654 access D 1 - radio_ng0 hive0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-4e3640#sh in w0 ¦ i mode Mode=access; Radio disabled=no; Result 2: wifi0 is wan mode Result 3: Similar as Result 1~2 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_05

| CaseID | WanRedundancy_CliMgt_05 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface <ethx ¦ wifix> mode wan; show wan interface <ethx ¦ wifix>; Restriction for max number of wan interfaces |  |  |
| PlatformDependence | BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default mode of each interface, result 1 2. Config mode of eth1 as wan, result 2 interface eth1 mode wan 3. Try to config mode of eth4 as wan, result 3 4. Try to config mode of eth3 as wan, result 4 5. Try to config mode of eth4 as wan, result 5 6. Try to config mode of wifi0 as wan, result 6 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable bonjour-gateway realm building01_floor01_hive0 Result 1: eth0/ usbnet is wan mode by default Result 2: eth1 is configured as wan mode Result 3: fail to config eth4 as wan Result 4: fail to config eth3 as wan Result 5: fail to config eth4 as wan Result 6: fail to config wifi0 as wan |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_06

| CaseID | WanRedundancy_CliMgt_06 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface <ethx ¦ wifix> mode wan; show wan interface <ethx ¦ wifix>; Restriction for max number of wan interfaces |  |  |
| PlatformDependence | BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default mode of each interface, result 1 2. Config mode of eth1 as wan, result 2 interface eth1 mode wan 3. Try to config mode of wifi0 as wan, result 3 4. Try to config mode of wifi1 as wan, result 4 |  |  |
| ExpectResult | BR config, zhxu-4e3640#sh ru interface eth0 mode wan interface eth1 mode bridge-access hostname zhxu-4e3640 no interface mgt0 dhcp client config version 3 config rollback enable snmp location change_me@ no capwap client dtls negotiation enable capwap client HTTP proxy name 10.155.60.114 port 808 application reporting collection-period 3600 report-period 3600 Result 1: eth0/ usbnet is wan mode by default Result 2: eth1 is configured as wan mode Result 3: fail to config wifi0 as wan Result 4: fail to config wifi1 as wan |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_07

| CaseID | WanRedundancy_CliMgt_07 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface ethx mode wan priority <number>; show wan interface ethx |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check the default wan priority of eth0, result 1 2. Config wan priority of eth0, echo the config, result 2 interface eth0 mode wan priority 5 3. Save config, then reboot BR 4. Check the wan priority of eth0, result 2 5. Delete the wan priority config of eth0, result 1 no interface eth0 mode wan priority 6. Config wan priority of eth0 with invalid value, result 3 7. Config eth1 as wan, repeat Step 1~ 6 for eth1, result 4 in eth1 mode wan 8. Remove wan mode from eth1, config eth4 as wan, repeat Step 1~ 6 for eth4, result 5 in e1 mode bridge-access in e2 mode wan 9. Remove wan mode from eth4, config eth3 as wan, repeat Step 1~ 6 for eth3, result 6 in e2 mode bridge-access in e3 mode wan 10. Remove wan mode from eth3, config eth4 as wan, repeat Step 1~ 6 for eth4, result 7 in e3 mode bridge-access in e4 mode wan |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server port 80 track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: wan priority of eth0 is 1 by default, and default route via eth0 is 1 accordingly AH-8c66c0#sh in e0 ¦ i pri Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in eth0 WAN Interface Status: ifname: eth0 wanif_type: 0 kifname: eth0 wifidx: 0 kifidx: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 0 lkup_time: Wed Oct 31 07:03:28 2012 lkdn_count: 0 lkdn_time: Wed Oct 31 07:03:28 2012 sm_name: eth0 SM curr_state: ATTACHED prev_state: ATTACHED prev_state_time: Wed Oct 31 07:03:28 2012 last_event: ETH_IF_CREATED last_evt_time: Wed Oct 31 07:03:28 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro ¦ i eth0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 Result 2: wan priority of eth0 is 5, and default route via eth0 is 5 accordingly AH-8c66c0#sh in e0 ¦ i pri Mode=wan; WAN Priority=5; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in e0 WAN Interface Status: ifname: eth0 wanif_type: 0 kifname: eth0 wifidx: 0 kifidx: 0 priority: 5 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:26 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 sm_name: eth0 SM curr_state: CONNECTED prev_state: CONNECTING prev_state_time: Thu Jan 1 00:00:26 1970 last_event: ETH_L3_CONNECTED last_evt_time: Thu Jan 1 00:00:26 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro ¦ i eth0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 0.0.0.0 10.155.30.254 0.0.0.0 UG 5 0 0 eth0 Result 3: Warning for invalid config AH-8c66c0#in e0 mode wan priority 0 ^-- unknown keyword or invalid input Result 4: Similar as Result 1~ 3 Default priority of eth1, AH-8c66c0#sh in e1 ¦ i pri Mode=wan; WAN Priority=10; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: …… ifname: wanif_type: 0 kifname: eth1 wifidx: 2 kifidx: 6 priority: 10 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:21 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro ¦ i eth1 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth1 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 10 0 0 eth1 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Customize the priority of eth1, in e1 mode wan priority 5 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in e1 ¦ i pri Mode=wan; WAN Priority=5; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: …… ifname: wanif_type: 0 kifname: eth1 wifidx: 2 kifidx: 6 priority: 5 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:21 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro ¦ i eth1 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth1 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 5 0 0 eth1 Result 5: Similar as Result 1~ 3 Default priority of eth4, AH-8c66c0#sh in e2 ¦ i pri Mode=wan; WAN Priority=20; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: …… ifname: wanif_type: 0 kifname: eth4 wifidx: 3 kifidx: 8 priority: 20 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 15:31:25 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 15:31:14 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro ¦ i eth4 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth4 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.155.30.254 0.0.0.0 UG 20 0 0 eth4 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Customize the priority of eth4, in e2 mode wan priority 5 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in e2 ¦ i pri Mode=wan; WAN Priority=5; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: …… ifname: wanif_type: 0 kifname: eth4 wifidx: 3 kifidx: 8 priority: 5 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 15:31:25 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 15:31:14 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro ¦ i eth4 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth4 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.155.30.254 0.0.0.0 UG 5 0 0 eth4 Result 6: Similar as Result 1~ 3 Default priority of eth3, AH-8c66c0#sh in e3 ¦ i pri Mode=wan; WAN Priority=30; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: …… ifname: wanif_type: 0 kifname: eth3 wifidx: 3 kifidx: 10 priority: 30 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:15:43 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:15:12 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro ¦ i eth3 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth3 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 10.155.30.254 0.0.0.0 UG 30 0 0 eth3 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Customize the priority of eth3, in e3 mode wan priority 5 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in e3 ¦ i pri Mode=wan; WAN Priority=5; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: …… ifname: wanif_type: 0 kifname: eth3 wifidx: 3 kifidx: 10 priority: 5 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:15:43 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:15:12 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro ¦ i eth3 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth3 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth3 0.0.0.0 10.155.30.254 0.0.0.0 UG 5 0 0 eth3 Result 7: Similar as Result 1~ 3 Default priority of eth4, AH-8c66c0#sh in e4 ¦ i pri Mode=wan; WAN Priority=40; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: …… ifname: wanif_type: 0 kifname: eth4 wifidx: 2 kifidx: 12 priority: 40 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:01:22 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:01:22 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro ¦ i eth4 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth4 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.155.30.254 0.0.0.0 UG 40 0 0 eth4 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Customize the priority of eth4, in e4 mode wan priority 5 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in e4 ¦ i pri Mode=wan; WAN Priority=5; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: …… ifname: wanif_type: 0 kifname: eth4 wifidx: 2 kifidx: 12 priority: 5 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:21 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro ¦ i eth4 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth4 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth4 0.0.0.0 10.155.30.254 0.0.0.0 UG 5 0 0 eth4 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_08

| CaseID | WanRedundancy_CliMgt_08 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface ethx mode wan priority <number>; show wan interface ethx |  |  |
| PlatformDependence | BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check the default wan priority of eth0, result 1 2. Config wan priority of eth0, echo the config, result 2 interface eth0 mode wan priority 5 3. Save config, then reboot BR 4. Check the wan priority of eth0, result 2 5. Delete the wan priority config of eth0, result 1 no interface eth0 mode wan priority 6. Config wan priority of eth0 with invalid value, result 3 7. Repeat Step 1~ 6 for eth1, result 4 |  |  |
| ExpectResult | BR config, zhxu-4e3640#sh ru hive zhxu interface eth0 mode wan interface eth1 mode wan interface mgt0 hive zhxu hostname zhxu-4e3640 no interface mgt0 dhcp client config version 3 config rollback enable snmp location change_me@ capwap client server name 192.168.10.100 no capwap client dtls negotiation enable capwap client HTTP proxy name 10.155.60.114 port 808 application reporting collection-period 3600 report-period 3600 Result 1: wan priority of eth0 is 1 by default, and default route via eth0 is 1 accordingly zhxu-4e3640#sh in e0 ¦ i pri Mode=wan; WAN Priority=1; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-4e3640#sh wan in eth0 WAN Interface Status: ifname: eth0 wanif_type: 0 kifname: eth0 wifidx: 0 kifidx: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:29 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 sm_name: eth0 SM curr_state: CONNECTED prev_state: CONNECTING prev_state_time: Thu Jan 1 00:00:29 1970 last_event: ETH_L3_CONNECTED last_evt_time: Thu Jan 1 00:00:29 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-4e3640#sh ip ro ¦ i eth0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 Result 2: wan priority of eth0 is 5, and default route via eth0 is 5 accordingly zhxu-4e3640#sh in e0 ¦ i pri Mode=wan; WAN Priority=5; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-4e3640#sh wan in eth0 WAN Interface Status: ifname: eth0 wanif_type: 0 kifname: eth0 wifidx: 0 kifidx: 0 priority: 5 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:29 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:11 1970 sm_name: eth0 SM curr_state: CONNECTED prev_state: CONNECTING prev_state_time: Thu Jan 1 00:00:29 1970 last_event: ETH_L3_CONNECTED last_evt_time: Thu Jan 1 00:00:29 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-4e3640#sh ip ro ¦ i eth0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 0.0.0.0 10.155.30.254 0.0.0.0 UG 5 0 0 eth0 Result 3: Warning for invalid config zhxu-4e3640#in e0 mode wan p 0 ^-- unknown keyword or invalid input Result 4: Similar as Result 1~ 3 Default priority of eth1, AH-8c66c0#sh in e1 ¦ i pri Mode=wan; WAN Priority=10; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: …… ifname: wanif_type: 0 kifname: eth1 wifidx: 2 kifidx: 6 priority: 10 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:21 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro ¦ i eth1 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth1 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 10 0 0 eth1 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - Customize the priority of eth1, in e1 mode wan priority 5 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in e1 ¦ i pri Mode=wan; WAN Priority=5; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: …… ifname: wanif_type: 0 kifname: eth1 wifidx: 2 kifidx: 6 priority: 5 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:21 1970 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro ¦ i eth1 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth1 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth1 0.0.0.0 10.155.30.254 0.0.0.0 UG 5 0 0 eth1 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_09

| CaseID | WanRedundancy_CliMgt_09 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface usbnetx mode wan priority <number>; show wan interface usbnetx |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check the default wan priority of usbnet0, result 1 2. Config wan priority of usbnet0, echo the config, result 2 interface usbnet0 mode wan priority 10 3. Save config, then reboot BR 4. Check the wan priority of usbnet0, result 2 5. Delete the wan priority config of usbnet0, result 1 no interface usbnet0 mode wan priority 6. Config wan priority of usbnet0 with invalid value, result 3 |  |  |
| ExpectResult | Result 1: wan priority of usbnet is 2000 by default Result 2: wan priority of usbnet is 10 Result 3: Warning for invalid config |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_10

| CaseID | WanRedundancy_CliMgt_10 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface wifi0 mode wan priority <number>; show wan interface wifi0 |  |  |
| PlatformDependence | BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check the default wan priority of wifi0, result 1 2. Config wan priority of wifi0, echo the config, result 2 interface wifi0 mode wan priority 10 3. Save config, then reboot BR 4. Check the wan priority of wifi0, result 2 5. Delete the wan priority config of wifi0, result 1 no interface wifi0 mode wan priority 6. Config wan priority of wifi0 with invalid value, result 3 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru hive zhxu interface eth0 mode wan interface eth1 mode bridge-access interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface wifi0 mode wan interface mgt0 hive zhxu interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.32.140 capwap client HTTP proxy name 10.155.60.114 port 808 usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth1 track-wan use-for-wan-eth1 ip 8.8.8.8 track-wan use-for-wan-eth1 interval 5 track-wan use-for-wan-eth1 interface eth1 track-wan use-for-wan-eth1 enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: wan priority of wifi0 is 1000 by default AH-8c66c0#sh in w0 ¦ i mode Mode=wan; Radio disabled=no; AH-8c66c0#sh in ¦ i wifi0 Wifi0 4018:b18c:66d4 wan U 6 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access U 6 - radio_ng0 zhxu - Wifi0.2 4018:b18c:66d5 client D 6 - radio_ng0 zhxu - Result 2: wan priority of wifi0 is 10 Result 3: Warning for invalid config |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_11

| CaseID | WanRedundancy_CliMgt_11 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface wifix mode wan priority <number>; show wan interface wifix |  |  |
| PlatformDependence | BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check the default wan priority of wifi0, result 1 2. Config wan priority of wifi0, echo the config, result 2 interface wifi0 mode wan priority 10 3. Save config, then reboot BR 4. Check the wan priority of wifi0, result 2 5. Delete the wan priority config of wifi0, result 1 no interface wifi0 mode wan priority 6. Config wan priority of wifi0 with invalid value, result 3 7. Repeat Step 1~ 6 for wifi1, result 4 |  |  |
| ExpectResult | Result 1: wan priority of wifi0 is 1000 by default Result 2: wan priority of wifi0 is 10 Result 3: Warning for invalid config Result 4: Similar as Result 1~ 3 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_12

| CaseID | WanRedundancy_CliMgt_12 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | interface ethx mode wan priority <number>; Restriction for different priority from one another |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check the default wan priority of eth0 and eth1, result 1 2. Config wan priority of eth0, echo the config, result 2 interface eth0 mode wan priority 10 3. Try to config wan priority of eth1 with same value, result 3 interface eth1 mode wan priority 10 |  |  |
| ExpectResult | Result 1: Default priority is * for eth0, * for eth1 Result 2: wan priority of eth0 is 10 Result 3: Fail to config same priority for eth1, and there is related warning |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_13

| CaseID | WanRedundancy_CliMgt_13 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | interface ethx mode wan priority <number>; Fail to config priority for non-wan interface |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default wan config, result 1 2. Try to config wan priority for eth1, result 2 interface eth1 mode wan priority 10 3. Repeat Step 1~ 2 for eth4, result 3 4. Repeat Step 1~ 2 for eth3, result 4 5. Repeat Step 1~ 2 for eth4, result 5 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usb0 track-wan use-for-wan-usbnet0 enable Result 1: eth1 is not wan mode AH-8c66c0#sh in e1 ¦ i mode Mode=bridge-access; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth1 Eth1 4018:b18c:66c2 access D - - - hive0 - Result 2: Fail to config priority for eth1 AH-8c66c0#in e1 mode wan priority 5 WAN priority can only be enabled on an interface in WAN mode ERROR: Invalid parameter(s) Result 3: Similar as Result 1~ 2 AH-8c66c0#sh in e2 ¦ i mode Mode=bridge-access; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth4 Eth4 4018:b18c:66c3 access D - - - hive0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#in e2 mode wan priority 5 WAN priority can only be enabled on an interface in WAN mode ERROR: Invalid parameter(s) Result 4: Similar as Result 1~ 2 AH-8c66c0#sh in e3 ¦ i mode Mode=bridge-access; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth3 Eth3 4018:b18c:66c3 access D - - - hive0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#in e3 mode wan priority 5 WAN priority can only be enabled on an interface in WAN mode ERROR: Invalid parameter(s) Result 5: Similar as Result 1~ 2 AH-8c66c0#sh in e4 ¦ i mode Mode=bridge-access; Mac learning= enabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i eth4 Eth4 4018:b18c:66c3 access D - - - hive0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#in e4 mode wan priority 5 WAN priority can only be enabled on an interface in WAN mode ERROR: Invalid parameter(s) |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_14

| CaseID | WanRedundancy_CliMgt_14 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | interface eth1 mode wan priority <number>; Fail to config priority for non-wan interface |  |  |
| PlatformDependence | BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default wan config, result 1 2. Try to config wan priority for eth1, result 2 interface eth1 mode wan priority 10 |  |  |
| ExpectResult | BR config, zhxu-4e3640#sh ru hive zhxu interface eth0 mode wan interface mgt0 hive zhxu hostname zhxu-4e3640 no interface mgt0 dhcp client config version 3 config rollback enable snmp location change_me@ capwap client server name 192.168.10.100 no capwap client dtls negotiation enable capwap client HTTP proxy name 10.155.60.114 port 808 application reporting collection-period 3600 report-period 3600 Result 1: eth1 is not wan mode zhxu-4e3640#sh in e1 ¦ i mode Mode=backhaul; Mac learning= disabled; Admin state=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-4e3640#sh in ¦ i eth1 Eth1 4018:b14e:3641 backhaul D - 1 - zhxu - Result 2: Fail to config priority for eth1 AH-8c66c0#in e1 mode wan priority 5 WAN priority can only be enabled on an interface in WAN mode ERROR: Invalid parameter(s) |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_15

| CaseID | WanRedundancy_CliMgt_15 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | interface wifi0 mode wan priority <number>; Fail to config priority for non-wan interface |  |  |
| PlatformDependence | BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default wan config, result 1 2. Try to config wan priority for wifi0, result 2 interface wifi0 mode wan priority 10 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable Result 1: wifi0 is not wan mode AH-8c66c0#sh in w0 ¦ i mode Mode=access; Radio disabled=no; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i wifi0 Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access D 1 - radio_ng0 hive0 - Result 2: Fail to config priority for wifi0 AH-8c66c0#in w0 mode wan p 5 WAN priority can only be enabled on an interface in WAN mode ERROR: Invalid parameter(s) |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_16

| CaseID | WanRedundancy_CliMgt_16 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | interface wifix mode wan priority <number>; Fail to config priority for non-wan interface |  |  |
| PlatformDependence | BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default wan config, result 1 2. Try to config wan priority for wifi0, result 2 interface wifi0 mode wan priority 10 3. Repeat Step 1~ 2 for wifi1, result 3 |  |  |
| ExpectResult | BR config, zhxu-4e3640#sh ru hive zhxu interface eth0 mode wan interface mgt0 hive zhxu hostname zhxu-4e3640 no interface mgt0 dhcp client config version 3 config rollback enable snmp location change_me@ capwap client server name 192.168.10.100 no capwap client dtls negotiation enable capwap client HTTP proxy name 10.155.60.114 port 808 application reporting collection-period 3600 report-period 3600 Result 1: wifi0 is not wan mode AH-8c66c0#sh in w0 ¦ i mode Mode=access; Radio disabled=no; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh in ¦ i wifi0 Wifi0 4018:b18c:66d0 access U 1 - radio_ng0 - - Wifi0.1 4018:b18c:66d4 access D 1 - radio_ng0 hive0 - Result 2: Fail to config priority for wifi0 zhxu-4e3640#in w0 mode wan p 5 ^-- unknown keyword or invalid input Result 3: Similar as Result 1~ 2 zhxu-4e3640#sh in w1 ¦ i mode Mode=dual; Radio disabled=no; Phymode=11na; Short preamble mode=enabled; - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-4e3640#sh in ¦ i wifi1 Wifi1 4018:b14e:3664 dual U 44 - radio_na0 - - Wifi1.1 4018:b14e:3664 backhaul U 44 1 radio_na0 zhxu - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - zhxu-4e3640#in w1 mode wan p 5 ^-- unknown keyword or invalid input |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_17

| CaseID | WanRedundancy_CliMgt_17 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | show wan failover |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Config wans' priority as default, show failover status, result 1 2. Unplug eth0, wan failover to usb modem, show failover status, result 2 3. Plug eth0 back, show failover status, result 1 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh usbm modem-id huawei_e1752 <Begin of USB modem configuration> Enabled: yes Mode: always-connected ----------------- Modem Id: huawei_e1752 USB Id(s): Vendor Id: 0x12d1, Product Id: 0x140c Vendor Id: 0x12d1, Product Id: 0x141b APN: 3gnet Dialup Username: Dialup Password: Dialup Number: ATD*99***1# <End of USB modem configuration> Result 1: wan failover state is “CONNECTED” AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 10.155.30.112 wan - 4018:b18c:66c0 U mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.27.181.89 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 1 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 0 lkdn_time: Thu Jan 1 00:00:10 1970 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 19 wanif_type: 1 priority: 600 is_default_wan: 0 cached_gwip: 0 lkup_count: 2 lkup_time: Fri Nov 2 01:40:19 2012 lkdn_count: 1 lkdn_time: Fri Nov 2 01:40:05 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 10.155.3.250 10.155.30.254 255.255.255.255 UGH 0 0 0 eth0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 10.155.30.0 0.0.0.0 255.255.255.0 U 0 0 0 eth0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.155.30.254 0.0.0.0 UG 1 0 0 eth0 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: CONNECTED prev_state: CONNECTED prev_state_time: Fri Nov 2 01:40:29 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 2 01:40:29 2012 Result 2: wan failover state is “FAILOVER” AH-8c66c0#sh l3 i Name IP Address Mode VLAN MAC State ----------- --------------- -------- ------ -------------- ----- eth0 0.0.0.0 wan - 4018:b18c:66c0 D mgt0 192.168.85.1 - 1 4018:b18c:66c0 U ppp0 10.27.181.89 wan - 0000:0000:0000 U - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan in WAN Interface Status: ifname: eth0 kifname: eth0 wifidx: 0 kifidx: 0 wanif_type: 0 priority: 1 is_default_wan: 0 cached_gwip: 0 lkup_count: 1 lkup_time: Thu Jan 1 00:00:27 1970 lkdn_count: 1 lkdn_time: Fri Nov 2 02:06:45 2012 ifname: usbnet0 kifname: ppp0 wifidx: 1 kifidx: 19 wanif_type: 1 priority: 600 is_default_wan: 1 cached_gwip: 0 lkup_count: 2 lkup_time: Fri Nov 2 01:40:19 2012 lkdn_count: 1 lkdn_time: Fri Nov 2 01:40:05 2012 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh ip ro Ref=references; Iface=interface; U=route is up;H=target is a host; G=use gateway; Destination Gateway Netmask Flags Metric Ref Use Iface --------------- --------------- --------------- ----- ------ ------ --- ----- 10.64.64.64 0.0.0.0 255.255.255.255 UH 0 0 0 ppp0 221.12.1.227 10.64.64.64 255.255.255.255 UGH 0 0 0 ppp0 192.168.85.0 0.0.0.0 255.255.255.0 U 0 0 0 mgt0 127.0.0.0 0.0.0.0 255.255.255.0 U 0 0 0 lo 0.0.0.0 10.64.64.64 0.0.0.0 UG 600 0 0 ppp0 - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#sh wan f WAN Failover Status: sm_name: WANFO SM curr_state: FAILOVER prev_state: FAILOVER prev_state_time: Fri Nov 2 02:07:57 2012 last_event: BACKUP_WAN_CONNECTED last_evt_time: Fri Nov 2 02:07:57 2012 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_18

| CaseID | WanRedundancy_CliMgt_18 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] track-wan <string> |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Config a track-wan group, result 1 track-wan test 2. Save config, then reboot BR 3. Check whether the track-wan group still exist, result 1 4. Delete the track-wan group, result 2 no track-wan test 5. Save config, then reboot BR 6. Check whether the track-wan group still exist, result 2 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable bonjour-gateway realm building01_floor01_hive0 Result 1: There is track-wan group “test” configured AH-8c66c0#sh ru ¦ i test track-wan test Result 2: There is no track-wan group “test” AH-8c66c0#sh ru ¦ i test AH-8c66c0# |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_19

| CaseID | WanRedundancy_CliMgt_19 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] track-wan <string> ip <ip_addr> |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Config track ip for track-wan group, result 1 track-wan test track-wan test ip 8.8.8.8 2. Save config, then reboot BR 3. Check whether the track ip config still exist, result 1 4. Delete the track ip, result 2 no track-wan test ip 8.8.8.8 5. Save config, then reboot BR 6. Check whether the track ip still exist, result 2 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable bonjour-gateway realm building01_floor01_hive0 Result 1: There is track ip config for track-wan group “test” AH-8c66c0#sh ru ¦ i test track-wan test track-wan test ip 8.8.8.8 Result 2: There is no track ip for track-wan group “test” AH-8c66c0#sh ru ¦ i test track-wan test AH-8c66c0# |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_20

| CaseID | WanRedundancy_CliMgt_20 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] track-wan <string> default-gateway |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Config track default-gateway for track-wan group, result 1 track-wan test track-wan test default-gateway 2. Save config, then reboot BR 3. Check whether the track default-gateway config still exist, result 1 4. Delete the track default-gateway, result 2 no track-wan test default-gateway 5. Save config, then reboot BR 6. Check whether the track default-gateway still exist, result 2 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable bonjour-gateway realm building01_floor01_hive0 Result 1: There is track default-gateway config for track-wan group “test” AH-8c66c0#sh ru ¦ i test track-wan test track-wan test default-gateway Result 2: There is no track default-gateway for track-wan group “test” AH-8c66c0#sh ru ¦ i test track-wan test AH-8c66c0# |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_21

| CaseID | WanRedundancy_CliMgt_21 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] track-wan <string> multi-dst-logic {or¦and} |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check the default multi-dst-logic for track-wan group, result 1 track-wan test 2. Config multi-dst-logic as "and" for track-wan group, result 2 track-wan test multi-dst-logic and 3. Save config, then reboot BR 4. Check the multi-dst-logic for track-wan group, result 2 5. Delete the multi-dst-logic for track-wan group, result 1 no track-wan test multi-dst-logic 6. Save config, then reboot BR 7. Check multi-dst-logic for track-wan group, result 1 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable bonjour-gateway realm building01_floor01_hive0 Result 1: The multi-dst-logic for track-wan group is "or" by default AH-8c66c0#sh ru ¦ i test track-wan test - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#track-wan test multi-dst-logic ? and Take action if none of the members in the group is responding (Default:or) or Take action if any single member in the group is not responding (Default:or) Result 2: The multi-dst-logic for track-wan group is "and" AH-8c66c0#sh ru ¦ i test track-wan test track-wan test multi-dst-logic and |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_22

| CaseID | WanRedundancy_CliMgt_22 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] track-wan <string> enable |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check the default toggle config for track-wan group, result 1 track-wan test 1. Enable track-wan group, result 2 track-wan test enable 2. Save config, then reboot BR 3. Check the toggle config for track-wan group, result 2 4. Delete the enabling for track-wan group, result 1 no track-wan test enable 5. Save config, then reboot BR 6. Check the toggle config for track-wan group, result 1 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable bonjour-gateway realm building01_floor01_hive0 Result 1: The track-wan group is disabled by default AH-8c66c0#sh ru ¦ i test track-wan test - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#track-wan test enable? enable Enable the group name for tracking (Default: Disable) Result 2: The track-wan group is enabled AH-8c66c0#sh ru ¦ i test track-wan test track-wan test enable |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_23

| CaseID | WanRedundancy_CliMgt_23 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] track-wan <string> interval [numbers] |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check the default interval for track-wan group, result 1 track-wan test 2. Config interval, echo the config, result 2 track-wan test interval 10 3. Save config, then reboot BR 4. Check the interval for track-wan group, result 2 5. Delete the interval, result 1 no track-wan test interval 6. Config the interval with invalid value, result 3 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable bonjour-gateway realm building01_floor01_hive0 Result 1: Interval is 6 by default AH-8c66c0#sh ru ¦ i test track-wan test - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#track-wan test interval ? <number> Enter the tracking interval (Default: 6 seconds; Range: 1-180; Note: The tracking interval must not be shorter than the probe timeout.) Result 2: Interval is 10 AH-8c66c0#sh ru ¦ i test track-wan test track-wan test interval 10 Result 3: Warning for invalid config AH-8c66c0#track-wan test interval 200 ^-- unknown keyword or invalid input AH-8c66c0#track-wan test interval 0 ^-- unknown keyword or invalid input |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_24

| CaseID | WanRedundancy_CliMgt_24 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] track-wan <string> timeout [numbers] |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check the default timeout for track-wan group, result 1 track-wan test 2. Config timeout, echo the config, result 2 track-wan test timeout 10 3. Save config, then reboot BR 4. Check the timeout for track-wan group, result 2 5. Delete the timeout, result 1 no track-wan test timeout 6. Config the timeout with invalid value, result 3 7. Config interval for track-wan test, then try to config timeout with a value greater than the interval, result 4 track-wan test interval 10 track-wan test timeout 20 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable bonjour-gateway realm building01_floor01_hive0 Result 1: Timeout is 2 by default AH-8c66c0#sh ru ¦ i test track-wan test - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#track-wan test timeout ? <number> Enter the timeout value (Default: 2 seconds; Range: 1-180; Note: The timeout value must not be more than the interval value.) Result 2: Timeout is 10 AH-8c66c0#sh ru ¦ i test track-wan test track-wan test timeout 10 Result 3: Warning for invalid config AH-8c66c0#track-wan test timeout 0 ^-- unknown keyword or invalid input AH-8c66c0#track-wan test timeout 181 ^-- unknown keyword or invalid input Result 4: Warning when value of timeout is greater than interval, AH-8c66c0#sh ru ¦ i test track-wan test track-wan test interval 10 track-wan test timeout 20 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_25

| CaseID | WanRedundancy_CliMgt_25 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] track-wan <string> retry [numbers] |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check the default retry for track-wan group, result 1 track-wan test 2. Config retry, echo the config, result 2 track-wan test retry 10 3. Save config, then reboot BR 4. Check the retry for track-wan group, result 2 5. Delete the retry, result 1 no track-wan test retry 6. Config the retry with invalid value, result 3 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable bonjour-gateway realm building01_floor01_hive0 Result 1: Retry is 2 by default AH-8c66c0#sh ru ¦ i test track-wan test - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - AH-8c66c0#track-wan test retry ? <number> Enter the retry value (Default: 2 times; Range: 0-1024) Result 2: Retry is 10 AH-8c66c0#sh ru ¦ i test track-wan test track-wan test retry 10 Result 3: Warning for invalid config AH-8c66c0#track-wan test retry 1025 ^-- unknown keyword or invalid input AH-8c66c0#track-wan test retry -1 ^-- unknown keyword or invalid input |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_26

| CaseID | WanRedundancy_CliMgt_26 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] track-wan <string> interface ethx; show track-wan status |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Bind track-wan group “test” to eth0, result 1 track-wan test interface eth0 2. Save config, then reboot BR 3. Check the binding of track-wan group “test” with eth0, result 1 4. Unlock track-wan group “test” from eth0, result 2 no track-wan test interface eth0 5. Save config, then reboot BR 6. Check the out-bound of track-wan group “test” with eth0, result 2 7. Config eth1 as wan, repeat Step 1~6 for eth1, result 3 8. Remove wan mode from eth1, config eth4 as wan, repeat Step 1~6 for eth4, result 4 9. Remove wan mode from eth4, config eth3 as wan, repeat Step 1~6 for eth3, result 5 10. Remove wan mode from eth3, config eth4 as wan, repeat Step 1~6 for eth4, result 6 |  |  |
| ExpectResult | BR config, AH-8c66c0#sh ru interface eth0 mode wan interface eth1 mode bridge-access interface eth1 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface eth3 mode bridge-access interface eth3 mac-learning enable interface eth4 mode bridge-access interface eth4 mac-learning enable interface mgt0 ip 192.168.85.1 255.255.255.0 no interface mgt0 dhcp client interface mgt0 dhcp-server options default-gateway 192.168.85.1 interface mgt0 dhcp-server options dns1 192.168.85.1 interface mgt0 dhcp-server ip-pool 192.168.85.10 192.168.85.99 interface mgt0 dhcp-server enable interface mgt0 dns-server enable capwap client server name 10.155.31.201 usbmodem mode always-connected usbmodem modem-id huawei_e1752 apn 3gnet track-wan use-for-wan-eth0 track-wan use-for-wan-eth0 ip 8.8.8.8 track-wan use-for-wan-eth0 interval 5 track-wan use-for-wan-eth0 interface eth0 track-wan use-for-wan-eth0 enable track-wan use-for-wan-usbnet0 track-wan use-for-wan-usbnet0 ip 8.8.8.8 track-wan use-for-wan-usbnet0 interval 5 track-wan use-for-wan-usbnet0 interface usbnet0 track-wan use-for-wan-usbnet0 enable bonjour-gateway realm building01_floor01_hive0 Result 1: track-wan group “test” is bound to eth0 AH-8c66c0#sh ru ¦ i test track-wan test track-wan test interface eth0 Result 2: track-wan group “test” is not bound to eth0 Result 3: Similar as Result 1~ 2 Result 4: Similar as Result 1~ 2 Result 5: Similar as Result 1~ 2 Result 6: Similar as Result 1~ 2 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_27

| CaseID | WanRedundancy_CliMgt_27 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] track-wan <string> interface ethx; show track-wan status |  |  |
| PlatformDependence | BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Bind track-wan group * to eth0, result 1 track-wan <string> interface eth0 show track-wan status 2. Save config, then reboot BR 3. Check the binding of track-wan group * with eth0, result 1 4. Unlock track-wan group * from eth0, result 2 no track-wan <string> interface eth0 show track-wan status 5. Save config, then reboot BR 6. Check the out-bound of track-wan group * with eth0, result 2 7. Config eth1 as wan, repeat Step 1~6 for eth1, result 3 |  |  |
| ExpectResult | Result 1: track-wan group * is bound to eth0 Result 2: track-wan group * is not bound to eth0 Result 3: Similar as Result 1~ 2 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_28

| CaseID | WanRedundancy_CliMgt_28 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] track-wan <string> interface usbnetx; show track-wan status |  |  |
| PlatformDependence | BR100, BR200, BR200WP, BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Bind track-wan group * to usbnet0, result 1 track-wan <string> interface usbnet0 show track-wan status 2. Save config, then reboot BR 3. Check the binding of track-wan group * with usbnet0, result 1 4. Unlock track-wan group * from usbnet0, result 2 no track-wan <string> interface usbnet0 show track-wan status 5. Save config, then reboot BR 6. Check the out-bound of track-wan group * with usbnet0, result 2 |  |  |
| ExpectResult | Result 1: track-wan group * is bound to usbnet Result 2: track-wan group * is not bound to usbnet |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_29

| CaseID | WanRedundancy_CliMgt_29 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] track-wan <string> interface wifi0; show track-wan status |  |  |
| PlatformDependence | BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Config wifi0 as wan, result 1 2. Bind track-wan group * to wifi0, result 2 track-wan <string> interface wifi0 show track-wan status 3. Save config, then reboot BR 4. Check the binding of track-wan group * with wifi0, result 1 5. Unlock track-wan group * from wifi0, result 3 no track-wan <string> interface wifi0 show track-wan status 6. Save config, then reboot BR 7. Check the out-bound of track-wan group * with wifi0, result 3 |  |  |
| ExpectResult | Result 1: wifi0 is wan mode Result 2: track-wan group * is bound to wifi0 Result 3: track-wan group * is not bound to wifi0 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_30

| CaseID | WanRedundancy_CliMgt_30 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] track-wan <string> interface wifix; show track-wan status |  |  |
| PlatformDependence | BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Config wifi0 as wan, result 1 2. Bind track-wan group * to wifi0, result 2 track-wan <string> interface wifi0 show track-wan status 3. Save config, then reboot BR 4. Check the binding of track-wan group * with wifi0, result 1 5. Unlock track-wan group * from wifi0, result 3 no track-wan <string> interface wifi0 show track-wan status 6. Save config, then reboot BR 7. Check the out-bound of track-wan group * with wifi0, result 3 8. Remove wan mode from wifi0, and config wifi1 as wan mode, result 4 9. Repeat Step 2~ 7 for wifi1, result 5 |  |  |
| ExpectResult | Result 1: wifi0 is wan mode Result 2: track-wan group * is bound to wifi0 Result 3: track-wan group * is not bound to wifi0 Result 4: wifi1 is wan mode Result 5: Similar as Result 2~ 3 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_31

| CaseID | WanRedundancy_CliMgt_31 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | track-wan <string> interface ethx; Fail to bind track group to non-wan interface |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Create a track wan group, result 1 2. Check default config of eth1, result 2 3. Try to bind track wan group to eth1, result 3 4. Repeat Step 2~ 3 for eth4, result 4 5. Repeat Step 2~ 3 for eth3, result 5 6. Repeat Step 2~ 3 for eth4, result 6 |  |  |
| ExpectResult | Result 1: track wan group is created Result 2: eth1 is not wan mode Result 3: Fail to bind track wan group to eth1 Result 4: Similar as Result 2~ 3 Result 5: Similar as Result 2~ 3 Result 6: Similar as Result 2~ |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_32

| CaseID | WanRedundancy_CliMgt_32 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | track-wan <string> interface eth1; Fail to bind track group to non-wan interface |  |  |
| PlatformDependence | BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Create a track wan group, result 1 2. Check default config of eth1, result 2 3. Try to bind track wan group to eth1, result 3 |  |  |
| ExpectResult | Result 1: track wan group is created Result 2: eth1 is not wan mode Result 3: Fail to bind track wan group to eth1 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_33

| CaseID | WanRedundancy_CliMgt_33 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | track-wan <string> interface wifi0; Fail to bind track group to non-wan interface |  |  |
| PlatformDependence | BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Create a track wan group, result 1 2. Check default config of wifi0, result 2 3. Try to bind track wan group to wifi0, result 3 |  |  |
| ExpectResult | Result 1: track wan group is created Result 2: wifi0 is not wan mode Result 3: Fail to bind track wan group to wifi0 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_CliMgt_34

| CaseID | WanRedundancy_CliMgt_34 |  |  |
| --- | --- | --- | --- |
| Priority | Low | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | track-wan <string> interface wifix; Fail to bind track group to non-wan interface |  |  |
| PlatformDependence | BRAP330, BRAP350 |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Create a track wan group, result 1 2. Check default config of wifi0, result 2 3. Try to bind track wan group to wifi0, result 3 4. Repeat Step 2~3 for wifi1, result 4 |  |  |
| ExpectResult | Result 1: track wan group is created Result 2: wifi0 is not wan mode Result 3: Fail to bind track wan group to wifi0 Result 4: Similar as Result 2~ 3 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

## GUI Management-HiveManager

### WanRedundancy_GuiHM_01

| CaseID | WanRedundancy_GuiHM_01 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface ethx mode wan; show wan interface ethx |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default mode of each interface, result 1 2. Try to change the wan mode from eth0, result 2 no interface eth0 mode wan 3. Check default mode of eth1, result 3 4. Config mode of eth1 as wan, result 4 interface eth1 mode wan 5. Save config, then reboot BR 6. Check the mode of eth1, result 4 7. Restore the mode of eth1, result 3 no interface eth1 mode wan 8. Save config, then reboot BR 9. Check the mode of eth1, result 3 10. Repeat Step 3~ 9 for eth4, result 5 11. Repeat Step 3~ 9 for eth3, result 6 12. Repeat Step 3~ 9 for eth4, result 7 |  |  |
| ExpectResult | Result 1: eth0 is wan mode by default Result 2: Fail to change the mode of eth0 Result 3: eth1 is lan mode Result 4: eth1 is wan mode Result 5: Similar as Result 3~ 4 Result 6: Similar as Result 3~ 4 Result 7: Similar as Result 3~ 4 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_GuiHM_01

| CaseID | WanRedundancy_GuiHM_01 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface ethx mode wan; show wan interface ethx |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default mode of each interface, result 1 2. Try to change the wan mode from eth0, result 2 no interface eth0 mode wan 3. Check default mode of eth1, result 3 4. Config mode of eth1 as wan, result 4 interface eth1 mode wan 5. Save config, then reboot BR 6. Check the mode of eth1, result 4 7. Restore the mode of eth1, result 3 no interface eth1 mode wan 8. Save config, then reboot BR 9. Check the mode of eth1, result 3 10. Repeat Step 3~ 9 for eth4, result 5 11. Repeat Step 3~ 9 for eth3, result 6 12. Repeat Step 3~ 9 for eth4, result 7 |  |  |
| ExpectResult | Result 1: eth0 is wan mode by default Result 2: Fail to change the mode of eth0 Result 3: eth1 is lan mode Result 4: eth1 is wan mode Result 5: Similar as Result 3~ 4 Result 6: Similar as Result 3~ 4 Result 7: Similar as Result 3~ 4 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_GuiHM_01

| CaseID | WanRedundancy_GuiHM_01 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface ethx mode wan; show wan interface ethx |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default mode of each interface, result 1 2. Try to change the wan mode from eth0, result 2 no interface eth0 mode wan 3. Check default mode of eth1, result 3 4. Config mode of eth1 as wan, result 4 interface eth1 mode wan 5. Save config, then reboot BR 6. Check the mode of eth1, result 4 7. Restore the mode of eth1, result 3 no interface eth1 mode wan 8. Save config, then reboot BR 9. Check the mode of eth1, result 3 10. Repeat Step 3~ 9 for eth4, result 5 11. Repeat Step 3~ 9 for eth3, result 6 12. Repeat Step 3~ 9 for eth4, result 7 |  |  |
| ExpectResult | Result 1: eth0 is wan mode by default Result 2: Fail to change the mode of eth0 Result 3: eth1 is lan mode Result 4: eth1 is wan mode Result 5: Similar as Result 3~ 4 Result 6: Similar as Result 3~ 4 Result 7: Similar as Result 3~ 4 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_GuiHM_01

| CaseID | WanRedundancy_GuiHM_01 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface ethx mode wan; show wan interface ethx |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default mode of each interface, result 1 2. Try to change the wan mode from eth0, result 2 no interface eth0 mode wan 3. Check default mode of eth1, result 3 4. Config mode of eth1 as wan, result 4 interface eth1 mode wan 5. Save config, then reboot BR 6. Check the mode of eth1, result 4 7. Restore the mode of eth1, result 3 no interface eth1 mode wan 8. Save config, then reboot BR 9. Check the mode of eth1, result 3 10. Repeat Step 3~ 9 for eth4, result 5 11. Repeat Step 3~ 9 for eth3, result 6 12. Repeat Step 3~ 9 for eth4, result 7 |  |  |
| ExpectResult | Result 1: eth0 is wan mode by default Result 2: Fail to change the mode of eth0 Result 3: eth1 is lan mode Result 4: eth1 is wan mode Result 5: Similar as Result 3~ 4 Result 6: Similar as Result 3~ 4 Result 7: Similar as Result 3~ 4 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_GuiHM_01

| CaseID | WanRedundancy_GuiHM_01 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface ethx mode wan; show wan interface ethx |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default mode of each interface, result 1 2. Try to change the wan mode from eth0, result 2 no interface eth0 mode wan 3. Check default mode of eth1, result 3 4. Config mode of eth1 as wan, result 4 interface eth1 mode wan 5. Save config, then reboot BR 6. Check the mode of eth1, result 4 7. Restore the mode of eth1, result 3 no interface eth1 mode wan 8. Save config, then reboot BR 9. Check the mode of eth1, result 3 10. Repeat Step 3~ 9 for eth4, result 5 11. Repeat Step 3~ 9 for eth3, result 6 12. Repeat Step 3~ 9 for eth4, result 7 |  |  |
| ExpectResult | Result 1: eth0 is wan mode by default Result 2: Fail to change the mode of eth0 Result 3: eth1 is lan mode Result 4: eth1 is wan mode Result 5: Similar as Result 3~ 4 Result 6: Similar as Result 3~ 4 Result 7: Similar as Result 3~ 4 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_GuiHM_01

| CaseID | WanRedundancy_GuiHM_01 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface ethx mode wan; show wan interface ethx |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default mode of each interface, result 1 2. Try to change the wan mode from eth0, result 2 no interface eth0 mode wan 3. Check default mode of eth1, result 3 4. Config mode of eth1 as wan, result 4 interface eth1 mode wan 5. Save config, then reboot BR 6. Check the mode of eth1, result 4 7. Restore the mode of eth1, result 3 no interface eth1 mode wan 8. Save config, then reboot BR 9. Check the mode of eth1, result 3 10. Repeat Step 3~ 9 for eth4, result 5 11. Repeat Step 3~ 9 for eth3, result 6 12. Repeat Step 3~ 9 for eth4, result 7 |  |  |
| ExpectResult | Result 1: eth0 is wan mode by default Result 2: Fail to change the mode of eth0 Result 3: eth1 is lan mode Result 4: eth1 is wan mode Result 5: Similar as Result 3~ 4 Result 6: Similar as Result 3~ 4 Result 7: Similar as Result 3~ 4 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_GuiHM_01

| CaseID | WanRedundancy_GuiHM_01 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface ethx mode wan; show wan interface ethx |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default mode of each interface, result 1 2. Try to change the wan mode from eth0, result 2 no interface eth0 mode wan 3. Check default mode of eth1, result 3 4. Config mode of eth1 as wan, result 4 interface eth1 mode wan 5. Save config, then reboot BR 6. Check the mode of eth1, result 4 7. Restore the mode of eth1, result 3 no interface eth1 mode wan 8. Save config, then reboot BR 9. Check the mode of eth1, result 3 10. Repeat Step 3~ 9 for eth4, result 5 11. Repeat Step 3~ 9 for eth3, result 6 12. Repeat Step 3~ 9 for eth4, result 7 |  |  |
| ExpectResult | Result 1: eth0 is wan mode by default Result 2: Fail to change the mode of eth0 Result 3: eth1 is lan mode Result 4: eth1 is wan mode Result 5: Similar as Result 3~ 4 Result 6: Similar as Result 3~ 4 Result 7: Similar as Result 3~ 4 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

## GUI Management-HiveUI

### WanRedundancy_GuiHiveUI_01

| CaseID | WanRedundancy_GuiHiveUI_01 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface ethx mode wan; show wan interface ethx |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default mode of each interface, result 1 2. Try to change the wan mode from eth0, result 2 no interface eth0 mode wan 3. Check default mode of eth1, result 3 4. Config mode of eth1 as wan, result 4 interface eth1 mode wan 5. Save config, then reboot BR 6. Check the mode of eth1, result 4 7. Restore the mode of eth1, result 3 no interface eth1 mode wan 8. Save config, then reboot BR 9. Check the mode of eth1, result 3 10. Repeat Step 3~ 9 for eth4, result 5 11. Repeat Step 3~ 9 for eth3, result 6 12. Repeat Step 3~ 9 for eth4, result 7 |  |  |
| ExpectResult | Result 1: eth0 is wan mode by default Result 2: Fail to change the mode of eth0 Result 3: eth1 is lan mode Result 4: eth1 is wan mode Result 5: Similar as Result 3~ 4 Result 6: Similar as Result 3~ 4 Result 7: Similar as Result 3~ 4 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

### WanRedundancy_GuiHiveUI_01

| CaseID | WanRedundancy_GuiHiveUI_01 |  |  |
| --- | --- | --- | --- |
| Priority | Middle | Automation Flag | Yes |
| Topology | Topo 1 |  |  |
| Description | [no] interface ethx mode wan; show wan interface ethx |  |  |
| PlatformDependence | BR100, BR200, BR200WP |  |  |
| Pre-condition |  |  |  |
| TestProcedure | 1. Check default mode of each interface, result 1 2. Try to change the wan mode from eth0, result 2 no interface eth0 mode wan 3. Check default mode of eth1, result 3 4. Config mode of eth1 as wan, result 4 interface eth1 mode wan 5. Save config, then reboot BR 6. Check the mode of eth1, result 4 7. Restore the mode of eth1, result 3 no interface eth1 mode wan 8. Save config, then reboot BR 9. Check the mode of eth1, result 3 10. Repeat Step 3~ 9 for eth4, result 5 11. Repeat Step 3~ 9 for eth3, result 6 12. Repeat Step 3~ 9 for eth4, result 7 |  |  |
| ExpectResult | Result 1: eth0 is wan mode by default Result 2: Fail to change the mode of eth0 Result 3: eth1 is lan mode Result 4: eth1 is wan mode Result 5: Similar as Result 3~ 4 Result 6: Similar as Result 3~ 4 Result 7: Similar as Result 3~ 4 |  |  |
| TestResult |  |  |  |
| Comment |  |  |  |

## Typical issue Test Case